TSG-RAN Working Group 4 meeting #100-E	R4-210xyz

Electronic Meeting, 16th - 27th August 2021

Source:	Ericsson

Title:	WF on the RedCap RF

Agenda item:	9.20.1

Document for:	Approval

# Introduction

During RAN4#100-E meeting a way-forward on RedCap UE is created based on the discussion in 1st round [1].

# Way-Forward

## 2.1 FR1

RedCap UE power class

Tentative agreements:

- PC3 should be specified for Redcap in FR1

- No PC1 and PC1.5 for RedCap UE in FR1

- PC2 for RedCap UE depending on operator request

Operating Bands:

Tentative agreements:

- ALL FDD band should be supported by RedCap UE

- ALL TDD band except n47, n79, n46 and n96 should be supported by RedCap UE

Other RX requirements than REFSENS

Moderator’s note: One Company wants to decide this next meeting, try to see if working assumption below would be ok:

Tentative working assumption:

- Reuse the ACS, maximum input level, Blocking, Spurious response, Rx IM, Spurious and based on Redcap REFSENS

Half-duplex FDD switching time

Moderator’s note: One Company wants to decide 2nd round. Could capture the discussion outcome later.

Issue 7-1: RX-TX switching time

- Proposals

  - Option 1: The Rx-Tx switching time for HD-FDD RedCap UE is 13 µs.

  - Option 2: TBA

## 2.2 FR2

Moderator’s note: Please add your company name at the end of the option so possibly the option that no companies prefer could be removed.

RAN4 Priority on RedCap UE in FR2

- Option 1: Deprioritize RedCap UE in FR2 in Rel-17

- Option 2: Specify RedCap UE in FR2 in Rel-17

- Option 3: FFS depending on discussion in Issue 6-1, 6-2, 6-3 and 6-4.

Issue 6-1: Use case for RedCap UE

  - Option 1: consider the below use case for RedCap UE in FR2

- Table 2-2 the characteristics of three use cases for Redcap UE

| Use cases | Specific characteristics | General characteristics |
| --- | --- | --- |
| Industrial wireless sensors | The device is stationary The battery should last at least few years | Lower cost and complexity Small and compact form factor Supporting all FR1/FR2 bands for FDD and TDD |
| Video surveillance | The device has low mobility |  |
| Wearables | The battery should last multiple days |  |

  - Option 2: FFS

Issue 6-2: new RedCap UE type

  - Option 1: define a new RedCap UE type

  - Option 2: define RedCap UE based on existing FR2 UE type

  - Option 3: FFS

Issue 6-3: Power class for RedCap UE in FR2

  - Option 1: define a new power class

  - Option 2: TBA

Issue 6-4: RF architecture for RedCap UE in FR2

  - Option 1: Reduction of RX branches: A simplification of only the baseband architecture, to a single baseband RX (rank 1) [Ericsson, Huawei, Xiaomi]

  - Option 2: Reduction of the number of elements in the antenna panel

  - Option 3: Reduction of the number of antenna panels [Xiaomi, Nokia, OPPO, Qualcomm]

  - Option 4: Option 1, option 2 and option 3 possible but depending on use case [Sony, Vivo]

# References

- R4-2114742, Email discussion summary for [100-e][142] NR_RedCap, Ericsson