ETSI TS 103 807 V1.0.0_0.2.0 (2021-06)

Task Force for European Standards for IMT-2000 (MSG)

IMT cellular networks;

Base Stations (BS);

Additional Regulatory Requirements

<

TECHNICAL SPECIFICATION

Reference

<Workitem>

Keywords

<keywords>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx.

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPPTM and LTETM are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

Introduction	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	6

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Regulatory requirements	6

4.1	Enhanced Selectivity for Band 1 BSs	6

4.1.1	Limits	7

4.1.2	Testing for compliance with technical requirements	8

4.2	Limits below 3400 MHz for protection of radiolocation for country-specific cases	8

4.2.1	Limits	9

4.2.2	Testing for compliance with technical requirements	9

4.3	Requirements above 2690 MHz for protection of the Radio Astronomy Service	9

4.3.1	Limits	9

4.3.2	Testing for compliance with technical requirements	10

Annex A (normative or informative): Title of annex	10

Annex (informative): Bibliography	11

Annex (informative): Change History	12

History	13

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Mobile Standards Group (MSG) .

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Introduction

- The present document addresses additional requirements arising from EC/CEPT spectrum regulatory framework that apply to specific equipment in specific cases in certain countries and/or in certain geographical areas.

# 1	Scope

The present document applies to the following equipment types:

E-UTRA, UTRA, NR, NB-IoT, MSR capable Base Stations

- The present document addresses additional requirements arising from EC/CEPT spectrum regulatory framework that apply to specific equipment in specific cases in certain countries and/or in certain geographical areas.

- The present document will be referenced in the relevant IMT Harmonised Standards – EN 301 908 Part 14 (E-UTRA BS), Part 18 (MSR BS), Part 23 (AAS BS), Part 24 (NR BS) – to address those additional requirements.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]		ETSI TS 137 141 V16.8.0 (2021-01): "Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing.

[2]	ETSI TS 136 104 (V16.8.0) (2021-01): "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

[3]	ETSI TS 125 105 (V16.0.0) (2020-11): "Universal Mobile Telecommunications System (UMTS); Base Station (BS) radio transmission and reception (TDD).

[4]	ETSI TS 138 104 (V16.6.0) (2021-01): “5G; NR; Base Station (BS) radio transmission and reception”

[5]	ETSI EN 301 908-18 (V13.1.1) (09-2019): "IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 18: E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS)

[6]	ETSI TS 138 141-2 (V16.4.0) (2020-07): “5G; NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing”

[7]	ETSI TS 137 145-1 (V16.5.0) (2021-01): “Universal Mobile Telecommunications System (UMTS); LTE; Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing”

[8]	ETSI TS 137 145-2 (V16.6.0) (2021-01): “Universal Mobile Telecommunications System (UMTS); LTE; Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing”

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ECC Decision (20)02 “Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)”

[i.2]	 ECC Report 318 “Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band”

[i.3]	COMMISSION IMPLEMENTING DECISION (EU) 2019/235 of 24 January 2019 on amending Decision 2008/411/EC as regards an update of relevant technical conditions applicable to the 3400-3800 MHz frequency band

[i.4]	ECC Report 281 “Analysis of the suitability of the regulatory technical conditions for 5G MFCN operation in the 3400-3800 MHz band”

[i.5]	COMMISSION IMPLEMENTING DECISION (EU) 2020/636 of 8 May 2020 amending Decision 2008/477/EC as regards an update of relevant technical conditions applicable to the 2 500–2 690 MHz frequency band

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the [following] terms [given in ... and the following] apply:

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

# 4	Regulatory requirements

## 4.1	Enhanced Selectivity for Band 1 BSs

The following requirement is applicable to E-UTRA, UTRA, NR, NB-IoT or MSR BSs that are intended to be deployed in close geographical proximity of FRMCS Base Stations.

This requirement is only applicable to BSs intended to transmit in the lowest channel of Band 1.

This requirement is only applicable to the blocking performance against an interfering signal in the frequency range 1900-1910 MHz and does not change the general blocking requirement between adjacent channels in-band.

Table 4.1.1: Base station operating band

| Band designation | Direction of transmission | Base Station operating bands |
| --- | --- | --- |
| 1 | Transmit | 2 110 MHz to 2 170 MHz |
|  | Receive | 1 920 MHz to 1 980 MHz |

### 4.1.1	Limits

The interfering signal shall be a [E-UTRA/NR] TDD signal with a 10 MHz channel for a UTRA, E-UTRA, NB-IOT, GSM/EDGE or NR (≤ 20 MHz) wanted signal.

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal is defined in the 1900-1910 MHz frequency range.

For the wanted and interfering signal coupled to the base station antenna input, using the parameters in Table 4.1.2 the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 136 104 [2], sub clause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 125 104 [3], sub clause 7.2.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 136 104 [2], sub clause 7.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 138 104 [4], sub clause 7.2.

The wanted signal shall be centred at the lowest channel of Band 1.

Table 4.1.2: General blocking requirement

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] |
| --- | --- | --- |
| Wide Area BS (option 1) | - 30 | PREFSENS + 1 dB |
| Wide Area BS (option 2) | -19.5 | PREFSENS + 6 dB |

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] (Note 1) | Centre Frequency of Interfering Signal |
| --- | --- | --- | --- |
| Wide Area BS | [- 17.5]+y (Note 7) | PREFSENS + x dB (Note 2) | 1905 MHz |
| Medium Range BS | [-12.5]+y (Note 7) | PREFSENS + x dB (Note 3, 5) |  |
| Local Area BS | [-7.5]+y (Note 7) | PREFSENS + x dB (Note 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see subclause 7.2 of .TS 137 141 [1] NOTE 2: For WA BS, "x" is equal to 6 in case of NR or E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE 3: For MR BS supporting GSM and/or UTRA, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of NR or E-UTRA or NB-IoT wanted signal and 3 in case of GSM/EDGE wanted signal. NOTE 4: For LA BS supporting GSM and/or UTRA, "x" is equal to 11 in case of NR or E-UTRA or NB-IoT wanted signal, 6 in case of UTRA wanted signal and equal to 3 in case of GSM/EDGE wanted signal. NOTE 5: For a BS neither supporting UTRA nor GSM, x is equal to 6 for all BS classes if NR is supported, otherwise "x" is equal to 9 for MR BS or 11 for LA BS if NR is not supported. NOTE 6: For a BS capable of multi-band operation, "x" in Note 2, 3, 4, 5 applies in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in an adjacent or overlapping band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. NOTE 7: For a BS supporting NR but neither supporting UTRA nor GSM, "y" is equal to -3 for the WA and MR BS class and -5 for the LA BS class. For all other cases, "y" is equal to zero for all BS classes |  |  |  |

### 4.1.2	Testing for compliance with technical requirements

- For MSR BS, the test suite specified in ETSI EN 301 908-18 [5], clause 5.3.7 shall be used

- For XXX.

## 4.2	Protection of radiolocation below 3400 MHz for country-specific cases

The requirement in this clause applies to non-AAS and AAS base stations BSs to ensure the protection of military radiolocation operating below 3400 MHz in some countries, in accordance to [i.3][i.4]. Applicability of this requirement is on national basis.

For non-AAS BSs, countries may decide between two different levels of protection (case A or case B) depending on the level of protection required for the radar in their territory or no limit (case C).

This requirement is applicable to E-UTRA, NR and MSR BSs.

This requirement is applicable to AAS and non-AAS BSs.

This requirement is applicable to BSs operating in the bands in table 4.2.1.

Table 4.2.1: Base station operating bands

| MSR and E-UTRA Band number | NR Band number | BS transmit & receive |  |  |
| --- | --- | --- | --- | --- |
| 42 |  | 3400 MHz | – | 3600 MHz |
| 43 |  | 3600 MHz | – | 3800 MHz |
| 77 (notes 1 and 3) | n77 | 3300 MHz | - | 4200 MHz |
| 78 (notes 2 and 3) | n78 | 3300 MHz | - | 3800 MHz |
| NOTE 1: In Europe according to [i.3], radio equipment in band 77 operates between 3 400 MHz and 4 200 MHz (FDL_low = 3 400 MHz and FDL_high =4 200 MHz). NOTE 2: In Europe according to [i.3], radio equipment in band 78 operates between 3 400 MHz and 3 800 MHz (FDL_low = 3 400 MHz and FDL_high = 3 800 MHz). NOTE 3: The band is for NR only. |  |  |  |  |

### 4.2.1	Limits

Table 4.2.2: Additional requirement below 3400 MHz for country specific cases, 
for non-AAS and AAS base stations

|  | Case | Frequency range | Non AAS limit | AAS TRP limit | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| Case A | Countries with military radiolocation systems below 3400 MHz | Below 3400 MHz | – 80 dBm | – 52 dBm | 1 MHz |
| Case B | Countries with military radiolocation systems below 3400 MHz | Below 3400 MHz | – 71 dBm |  | 1 MHz |
| Case C | Countries without adjacent band usage or with usage that does not need extra protection | Below 3400 MHz | Not applicable | Not applicable | Not applicable |

These requirements are derived from Table 6 of COMMISSION IMPLEMENTING DECISION (EU) 2019/235 [i.3] assuming a 21 dBi antenna gain for the non-AAS cases.

### 4.2.2	Testing for compliance with technical requirements

The following test suites apply:

- For MSR BS, the test suite specified in ETSI EN 301 908-18 [5], clause 5.3.1 shall be used. Test between X and Y MHz

- For E-UTRA BS, ...

Include references to Part 3, 14, 18, 23, 24

The results obtained shall be compared to the limits in clause 4.2.1 in order to prove compliance

## 4.3	Protection of the Radio Astronomy Service operating above 2690 MHz

The following requirement is applicable in specific geographical areas to reduce the size of the coordination zone with RAS stations.

This requirement is applicable to FDD AAS BSs and NR BSs Type 1-H and Type 1-O operating in the frequency range in table 4.3.1.

Table 4.3.1: Base station operating band

| MSR and E-UTRA Band number | NR Band number | UTRA Band number | Uplink (UL) BS receive UE transmit | Downlink (DL) BS transmit UE receive |
| --- | --- | --- | --- | --- |
| 7 | n7 | VII | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz |

### 4.3.1	Limits

Table 4.3.2: Additional requirement for FDD AAS BSs 
with regard to Radio Astronomy Service

| Frequency range | Case (Note 1) | TRP limit |
| --- | --- | --- |
| 2690-2700 MHz | A | + 3 dBm/10 MHz |
|  | B | Not applicable |

Note 1:	Case A: This limit yields a reduced coordination zone with respect to RAS stations. 
Case B: For situations where additional baseline is not considered necessary by the concerned Member State (e.g. where there is no nearby RAS station or situation where no coordination zone is required).

These requirements are based on table 7 of COMMISSION IMPLEMENTING DECISION (EU) 2020/636 [i.5].

### 4.3.2	Testing for compliance with technical requirements

For single RAT NR BS type 1-0 the test shall be in accordance with TS 138 141-2 [6] clause 6.7.4.4.

For AAS BSs the test shall be in accordance with TS 137 145-2 [8] clause 6.7.5.4.

Annex A (normative or informative):
Title of annex

Annex (informative):
Bibliography

Annex (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| <Month year> | <#> | <Changes made are listed in this cell> |

# History

| Document history |  |  |
| --- | --- | --- |
| <Version> | <Date> | <Milestone> |

Latest changes made on 2019-01-29