3GPP TSG-RAN WG4 Meeting #100-e	R4-2114895

Electronic Meeting, 17-28 August 2021

Title:	[Draft] LS on NB-IoT testing issues

Reply To:

Release:	Rel-14

Work Item:	TEI-14

Source:	RAN WG4

To:	RAN WG5, PTCRB, CTIA CPWG

Cc:

Contact Person:

Name:	Bill Shvodian

E-mail Address:	bill.shvodian@t-mobile.com

Attachments:

1. Background information:

In August of 2020 RAN4 sent an LS to the FCC asking for guidance on NB-IoT testing issues near the edges of bands [1]. RAN4 received indirect feedback from the FCC on NB-IoT testing in [2]. RAN4 subsequently agreed CRs to solve the NB-IoT testing issue in Rel-14 [3], Rel-15 [4], Rel-16 [15] and Rel-17 [16]. The changes are summarized in the following table for NB-IoT operation in the USA, and especially that the lowest 100 kHz of Band 12 and Band 13 are not excluded for NB-IoT operation in the USA:

Table 5.5F-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850.1 MHz | – | 1909.9 MHz | 1930.1 MHz | – | 1989.9 MHz | FDD |
| 4 | 1710.1 MHz | – | 1754.9 MHz | 2110.1 MHz | – | 2154.9 MHz | FDD |
| 5 | 824.1 MHz | – | 848.9 MHz | 869.1 MHz | – | 893.9MHz | FDD |
| 12 | 699 MHz | – | 715.9 MHz | 729 MHz | – | 745.9 MHz | FDD |
| 13 | 777 MHz | – | 786.9 MHz | 746 MHz | – | 755.9 MHz | FDD |
| 17 | 704.1 MHz | – | 715.9 MHz | 734.1 MHz | – | 745.9 MHz | FDD |
| 25 | 1850.1 MHz | – | 1914.9 MHz | 1930.1 MHz | – | 1994.9 MHz | FDD |
| 26 | 814.1 MHz | – | 848.9 MHz | 859.1 MHz | – | 893.9 MHz | FDD |
| 66 | 1710.1 MHz | – | 1779.9 MHz | 2110.1 MHz | – | 2199.0 MHz | FDD4 |
| 71 | 663.1 MHz | – | 697.9 MHz | 617.1 MHz | – | 651.9 MHz | FDD |
| 85 | 698.1 MHz | – | 715.9 MHz | 728.1 MHz | – | 745.9 MHz | FDD |

Please note that 2199.0 MHz for B66 is an error. It should be 2179.9 MHz. This is being corrected.

In 36.101 the NB-IoT restriction for operation in the USA is tied to NS_04 signalling, but our understanding is that FCC certification does not use NS signalling. Therefore NB-IoT UEs shall not operate in the 100 kHz at the edge of the band when operating in the USA or/and tested for FCC compliance even when NS_04 is not signalled.

- Summary

TS 36.101 has been updated to exclude 100 kHz at the edge of NB-IoT bands for operation in the USA, except for the lower edge of Band 12 and Band 13 which are not excluded.

- Actions:

To RAN5, PTCRB and CTIA CPWG,

ACTION: 	RAN4 kindly asks RAN5, PTCRB and CPWG to take the above into consideration for NB-IoT testing.

- References:

- R4-2011913, 	“LS on NB-IoT testing issues,” RAN4

- R4-2107330,	“NB-IoT Testing,” T-Mobile USA, Qualcomm

- R4-2108012,	“CR for 36.101: Introduction of NS Signalling for NB-IoT in the USA,” T-Mobile USA, Qualcomm

- R4-2111484,	“Mirror CR for 36.101: Introduction of NS Signalling for NB-IoT in the USA,” T-Mobile USA, Qualcomm

- R4-2111485,	“Mirror CR for 36.101: Introduction of NS Signalling for NB-IoT in the USA,” T-Mobile USA, Qualcomm

- R4-2111486,	“Mirror CR for 36.101: Introduction of NS Signalling for NB-IoT in the USA,” T-Mobile USA, Qualcomm

- Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #101-e	1-12 November   2021 		Online

TSG-RAN4 #102		21-25 February 2022 		Athens, Greece?