3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115339

Electronic Meeting, August 16-27, 2021

Title:	LS on beam information of PUCCH SCell in PUCCH SCell activation procedure

Response to:

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:

Contact Person:

Name:	Zhongyi Shen

E-mail Address:	shenzhongyi3@huawei.com

Attachments:

1. Overall Description:

RAN4 is currently discussing the requirements for PUCCH SCell activation. For unknown PUCCH SCell activation (known cell conditions as defined in TS 38133 clause 8.3.2), from RAN4 perspective, we observe that UE may have problems supporting the following cases under the current NR specification:

- unknown FR1 PUCCH SCell activation with a valid TA

- unknown FR2 PUCCH SCell activation with a valid TA

- unknown FR1 PUCCH SCell activation without a valid TA

- unknown FR2 PUCCH SCell activation without a valid TA

One issue among the above identified cases is the beam information cannot be reported to network via the PUCCH of target being-activated SCell during the PUCCH SCell activation procedure. From RAN4’s perspective, the beam information reporting may be needed for following purposes:

1. Determine the associated SSB in PDCCH order for CFRA for TA updating when TimeAlignmentTimer associated with the TAG containing the PUCCH SCell is not running.

2. Determine the TCI state for PDCCH and PDSCH(when applicable) on target being-activated SCell

3. Determine the UL spatial relation for PUCCH on target being-activated FR2 SCell

4. Determine the Rx beam for PUCCH of target being-activated SCell at network reception

RAN4 sees benefits in supporting PUCCH SCell activation for the above cases in terms of network operation flexibility and UE power consumption. RAN4 would like RAN1 and RAN2 to answer the following questions:

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

Q2: Whether the above observation is correct, i.e. the identified four cases are not supported by the current RAN1 and RAN2 specification

Q3: Whether the above identified cases can be supported by RAN1 and RAN2 spec updates within Rel-17 timeframe.

RAN4 will further discuss whether/how to define requirements of PUCCH SCell activation for the above cases based on RAN1 and RAN2 reply to above questions.

2. Action

To: RAN1, RAN2

ACTION: RAN4 respectfully asks RAN1 and RAN2 to provide answers for the above questions

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101-e		1 – 12 November 2021

TSG-RAN4 Meeting #102		21 – 25 February 2022