3GPP TSG-RAN WG4 Meeting #101e-bis	R4-2200024

Electronic Meeting, January 17-25, 2022

Source:	China Telecom

Title:	On UE autonomous adjustment and DL reception in-between transmission

Agenda Item:	6.18.2.4

Document for:	Discussion

- Introduction

In RAN4 #101e meeting, one more reply LS on PUCCH and PUSCH transmission was sent to RAN1 [1], and the WF on phase continuity and power consistency was approved in [2].

This contribution provides our views on UE autonomous adjustment and DL reception in-between transmission.

- Discussion

2.1  UE autonomous adjustment

In RAN4 #101e, it is still open on how to handle UE autonomous adjustment across the JCE bundle:

For UE autonomous adjustment

- Option 1: Up to UE implementation, while maintaining the power consistency and phase continuity tolerance.

- Option 2: The autonomous adjustment is not expected if DL timing is not changed. It is up to UE implementation if DL timing is changed. FFS how to capture this in RF requirement/test.

Firstly, we’d like to discuss whether there is any real difference between the two options? As we know, for UE autonomous adjustment, it applies when the transmission timing error between the UE and the reference timing exceeds a threshold, i.e., when the propagation delay between the BS and UE changes due to UE movement. So, typically the UE autonomous adjustment applies when the DL timing is changed. From this sense, option 1 and option 2 are identical.

Secondly, as summarized in RAN4 #101e, there can be different interpretations on the description of “up to UE implementation”. One interpretation is that option 1 also implies UE would not adjust timing, so as to maintain the power consistency and phase continuity tolerance. Another interpretation is that, in option 1, UE autonomous timing adjustment and compensation on phase offset are not precluded. With either of the two interpretations, we see no issue to go with option 1.

Thirdly, as calculated in our contribution to RAN4 #101e in [3], it is obvious that the possible maximum change of propagation delay within a JCE bundle is very small compared to the Te. More specifically, the prorogation delay between the UE and the BS can change at maximum 10.37 ns (assuming 15kHz SCS, 32 slots for the bundle, UE speed of 350km/h), and meanwhile, the Te = 12*64*Tc = 390.63 ns. So, we do not need to be too anxious about this issue.

Observation 1: Typically the UE autonomous adjustment applies when the DL received timing (i.e., the propagation delay between the BS and UE) is changed. From this sense, option 1 and option 2 are identical.

Observation 2: For option 1 (i.e., up to UE implementation, while maintaining the power consistency and phase continuity tolerance), it can be interpreted by either: 1) not adjusting timing, or 2) UE compensation on phase offset in case timing is adjusted. With either interpretation, the power consistency and phase continuity tolerance can be maintained, and there is no issue identified.

Observation 3: The possible maximum change of propagation delay within a JCE bundle is very small compared to the Te.

Proposal 1: Option 1 for UE autonomous adjustment (i.e., up to UE implementation, while maintaining the power consistency and phase continuity tolerance) is preferred.

2.2  DL reception in-between transmission

For the DL reception in-between transmission, the following agreements were captured in the RAN4 WF:

- For the case of “with DL reception (including monitoring and/or measurements)”:

  - FFS: Whether this case will be discussed in RAN4 anymore in Rel-17.

For the scenario of “with DL reception” in-between transmission, based on the following RAN1 agreements made in RAN1 #106e-bis and RAN1 #107e, the “with DL reception” scenario is not supported in RAN1.

RAN1 #106e-bis Agreement

- Support at least the following events that violate power consistency and phase continuity.

  - Dropping/cancellation based on Rel-15/16 collision rules.

  - FFS: Rel-17 collision rules.

  - DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

  - FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

  - Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

RAN1 #107e Agreement:

- For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.

Therefore, to save efforts in RAN4 discussion, it is proposed that RAN4 not further discuss the scenario of “with DL reception” in-between transmission in Rel-17.

Observation 4: The “with DL reception” scenario is not supported in RAN1.

Proposal 2: RAN4 not further discuss the scenario of “with DL reception” in-between transmission in Rel-17.

- Conclusion

This contribution presented our views on UE autonomous adjustment and DL reception in-between transmission, with the following observations and proposals:

Observation 1: Typically the UE autonomous adjustment applies when the DL received timing (i.e., the propagation delay between the BS and UE) is changed. From this sense, option 1 and option 2 are identical.

Observation 2: For option 1 (i.e., up to UE implementation, while maintaining the power consistency and phase continuity tolerance), it can be interpreted by either: 1) not adjusting timing, or 2) UE compensation on phase offset in case timing is adjusted. With either interpretation, the power consistency and phase continuity tolerance can be maintained, and there is no issue identified.

Observation 3: The possible maximum change of propagation delay within a JCE bundle is very small compared to the Te.

Proposal 1: Option 1 for UE autonomous adjustment (i.e., up to UE implementation, while maintaining the power consistency and phase continuity tolerance) is preferred.

Observation 4: The “with DL reception” scenario is not supported in RAN1.

Proposal 2: RAN4 not further discuss the scenario of “with DL reception” in-between transmission in Rel-17.

- References

R4-2120002, Reply LS on PUCCH and PUSCH transmissions, Qualcomm, RAN4 #101e, Nov 2021.

R4-2120003, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, RAN4 #101e, Nov 2021.

R4-2117187, On UE autonomous adjustment and DL symbol(s) in-between transmission, China Telecom, RAN4 #101e, Nov 2021.