3GPP TSG-RAN4 Meeting #101bis-e	                                                         R4-2200120

Electronic meeting, Jan. 17 – 25, 2022

Title:	Discussion on latency reduction of positioning measurement

Source:	CATT

Agenda Item:	6.21.2.2

Document for:	Discussion

1 Introduction

In RAN4#101e meeting, the issues on latency reduction of positioning measurement were discussed which include the processing samples reduction, UE processing capability, MG enhancement, PRS measurement outside gap and measurement period optimization. And the following conclusions were made and captured in the approved WF [1] as below:

If AGC is required for PRS measurements (i.e. if none of conditions is met), then number of required samples (M2) for AGC? - M2=1 Reducing Rx beam sweeping factor - Introduce a new UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers under certain conditions. Conditions are FFS. Defining reduced number of samples (M) or associated parameters as UE capability - Defer the discussion until more input is received from RAN1 on the UE capability agreements. Work needed for PRS measurements without gaps - MGRP is not needed in the PRS measurement period. New MG patterns and MG enhancement for PRS measurements: - New gap pattern will not be defined for PRS measurements in Rel-17. - Multiple gap patterns for PRS and other measurements are already discussed under the current Rel-17 NR WI on Measurement Gap Enhancement. No need to further discuss this under R-17 ePos WI. - MG enhancement for PRS measurements is not discussed under Rel-17 ePos WI. - Wait for RAN1 input on MG activation and deactivation mechanisms before discussing need for RAN4 requirements

Besides the conclusions above, there are still some remaining issues. In this paper, we further discuss the latency reduction of positioning measurements and give our proposals.

2 Discussion

2.1 Number of samples

In last meeting, RAN4 discussed the reduced PRS processing samples including the number of samples for measurement and the number of samples for AGC.

Regarding the number of samples for measurement, it was agreed to determine based on the link simulation and the simulation assumption was agreed in [2]. Based on the simulation assumption, we provide the simulation results of PRS-RSRP measurement in Annex. From the results, there are the following observations:

Observation 1: For PRS-RSRP measurement, the similar accuracy requirements as R16 can be achieved with 1/2 samples at -6dB Es/Iot under all propagation conditions.

Observation 2: For PRS-RSRP measurement, the similar accuracy requirements as R16 can be achieved with 1/2 samples at -13dB Es/Iot under all propagation conditions.

Observation 3: For PRS-RSRP measurement, the accuracy requirements under TDL-D channel have no improvement compared with TDL-A or TDL-C channel.

Proposal 1: For defining the number of samples for PRS-RSRP measurement, suggest reusing the R16 side condition and propagation conditions for latency reduction of positioning measurement.

Regarding the number of PRS processing samples for AGC, it was agreed that one sample is used if AGC is required. Then it is still under discussion the conditions under which the sample for AGC is not needed. The candidate options are captured in [1] as following:

One or more conditions under which samples for AGC is reduced or not required for PRS measurements - Agreements - Additional samples for AGC for PRS measurements are not required in case at least one of the following conditions is met - Condition #1: - 1A) PRS bandwidth is within the active BWP and - FFS: 1B) Certain power difference between serving and neighbor cell signal power is maintained - Option 1: Target PRS Es/Iot side condition is ≥ -6dB - Option 2: Difference between serving and neighboring cell Es/Iot is within X dB - FFS: Additional conditions 2-3 under which AGC is not needed are: - Condition 2: QC, CMCC - When UE is provided with the QCL information of the PRS (dl-PRS-QCL-Info) - Condition 2a (QC): - If PRS QCL information is provided with SSB as reference with QCL Type A, Type D and average gain - Condition 3: QC, CATT - Based on PRS configuration parameters: - Condition 3a: QC, OPPO - PRS resource repetitions (in different slots) within one PRS instance. Number of repetitions is FFS - Condition 3b: CATT - For the PRS measurement with small periodicity or the PRS measurement with resources having multiple PRS symbols in one sample or for the UE which have higher processing capability

For condition 1, it was agreed that AGC is not needed when PRS bandwidth is within the active BWP. In this case, UE can receive the PRS of neighbour cells and the signals of serving cell simultaneously. If the difference between serving cell and neighbour cell Es/Iot is large, UE may not receive the signals simultaneously and AGC is required to adjust the received signal power of different cells. So it seems option 2 is more appropriate. But considering the side condition for serving cell is -6dB, if the target PRS side condition is also -6dB, the difference between serving cell and target cell may not be large. And since in the discussion of number of samples for measurement, the target PRS side condition will be defined and is possible to be -6dB, it can be better if the conditions for measurement and for AGC are the same. So if the side condition in simulation is -6dB, we are also fine to compromise to option 1 in this issue.

Proposal 2: Condition 1B can be option 2 that the difference between serving and neighboring cell Es/Iot is within X dB. But if the side condition of target cell in simulation part is defined as -6dB, we are also fine with option 1.

For condition 2, we think there is no power related information is provided in the QCL information, so it seems the AGC adjustment for SSB measurement cannot help the PRS receiving i.e. the AGC for PRS measurement is still needed. So we think condition may not reduce the number of samples for AGC.

For condition 3, we think condition 3a and 3b is similar, and both of them are suggesting more PRS symbols (e.g. with more repetitions in one sample). In this case, the AGC can be completed in the same samples as measurement and no additional samples are needed for AGC. Also the small periodicity PRS case in condition 3b should be considered. Since if the interval time between two measurements is short and the AGC has been done in the first measurement, then it is not needed for the second measurement. And if the interval time is short, the receiving gain change is small, and the symbols for AGC settling can be reduced. So no additional samples are needed for AGC.

Proposal 3: The following condition can be considered under which the AGC is not needed:

- Condition 3a: Enough resource repetitions in one sample. Number of repetitions is FFS.

- Condition 3b: When PRS periodicity is small (e.g. smaller than X ms).

Regarding the Rx beam sweeping factor, it was agreed to introduce a new UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers under certain conditions. The conditions are FFS. In our understanding, no conditions are needed since it is UE capability and depends on UE implementation.

Proposal 4: For introducing UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers, no other conditions are needed.

2.2 PRS measurement without MG

In last meeting, based on RAN1 agreements on the PRS measurement without gap, RAN4 initially discussed the issues to be solved.  And the following potential additional parameters/aspects in the PRS measurement requirements for gapless measurements are listed for further studies:

| No. | Parameters/issues |
| --- | --- |
| 1 | Tavailable_PRS,i |
| 2 | LavailablePRS,i |
| 3 | Teffect,i |
| 4 | Applicable number of PFLs |
| 5 | Applicable number of samples |
| 6 | Approach on the calculation of multiple positioning frequency layers |
| 7 | PRS processing window |
| 8 | Requirement applicability |
| 9 | CSSF outside MG |

For the measurement requirements outside gap, we think the requirements of the measurement within gap in R16 can be a reference. Based on RAN1 agreements, PRS processing window is configured for the PRS measurement without gap, so regarding Tavailable_PRS,i, the periodicity of PRS processing window should be used. For UE processing capability, we think the definition in R16 is also applied for the measurement without gap, so the definition of  Teffect,i can be reused.

For LavailablePRS,ithe calculation approach and the impact of PRS resources configuration (including number of resources in a slot and muting patterns) in R16 can be reused for the PRS measurement without gap.

The number of samples and the Rx beam sweeping factor is still under discussion in section 2.1, and if decided, they should be also applied for both PRS measurement with and without gap.

For calculating CSSF outside gap, we think the assumption of processing unit on PRS measurement need to be discussed first. If the PRS measurement is performed with dedicated processing unit, the scaling factor between PRS measurement and RRM measurement is not needed. Then the CSSF for PRS measurement should be discussed among the multiple PFLs. If the PRS measurement is performed with the same processing unit as RRM measurement, then scaling factor between PRS and RRM measurement should be considered. And in this case, we think the same principle as R16 for PFL counting can be used i.e. only one PFL is counted when defining CSSF for RRM measurement.

For the calculation of multiple positioning frequency layers, we think if the PRS processing windows among PFLs are overlapped, the sum approach can be used. But if the processing windows are non-overlapped, the max approach can be used.

Proposal 5: For the measurement requirements of PRS measurement without gap, Tavailable_PRS,i can be the periodicity of PRS processing window.

Proposal 6: For the measurement requirements of PRS measurement without gap, the UE processing capability, the definition of Teffect,i and the calculation of LavailablePRS,i in R16 can be reused.

Proposal 7: For the measurement requirements of PRS measurement without gap, the number of samples and the Rx beam sweeping factor decided in section 2.1 can be used.

Proposal 8: For calculating CSSF outside gap, the assumption of processing unit on PRS measurement needs to be discussed first.

Proposal 9: For the calculation of multiple positioning frequency layers, if the PRS processing windows among PFLs are overlapped, the sum approach can be used. But if the processing windows are non-overlapped, the max approach can be used.

2.3 MG enhancement for PRS measurement

In last meeting, the MG enhancement for PRS measurement was discussed and the following two remaining issues were captured in the WF [1]:

Support of per-FR MG for PRS measurement - FFS: whether RAN4 to define support of per-FR MG for PRS measurement. Optimization of PRS measurements with gaps - FFS: Options for optimization of PRS measurements with gaps - Option 1: QC - For a low-latency PFL i with CSSFPRS,i=1, λPRS, i=1 and Nsample=1, set Tlast,i=MGLi+Ti in the measurement period requirement if all the PRS resources in Tavailable_PRS,i are contained within a single measurement gap instance. - Option 2: OPPO - Discuss the following rules to achieve the alignment among PRS configuration, MG configuration and UE processing capability - The time offset difference between PRS resources in the same positioning frequency layer should be small so that all PRS resources could be covered by MGL. - The periodicity of PRS resources and MG should be configured as the same value, and they should be very close to, but no shorter than the UE capability T. - The time duration of available PRS in the positioning frequency layer i should be no larger than the UE capability N. - The number of PRS resources in each slot in the positioning frequency layer i should be no larger than the UE capability N'. - Option 3: HW - Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe.

Regarding the support of per-FR MG for PRS measurement, we have proposed in R16 positioning feature but some companies have concerns on the implementation. If it can be implemented in R17, we are fine with it.

Regarding the enhancement on Tlast, our preference is not to consider the R16 optimization in limited configuration and scenarios unless the gain of this enhancement is not evident. But if the optimization on Tlast is introduced, we think option 3 is valid.

Proposal 10: Define the support of per-FR MG for PRS measurement.

Proposal 11: Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe.

3 Summary

In this paper, the issues on latency reduction of positioning measurement are discussed, and the following observations and proposals are given:

Observation 1: For PRS-RSRP measurement, the similar accuracy requirements as R16 can be achieved with 1/2 samples at -6dB Es/Iot under all propagation conditions.

Observation 2: For PRS-RSRP measurement, the similar accuracy requirements as R16 can be achieved with 1/2 samples at -13dB Es/Iot under all propagation conditions.

Observation 3: For PRS-RSRP measurement, the accuracy requirements under TDL-D channel have no improvement compared with TDL-A or TDL-C channel.

Proposal 1: For defining the number of samples for PRS-RSRP measurement, suggest reusing the R16 side condition and propagation conditions for latency reduction of positioning measurement.

Proposal 2: Condition 1B can be option 2 that the difference between serving and neighboring cell Es/Iot is within X dB. But if the side condition of target cell in simulation part is defined as -6dB, we are also fine with option 1.

Proposal 3: The following condition can be considered under which the AGC is not needed:

- Condition 3a: Enough resource repetitions in one sample. Number of repetitions is FFS.

- Condition 3b: When PRS periodicity is small (e.g. smaller than X ms).

Proposal 4: For introducing UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers, no other conditions are needed.

Proposal 5: For the measurement requirements of PRS measurement without gap, Tavailable_PRS,i can be the periodicity of PRS processing window.

Proposal 6: For the measurement requirements of PRS measurement without gap, the UE processing capability, the definition of Teffect,i and the calculation of LavailablePRS,i in R16 can be reused.

Proposal 7: For the measurement requirements of PRS measurement without gap, the number of samples and the Rx beam sweeping factor decided in section 2.1 can be used.

Proposal 8: For calculating CSSF outside gap, the assumption of processing unit on PRS measurement needs to be discussed first.

Proposal 9: For the calculation of multiple positioning frequency layers, if the PRS processing windows among PFLs are overlapped, the sum approach can be used. But if the processing windows are non-overlapped, the max approach can be used.

Proposal 10: Define the support of per-FR MG for PRS measurement.

Proposal 11: Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe.

4 References

- R4-2120329, WF on NR Positioning Enhancements (Part 1), Ericsson, RAN4#101e

- R4-2120330, Link level simulation assumptions for NR PRS measurements with reduced latency, Huawei, RAN4#101e

- R4-2117013, LS on PRS measurement outside the measurement gap, Huawei

Annex

Simulation results

The simulation results of PRS-RSRP measurement based on simulation assumption [2] is provided below. DL PRS-RSRP measurement errors of 3 cells for AWGN, TDL-A/C and TDL-D channels are listed respectively in the tables. And the results are the maximum between 5% and 95% of PRS-RSRP error in absolute value.

Table 1 PRS-RSRP measurement error for Es/Iot = (-6,-13,-13)dB

| FR | PRS BW (PRBs) | SCS(kHz) | AWGN |  |  | TDL-A（C） |  |  | TDL-D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 4 | 1 | 2 | 4 | 1 | 2 | 4 |
| FR1 | 24 | 15 | (1.0,2.6,2.6) | (0.8,2.1,2.2) | (0.7,1.8,1.9) | (1.0,3.9,3.6) | (0.7,2.8,3.1) | (0.6,2.1,2.3) | (1.1,3.8,4.0) | (0.8,2.9,3.1) | (0.6,2.1,2.3) |
|  | 52 |  | (0.7,1.8,2.0) | (0.5,1.6,1.6) | (0.4,1.3,1.3) | (0.6,2.3,2.3) | (0.4,1.6,1.7) | (0.3,1.3,1.2) | (0.8,2.5,2.6) | (0.6,1.9,2.1) | (0.4,1.4,1.6) |
|  | 104 |  | (0.5,1.5,1.4) | (0.4,1.2,1.2) | (0.3,0.9,0.9) | (0.4,1.4,1.4) | (0.3,1.0,1.1) | (0.2,0.8,0.8) | (0.5,2.0,1.8) | (0.4,1.6,1.4) | (0.2,1.2,1.1) |
|  | 24 | 30 | (1.0,2.6,2.7) | (0.8,2.1,2.2) | (0.7,1.8,1.8) | (1.0,3.1,3.3) | (0.7,2.5,2.6) | (0.5,1.9,1.9) | (1.0,3.6,3.8) | (0.8,2.9,2.9) | (0.6,2.5,2.2) |
|  | 48 |  | (0.7,1.9,1.9) | (0.5,1.6,1.6) | (0.4,1.4,1.3) | (0.6,2.1,2.0) | (0.4,1.6,1.6) | (0.3,1.1,1.2) | (0.8,2.6,2.7) | (0.6,2.1,2.4) | (0.4,1.4,1.7) |
|  | 132 |  | (0.4,1.2,1.3) | (0.3,1.0,1.1) | (0.3,0.8,0.8) | (0.4,1.6,1.4) | (0.3,1.1,1.0) | (0.2,0.8,0.7) | (0.5,1.5,1.6) | (0.3,1.2,1.4) | (0.3,0.8,1.0) |
| FR2 | 32 | 120 | (1.4,3.7,3.6) | (1.1,2.9,2.8) | (0.9,2.6,2.5) | (1.2,4.0,3.8) | (0.9,3.2,2.9) | (0.7,2.4,2.3) | (1.6,4.9,5.3) | (1.1,3.9,4.3) | (0.8,3.3,3.2) |
|  | 64 |  | (1.0,2.7,2.7) | (0.8,2.1,2.2) | (0.7,1.7,1.8) | (0.9,2.7,2.7) | (0.6,1.9,1.9) | (0.4,1.5,1.4) | (1.1,4.0,3.8) | (0.7,3.1,3.1) | (0.5,2.1,2.1) |
|  | 128 |  | (0.7,2.0,2.0) | (0.6,1.7,1.7) | (0.5,1.4,1.5) | (0.6,2.0,1.8) | (0.4,1.5,1.4) | (0.3,1.1,1.2) | (0.7,2.7,2.6) | (0.6,2.4,2.2) | (0.5,1.7,1.8) |

Table 2 PRS-RSRP measurement error for Es/Iot = (-3,-6,-6)dB

| FR | PRS BW (PRBs) | SCS(kHz) | AWGN |  |  | TDL-A（C） |  |  | TDL-D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 4 | 1 | 2 | 4 | 1 | 2 | 4 |
| FR1 | 24 | 15 | (0.4,0.5,0.5) | (0.3,0.4,0.4) | (0.3,0.3,0.3) | (0.3,0.6,0.5) | (0.2,0.4,0.3) | (0.2,0.3,0.3) | (0.4,0.5,0.5) | (0.2,0.4,0.4) | (0.2,0.3,0.3) |
|  | 52 |  | (0.2,0.3,0.4) | (0.2,0.3,0.3) | (0.2,0.2,0.2) | (0.2,0.3,0.3) | (0.2,0.3,0.2) | (0.1,0.2,0.2) | (0.3,0.3,0.4) | (0.2,0.2,0.2) | (0.1,0.2,0.2) |
|  | 104 |  | (0.2,0.2,0.2) | (0.1,0.2,0.2) | (0.1,0.1,0.1) | (0.2,0.2,0.2) | (0.1,0.2,0.2) | (0.1,0.1,0.1) | (0.2,0.2,0.2) | (0.1,0.2,0.2) | (0.1,0.1,0.1) |
| FR2 | 32 | 120 | (0.6,0.7,0.8) | (0.5,0.6,0.6) | (0.4,0.5,0.5) | (0.5,0.7,0.7) | (0.3,0.5,0.4) | (0.2,0.4,0.3) | (0.5,0.7,0.7) | (0.4,0.5,0.6) | (0.3,0.4,0.3) |
|  | 64 |  | (0.4,0.6,0.5) | (0.3,0.4,0.4) | (0.3,0.3,0.3) | (0.3,0.4,0.4) | (0.2,0.3,0.3) | (0.2,0.2,0.2) | (0.4,0.5,0.5) | (0.3,0.4,0.4) | (0.2,0.2,0.3) |