3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2200141

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | - | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-12-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed no upper power bound is considered for inter-band V2X con-current operation. The MOP requirements specified in clause 6.2E.1.2 are not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the description to indicate the respective MOP requirements specified for single carrier of Uu and SL apply for uplink transmission and sidelink transmission of inter-band con-current operation. - Leave table 6.2E.1.3-1 as void. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MOP requirements specified for inter-band con-current operation would cause confusion and contradiction with configured transmitted power. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.2E.1.3	UE maximum output power for Inter-band V2X

For inter-band V2X with transmission assigned to one E-UTRA band and one NR band, the maximum output power shall apply on each component carrier as defined in clause 6.2.2 or clause 6.2.2G in TS 36.101 [4] and in clause 6.2.E.1.1 or clause 6.2.1.1  in TS 38.101-1 [2], respectively. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.3-1: void

<End of Change 1>