3GPP TSG-RAN WG4 Meeting # 101-bis-e 		                        R4-2200149

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on system parameters for 6GHz licensed band

Source: 	CATT

Agenda item:	5.3.2

Document for:	Approval

- Introduction

In RAN#94-e meeting, a WI on introduction of 6GHz NR licensed bands was approved [1]. This contribution will provide analysis on operating band, channel bandwidth and channel arrangement of 6GHz NR licensed bands.

- Discussion

Based on the reply LS from RCC [2], 3GPP decide to start the work on 6425-7125 MHz frequency range according to RCC Recommendation 1/21. The frequency plan recommended by RCC is provided as below:

Harmonised Technical Conditions for 5G-NR/IMT-2020 Systems in the 6425-7125 MHz Frequency Band - Frequency Plan It is recommended that for 5G-NR/IMT-2020 systems operating in the 6425-7125 MHz band be used the frequency band plan as shown on Fig. 1 and a Time Division Duplex (TDD) mode. The frequency band plan in the 6425-7125 MHz band consists of 35 reference frequency blocks of 20 MHz each. 6425 MHz; →; ← 20 MHz; 7125 MHz. B1; B2; B3; B4; B5; …; B35. Figure 1. Frequency band plan for 5G-NR/IMT-2020 systems in the 6425-7125 MHz frequency band A frequency channel used by 5G/IMT-2020 systems can consist of one or more adjacent frequency blocks but should not exceed 400 MHz. In order to maximize the usable spectrum within the 6425-7125 MHz by 5G-NR/IMT-2020 systems and taking into account that this and adjacent frequency bands are used by other systems: - the bandwidth of a frequency block could be increased in relation to the reference block by multiples of 10 MHz; - the central frequency of a frequency block can be shifted in relation to the central frequency of a reference block by multiples of 5 MHz; It is recommended, as far as possible, to assign frequency channels as continuous spectrum.

From the above recommendations, the frequency range of 6425-7125MHz should be operated with a TDD mode. The minimum channel bandwidth is 20MHz and the maximum channel bandwidth should not exceed 400MHz. Also, the channel bandwidth can be increase by multiples of 10MHz. Hence, it is proposed to introduce licensed band n103 for 6425-7125MHz as presented in table 2-1.

Proposal 1: To introduce licensed band n103 for 6425-7125MHz as presented in table 2-1.

Table 2-1: Operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n10317 | 6425 MHz – 7125MHz | 6425 MHz – 7125MHz | TDD |

Based on the study outcome in TR 38.921, the 100 MHz has been considered as a representative channel bandwidth that will be used. So it is proposed to introduce the channel bandwidth in table 2-2 for 6425-7125MHz.

Proposal 2: To introduce the channel bandwidth in table 2-2 for 6425-7125MHz.

Table 2-2: UE channel bandwidth

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n100 | 15 |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |

Based on the NR-ARFCN equation, it is proposed to introduce the applicable NR ARFCN in table 2-3 for 6425-7125MHz.

Proposal 3: To introduce the applicable NR ARFCN in table 2-3 for 6425-7125MHz.

Table 2-3: Applicable NR ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n103 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |

Based on GSCN definition, it is proposed to introduce the applicable SS raster entries in table 2-4 for 6425-7125MHz.

Proposal 4: To introduce the applicable SS raster entries in table 2-4 for 6425-7125MHz.

Table 2-4: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n103 | 30 kHz | Case C | 9881 – <1> – 10360 |

Our companion draft CR is also provided in [5].

- Conclusion

This contribution mainly discusses system parameters for 6GHz NR licensed band. The following proposals are derived:

Proposal 1: To introduce licensed band n103 for 6425-7125MHz as presented in table 2-1.

Table 2-1: Operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n10317 | 6425 MHz – 7125MHz | 6425 MHz – 7125MHz | TDD |

Proposal 2: To introduce the channel bandwidth in table 2-2 for 6425-7125MHz.

Table 2-2: UE channel bandwidth

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n100 | 15 |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |

Proposal 3: To introduce the applicable NR ARFCN in table 2-3 for 6425-7125MHz.

Table 2-3: Applicable NR ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n103 | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |

Proposal 4: To introduce the applicable SS raster entries in table 2-4 for 6425-7125MHz.

Table 2-4: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n103 | 30 kHz | Case C | 9881 – <1> – 10360 |

- Reference

- RP-213696, Revised WID: Introduction of 6GHz NR licensed bands, Huawei, HiSilicon, RAN#94e

- RP-213605, LS on inclusion of the 6425-7125 MHz frequency band in the 3GPP specification for 5G-NR/IMT-2000 systems, Regional Commonwealth in the field of Communication (RCC)

- RCC Recommendation 1/21, Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz (Release 17)

- Draft CR for TS 38.101-1, Introduction of 6GHz licensed band for UE, CATT, RAN4#101bis-e