3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2200236

Online, 17th – 25th Jan. 2022

Agenda item:	5.19.1

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_11_n3-n79

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_11_n3-n79 was approved in RAN#94 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_11_n3-n79

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n79 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  |  |  | n3 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |
| Note 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n79A, 3rd order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of band n79 and 3rd order IMD generated by dual uplink of Band 11 + Band n79 may also fall into own Rx of band n3.

6.X.4	∆TIB and ∆RIB values

For DC_11_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n79 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n79 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n79 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n79 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, IMD3 of B11 and n3 to Band n79 Rx and IMD3 of B11 and n79 to Band n3 Rx need to be addressed for REFSENS relaxation. Based on the MSD analysis results in [2][3], the following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A_n3A-n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | 16.2 | TDD | IMD3 |
|  | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n79 | 4735 | 40 | 216 | 4735 | N/A | TDD | N/A |
|  |  | n3 |  | 1770 |  | 5 |  | 25 |  | 1865 |  | 17.8 |  | FDD |  | IMD3 |  |

[Unchanged Parts Skipped]

3	Reference

[1] RP-212876	Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics

[2] R4-2200793	TP on summary of self-interference analysis for new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17, LG Electronics

[3] R4-2200794	MSD test results and test configurations for new NR DC band combinations, LG Electronics