3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200280

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.19.4

Source:	Apple Inc.

Title:	On scope of demod requirements for FeMIMO

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In this first meeting to discuss demod requirements for FeMIMO we provide our views on the scope of demod and CSI requirements.

# 2. Discussion

For FeMIMO the following enhancements have been introduced by RAN1:

- Enhancements to multi-beam operation

- Enhancements for multi-TRP deployment

  - PDCCH Reliability Enhancement

  - PUSCH/ PUCCH Reliability Enhancement

  - Inter-cell multi-TRP operation

  - HST-SFN Enhancements

- SRS Enhancements

- CSI Enhancements

  - Enhanced Type II port selection codebook

  - CSI enhancement for multi-TRP

Table 1: FeMIMO Enhancement impact to UE demod

| Feature | Impact to UE demodulation requirements |
| --- | --- |
| Enhancements to multi-beam operation | No impact to UE demod |
| Multi-TRP PDCCH | Impacts UE processing |
| Multi-TRP PUCCH/PUSCH | No impact to UE demod |
| Inter-cell multi-TRP | Impacts UE processing |
| HST SFN Enahncement | Impacts UE processing |
| SRS Enhancements | No impact to UE demod |
| Further enhanced Type II port selection codebook | Impacts UE processing but no UE requirements for port selection |
| CSI enhancement for multi-TRP | Impacts UE processing |

For FeMIMO we don’t see impacts to UE demod requirements with the following enhancements:

- Enhancements to multi-beam operation

- Multi-TRP PUCCH/PUSCH

- SRS Enhancements

- Further enhanced Type II port selection codebook

Proposal #1: No impact to UE demodulation with the following enhancements

- Enhancements to multi-beam operation

- Multi-TRP PUCCH/PUSCH

- SRS Enhancements

- Further enhanced Type II port selection codebook

For multi-TRP enhancements for PDCCH the enhancements are to improve PDCCH reliability by transmitting 2 PDCCH on two linked search space. It impacts UE processing, but we don’t see a benefit of introducing requirements for this enhancement for UE PDCCH demodulation.

For inter-cell multi-TRP enhancements, the enhancements in Rel-17 are to enable multi-TRP transmission from 2 intra-frequency cells. From UE PDSCH processing there should be no difference compared to Rel-16 multi-TRP transmission processing in Rel-16. The main enhancement is to enable UE to report L1-RSRP for cell with different PCI from serving cell.  From PDSCH demodulation requirements we don’t see a benefit to introduce requirements for inter-cell multi-TRP as the requirements should be very similar to multi-TRP requirements for eMIMO.

Proposal #2: Do not define UE demod requirements for the following FeMIMO enhancements:

- Multi-TRP PDCCH enhancements

- Inter-cell multi-TRP operation

For HST SFN enhancements the enhancement is to transmit PDCCH/PDSCH in SFN manner with 2 TCI states and TRS in non-SFN mode from each TRP. Also, 2 schemes – with and without pre-compensation are introduced.

Figure 1: HST-SFN Enhancement

HST SFN enhancements would impact UE processing and it would be beneficial to discuss UE demodulation requirements with this enhancement.

CSI enhancement for multi-TRP Single-DCI SDM transmission scheme is introduced in Rel-17 to report PMI RI for each TRP in single DCI SDM transmission scheme. It would be beneficial to further discuss if we define PMI reporting requirements for CSI enhancements for SDM transmission scheme.

Proposal #3: Further discuss if requirements are introduced for the following enhancements for FeMIMO

- HST-SFN enhancements

- CSI enhancements for single DCI SDM transmission scheme

# 3. Conclusion

In this paper, we provide our views on the scope of demod and CSI requirements for FeMIMO. Our proposals are captured below:

Proposal #1: No impact to UE demodulation with the following enhancements

- Enhancements to multi-beam operation

- Multi-TRP PUCCH/PUSCH

- SRS Enhancements

- Further enhanced Type II port selection codebook

Proposal #2: Do not define UE demod requirements for the following FeMIMO enhancements:

- Multi-TRP PDCCH enhancements

- Inter-cell multi-TRP operation

Proposal #3: Further discuss if requirements are introduced for the following enhancements for FeMIMO

- HST-SFN enhancements

- CSI enhancements for single DCI SDM transmission scheme

# Reference