3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200364

Electronic Meeting, January 17-25, 2022

Agenda item:	6.8.3.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on HST CA tests for FR1

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed HST CA requirements and RAN4 agreed a WF [1]. This contribution presents our views on HST CA requirements.

2. Discussion

2.1. UE capability for HST-SFN CA

We show our views on following options [1].

UE capability for HST-SFN CA - Option 1: define a new Rel-17 UE capability to indicate the support of HST-SFN CA - Option 1a: the granularity of the capability is a per-UE - Option 1b: the granularity of the capability is per band combination - Option 1c: the granularity of the capability is per band - Option 2: Do not introduce additional UE capability for HST-SFN CA.

For LTE HST-SFN test, UE that is capable of highSpeedEnhancedDemodulationFlag should pass both HST-SFN for single carrier test and CA test, so the motivation for introducing new UE capability for NR HST-SFN CA is not clear. On the other hand, from the perspective of Rel-17 HST RRM discussion, RAN4 agreed to introduce the UE capability to indicate whether the enhanced requirements for HST CA are supported or not [2]. Considering this RRM agreement, we think it would be better to align UE side with RRM side. Thus, we are fine to define the new Rel-17 UE capability to indicate the support of HST-SFN CA.

Proposal 1: Define the new Rel-17 UE capability to indicate the support of HST-SFN CA

3. Conclusion

In this contribution, we present our views on HST CA requirements. Our proposals is summarized below.

Proposal 1: Define the new Rel-17 UE capability to indicate the support of HST-SFN CA

References

- R4-2120700, WF on FR1 HST demodulation, CMCC

- R4-2120285, WF on RRM for FR1 HST, CMCC