3GPP TSG-RAN WG4 #101-bis-e	R4-2200379

Electronic Meeting, January 17-25, 2022

Agenda item:	6.12.2.2

Source:	MediaTek Inc.

Title:	Discussion on the MMSE-IRC receiver for intra-cell interference

Document for:	Discussion

- Introduction

In RAN4#101-e, a way forward for MMSE-IRC receiver for intra-cell inter-user interference was agreed [1] and the remaining issues in the WF are captured as below.

PMI matrix selection for Co-scheduled UE for 2TX and 4TX - Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE - Option 3: Use option 2 for rank 1+1 and option 1 for rank 2+2. DMRS ports for case with rank 1+1 - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Option 3: Variable DMRS port mapping during the test. DMRS scrambling ID for target UE and co-scheduled UE - Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2: Same scrambling ID for all cases - Option 3: Configure variable scrambling ID during the test.

In this contribution, we present simulation results based on the agreed simulation assumptions and provide our views for the remaining issues.

- Discussion

PMI matrix selection for Co-scheduled UE for 2TX and 4TX

From the simulation results, with MCS13, it showed that there are 1.5~3.8dB performance degradation with random precoder compared to orthogonal precoder. We agree that neither random nor orthogonal precoder reflect the realistic precoder selection. However, for the scenario of MU-MIMO, we think network would try to select the orthogonal precoder to reduce the inter-user interference for better performance. Hence, we propose to define test case with precoder to ensure orthogonality.

Proposal 1: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

DMRS ports for case with rank 1+1

It was agreed to have co-scheduled UEs on different CDM groups for rank 2+2. Hence, for the test coverage, we could configure same CDM group for rank 1+1 cases to verify UE performance. That is, configure DMRS port 0 for target UE and DMRS port 1 for the interference UE.

Proposal 2: For the test cases with rank 1+1, configure DMRS port 0 for target UE and DMRS port 1 for the interference UE, i.e., same CDM group.

DMRS scrambling ID for target UE and co-scheduled UE

From our simulation results, the performance of MMSE-IRC with different scrambling ID is slightly worse than that with same scrambling ID for the case of same CDM group. Also, it shows that the performance gain of MMSE-IRC with different scrambling ID is still large compared to MMSE-MRC. According to the specification, it is possible for gNB to configure different scrambling ID to co-scheduled UE. Hence, we propose to consider both same and different scrambling ID in the test cases.

Proposal 3: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Simulation Results

| Figure 1. Simulation results for 2Tx |  |
| --- | --- |

| Figure 2. Simulation results for 4Tx |  |
| --- | --- |

Table 1: Summary of simulation results for 2 Tx, TDL-C channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 15.6 | Inf |
|  |  |  | Orthog | Inf | 13.5 | Inf |
|  | 4 | 13 | Rand | Inf | 9.3 | Inf |
|  |  |  | Orthog | 11 | 7.5 | 3.5 |
| 2 | 2 | 13 | Rand | Inf | 15.5 | Inf |
|  |  |  | Orthog | Inf | 13.2 | Inf |
|  | 4 | 13 | Rand | Inf | 9.1 | Inf |
|  |  |  | Orthog | 10.9 | 7.5 | 3.4 |

Table 2: Summary of simulation results for 2 Tx, TDL-C channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 1 | 2 | 13 | Rand | Inf | 16.1 | Inf |
|  |  |  | Orthog | Inf | 13.8 | Inf |
|  | 4 | 13 | Rand | Inf | 9.3 | Inf |
|  |  |  | Orthog | 13.4 | 7.6 | 5.8 |
| 2 | 2 | 13 | Rand | Inf | 15.6 | Inf |
|  |  |  | Orthog | Inf | 13.3 | Inf |
|  | 4 | 13 | Rand | Inf | 9.1 | Inf |
|  |  |  | Orthog | 10.9 | 7.6 | 3.3 |

Table 3: Summary of simulation results for 4 Tx, TDL-A channel and same scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+2 | 4 | 13 | Rand | Inf | 16.9 | Inf |
|  |  |  | Orthog | Inf | 13.1 | Inf |

Table 4: Summary of simulation results for 4 Tx, TDL-A channel and different scrambling ID

| Parameters |  |  |  | SNR for 70% of max T-put, [dB] |  | MMSE-IRC SNR gain, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | #Rx | MCS | PMI | MMSE-MRC | MMSE-IRC |  |
| 2+2 | 4 | 13 | Rand | Inf | 16.9 | Inf |
|  |  |  | Orthog | Inf | 13.1 | Inf |

- Conclusion

In this contribution, we provide simulation results and our views on the remaining issues for the requirements of MU-MIMO. The proposals are summarized as below:

Proposal 1: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

Proposal 2: For the test cases with rank 1+1, configure DMRS port 0 for target UE and DMRS port 1 for the interference UE, i.e., same CDM group.

Proposal 3: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Reference

- R4-2120709, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon

Appendix

Table 1. Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| CBW/SCS |  | 10 MHz/15 kHz for FDD |
| Antenna configuration |  | 2T2R and 2T4R ULA Low for case with rank1+1 4T4R XPL ULA Low for case with rank 2+2 |
| Channel model |  | TDLA30-10 (for rank 2+2) and TDLC300-100 (for rank 1+1) |
| Codebook |  | Type I SP codebook |
| Precoder selection for target UE |  | Random |
| Precoder selection for interference co-scheduled UE |  | - Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal - Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE |
| Precoding granularity |  | 2 PRBs |
| PRB bundling size |  | 2 PRBs |
| Scrambling ID for target UE and co-scheduled UE |  | Option1: same Option2: different |
| Rank and DMRS ports |  | - [1+1]: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - [1+1]: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - [2+2] : DMRS port 0 (and 1) for target UE, port 2 (and 3) for the interference UE, i.e., use different CDM groups for the target and interference UEs |
| Number of CDM groups without data configuration for case with rank 1+1 if same CDM group |  | 1 for target UE and co-scheduled UE |
| Target NR PDSCH parameters | Mapping type | Type A |
|  | Number of front-loaded DMRS | 1 |
|  | Number of additional DMRS | 1 |
|  | MCS | 16QAM (MCS 13) |
|  | HARQ process number for target NR PDSCH | 4 for FDD, 8 for TDD |
| Signal power and SNR assumptions |  | - Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1 - SNR = STargetUE/N |