3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200417

# Electronic Meeting, January 17-25, 2021

Agenda Item:	6.25.2

Source: 		Qualcomm Incorporated

Title: 			RRM requirements for NR-SDT

Document for:	Discussion

# 1.	Introduction

In the last RAN4 meeting, the requirements for NR-SDT have been discussed in [1]. However, there are open issues from the last meeting. And we continue to discuss the RRM requirements for NR-SDT in this paper.

# 2. 	Discussion

In the previous RAN4#101 e-meeting, it was agreed to consider the new requirements for CG-SDT like LTE-PUR. But there are more points to discuss at CG-SDT as following:

- Separate TA validation requirements for CG-SDT per usages.

- Design the TA validation window for CG-SDT

  - Separate TA validation requirements for CG-SDT per usages

In last meeting, RAN4 agreed that similar requirements need to be defined as LTE-PUR. Similar to LTE-PUR defined two different sets of requirements such as eMTC and NB-IoT, different TA validation requirements can be defined for different use cases such as NR-RedCap.

Proposal 1: Different TA validation requirements can be defined for different use cases.

  - Design the TA validation window for CG-SDT

In the previous meeting, RAN4 agreed that two different window size shall be defined to validate TA and follow the same principle of LTE-PUR. The LTE-PUR TA validation for UE category M1 in [2] is duplicated as

RSRP1 is considered valid provided that the following condition is met when in normal coverage: (T1 – min(480 ms, N× DRX cycle)) ≤ T1’ ≤ (T1 + min(480 ms, N×DRX cycle)) RSRP1 is considered valid provided that the following condition is met when in enhanced coverage: (T1 – min(800 ms, N× DRX cycle)) ≤ T1’ ≤ (T1 + min(800 ms, N×DRX cycle)) RSRP2 is considered valid provided that the following condition is met when in normal coverage: T2 – min(480 ms, N×DRX cycle) ≤ T2’ ≤ T2 RSRP2 is considered valid provided that the following condition is met when in enhanced coverage: T2 – min(800 ms, N×DRX cycle) ≤ T2’ ≤ T2

However, some parameters shall be defined for NR specific.

Proposal 2: The principle of TA validation window parameters design for CG-SDT

- RSRP1 is considered valid that the following condition is met

(T1-min (X,N_serv)) <= T1’ <= (T1+min(X,N_serv)

- RSRP2 is considered valid that the following condition is met

(T2-min (X,N_serv)) <= T2’ <= T2

- where X can be the measurement period for intra-frequency measurements without gap for No DRX,

Nserv can be the consecutive DRX cycles for UE to evaluate a serving cell, and other definitions of parameters such as T1, T1’, T2, T2’, RSRP1, RSRP2 can be same as LTE-PUR.

# 3.	Conclusion

In this contribution, we discussed RRM requirements for CG-SDT based on the LTE-PUR procedure. Based on the discussion, we proposed followings:

Proposal 1: Different TA validation requirements can be defined for different use cases.

Proposal 2: The principle of TA validation window parameters design for CG-SDT

- RSRP1 is considered valid that the following condition is met

(T1-min (X,N_serv)) <= T1’ <= (T1+min(X,N_serv)

- RSRP2 is considered valid that the following condition is met

(T2-min (X,N_serv)) <= T2’ <= T2

- where X can be the measurement period for intra-frequency measurements without gap for No DRX,

Nserv can be the consecutive DRX cycles for UE to evaluate a serving cell, and other definitions of parameters such as T1, T1’, T2, T2’, RSRP1, RSRP2 can be same as LTE-PUR.

# 4.    References

- R4-2120338, WF on RRM requirements for NR SDT in INACTIVE state, ZTE Corporation, RAN4 #101e

- TS 36.133 V16.11.0