3GPP RAN WG4 Meeting #101bis		R4-2200433

Electronic meeting, 17 – 26th January 2022

Agenda item:	5.2.2

Source:	Apple

Title:	Applicability of band n96

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently (refer to the latest regulatory status updated in  [2]). Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements [3].

During the previous meetings, RAN WG4 analysed existing Region 2 and Region 3 regulatory requirements for which it has been concluded that band n96 can be re-used because the full frequency range of 5925-7125MHz has been adopted for the license-exempt operation. However, as discussed during the RAN4#101 meeting, the existing NOTE for band n96 limits its applicability to US, whereas it was agreed before that band n96 will be-reused for the Region 2 countries (e.g. US, Canada, Brasil, Peru, Chile, Costa Rica, Colombia) and Region 3 (e.g. South Korea).

# 2	Applicability of band n96

As already mentioned in the Introduction part, when the band n96 was introduced back in Rel-16, a NOTE was added clarifying that it is applicable only to US. However, there have been other countries, in Region 1, 2 and 3, which have announced that they would also allow license-exempt operation in the 5925-7125MHz frequency range. Thus, the existing NOTE is not valid anymore and should be changed or removed, as was discussed during the RAN4#101 meeting.

As captured in [4], some companies prefer removing the NOTE completely while some companies suggest keeping it with more generic wording, whereupon the potential wording could be "This band is only applicable subject to regional and/or country specific restrictions". Our view is that even though the proposed wording does not restrict band n96 applicability only to US, it does not convey much technical information because the same statement can be applied to almost any band. Thus, our preference is to remove the NOTE completely.

Proposal 1a:	Remove (preferred option) or change the existing NOTE for band n96.

Proposal 1b:	If the NOTE is kept, the following wording can be adopted "This band is only applicable subject to regional and/or country specific restrictions".

Another aspect raised during the meeting was the release in which the corresponding NOTE should be changed. Since the band n96 was added in Rel-16, it is natural to change/remove the corresponding NOTE in Rel-16 otherwise it may give a wrong impression that Rel-16 band n96 cannot be used in other countries. In fact, since Region 2 LAM countries regulatory requirements are generally aligned with US (refer to Annex A), Rel-16 band n96 with NS_53 can be used safely used in e.g. Brazil or Chile.

Proposal 2:	Removing/changing the NOTE should start from Rel-16.

# 3	Conclusions

In this contribution we have presented our view on the applicability of band n96 to other countries/regions which have adopted the 5925-7125MHz frequency range for the license-exempt operation. As a summry of our paper, we suggest changing/removing the NOTE, whereupon the corresponding modification should be applied from Rel-16.

Proposal 1a:	Remove (preferred option) or change the existing NOTE for band n96.

Proposal 1b:	If the NOTE is kept, the following wording can be adopted "This band is only applicable subject to regional and/or country specific restrictions".

Proposal 2:	Removing/changing the NOTE should start from Rel-16.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- 3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.13.0

- RP-211602, "New WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- R4-2119863, "WF on introduction of the full unlicensed band ", Apple

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |