3GPP RAN WG4 Meeting #101bis-e		R4-2200450

Electronic meeting, January 17 – 26, 2022

Agenda item:	6.2.2.1

Source:	Apple

Title:	TP to TR 38.834 on P-MPR handling

WI/SI:	NR_FR1_TRP_TRS-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

In an effort to ensure that the TRP of all UEs is measured at their maximum configured output power, it is necessary tspecify a general test condition that for UE radiated testing, P-MPR shall be assumed to be 0 dB.  This contribution provides a text proposal to TR38.834 in accordance with this observation. We note that this proposal is also well aligned with the CTIA Test Plan, which specifies that the device needs to operate at full RF power during the OTA test.

# 2	Text proposal

<begin text proposal>

# 4	General

## 4.1	Device types

The following device types are within the scope of FR1 TRP TRS WI:

-	Smartphone

-	Considering UEs with antenna configurations of 1Tx, 2Tx, 2 Rx and 4 Rx

-	Tablet

-	Laptop embedded equipment (LEE)

-	Laptop mounted equipment (LME)

The development of test methodology aspects shall initially focus on the smartphone device type.

For UE radiated conformance testing P-MPRc shall be 0 dB.

## 4.2	Testing configuration

### 4.2.1	UE use scenarios for TRP TRS test

The following use scenarios are considered for TRP TRS test:

-	Talk mode using head & hand phantom for narrow devices between 56 mm and 72 mm and for wide devices with a width >72 mm and <92 mm.

-	Browsing mode using hand phantom for narrow and wide phones

-	Free Space for devices not used in talk or browsing mode

For smartphone, head/hand phantoms testing configuration is the first priority. How to select Talk mode or Browsing mode is FFS.

For other device types, free space (FS) testing configuration is the first priority.

### 4.2.2	UE mechanical mode description

Primary mechanical mode means the main operation mode of a UE, i.e., portrait slide open, fold open, or other mechanical configuration, depends on form factor. The primary mechanical mode should be declared by manufacturer instructions. The test methods for TRP TRS testing are defined based on UE primary mechanical mode.

<end text proposal>