3GPP TSG-RAN4 Meeting # 101bis-e	R4-2200474

# Electronic Meeting,  17th – 27th January, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.141-1 adding MU and TT for FR1 PUSCH with 256QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2021-12-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed to add requirements for FR1 PUSCH with 256QAM. The MU and TT should be added for the corresponding comformance tests. Propose TE venders check the MU and TT value for 256QAM at the first. If MU and TT is the same as legacy value, then it seems unnecessary to add a new row for 256QAM then this CR is not needed. Otherwise, this CR with new MU and TT value could be needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table 4.1.2.4-1 and C.3-1 with new row for PUSCH with 256QAM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no MU and TT for FR1 PUSCH with 256QAM comformance tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, Annex C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#########################   Start of change#1  ############################

#### 4.1.2.4	Measurement of performance requirements

Table 4.1.2.4-1: Maximum Test System Uncertainty for performance requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8 PUSCH, PUCCH, PRACH with single antenna port and fading channel | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8 PUSCH, PRACH with single antenna port and AWGN | ± 0.3 dB | Signal-to-noise ratio uncertainty ±0.3 dB |
| 8 PUSCH with two antenna port and fading channel | ± 0.8 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for MIMO |
| 8 PUSCH with single antenna port and 256QAM | [± 0.6] dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |

#########################   End of change#1  ############################

#########################   Start of change#2  ############################

# C.3	Measurement of performance requirements

Table C.3-1: Derivation of Test Requirements (Performance tests)

| Test | Minimum Requirement in TS 38.104 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | SNRs as specified | 0.6 dB for 1Tx cases 0.8 dB for 2Tx cases | Formula: SNR + TT T-put limit unchanged |
| 8.2.1 Performance requirements for PUSCH with 256QAM | SNRs as specified | [0.6] dB for 1Tx cases | Formula: SNR + TT T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3 Performance requirements for UCI multiplexed on PUSCH | SNRs as specified | 0.6 dB | Formula: SNR + TT BLER limit unchanged |
| 8.2.4 Performance requirements for PUSCH for high speed train | SNRs as specified | 0.3 dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.5 Performance requirements for UL timing adjustment | SNRs as specified | 0.3 dB for AWGN | Formula: SNR + TT T-put limit unchanged |
| 8.2.6 Performance requirements for PUSCH with 0.001% BLER | SNRs as specified | 0.3 dB | Formula: SNR + TT + 1dB 1dB is added to the test requirement to facilitate early test pass. The BLER delivered by the device during the test will be lower than the test requirement, which enables compliance to the requirement to be demonstrated with a number of observed block errors lower than a certain threshold. |
| 8.2.7 Performance requirements for PUSCH repetition Type A | SNRs as specified | 0.6 dB | Formula: SNR + TT BLER limit unchanged |
| 8.2.8 Performance requirements for PUSCH Mapping Type B with non-slot transmission | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | SNRs as specified | 0.6 dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | SNRs as specified | 0.6 dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.6 Performance requirements for multi-slot PUCCH | SNRs as specified | 0.6 dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | SNRs as specified | 0.6 dB for fading cases 0.3 dB for AWGN cases | Formula: SNR + TT PRACH false detection limit unchanged PRACH detection limit unchanged |

#########################   End of change#2 ############################