3GPP TSG-RAN WG4 Meeting #101-bis-e 	R4-2200501

Electronic Meeting, 17-25 Jan., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On CQI requirements for intercell interference MMSE-IRC

Agenda item:	6.12.2.1.2

Document for:	Discussion

Introduction

During the last RAN4#101-e meeting, good progress was made on the topic of CQI requirements for inter-cell interference MMSE-IRC.

Some remaining issues are captured in the corresponding WF [1]. The major open topics being:

- Antenna configuration of serving cell.

- Requirement’s definition.

In this contribution we will express our views on the captured open issues and open new discussions, if necessary.

CQI requirements for inter-cell interference MMSE-IRC

Here we discuss open issues, as are left over from the last meeting.

Antenna configuration for serving cell

During the discussion in RAN4#101e multiple antenna configuration for serving cell were discussed [1]:

- Option 1: 2T2R, 2T4R, ULA Low - Option 2: 1x2/1x4 ULA Low

Figure 1 below shows the results captured for option 1, while Figure 2 below captures the results for option 2. Results in interference scenario and in AWGN are captured. For interference scenario, curves with both MMSE-IRC is used for both CQI calculation as well as PDSCH demodulation. For both antenna configurations a DIP value of -0.41 dB is used.

It is seen that both antenna configurations are suitable and have SINR regions where ratio of throughputs (γ) is higher than 1.5. SINRs above 2 dB are suitable with option 1, while SINR above -2 dB are suitable with option 2.

Both antenna configurations have SINR regions with γ higher than 1.5. SINR values with option 2 are lower and arguably closer to cell edge conditions. Furthermore, option 2 is similar to what was used in LTE.

Prefer option 2 to define the requirements.

Figure 1 Throughput with antenna configuration as per option 1

Figure 2 Throughput with antenna configuration as per option 2

Requirements definition

During the discussion in RAN4#101e it was decided to reuse the LTE metric for CQI reporting in ICI and following agreement was reached for requirements definition[1]:

- Agreement: - SINR, γ and X values should be decided based on the simulation results - Criteria will be further decided in the next meeting based on simulation results - Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing (example).

Based on the results captured in Figure 1 and 2 we can see that a value of γ in the range of 1.5 to 2 can be defined for 2Rx configurations.

A suitable SINR value could be around -2 dB. This SINR will be found closer to cell edge and has a γ value of around 1.5 with 1Tx2Rx ULA low antenna configuration.

SINR of ~-2 dB has a ratio γ of ~1.5 for UEs with 2 Rx antenna configurations.

SINR of -2 dB and ratio γ of 1.5 can be used for 2 Rx antenna configuration.

Conclusion

In this contribution we have provided our views on various open issues with relation to CQI requirements for intercell interference MMSE-IRC. We make proposals concerning Antenna configuration and Requirement definition.

We have made the following observations and proposals:

Antenna configuration for serving cell

- Both antenna configurations have SINR regions with γ higher than 1.5. SINR values with option 2 are lower and arguably closer to cell edge conditions. Furthermore, option 2 is similar to what was used in LTE.

- Prefer option 2 to define the requirements.

Requirements definition

SINR of ~0 dB has a ratio γ of ~1.5 for UEs with 2 Rx antenna configurations.

- SINR of -2 dB and ratio γ of 1.5 can be used for 2 Rx antenna configuration.

- References

- R4-2120708 - WF on CSI requirements for inter-cell interference MMSE-IRC