3GPP TSG-RAN4 Meeting #101-bis-e	R4-2200601

# Electronic Meeting, Jan. 17 – 25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce relaxed requirements for CSI-RS based RLM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce relaxed requirements for CSI-RS based RLM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relaxed requirements for CSI-RS based RLM is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

8.1.3.X	Minimum requirement for UE fulfilling relaxed measurement criterion

The requirements defined in 8.1.3.2 apply for the clause except that:

-	TEvaluate_out_CSI-RS  when DRX ≤ 160ms are defined in Table 8.1.3.X-1 for the first out-of-sync indication to the higher layer in FR1.

-	TEvaluate_out_CSI-RS when DRX ≤ 160ms are defined in Table 8.1.3.X-2 for the first out-of-sync indication to the higher layer in FR2 with scaling factor N=1.

Table 8.1.3.X-1: Evaluation period TEvaluate_out_CSI-RS of the first out-of-sync indication for UE fulfilling relaxed measurement criteria in FR1

| Configuration | TEvaluate_out_CSI-RS (ms) |  |
| --- | --- | --- |
| Max(TDRX, TCSI-RS) ≤ 40 ms | Max(200, Ceil(1.5×Mout×P×K1)× Max(TDRX, TCSI-RS)) |  |
| 40 ms < Max(TDRX, TCSI-RS) ≤ 80 ms |  | Max(200, Ceil(1.5×Mout×P×K2)× Max(TDRX, TCSI-RS)) |  |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) |  |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE2: K1 = [3], K2 = [2], Mout, P are specified in clause 8.1.3.2. |  |  |

Table 8.1.3.X-2: Evaluation period TEvaluate_out_CSI-RS of the first out-of-sync indication for UE fulfilling relaxed measurement criteria in FR2

| Configuration | TEvaluate_out_CSI-RS (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max(200, Ceil(1.5×Mout×P×N×K3)× Max(TDRX, TCSI-RS)) |
| 80 ms < Max(TDRX, TCSI-RS) ≤ 160 ms | Max(200, Ceil(1.5×Mout×P×N)× Max(TDRX, TCSI-RS)) |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE2: K3 = 2, Mout, P are specified in clause 8.1.3.2. |  |

<< End of change 1>>