3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200647

Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Impact on NR positioning RRM requirements

Agenda item:	6.21.2.4

Document for:	Discussion

# Introduction

In Rel-17, there are various positioning features to improve positioning accuracy, latency and resource efficiency as discussed in WF [2,4]. In this contribution, we discuss on impact on NR positioning RRM requirements that has been discussed in RAN4#101.

# Discussion

Impact on existing UE positioning and RRM requirements SRS antenna port switching on UE Rx-Tx time difference - FFS: whether Rel-15 SRS (SRS-Resource) can be used for UE Rx-Tx time difference - FFS: impact of SRS antenna switching on UE behaviour and UE Rx-Tx time difference accuracy SRS antenna port switching on gNB Rx-Tx time difference and UL RTOA - FFS: whether Rel-15 SRS (SRS-Resource) can be used for gNB Rx-Tx - FFS: impact of SRS antenna switching on gNB Rx-Tx time difference accuracy - Rel-15 SRS (SRS-Resource) be used for UL RTOA - FFS: impact of SRS antenna switching on UL RTOA reporting (report mapping in 38.133) if different SRS antenna ports used for UL RTOA

First, we wonder if SRS antenna port needs to be switched for positioning measurement purpose. Only a single SRS antenna port is used for positioning measurements, so positioning SRS port control is different from sounding SRS purpose. If SRS antenna port switches during the UE Rx-Tx timing measurement period, it impacts on accuracy, since SRS antenna port has a unique CS rotation, CS rotation makes different timing in an OFDM symbol. However, we don’t have clear understanding why SRS antenna port needs to be switched for positioning. In addition, positioning measurements is not supposed to be impacted by physical TX antenna switching due to RF circumstance like hand grip etc. Once a SRS antenna port is configured, one of UE physical TX antennas can transmit the configured SRS sequence without SRS antenna port switching.

Observation 1 : If SRS antenna port switching happens during UE RXTX TD or gNB TXRX TD measurement, it impacts on measurement accuracy.

38.214 clause 5.1.6.5 The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers.

RAN1 agreement: Support reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR. - Note: There is no impact to specifications managed by RAN1 - Note: There is no impact to specifications managed by RAN4 for UE requirements - Note: No new UE behaviour is expected

Observation 2 : RAN1 agreement supports reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements, not  for UE RX-TX TD

Under the current Rel-15/16 spec, no impact to specifications managed by RAN4 for UE requirements when Rel-15 SRS-Resource is reused. Only controversial point is about UE RX-TX TD measurement. In fact, as we assume, there is no significant UE impact even if reusing the Rel-15 SRS for UE RX-TX time measurement. Probably impact is found on gNB RX to support gNB TX-RX TD using the Rel-15 SRS.

Currently, Rel-17 NR-IIoT WI is working to measure TA value using the UE RX-TX TD [7]. In this case, there is no need to restrict Rel-15 SRS for this TA measurement purpose toward a serving cell. Although SRS antenna port switching impact on TA value determination is same, the serving cell can control the AP switching at its intention.

Observation 3 : RAN1/4 is studying to measure TA value using the UE RX-TX TD in IIoT WI. In this case, there is no need to restrict Rel-15 SRS for this TA measurement purpose toward a serving cell.

Orignally, a single port of SRS is supposed to use for SRS-positioning, so SRS antenna port switching for MIMO is not considered in Rel-16 SRS-positioning. If using the Rel-15 SRS, then it is questionable how to treat Rel-15 SRS antenna port switching for UE RX-TX TD measurement purpose.

Proposal 1 : Follow the RAN1 agreement and the current spec statement. Rel-15 SRS resource set is not supported for Rel-16/17 UE RXTX TD measurement,

Altanetively, we can consider to allow Rel-15 SRS without antenna port switching to be used for UE RX-TX TD measurement in Rel-17 specification, but we have concern that it may cause fragmentation of UE and gNB implementations.

# Conclusion

This contribution has discussed problems and approaches on SRS antenna port switching impacts. Our observation and proposals are provided as

Observation 1 : If SRS antenna port switching happens during UE RXTX TD or gNB TXRX TD measurement, it impacts on measurement accuracy.

Observation 2 : RAN1 agreement supports reuse of Rel-15 SRS resource set for NR UL RTOA, AoA and gNB RSRP measurements, but not for UE RX-TX TD

Observation 3 : RAN1/4 is studying to measure TA value using the UE RX-TX TD in IIoT WI. In this case, there is no need to restrict Rel-15 SRS for this TA measurement purpose toward a serving cell.

Proposal 1 : Follow the RAN1 agreement and the current spec statement. Rel-15 SRS resource set is not supported for Rel-16/17 UE RXTX TD measurement,

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson

[2] R4-2115365 R4-2120419 WF on NR Positioning Enhancements (Part 1), RAN4#101

[3] R4-2115367 R4-2120377 Email discussion summary for [101-e][235], RAN4#101

[4] R4-2120333 WF on NR Positioning Enhancements (Part 2), RAN1#101

[5] R4-2120378 Email discussion summary for [101-e][236] , RAN1#101

[6] R1-2112744 LS on UL SRS-RSRPP definition, RAN1#107

[7] R4-2201723 Discussion of RRM Requirements for Propagation Delay Compensation Enhancements, Nokia, NSB, RAN4#101b