3GPP TSG-RAN WG4 Meeting#101-bis-e      	R4-2200702

E-meeting, 17 – 25 Jan, 2022

Title:	On RMR900 sensitivity

Source:	Nokia

Agenda item:	5.4.2

Document for:	Approval

# 1	Introduction

This contribution discussed reference sensitivity for RMR900 band.

# 2	Discussion

There  was draftCR [1] endorsed in previous RAN4 meeting which had REFSENS of -100 for RMR900 i.e. n100. However during meeting there was also another proposal which had REFSENS of -97 for n100. Due to usual chaotic e-meeting arrangements it was decided to open the discussion for n100 REFSENS.

When REFSENS for a band is defined it is typically done by looking band plan and it is implications to duplex-filter. One important figure of merit is relative duplex gap which is ratio of duplex gap size to absolute frequency, smaller the value more challenging the filter design is and insertion loss becomes bigger. Table 1 lists bands below 1 GHz in terms of relative duplex gap from most challenging to most easy. We can observe that when we go down the list REFSENS decreases as expected. Bands with -100 dBm REFSENS are three last one and n100 is the easiest one of all.

Observation 1: n100 is easiest sub 1 GHz band in terms of relative duplex gap

Table 1: Relative duplex gap

| Band | Uplink (UL) |  |  | Downlink (DL) |  |  | BW | Duplex | UL-DL | Relative | max | UL-DL sep. | REF SENS 5 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE transmit |  |  | UE receive |  |  |  | Gap | separation | Duplex gap | CH BW | / max CH BW |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  | MHz | MHz | MHz | % | MHz |  | dBm |
| 8 | 880 | – | 915 | 925 | – | 960 | 35 | 10 | 45 | 1.09 | 10 | 4.5 | -97 |
| 26 | 814 | – | 849 | 859 | – | 894 | 35 | 10 | 45 | 1.17 | 15 | 3.0 | -97.5 |
| 20 | 832 | – | 862 | 791 | – | 821 | 30 | 11 | 41 | 1.27 | 20 | 2.1 | -97 |
| 28 | 703 | – | 748 | 758 | – | 803 | 45 | 10 | 55 | 1.33 | 20 | 2.8 | -98.5 |
| 71 | 663 | - | 698 | 617 | - | 652 | 35 | 11 | 46 | 1.56 | 20 | 2.3 | -97.2 |
| 85 | 698 | - | 716 | 728 | - | 746 | 18 | 12 | 30 | 1.66 | 5 | 6.0 | -97 |
| 12 | 699 | – | 716 | 729 | – | 746 | 17 | 13 | 30 | 1.80 | 10 | 3.0 | -97 |
| 5 | 824 | – | 849 | 869 | – | 894 | 25 | 20 | 45 | 2.33 | 10 | 4.5 | -98 |
| 14 | 788 | – | 798 | 758 | – | 768 | 10 | 20 | 30 | 2.48 | 10 | 3.0 | -97 |
| 17 | 704 | – | 716 | 734 | – | 746 | 12 | 18 | 30 | 2.48 | 10 | 3.0 | -97 |
| 13 | 777 | – | 787 | 746 | – | 756 | 10 | 21 | 31 | 2.63 | 10 | 3.1 | -97 |
| 27 | 807 | – | 824 | 852 | – | 869 | 17 | 28 | 45 | 3.34 | 10 | 4.5 | -98 |
| 68 | 698 | - | 728 | 753 | - | 783 | 30 | 25 | 55 | 3.38 | 15 | 3.7 | -98.5 |
| 19 | 830 | – | 845 | 875 | – | 890 | 15 | 30 | 45 | 3.49 | 15 | 3.0 | -100 |
| 18 | 815 | – | 830 | 860 | – | 875 | 15 | 30 | 45 | 3.55 | 15 | 3.0 | -100 |
| 6 | 830 | – | 840 | 875 | – | 885 | 10 | 35 | 45 | 4.08 | 10 | 4.5 | -100 |
| 100 | 874.4 | - | 880 | 919.4 | - | 925 | 5.6 | 39.4 | 45 | 4.38 | 5 | 9.0 |  |

Absolute duplex gap size is also impacting de-sense due to TX noise, RX IM2. Table 2 lists bands in terms of absolute duplex gap from from most challenging to most easy. Like for relative duplex gap three easiest bands all have REFSENS of -100 dBm and n100 is even easier.

Observation 2: n100 is easiest sub 1 GHz band in terms of absolute duplex gap

Table 2: Absolute duplex gap

| Band | Uplink (UL) |  |  | Downlink (DL) |  |  | BW | Duplex | UL-DL | Relative | max | UL-DL sep. | REF SENS 5 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE transmit |  |  | UE receive |  |  |  | Gap | separation | Duplex gap | CH BW | / max CH BW |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  | MHz | MHz | MHz | % | MHz |  | dBm |
| 8 | 880 | – | 915 | 925 | – | 960 | 35 | 10 | 45 | 1.09 | 10 | 4.5 | -97 |
| 26 | 814 | – | 849 | 859 | – | 894 | 35 | 10 | 45 | 1.17 | 15 | 3.0 | -97.5 |
| 28 | 703 | – | 748 | 758 | – | 803 | 45 | 10 | 55 | 1.33 | 20 | 2.8 | -98.5 |
| 20 | 832 | – | 862 | 791 | – | 821 | 30 | 11 | 41 | 1.27 | 20 | 2.1 | -97 |
| 71 | 663 | - | 698 | 617 | - | 652 | 35 | 11 | 46 | 1.56 | 20 | 2.3 | -97.2 |
| 85 | 698 | - | 716 | 728 | - | 746 | 18 | 12 | 30 | 1.66 | 5 | 6.0 | -97 |
| 12 | 699 | – | 716 | 729 | – | 746 | 17 | 13 | 30 | 1.80 | 10 | 3.0 | -97 |
| 17 | 704 | – | 716 | 734 | – | 746 | 12 | 18 | 30 | 2.48 | 10 | 3.0 | -97 |
| 5 | 824 | – | 849 | 869 | – | 894 | 25 | 20 | 45 | 2.33 | 10 | 4.5 | -98 |
| 14 | 788 | – | 798 | 758 | – | 768 | 10 | 20 | 30 | 2.48 | 10 | 3.0 | -97 |
| 13 | 777 | – | 787 | 746 | – | 756 | 10 | 21 | 31 | 2.63 | 10 | 3.1 | -97 |
| 68 | 698 | - | 728 | 753 | - | 783 | 30 | 25 | 55 | 3.38 | 15 | 3.7 | -98.5 |
| 27 | 807 | – | 824 | 852 | – | 869 | 17 | 28 | 45 | 3.34 | 10 | 4.5 | -98 |
| 19 | 830 | – | 845 | 875 | – | 890 | 15 | 30 | 45 | 3.49 | 15 | 3.0 | -100 |
| 18 | 815 | – | 830 | 860 | – | 875 | 15 | 30 | 45 | 3.55 | 15 | 3.0 | -100 |
| 6 | 830 | – | 840 | 875 | – | 885 | 10 | 35 | 45 | 4.08 | 10 | 4.5 | -100 |
| 100 | 874.4 | - | 880 | 919.4 | - | 925 | 5.6 | 39.4 | 45 | 4.38 | 5 | 9.0 |  |

Based on these two observations we made following proposal.

Proposal: n100 REFSENS is kept as -100 dBm

# 3	Conclusion

In this contribution we have made following observations and a proposal.

Observation 1: n100 is easiest sub 1 GHz band in terms of relative duplex gap

Observation 2: n100 is easiest sub 1 GHz band in terms of absolute duplex gap

Proposal: n100 REFSENS is kept as -100 dBm

# 4 	References

[1] R4-2120040, DraftCR 38.101-1: Introduction of 900 MHz to 5G NR for RMR, Nokia, RAN4#101e