3GPP TSG-RAN WG4 Meeting #101-e-bis	R4-2201329

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			1900MHz RMR band – BS RF

Agenda item:		5.5.3

Document for:		Approval

# Introduction

The 1900MHz NR band for European RMR WI ([1]) has been revised in RAN#92e meeting to introduce a new 1900MHz band for Railway in Europe.

Following the discussion from last RAN4#100-e meeting, this contribution is addressing one of the major blocking issue to progress on this WI and makes proposal on how to capture EC Decision(20)02 limits.

# Discussion

## ECC Decision(20)02 analysis

CEPT ([2]) has specified following Least Restrictive Technical Conditions for BS operating in the new 1900MHz RMR band:

- General in-block requirement mandatory for uncoordinated deployment:

- Baseline requirement:

- Receiver characteristics:

It should be noted that ECC Decision(20)02 also specified LRTC for cab-radio equipment, but this is out of scope of this WI ([1]).

It should also be noted that the CEPT studies ([3] and [4]) were only considering a 10MHz channel BW signal, and not a 5 MHz one as it has been requested in this WI ([1]).

## TS 38.104 impacts

### BS maximum output power

In last RAN4#101-e meeting, even if not captured in any WF, it was agreed ([8]) to capture in TS 38.104 that for BS operating in n101, the BS rated output power Prated,c,AC shall not exceed 47 dBm/10MHz.

As 5 MHz channel bandwidth will also be supported, BS rated output power would also need to be capped. We make then following proposal:

Proposal1:  For BS operating in n101, the BS rated output power Prated,c,AC  shall not exceed 44 dBm/5MHz (considering a 18 dBi antenna gain).

### Additional baseline

In last RAN4#101-e meeting, even if not captured in any WF, it was agreed ([8]) to capture the following additional requirement as an additional spurious limit for band n101 in TS 38.104:

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | -61 dBm (NOTE) | 5 MHz |
| NOTE: considering a 18 dBi antenna gain |  |  |

### Blocking requirement

In last RAN4#101-e, it was suggested to have more discussion on the interfering signal.

The main difference between ECC Decision(20)02 and our proposal is that ECC Decision(20)02 has specified a 5 MHz LTE signal as interferer while we are proposing a 5 MHz NR signal.

As the channel BW of the interferer would be 5 MHz and the subcarrier spacing 15kHz, looking at the spectrum utilization, there is no difference between LTE an NR. And considering this new band n101 will be specified in NR specification, we think it’s relevant then to specify the interferer signal as a NR signal and not a LTE signal. We make then following proposal:

Proposal2: Capture the following additional BS blocking requirement for band n101 in TS 38.104:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Centre Frequency of Interfering Signal [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz NR signal |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |  |  |

# Conclusion

In this contribution, we further studied the impacts of introducing the 1900MHz RMR band in TS 38.104, based on ECC Decision (20)02. Even if this was not captured ion any Way Forward, we assumed the following was agreed:

Previous agreement (to be captured in an approved tdoc):

- To convert the CEPT EIRP limits in conducted limits assuming a 18dBi antenna gain and capture those conducted limits in TS 38.104 together with the antenna gain assumption.

- To capture in TS 38.104 that for BS operating in n101, the BS rated output power Prated,c,AC  shall not exceed shall not exceed 47 dBm/10MHz.

- To capture in TS 38.104 the additional spurious limit for band n101 in TS 38.104:

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | -61 dBm (NOTE) | 5 MHz |
| NOTE: considering a 18 dBi antenna gain |  |  |

We made the following observations and proposals:

Proposal1:  For BS operating in n101, the BS rated output power Prated,c,AC  shall not exceed 44 dBm/5MHz (considering a 18 dBi antenna gain).

Proposal2: Capture the following additional BS blocking requirement for band n101 in TS 38.104:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Centre Frequency of Interfering Signal [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz NR signal |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |  |  |

# References

- RP-211542, Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), UIC, Ericsson

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

- ECC Report 314, Co-existence between Future Railway Mobile Communication System (FRMCS) in the frequency range 1900-1920 MHz and other applications in adjacent bands

- ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band

- R4-2118164, 900MHz RMR band – BS RF, Ericsson

- EN 301 908-14, IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS) Release 15 - E-UTRA BS v15.1.1

- EN 301 908-18, IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 18: E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS) Release 15 - 3GPP Rel-15 update including NR

- R4-2120725, Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM, Huawei