3GPP TSG-RAN WG4 Meeting #101-bis-e 	       R4-2201459

# E-meeting, 17th Jan– 25th Feb, 22

Agenda item:	6.5.2.1

Source: 	ZTE Corporation

Title: 	Further discussions on power related requirements for conducted repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of discussion on repeater conducted output power and emission requirements and there were lots of open issues left for further discussions. Therefore in this contribution, we want to share some further considerations on those remaining open issues [1].

Discussion

- 2.1. UL output power levels

Further discuss whether to explicitly define RF requirements in repeater spec to avoid inter-operator interference, some options are listed as below: - It’s left to deploying operators how to avoid interference for UL without power limitation. Candidate solutions include planned deployment, potential antenna gain limit or UL beamwidth limit. - Other options are not excluded.

- The motivation to avoid the inter-operator interference in UL is adorable and understandable. For UL beamwidth and antenna gain related requirement, this should be up to the implementation and it’s difficult to specify the exact values. From our understanding, antenna gain or beamwidth should be as part of declaration for conformance testing. Whether planned or unplanned restriction, this might be impossible to explicitly specify it in the spec which is totally up to the practical deployment.

Proposal 1: antenna gain or UL beamwidth could be part of declaration for conformance testing.

- 2.3. ALC/AGC

Agreement on Nov.4 GTW For ALC core requirements, including below requirements OBUE, ACLR , output power, spurious emission and EVM requirements FFS whether spurious emission and EVM requirements need to be test under ALC test condition which can be further discussed in conformance phase

Regarding whether the spurious emission requirements should be tested under the ALC condition, indeed in the existing LTE base repeater spec, the spurious emission requirement has already been tested with 10dB higher input power, therefore for NR based repeater, we could follow the same approach. For spurious emission requirement, usually it is determined by different RF component e.g. input signal spurious emission (N0), 1st LNA PA noise floor (N1), front-end PA noise floor (N2) and other duplex noise floor (N3), .

Spurious emission@normal ={N0+（G1+G2）}+{N1+（G1+G2）}+{N2+G2}；in dB

Spurious emission@ALC={N0’+（G1-X+G2）}+{N1+（G1-X+G2）}+{N2+G2}；in dB

Where:

- G1 is gain for 1st LNA;

- G2 is gain setting for 2nd front-end PA;

- X is gain reduction for 1st LNA due to 10dB higher input power;

It could be easily found that spurious emission in two cases are potentially different, therefore we think that it’s necessary to verify the spurious emission requirement under the ALC mode;

Similar as spurious emission requirements, we could reuse the similar approach to analyze the EVM performance as following:

Regarding EVM performance should be different

SNR@normal ={S+G1+G2}-{{M0+（G1+G2）}+{N1+（G1+G2）}+{N2+G2}} in dB;

SNR@ALC={S’+G-X+G2}-{{M0’+（G1-X+G2）}+{N1+（G1-X+G2）}+{N2+G2}} in dB;

It could be easily found that EVM performance in two cases are potentially different, therefore we think that it’s necessary to verify the EVM requirement under the ALC mode;

Proposal 2: the spurious emission requirement and EVM requirement should also been tested under the ALC mode;

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: antenna gain or UL beamwidth could be part of declaration for conformance testing;

Proposal 2: the spurious emission requirement and EVM requirement should also been tested under the ALC mode;

References

- R4-2120666, WF on repeater conducted requirements, approved.