3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2201613

Electronic Meeting, January 17 – 25, 2022

Title: 	Discussion on RRM requirements for intra-band con-current V2X operation

Source: 	Huawei, HiSilicon

Agenda item:	6.15.5.1

Document for:	Discussion

Introduction

In RAN4 #101-e meeting, a WF [1] was approved to capture the agreements for R17 V2X enhancements. In this contribution, we provide further discussion on how to define RRM requirements related to new operating scenarios.

Discussion

- NTA-offset and NTA,SL for NR Cell configured as synchronization reference

For intra-band con-current V2X operation, RAN4 has agreed to consider SL NTA_offset = UL NTA_offset for NR cell as synchronization reference source in R17. However, the current UE transmit timing requirements are only applied for NR cell on a non-V2X sidelink carrier and need to be updated in R17 for considering intra-band con-current SL scenarios.

Proposal 1: In R17, the requirements on NTA,SL and NTA-offset for NR cell as synchronization reference need to be updated as follows:

The sidelink transmissions takes place (NTA,SL+NTA-offset)×TC before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where NTA,SL=0. If uplink transmission and sidelink transmission are in the same band, NTA-offset is defined in Table 7.1.2-2, otherwise NTA-offset is defined as 0.

- Scheduling availability requirements for TDM based intra-band con-current SL operation

In R16, the scheduling availability requirements for UE switching between LTE SL and NR SL are defined as follows:

When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in NR slot ‘n’, - UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n’. When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in NR slot ‘n-1’, - UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n-1’. When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in E-UTRA subframe ‘n’, - UE is not expected to transmit or receive on E-UTRA V2X sidelink on the subframe ‘n’. When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in E-UTRA subframe ‘n-1’, - UE is not expected to transmit or receive E-UTRA on V2X sidelink on the subframe ‘n-1’.

In RAN4, it has been agreed to reuse the principles defined in R16 NR V2X for transmission between SL and Uu in R17. Then, the following is proposed:

Proposal 2: The scheduling availability requirements for UE switching between uplink transmission and V2X sidelink transmission can be defined as:

When switch from uplink transmision to V2X sidelink transmision occurs in sidelink slot ‘n’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n’. When switch from V2X sidelink transmision to uplink transmision occurs in sidelink slot ‘n-1’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n-1’. When switch from V2X sidelink transmision to uplink transmision occurs in uplink slot ‘n’, - UE is not expected to transmit uplink or receive downlink on the uplink slot ‘n’. When switch from uplink transmision to V2X sidelink transmision occurs in uplink slot ‘n-1’, - UE is not expected to transmit uplink or receive downlink on the uplink slot ‘n-1’.

- Interruptions SL due to Uu BWP switching

For FDM based intra-band con-current SL operation, RAN4 agreed to define the interruptions on SL due to Uu BWP switch which are similar to the interruptions of NR intra-band CA/DC. In TS38.133, the existing interruption requirements due to active BWP in EN-DC are defined as follows:

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Similarly, the interruption requirements on SL due to Uu BWP switch can be defined as the above table.

Proposal 3: For Rel-17 NR V2X, the interruption requirements on SL due to can be defined as:

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Conclusions

For Rel-17 NR V2X enhancements, this contribution provides our analysis on RRM impacts. The following are provided:

Proposal 1: In R17, the requirements on NTA,SL and NTA-offset for NR cell as synchronization reference need to be updated as follows:

The sidelink transmissions takes place (NTA,SL+NTA-offset)×TC before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where NTA,SL=0. If uplink transmission and sidelink transmission are in the same band, NTA-offset is defined in Table 7.1.2-2, otherwise NTA-offset is defined as 0.

Proposal 2: The scheduling availability requirements for UE switching between uplink transmission and V2X sidelink transmission can be defined as:

When switch from uplink transmision to V2X sidelink transmision occurs in sidelink slot ‘n’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n’. When switch from V2X sidelink transmision to uplink transmision occurs in sidelink slot ‘n-1’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n-1’. When switch from V2X sidelink transmision to uplink transmision occurs in uplink slot ‘n’, - UE is not expected to transmit uplink or receive downlink on the uplink slot ‘n’. When switch from uplink transmision to V2X sidelink transmision occurs in uplink slot ‘n-1’, - UE is not expected to transmit uplink or receive downlink on the uplink slot ‘n-1’.

Proposal 3: For Rel-17 NR V2X, the interruption requirements on SL due to can be defined as:

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Reference

- R4-2120315, “WF on NR SL enhancements RRM requirements”, LG Electronics