3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2203016

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	6.16.4

Source: 	Nokia

Title: 	WF on BS RF Tx requirements

Document for:	Approval

# 1.	Background

Contributions on BS RF TX requirements for 52.6 – 71 GHz in RAN4#101-e have a variety of proposals:

[1]	R4-2200136	Discussion on BS TX RF requirements for 52 6-71GHz	CATT

[2]	R4-2200411	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

[3]	R4-2200843	On BS RF transmitter requirements for the frequency range 52 to 71 GHz	Ericsson

[4]	R4-2201456	Further discussion on BS Tx requirements for 52.6-71GHz	ZTE Corporation

[5]	R4-2201821	Discussion on the remaining BS RF requirements for FR2-2: Tx requirements	Huawei

The way forward from the discussion of the contributions are summarized as follows.

# 2.	Way forward

2.1	TAE

-	Inter-band CA: 3us.

-	Contiguous intra-band CA (baseline assumption): 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. RAN4 will make final decision on February RAN4#102-e meeting.

-	Non-contiguous intra-band CA: [260 ns]. (Note: Whether the requirement need to be specified in Rel-17 specification pending on further confirmation whether NC CA is within Rel-17 scope).

-	MIMO: Option 1 baseline assumption; RAN4 will make final decision on February RAN4#102-e meeting.

-	Option 1: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS.

-	Option 2: 32.5 ns for 480 kHz, 16.25 ns for 960kHz SCS.

2.2	EVM

-	Re-use 50% of the normal CP EVM window length for FR-2-2 for 120 kHz SCS.

-	FFS for 480kHz and 960kHz SCSs; RAN4 will make conclusion on February RAN4#102-e meeting.

2.3	Emissions: OBUE

-	ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

-	FFS for the upper bound of FDL,high – FDL,low.

-	OBUE limits as shown in Table 1 and Table 2 below:

Table 1: OBUE limits (Category A) applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Table 2: OBUE limits (Category B) applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 29 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

2.4	Emissions: Spurious emissions

-	Re-use Cat A and Cat B emission limits, with new step frequencies defined for Cat B.

-	Step frequencies are set as in Table 3 below.

Table 3: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| [Licensed band] | [46] | [61] | [62.5] | [74.5] | [76] | [91] |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |