3GPP TSG-RAN WG4 Meeting #102-e	R4-22xxxxx

Online Meeting, 21 Febuary – 03 March 2022

Third Generation Partnership Project (3GPP™)

DRAFT Meeting Report
for
TSG RAN WG4
meeting: 101-bis-e

Electronic Meeting, Online, 17/01/2022 to 25/01/2022

Report generated on Thursday, 2022-01-13 14:32  UTC

Contents:

1	Opening of the E-meeting	11

2	Approval of the agenda	11

3	Letters / reports from other groups / meeting	11

4	Rel-17 feature list	17

5	Rel-17 spectrum related WIs for NR	17

5.1	Introduction of lower 6GHz NR unlicensed operation for Europe	17

5.1.1	General	17

5.1.2	Band definition and channel arrangement	18

5.1.3	UE RF requirements	18

5.1.4	BS RF requirements	19

5.1.5	Others	20

5.2	Introduction of operation in full unlicensed band 5925-7125MHz for NR	21

5.2.1	General	21

5.2.2	Regulatory requirements and evaluation for re-using existing NS	21

5.2.3	UE RF requirements	21

5.2.4	BS RF requirements	22

5.2.5	Others	22

5.3	Introduction of 6GHz NR licensed bands	23

5.3.1	General	23

5.3.2	System parameters	23

5.3.3	UE RF requirements	25

5.3.4	BS RF requirements	26

5.3.5	Others	29

5.4	Introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio	29

5.4.1	General	29

5.4.2	UE RF requirements	30

5.4.3	BS RF requirements	30

5.4.4	Others	32

5.5	Introduction of 1900 MHz spectrum to 5G NR applicable for Rail Mobile Radio	33

5.5.1	General	33

5.5.2	UE RF requirements	34

5.5.3	BS RF requirements	34

5.5.4	Others	34

5.6	Issues arising from basket WIs but not subject to block approval	35

5.6.1	UE RF requirements	35

5.6.2	NR-U intra-band contiguous UL CA	36

5.7	NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)	37

5.7.1	UE RF requirements for FR1	37

5.7.2	UE RF requirements for FR2	37

5.8	NR inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2)	37

5.8.1	NR inter band CA requirements without any FR2 band(s)	37

5.8.2	NR inter band CA requirements with at least one FR2 band	42

5.9	NR Inter-band Carrier Aggregation for 3 bands DL with 1 band UL	43

5.9.1	UE RF requirements	43

5.10	NR Inter-band Carrier Aggregation for 4 bands DL with 1 band UL	48

5.10.1	UE RF requirements	48

5.11	NR Inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL	52

5.11.1	UE RF requirements	52

5.12	NR inter-band Carrier Aggregation and Dual connectivity for DL 4 bands and 2UL bands	62

5.12.1	UE RF requirements	62

5.13	NR inter-band CA for 5 bands DL with x bands UL (x=1, 2)	66

5.13.1	UE RF requirements	66

5.14	DC of 1 LTE band and 1 NR band	66

5.14.1	EN-DC requirements without FR2 band	66

5.14.2	EN-DC requirements with FR2 band	68

5.15	DC of 2 LTE band and 1 NR band	68

5.15.1	EN-DC requirements without FR2 band	69

5.15.2	EN-DC requirements with FR2 band	71

5.16	DC of 3 LTE band and 1 NR band	71

5.16.1	EN-DC requirements without FR2 band	71

5.16.2	EN-DC requirements with FR2 band	75

5.17	DC of 4 LTE band and 1 NR band	76

5.17.1	EN-DC requirements without FR2 band	76

5.17.2	EN-DC requirements with FR2 band	77

5.18	DC of 5 bands LTE inter-band CA (5DL/1L) and 1 NR band (1DL/1UL)	78

5.18.1	UE RF requirements	78

5.19	DC of x bands (x=1,2, 3, 4) LTE inter-band CA and 2 bands NR inter-band CA	78

5.19.1	EN-DC requirements including NR inter CA without FR2 band	79

5.19.2	EN-DC requirements including NR inter CA with FR2 band	90

5.20	DC of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA	91

5.20.1	UE RF requirements	91

5.21	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)	91

5.21.1	UE RF requirements	91

5.22	DC of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL)	92

5.22.1	UE RF requirements	92

5.23	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 4 bands NR inter-band CA (4DL/1UL)	92

5.23.1	UE RF requirements	92

5.24	Band combinations for SA NR supplementary uplink (SUL) NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)	92

5.24.1	UE RF requirements	92

5.25	Band combinations for Uu and V2X con-current operation	92

5.25.1	UE RF requirements	92

5.26	Adding channel bandwidth support to existing NR bands	93

5.26.1	UE RF requirements	93

5.26.1.1	Addition of bandwidth and Tx/Rx requirements	94

5.26.1.2	NR-U 100MHz bandwidth	94

5.26.2	BS RF requirements	94

5.27	Introduction of bandwidth combination set 4 (BCS4) for NR	94

5.27.1	UE RF requirements for BCS4/BCS5	94

5.27.2	Discussion of LS on NR CA capability for BCS5 (R2-1209073)	96

5.28	Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths	96

5.28.1	UE RF requirements	96

5.29	High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, Band 13, Band n5, Band n13, and Band n71	97

5.29.1	General	97

5.29.2	Feasibility study	97

5.29.2.1	Coexistence study between B5 and adjacent bands	97

5.29.2.2	Coexistence study between B13/n13 and adjacent bands	97

5.29.2.3	Filter with smaller duplex for B13, n13 and n71	97

5.29.2.4	PA related to MPR and A-MPR for B13, n13, and n71	97

5.29.3	UE RF requirements	97

5.29.3.1	UE REFSENS	97

5.29.3.2	UE Tx requirements (MOP, MPR, A-MPR, and ACLR)	97

5.30	High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink	98

5.30.1	UE RF requirements	98

5.31	High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band	100

5.31.1	UE RF requirements	100

5.32	Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL	100

5.32.1	UE RF requirements	100

5.33	Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)	100

5.33.1	UE RF requirements	100

5.34	High power UE for NR TDD intra-band carrier aggregation in frequency range FR1	102

5.34.1	UE RF requirements	102

5.35	Increasing UE power high limit for CA and DC	102

5.35.1	General and work plan	102

5.35.2	Feasibility and impact study	102

5.35.3	UE RF requirements	103

5.36	High power UE (power class 2) for NR FDD band	104

5.36.1	General and work plan	104

5.36.2	UE RF requirements	104

5.36.2.1	UE maximum output power and power tolerance	104

5.36.2.2	A-MPR requirements	104

5.36.2.3	PC2 MSD requirements (investigation for HD-FDD)	105

5.37	Additional NR bands for UL-MIMO	105

5.37.1	UE RF requirements	105

5.38	Downlink interruption for band combinations to conduct dynamic Tx Switching	106

5.38.1	Determination of inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed	106

5.39	Simultaneous Rx/Tx band combinations for CA, SUL, MR-DC and NR-DC	106

5.39.1	MSD threshold principle	106

5.39.2	FR2 band combinations with simultaneous Rx/Tx	107

5.40	4Rx support for NR band n8	108

5.40.1	UE RF requirements (delta_R_IB,4Rx)	108

5.40.2	Release independency	108

6	Rel-17 non-spectrum related work items for NR	108

6.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs	108

6.1.1	General	108

6.1.2	Performance requirements	109

6.1.2.1	Performance Requirements for FR1	109

6.1.2.2	Performance Requirements for FR2	110

6.1.2.3	MU assessment for FR1 and FR2	110

6.1.3	Testing methodologies	110

6.1.3.1	Testing parameters for Performance	110

6.1.3.2	Optimization of test methodologies	111

6.1.3.3	Channel model validation	111

6.2	Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)	112

6.2.1	General and work plan	112

6.2.2	Test methodology	113

6.2.2.1	SA test methodology	114

6.2.2.2	EN-DC test methodology	114

6.2.2.3	UE with multiple antennas test methodology	115

6.2.2.4	Test time reduction	115

6.2.3	Performance requirements	115

6.2.3.1	Framework for lab alignment and requirements	115

6.2.3.2	SA requirements	116

6.2.3.3	EN-DC requirements	116

6.3	RF requirements enhancement for NR frequency range 1 (FR1)	116

6.3.1	General	116

6.3.2	RF core requirements	117

6.3.2.1	UL MIMO configuration for SUL band configurations	117

6.3.2.2	HPUE for TDD intra-band contiguous UL CA	117

6.3.2.3	HPUE for TDD intra-band non-contiguous UL CA	118

6.3.2.4	Intra-band UL contiguous CA for UL MIMO (n41C and n78C)	119

6.3.2.5	Solution preventing transmission power dropping on cell with lower priority	120

6.3.2.5.1	FR1 related	120

6.3.2.5.2	FR2 related	121

6.3.3	RRM core requirements	121

6.3.4	RRM performance requirements	121

6.4	NR RF requirement enhancements for frequency range 2 (FR2)	121

6.4.1	General	121

6.4.2	UE RF requirements for inter-band CA	122

6.4.2.1	Inter-band DL CA requirements	122

6.4.2.1.1	CA configurations within the same frequency group based on CBM	122

6.4.2.1.2	CA configurations between different frequency groups based on CBM	123

6.4.2.1.3	Feasibility study for DL inter-band CA for IBM within the same frequency group	125

6.4.2.1.4	Rx beam switch value	125

6.4.2.2	Inter-band UL CA requirements	125

6.4.2.2.1	Inter-band UL CA for two bands	125

6.4.2.2.2	CA configuration CA_n257A-n259A based on IBM	126

6.4.3	UL gaps for self-calibration and monitoring	127

6.4.3.1	UE Tx power management	127

6.4.3.2	Coherent UL-MIMO	128

6.4.4	DC location for intra-band UL CA with > 2 CCs for both FR2 and FR1	128

6.4.5	CA BW classes	129

6.4.5.1	New FR2 CA BW classes	129

6.4.5.2	Fallback group	130

6.4.6	RRM core requirements	130

6.4.6.1	Inter-band DL CA requirements for CBM	130

6.4.6.1.1	MRTD requirements	130

6.4.6.1.2	Other RRM requirements	132

6.4.6.2	Inter-band UL CA for IBM	134

6.4.6.3	UL gaps for self-calibration and monitoring	134

6.5	NR repeater	135

6.5.1	General	135

6.5.1.1	System parameters	135

6.5.1.2	Repeater Class/Type	136

6.5.1.3	TDD repeater switching requirements	136

6.5.1.4	Others	137

6.5.2	Conductive RF core requirements	137

6.5.2.1	Transmitted power related requirements	138

6.5.2.2	Emission requirements	138

6.5.2.3	Others	139

6.5.3	Radiated RF core requirements	140

6.5.3.1	Transmitted power related requirements	141

6.5.3.2	Emission requirements	141

6.5.3.3	Others	142

6.5.4	EMC core requirements	143

6.6	Introduction of DL 1024QAM for NR FR1	144

6.6.1	General	144

6.6.2	UE RF requirements maintenance	144

6.6.3	BS TX RF requirements maintenance	144

6.6.4	BS RF conformance testing	144

6.6.5	Demodulation and CSI requirements	145

6.6.5.1	General	145

6.6.5.2	PDSCH requirements	145

6.6.5.3	SDR requirements	146

6.6.5.4	CQI requirements	146

6.7	UE RF requirements for Transparent Tx Diversity (TxD) for NR	147

6.7.1	General	147

6.7.2	UE RF requirements for phase 1 (38.101-1)	147

6.7.2.1	UL MIMO requirement for TxD except ULFPTx	147

6.7.3	UE RF requirements for phase 2 (38.101-1)	148

6.7.3.1	SRS antenna switching related	148

6.7.3.2	ULFPTx related	149

6.8	Enhancement for NR high speed train scenario in FR1	151

6.8.1	General	151

6.8.2	RRM core requirements	151

6.8.2.1	Intra-frequency measurements	151

6.8.2.2	Inter-frequency measurements	151

6.8.2.3	L1-SINR measurements	152

6.8.2.4	Others	153

6.8.3	UE demodulation requirements (38.101-4)	154

6.8.3.1	General	154

6.8.3.2	PDSCH requirements for CA scenarios	154

6.9	NR support for high speed train scenario in FR2	156

6.9.1	General	156

6.9.2	High speed train deployment scenario in FR2	156

6.9.3	UE RF core requirements	156

6.9.3.1	UE Tx requirements	156

6.9.3.1.1	UE RF framework and power class	157

6.9.3.1.2	Spherical coverage requirements	157

6.9.3.1.3	Beam correspondence	157

6.9.3.2	UE Rx requirements	158

6.9.4	RRM core requirements	158

6.9.4.1	General	158

6.9.4.2	Number of RX beams	160

6.9.4.3	RRC Idle/Inactive and connected state mobility requirements	160

6.9.4.4	Timing requirements	161

6.9.4.5	Signalling characteristics requirements	162

6.9.4.6	Measurement procedure requirements	164

6.9.5	Demodulation requirements	164

6.9.5.1	General	164

6.9.5.2	UE demodulation requirements	165

6.9.5.2.1	PDSCH requirements under Uni-directional scenario	165

6.9.5.2.2	PDSCH requirements under Bi-directional scenario	165

6.9.5.3	BS demodulation requirements	166

6.9.5.3.1	PUSCH requirements	166

6.9.5.3.2	PUSCH with UL timing adjustment requirements	167

6.9.5.3.3	PRACH requirements	168

6.10	Further RRM enhancement for NR and MR-DC	169

6.10.1	General	169

6.10.2	RRM core requirements	169

6.10.2.1	SRS antenna port switching	169

6.10.2.2	HO with PSCell	171

6.10.2.3	PUCCH SCell activation/deactivation	173

6.11	NR and MR-DC measurement gap enhancements	176

6.11.1	General	176

6.11.2	RRM core requirements	176

6.11.2.1	Pre-configured MG pattern(s)	176

6.11.2.2	Multiple concurrent and independent MG patterns	179

6.11.2.3	Network Controlled Small Gap	182

6.12	Further enhancement on NR demodulation performance	185

6.12.1	General	185

6.12.2	UE demodulation and CSI requirements	186

6.12.2.1	MMSE-IRC receiver for inter-cell interference	186

6.12.2.1.1	PDSCH requirements	186

6.12.2.1.2	CQI requirements	188

6.12.2.2	MMSE-IRC receiver for intra-cell inter-user interference	190

6.12.2.3	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR	192

6.12.2.3.1	General	192

6.12.2.3.2	Necessity of Network assistant signaling	193

6.12.2.3.3	Test set-up	195

6.12.3	BS demodulation requirements	196

6.12.3.1	PUSCH demodulation requirements for FR1 256QAM	196

6.13	Solutions for NR to support non-terrestrial networks (NTN)	198

6.13.1	General	198

6.13.1.1	System parameters	198

6.13.1.2	NTN Satellite Access Node Class/Type	199

6.13.1.3	Regulatory information	199

6.13.1.4	Others	200

6.13.2	Coexistence aspects	201

6.13.2.1	NTN coexistence scenarios and simulations	201

6.13.2.2	HAPS coexistence scenarios and simulations	203

6.13.2.3	ACLR/ACS proposals	203

6.13.3	Satellite Access Node RF requirements	204

6.13.3.1	TX requirements for radiated characteristics	204

6.13.3.2	RX requirements for radiated characteristics	205

6.13.3.3	Tx requirements for conducted characteristics	206

6.13.3.4	Rx requirements for conducted characteristics	207

6.13.4	UE RF requirements	207

6.13.4.1	TX requirements	207

6.13.4.2	RX requirements	208

6.13.5	RRM core requirements	209

6.13.5.1	General	209

6.13.5.2	GNSS-related requirements	211

6.13.5.3	Mobility requirements	211

6.13.5.4	Timing requirements	212

6.13.5.5	Measurement procedure requirements	214

6.13.6	Demodulation requirements	216

6.13.6.1	General	216

6.13.6.2	Satellite Access Node demodulation requirements	216

6.13.6.3	UE demodulation requirements	216

6.14	UE Power Saving Enhancements for NR	217

6.14.1	General	217

6.14.2	RRM core requirements	217

6.14.2.1	UE measurements relaxation for RLM and/or BFD	217

6.15	NR Sidelink enhancement	220

6.15.1	General	220

6.15.2	UE RF requirements for NR SL enhancement	220

6.15.2.1	Configured Tx power requirements	221

6.15.2.2	REFSENS requirements	221

6.15.2.3	Other RF requirements	221

6.15.3	Intra-band con-current operation between NR SUL and NR Uu	222

6.15.3.1	RF requirements for intra-band V2X con-current (including MPR)	222

6.15.3.2	Synchronous operation between SL and Uu (including switching time mask, SL transmission timing)	224

6.15.4	High power UE(PC2) for SL	224

6.15.4.1	TX requirements (Power class)	224

6.15.4.2	Coexistence study	225

6.15.4.3	Others	225

6.15.5	RRM core requirements	225

6.15.5.1	Intra-band con-current V2X operation	225

6.15.5.2	SL-DRX	226

6.15.5.3	Others	227

6.16	Extending current NR operation to 71GHz	228

6.16.1	General	228

6.16.2	Operation bands and system parameters (channelization, raster, CBW, etc)	229

6.16.3	UE RF requirements	231

6.16.3.1	TX requirements	231

6.16.3.2	RX requirements	233

6.16.4	BS RF requirements	234

6.16.4.1	TX requirements	234

6.16.4.2	RX requirements	235

6.16.5	Co-existence simulations	236

6.16.6	FR1+FR2-2 DC/CA band combinations	237

6.16.7	RRM core requirements	238

6.16.7.1	General	238

6.16.7.2	Timing requirements	240

6.16.7.3	Interruption requirements	241

6.16.7.4	Active BWP switching delay requirements	242

6.16.7.5	Measurement gap interruption requirements	243

6.16.7.6	LBT impacts on RRM requirements	244

6.16.8	Others	245

6.17	Enhancements to Integrated Access and Backhaul (IAB) for NR	246

6.17.1	General	246

6.17.2	RF requirements	246

6.17.2.1	Impact for Simultaneous operation of IAB child and parent links	246

6.17.2.2	Impact for Timing enhancement	246

6.17.2.3	Others	247

6.17.3	RRM core requirements	247

6.17.4	Others	248

6.18	NR coverage enhancements	248

6.18.1	General and CR structure	248

6.18.2	UE RF requirements	249

6.18.2.1	Requirements for non-scheduled gap	249

6.18.2.2	Tolerance for power consistency/phase continuity	249

6.18.2.3	Maximum duration for joint channel estimation	250

6.18.2.4	Others	251

6.18.3	BS demodulation requirements	251

6.19	Further enhancements on MIMO for NR	252

6.19.1	General	252

6.19.2	UE RF requirements	253

6.19.2.1	Additional requirement for multi-panel reception	253

6.19.2.2	Impact of MPE enhancements	253

6.19.2.3	SRS related impact	254

6.19.3	RRM core requirements	254

6.19.3.1	Unified TCI for DL and UL	254

6.19.3.2	Inter-cell beam management	255

6.19.3.3	Others	256

6.19.4	UE Demodulation and CSI requirements	257

6.20	Support of reduced capability NR devices	259

6.20.1	General	259

6.20.2	UE RF requirements	259

6.20.2.1	FR1	259

6.20.2.1.1	Tx requirements (power class)	259

6.20.2.1.2	Rx requirements (REFSENS, etc)	260

6.20.2.2	FR2	261

6.20.2.2.1	Tx requirements (power class, UE type)	261

6.20.2.2.2	Rx requirements	262

6.20.2.3	Others	262

6.20.3	RRM core requirements	263

6.20.3.1	Impacts from UE complexity reduction	263

6.20.3.1.1	General	263

6.20.3.1.2	Mobility requirements	264

6.20.3.1.3	Timing requirements	265

6.20.3.1.4	Signalling characteristics	266

6.20.3.1.5	Measurement procedure	267

6.20.3.2	Extended DRX enhancements	269

6.20.3.3	RRM measurement relaxations	270

6.20.3.4	Others	272

6.20.4	UE demodulation and CSI requirements	273

6.21	Positioning enhancements for NR	274

6.21.1	General	274

6.21.2	RRM core requirements	275

6.21.2.1	UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation	275

6.21.2.2	Latency reduction of positioning measurement	276

6.21.2.3	Measurement in RRC_INACTIVE state	279

6.21.2.4	Impact on existing UE positioning and RRM requirements	280

6.21.2.5	Enhancements of A-GNSS positioning	282

6.21.2.6	Others	282

6.22	Multi-Radio Dual-Connectivity enhancements	283

6.22.1	General	283

6.22.2	RRM core requirements	283

6.22.2.1	Efficient activation/de-activation mechanism for SCells	283

6.22.2.2	Efficient activation/de-activation mechanism for one SCG	284

6.22.2.3	Conditional PSCell change and addition	286

6.22.2.4	Others	287

6.23	Enhanced IIoT and URLLC support	288

6.23.1	General	288

6.23.2	RRM core requirements	288

6.23.2.1	Propagation delay compensation enhancements	288

6.23.2.2	Reference point for Te requirements	289

6.23.2.3	Others	290

6.24	NR Sidelink Relay	290

6.24.1	General	290

6.24.2	RRM core requirements	290

6.25	NR small data transmissions in INACTIVE state	291

6.25.1	General and work plan	291

6.25.2	RRM core requirements	291

6.26	Support for Multi-SIM devices for LTE/NR	293

6.26.1	General and work plan	293

6.26.2	RRM core requirements	293

7	Rel-17 Study Items for NR	295

7.1	Study on enhanced test methods for FR2 in NR	295

7.1.1	Maintenance on objectives 1~6	295

7.1.2	OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz	295

7.1.2.1	General	295

7.1.2.1.1	Test system assumption	295

7.1.2.1.2	UE types	295

7.1.2.1.3	MU assessment	296

7.1.2.1.4	Others	296

7.1.2.2	Test methodology for UE RF	296

7.1.2.3	Test methodology for RRM	296

7.1.2.4	Test methodology for UE demodulation and CSI	296

7.2	Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths	296

7.2.1	General and TR	296

7.2.2	Evaluation of use of larger channel bandwidths than licensed bandwidth	297

7.2.2.1	Channel filter assumptions and RB blanking with impacts on UE (ACS, blocking)	297

7.2.2.2	Signaling and configuration (RAN1/RAN2 impacts) aspects	297

7.2.2.3	Other aspects such as detailed solution, complexity, legacy UE, etc	298

7.2.3	Evaluation of use of overlapping UE channel bandwidths	298

7.2.3.1	Overlapping CBWs from network perspective	298

7.2.3.2	Combined UE CBWs (one cell)	299

7.2.3.2.1	Signaling and configuration (RAN1/RAN2 impacts) aspects	299

7.2.3.2.2	Other aspects such as detailed solution, complexity, legacy UE, etc	299

7.2.3.3	Overlapping CA (two cells)	299

7.2.3.3.1	Signaling and configuration (RAN1/RAN2 impacts) aspects	299

7.2.3.3.2	Other aspects such as detailed solution, complexity, legacy UE, etc	300

7.2.3.4	Overall method comparisons	300

7.3	Study on band combination handling in RAN4	301

7.3.1	General and TR	301

7.3.2	Information of rules and guidelines of specifying band combinations (TP format, notation, band configurations, BCS)	301

7.3.2.1.1	Signaling and configuration (RAN1/RAN2 impacts) aspects	302

7.3.2.1.2	Other aspects such as detailed solution, complexity, legacy UE, etc	302

7.3.3	Improving RAN4 specification structures and reducing redundant contents	302

7.3.3.1	Optimization of delta TIB and delta RIB	302

7.3.3.2	Optimizations for other redundancy	303

7.4	Optimizations of pi/2 BPSK uplink power in NR	303

7.4.1	General and TR	303

7.4.2	UE Tx power and related issues	303

7.4.3	Evaluation of filter requirements applicable to identified new UE power capability	304

7.4.4	Link level simulations	304

7.4.5	SAR analysis	305

7.4.6	Identify RAN4 requirements	305

8	Rel-17 Work Items for LTE	305

8.1	LTE inter-band Carrier Aggregation for 2 bands DL with 1 band UL	305

8.1.1	UE RF with harmonic, close proximity and isolation issues	305

8.1.2	UE RF without specific issues	305

8.2	LTE inter-band Carrier Aggregation for 3 bands DL with 1 band UL	306

8.2.1	UE RF with harmonic, close proximity and isolation issues	306

8.2.2	UE RF without specific issues	306

8.3	LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL	307

8.3.1	UE RF with 4 LTE bands CA	307

8.3.2	UE RF with 5 LTE bands CA	307

8.4	LTE inter-band Carrier Aggregation for 2 bands DL with 2 band UL	307

8.4.1	UE RF with harmonic, close proximity and isolation issues	307

8.4.2	UE RF without specific issues	307

8.5	LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 band UL	307

8.5.1	UE RF with MSD	308

8.5.2	UE RF without MSD	308

8.6	RRM for LTE CA basket WIs	308

8.6.1	RRM Core (36.133)	308

8.6.2	RRM Perf (36.133)	308

8.7	New WID on Additional LTE bands for UE category M1&M2 and/or NB1&NB2 in Rel-17	308

8.7.1	RF requirements	308

8.7.2	Others	308

8.8	Upper 700MHz A Block new E-UTRA band in US	309

8.8.1	General	309

8.8.2	Study for co-existence requirements	310

8.8.3	UE RF requirements	310

8.8.4	BS RF requirements	310

8.8.5	Others	311

8.9	Additional enhancements for NB-IoT and LTE-MTC	311

8.9.1	General and work plan	311

8.9.2	Support of 16QAM in NB-IoT	311

8.9.2.1	BS RF requirements	311

8.9.2.2	UE RF requirements	312

8.9.3	Support of power reduction for PRACH, PUCCH, and full-PRB PUSCH in MTC	312

8.9.3.1	UE RF requirements	312

8.9.4	RRM core requirements	312

8.9.4.1	Neighbour cell measurement in RRC Connected state for NB-IoT	312

8.9.5	Others	313

8.9.6	Demodulation requirements	313

8.9.6.1	General	313

8.9.6.2	Demodulation requirements for NB-IoT	313

8.9.6.2.1	UE demodulation requirements	313

8.9.6.2.2	BS demodulation requirements	314

8.9.6.3	Demodulation requirements for MTC	314

9	Liaison and output to other groups	315

9.1	R17 related	315

9.1.1	LS reply for beam correspondence with SDT in RRC_INACTIVE	315

9.1.2	RAN5 response LS on LTE REFSENS exception simplification (R5-215803)	315

9.1.3	Others	316

10	Any other business	317

11	Close of the E-meeting	320

## 1	Opening of the E-meeting

The Chairman Xizeng Dai (Huawei) opened the meeting on RAN4 reflector on 17/01/2022.

Intellectual Property Rights Declaration Policy

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

Statement regarding competition law

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chairman and Vice-Chairmen and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG meetings was important to allow for full and fair consideration of such matters.

Meeting arrangements

The meeting was conducted on three parallel sessions; Main session, RRM session and BS RF Test Demod session. The Main session was chaired by RAN4 Chair Xizeng Dai (Huawei), RRM session was chaired by RAN4 Vice Chair Andrey Chervyakov (Intel) and BS RF Test Demod session was chaired by RAN4 ViceChair Haijie Qiu (Samsung). The sessions were further broken down into separate email threads to address specific technical topics lead by assigned discussion moderators. Webinar sessions were used to summarize progress, resolve controversial issues and decide way forward.

## 2	Approval of the agenda

R4-2200001	RAN4#101-e Meeting Report

Type: report		For: Approval
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2200002	Agenda for RAN4#101-bis-e

Type: agenda		For: Approval
					Source: RAN4 Chair (Huawei)

Decision: 		The document was not treated.

R4-2200003	RAN4#101-bis-e E-Meeting Arrangements and Guidelines

Type: other		For: Approval
					Source: RAN4 Chair (Huawei)

Decision: 		The document was not treated.

## 3	Letters / reports from other groups / meeting

R4-2200005	LS on Energy Efficiency as guiding principle for new solutions	SA

R4-2200006	Reply LS on inclusion of the 6425-7125 MHz frequency band in the 3GPP specification for 		5G-NR/IMT-2000 systems	RAN	Response to: (RP-213605)

R4-2200007	LS on updates to study on optimizations of pi/2 BPSK uplink power in NR	RAN

R4-2200008	Liaison statement to 3GPP TSG RAN on the inclusion of the 6425-7125 MHz frequency band 		in the 3GPP specification for 5G-NR / 1MT-2020 systems	RCC Commission on Spectrum 		and Satellite Orbits

R4-2200009	Reply LS to RP-211577 on Test methods for over-the-air Total Radiated Power (TRP) field 		measurements of unwanted emissions from IMT radio equipment utilizing active antennas		ITU-R WP1C	Response to: (RP-211577)

R4-2200010	LS on work towards two new recommendations "Generic unwanted emission characteristics 		of base / mobile stations using the terrestrial radio interfaces of IMT-2020"	RAN5			Response to: (R5-216419)

R4-2200011	LS on configuration of p-MaxEUTRA and p-NR-FR1	RAN5

R4-2200012	Reply to LS (in R4-2120025) on FR2 UE relative power control tolerance requirements			RAN5	Response to: (R4-2120025)

R4-2200013	Reply LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility to RAN3	RAN3			Response to: (R1-2108527)

R4-2200014	LS on TRS-based SCell activation details	RAN2

R4-2200015	LS response on PC5 DRX for ProSe	RAN2	Response to: (R2-2111237)

R4-2200016	Reply LS on signalling for intra-band CA with UL-MIMO	RAN2					Response to: (R4-2114754)

R4-2200026	Reply LS on R17 NR MG enhancements – Concurrent MG	RAN2					Response to: (R4-2115343)

R4-2200027	Response LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#105-e			RAN2	Response to: (R1-2108427)

R4-2200028	Reply LS on the use of NCD-SSB instead of CD-SSB for RedCap UEs	RAN2				Response to: (R2-2110727)

R4-2200029	Reply LS to RAN4 on UL gap in FR2 RF enhancement	RAN2					Response to: (R4-2114965)

R4-2200030	LS to RAN4 on L3 filter configuration	RAN2

R4-2200031	Reply LS on specification impact for methods on efficient utilization of licensed spectrum that 		is not aligned with existing NR channel bandwidths	RAN2

R4-2200032	LS on MPE information signalling	RAN2

R4-2200033	Reply LS on Pre-configured MG	RAN2	Response to: (R4-2115438)

R4-2200034	LS on the minimum time gap for wake-up and Scell dormancy indication for NR operation in 		52.6 to 71 GHz	RAN1

R4-2200035	Follow-up reply LS on inter-cell beam management and multi-TRP in Rel-17	RAN1			Response to: (R2-2108925)

R4-2200036	LS on DL PRS processing by UEs in RRC_INACTIVE state	RAN1

R4-2200037	LS on UL SRS-RSRPP definition	RAN1

R4-2200038	Reply LS on capability related RAN2 agreements for RedCap	RAN1					Response to: (R1-2108714)

R4-2200040	LS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap		RAN1

R4-2200041	LS on lower Rx beam sweeping factor for latency improvement	RAN1

R4-2200042	LS on DL synchronization enhancements for IoT NTN	RAN1

R4-2200043	LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#107-e	RAN1

R4-2200044	LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE	RAN1

R4-2200045	LS on sensing beam selection	RAN1

R4-2200046	LS on BFR for CORESET with two activated TCI states	RAN1

R4-2200047	LS on propagation delay compensation	RAN1

R4-2200048	LS on Rel-17 MAC-CE impacts	RAN1

R4-2200049	Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure			RAN1	Response to: (R4-2115339)

R4-2200050	Reply LS on initial state of elements controlled by MAC CEs	RAN1					Response to: (R1-2110756)

R4-2200051	LS on the condition of PRS measurement outside the MG	RAN1

R4-2200052	LS on updated Rel-17 RAN1 UE features list for LTE	RAN1

R4-2200053	LS on updated Rel-17 RAN1 UE features list for NR		RAN1

R4-2200054	LS on the reporting of the Tx TEG association information	RAN1

R4-2200055	LS on channel quality reporting for NB-IoT		RAN1

R4-2200056	LS on range of power control parameters for eIAB	RAN1

R4-2200057	LS on updated Rel-17 LTE and NR higher-layers parameter list	RAN1

R4-2200058	LS on triggering signalling of temporary RS for SCell activation	RAN1

R4-2200005	LS on Energy Efficiency as guiding principle for new solutions (SA)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, CT, SA1, SA2, SA3, SA4, SA5, SA6, RAN1, RAN2, RAN3, RAN4, RAN5, CT1, CT3, CT4, CT6, cc -

Decision: 		The document was not treated.

R4-2200006	Reply LS on inclusion of the 6425-7125 MHz frequency band in the 3GPP specification for 5G-NR/IMT-2000 systems (RAN)

Type: LS in		For: Information
					Original outgoing LS: -, to RCC Commission on Spectrum and Satellite Orbits, cc RAN4
Decision: 		The document was not treated.

R4-2200007	LS on updates to study on optimizations of pi/2 BPSK uplink power in NR (RAN)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -
Decision: 		The document was not treated.

R4-2200008	Liaison statement to 3GPP TSG RAN on the inclusion of the 6425-7125 MHz frequency band in the 3GPP specification for 5G-NR / 1MT-2020 systems (RCC Commission on Spectrum and Satellite Orbits)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, cc RAN4
Decision: 		The document was not treated.

R4-2200009	Reply LS to RP-211577 on Test methods for over-the-air Total Radiated Power (TRP) field measurements of unwanted emissions from IMT radio equipment utilizing active antennas (ITU-R WPIC)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, cc ITU-R WP5D, RAN4

Decision: 		The document was not treated.

R4-2200010	LS on work towards two new recommendations "Generic unwanted emission characteristics of base / mobile stations using the terrestrial radio interfaces of IMT-2020 (RAN5)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, cc RAN4

Decision: 		The document was not treated.

R4-2200011	LS on configuration of p-MaxEUTRA and p-NR-FR1 (RAN5)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN1, RAN2, RAN4, cc -

Decision: 		The document was not treated.

R4-2200012	Reply to LS (in R4-2120025) on FR2 UE relative power control tolerance requirements (RAN5)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200013	Reply LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility to RAN3 (RAN3)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, RAN1, RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200014	LS on TRS-based SCell activation details (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN1, RAN4, cc -

Decision: 		The document was not treated.

R4-2200015	LS response on PC5 DRX for ProSe (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to SA2, cc CT1, RAN1, RAN4

Decision: 		The document was not treated.

R4-2200016	Reply LS on signalling for intra-band CA with UL-MIMO (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200026	Reply LS on R17 NR MG enhancements – Concurrent MG (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN1

Decision: 		The document was not treated.

R4-2200027	Response LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#105-e (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN1, cc RAN4

Decision: 		The document was not treated.

R4-2200028	Reply LS on the use of NCD-SSB instead of CD-SSB for RedCap Ues (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN1, cc RAN4

Decision: 		The document was not treated.

R4-2200029	Reply LS to RAN4 on UL gap in FR2 RF enhancement (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200030	LS to RAN4 on L3 filter configuration (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200031	Reply LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN1

Decision: 		The document was not treated.

R4-2200032	LS on MPE information signalling (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN1, cc RAN4

Decision: 		The document was not treated.

R4-2200033	Reply LS on Pre-configured MG (RAN2)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200034	LS on the minimum time gap for wake-up and Scell dormancy indication for NR operation in 52.6 to 71 GHz (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200035	Follow-up reply LS on inter-cell beam management and multi-TRP in Rel-17 (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200036	LS on DL PRS processing by UEs in RRC_INACTIVE state (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200037	LS on UL SRS-RSRPP definition (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200038	Reply LS on capability related RAN2 agreements for RedCap (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200040	LS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN2

Decision: 		The document was not treated.

R4-2200041	LS on lower Rx beam sweeping factor for latency improvement (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200042	LS on DL synchronization enhancements for IoT NTN (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200043	LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#107-e (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200044	LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200045	LS on sensing beam selection (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200046	LS on BFR for CORESET with two activated TCI states (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN2

Decision: 		The document was not treated.

R4-2200047	LS on propagation delay compensation (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200048	LS on Rel-17 MAC-CE impacts (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200049	Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN2

Decision: 		The document was not treated.

R4-2200050	Reply LS on initial state of elements controlled by MAC Ces (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200051	LS on the condition of PRS measurement outside the MG (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -

Decision: 		The document was not treated.

R4-2200052	LS on updated Rel-17 RAN1 UE features list for LTE (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200053	LS on updated Rel-17 RAN1 UE features list for NR (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

R4-2200054	LS on the reporting of the Tx TEG association information (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, RAN4, cc RAN3

Decision: 		The document was not treated.

R4-2200055	LS on channel quality reporting for NB-IoT (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, RAN4, cc -

Decision: 		The document was not treated.

R4-2200056	LS on range of power control parameters for eIAB (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN2

Decision: 		The document was not treated.

R4-2200057	LS on updated Rel-17 LTE and NR higher-layers parameter list (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, RAN3, cc RAN4

Decision: 		The document was not treated.

R4-2200058	LS on triggering signalling of temporary RS for SCell activation (RAN1)

Type: LS in		For: Information
					Original outgoing LS: -, to RAN2, cc RAN4

Decision: 		The document was not treated.

## 4	Rel-17 feature list

R4-2200286	Initial views on R17 feature list

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200485	Inputs to Rel-17 NR UE features for measurement gap enhancement

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200544	Discussion on Rel-17 RAN4 UE feature list

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200641	Discussion on UE capability for FR1 HST enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2201446	Initial review of RAN4 UE feature in Rel-17

Type: other		For: Decision
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201803	A new Rel-17 per-FR MG capability based on Per BC

Type: discussion		For: Decision
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

## 5	Rel-17 spectrum related WIs for NR

### 5.1	Introduction of lower 6GHz NR unlicensed operation for Europe

#### 5.1.1	General

R4-2200429	Overview of the Region 1 countries implementing lower 6GHz unlicensed band

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2201080	draft TR 38.849 v0.6.0

Type: draft TR		For: Agreement
					38.849 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Inclusion of agreements and TPs provided at RAN4#101bis

Decision: 		The document was not treated.

#### 5.1.2	Band definition and channel arrangement

R4-2200910	On channel rasters for the 6GHz unlicensed band

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2201081	On band definition for the lower 6GHz NR unlicensed operation

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 5.1.3	UE RF requirements

R4-2200430	Introduction of the lower 6GHz unlicensed band

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Apple, Skyworks Solutions Inc., MediaTek Inc.

Decision: 		The document was not treated.

R4-2200849	Unwanted emissions requirements for lower 6GHz NR unlicensed operation for Europe

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss the unwanted emissions requirements for operations in 5925-6425 MHz

Decision: 		The document was not treated.

R4-2200850	Unwanted emissions requirements for Band n102

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to introduce unwanted emissions requirements for Band n102

Decision: 		The document was not treated.

R4-2201854	NS signaling, MPR, and A-MPR for the 5925 – 6425 MHz NR-U band

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 5.1.4	BS RF requirements

R4-2201082	Running CR for 38.104 to add n102

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, ZTE Corporation

Abstract:

draftCR including agreements at RAN4#101 intended revised to include agreements at RAN4#101bis

Decision: 		The document was not treated.

R4-2201474	Discussion on MU for EU band n102

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201475	draft CR to TS38.141-2 the introduction of EU unlicensed band n102

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201476	draft CR to TS36.104 the introduction of EU unlicensed band n102

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201918	draft CR 37.105 on Introduction of lower 6GHz NR unlicensed operation for Europe

Type: draftCR		For: Endorsement
					37.105 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

draft CR 37.105 on Introduction of lower 6GHz NR unlicensed operation for Europe

Decision: 		The document was not treated.

R4-2201928	draftCR to 37.145-1 - adding band n102

Type: draftCR		For: Endorsement
					37.145-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

Adding the new UL band to the AAS conducted conformance specification

Decision: 		The document was not treated.

R4-2201929	draftCR to 37.145-2 - adding band n102

Type: draftCR		For: Endorsement
					37.145-2 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

Adding the new UL band to the AAS radiated conformance specification

Decision: 		The document was not treated.

R4-2201998	draft CR to 37.104 on introduction of n102 co-existence requirements

Type: draftCR		For: Endorsement
					37.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 5.1.5	Others

R4-2200431	TP for TR 38.849

Type: pCR		For: Approval
					38.849 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Abstract:

Text proposal with a summary of which NS values are applicable to Region 1 countries.

Decision: 		The document was not treated.

R4-2201083	TP to TR 38.849 updating clause 5.1 for the lower 6GHz band

Type: pCR		For: Approval
					38.849 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202001	draft CR to 37.141 on introduction of n102 co-existence requirements

Type: draftCR		For: Endorsement
					37.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202002	draft CR to 36.141 on introduction of n102 co-existence requirements

Type: draftCR		For: Endorsement
					36.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

### 5.2	Introduction of operation in full unlicensed band 5925-7125MHz for NR

#### 5.2.1	General

R4-2200432	Update of the 6GHz unlicensed band system and regulatory requirements in Region 2 and Region 3 countries

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2201084	On band definition for 6GHz NR unlicensed operation

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 5.2.2	Regulatory requirements and evaluation for re-using existing NS

R4-2200374	Introduction of the VLP mode for the 6GHz unlicensed band

Type: discussion		For: Information
					Source: TTA

Abstract:

Korean regulation includes the LPI/VLP operation modes. Regarding the VLP operation, MSIT and domestic industries prefer to standardize the related requirement for VLP mode to enrich the echo system of the 6 GHz unlicensed band. TTA kindly ask 3GPP RAN4 t

Decision: 		The document was not treated.

R4-2200433	Applicability of band n96

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

#### 5.2.3	UE RF requirements

R4-2200434	Draft CR for TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Apple

Abstract:

Draft running CR for endorsing technical content of existing agreements.

Decision: 		The document was not treated.

R4-2201125	Discussion on the impact of the New power class definitions for NR-U UE in other countries

Type: discussion		For: Approval
					Source: LG Electronics Inc.

Abstract:

In this contribution, We propose new power class and MPR/A-MPR approach for other countries that require VLP operating mode.

Decision: 		The document was not treated.

R4-2201128	A-MPR analysis results for NR-U(VLP) considering regulatory parameters in Korea

Type: discussion		For: Approval
					Source: LG Electronics Inc.

Abstract:

In this contribution, we provide A-MPR analysis results for NR-U(VLP) considering regulatory parameters in South Korea.

Decision: 		The document was not treated.

#### 5.2.4	BS RF requirements

R4-2201515	TP for BS RF requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 5.2.5	Others

R4-2200435	TP for TR 38.849

Type: pCR		For: Approval
					38.849 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Abstract:

Text proposal with a summary of which NS values are applicable to Region 2 and Region 3 countries.

Decision: 		The document was not treated.

R4-2201085	TP to TR 38.849 updating clause 5.1 for the full 6GHz band

Type: pCR		For: Approval
					38.849 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

### 5.3	Introduction of 6GHz NR licensed bands

#### 5.3.1	General

R4-2200436	Initial considerations on requirements for the licensed operation in the upper 6GHz frequency range

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2201231	Discussion on general issues for 6GHz licensed band

Type: discussion		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2201330	6GHz licensed band - General

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is discussing general aspects and system parameters for the new 6GHz license band

Decision: 		The document was not treated.

R4-2201503	General aspects for 6GHz NR licensed band

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201987	6GHz licensed band aspects

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

#### 5.3.2	System parameters

R4-2200149	Discussion on system parameters for 6GHz licensed band

Type: discussion		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200152	draft CR on introduction of 6GHz licensed band for 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2201304	Discussion on system parameters for 6G license band

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201331	6GHz licensed band - BS requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is going through BS RF requirements, proposing a way forward to specify the corresponding limits for the new 6GHz license band.

Decision: 		The document was not treated.

R4-2201447	Discussion on system parameters for 6425-7125MHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201504	System parameters for 6GHz NR licensed band

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201545	6GHz licensed band attributes based on RCC recommendations

Type: discussion		For: Discussion
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we examine the RCC recommendation document and applicable ITU documents referred to and provide our understanding on how it can translate into RAN4 specifications.

Decision: 		The document was not treated.

R4-2201824	draft CR on introduction of 6GHz system parameters for 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201825	draft CR on introduction of 6GHz system parameters for 38.104

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

#### 5.3.3	UE RF requirements

R4-2200150	Discussion on UE RF requirements for 6GHz licensed band

Type: discussion		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2201305	Discussion on UE RF requirements for 6G license band

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201332	6GHz licensed band - UE requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is going through UE RF requirements, proposing a way forward to specify the corresponding limits for the new 6GHz license band.

Decision: 		The document was not treated.

R4-2201448	Discussion on UE RF requirements for 6425-7125MHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201505	UE RF requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon, CMCC, China Unicom, OPPO

Decision: 		The document was not treated.

R4-2201506	Draft CR for 38.101-1: 6GHz NR licensed band

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon,CMCC, China Telecom, China Unicom, OPPO

Decision: 		The document was not treated.

R4-2201826	draft CR on introduction of 6GHz UE RF requirements for 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201855	6 GHz licensed band

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 5.3.4	BS RF requirements

R4-2200153	Analysis on BS requirements for operation in 6GHz band

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200154	draft CR on introduction of 6GHz licensed band for 38.104

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200155	draft CR on introduction of 6GHz licensed band for 38.141-1

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200156	draft CR on introduction of 6GHz licensed band for 38.141-2

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200157	draft CR on introduction of 6GHz licensed band for 37.104

Type: draftCR		For: Endorsement
					37.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200158	draft CR on introduction of 6GHz licensed band for 37.105

Type: draftCR		For: Endorsement
					37.105 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200159	draft CR on introduction of 6GHz licensed band for 37.141

Type: draftCR		For: Endorsement
					37.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200160	draft CR on introduction of 6GHz licensed band for 37.145-1

Type: draftCR		For: Endorsement
					37.145-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200161	draft CR on introduction of 6GHz licensed band for 37.141-2

Type: draftCR		For: Endorsement
					37.145-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200480	draft CR on introduction of 6GHz licensed band for 38.174

Type: draftCR		For: Endorsement
					38.174 v16.5.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200481	draft CR on introduction of 6GHz licensed band for 38.176-1

Type: draftCR		For: Endorsement
					38.176-1 v16.2.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200482	draft CR on introduction of 6GHz licensed band for 38.176-2

Type: draftCR		For: Endorsement
					38.176-2 v16.2.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2201449	Discussion on BS RF requirements for 6425-7125MHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201450	draft CR to TS38.104 the introduction of 6425-7125MHz

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201451	draft CR to TS36.104 the introduction of coexistence requirements of licensed band 6425-7125MHz

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201452	draft CR to TS36.141 the introduction of coexistence requirements of licensed band 6425-7125MHz

Type: draftCR		For: Endorsement
					36.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201507	BS RF requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon, China Unicom, CMCC

Decision: 		The document was not treated.

R4-2201508	Draft CR for 38.104: 6GHz NR licensed band

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201827	draft CR to TS 38.174 introduction of 6GHz coexistence requirement in IAB spec

Type: draftCR		For: Endorsement
					38.174 v16.5.0	  CR-  rev  Cat: B (Rel-16)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201828	draft CR to TS 38.176-1 on introduction of coexistence requirement for 6GHz

Type: draftCR		For: Endorsement
					38.176-1 v16.2.0	  CR-  rev  Cat: B (Rel-16)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201829	draft CR to TS 38.176-2 on introduction of coexistence requirement for 6GHz

Type: draftCR		For: Endorsement
					38.176-1 v16.2.0	  CR-  rev  Cat: B (Rel-16)

					Source: ZTE Corporation

Decision: 		The document was not treated.

#### 5.3.5	Others

R4-2201453	Discussion on MR/LA BS UEM requirements for 6425-7125MHz and 10-10.5GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201454	draft CR to TR38.921 MR and LA BS requirements for 6425-7125MHz and 10-10.5GHz

Type: draftCR		For: Endorsement
					38.921 v17.0.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

### 5.4	Introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio

#### 5.4.1	General

R4-2201687	Revised TR 38.853 version 0.2.0

Type: draft TR		For: Approval
					38.853 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Abstract:

In the Copyright Notification section, update the copyright year to 2022.

Decision: 		The document was not treated.

R4-2201805	Cross-check on the implementation of ECC Decision (20)02 for RMR900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we analyse progress on translation of the ECC decision (20)02 [1] into RAN4 BS RF requirements for RMR900, and provide further proposals on the remaining issues identified after analysis of the ECC (20)02 regulations.

Decision: 		The document was not treated.

R4-2201806	draft CR to TS 38.104: consideration of regional requirements for RMR900

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Based on the related discussion paper, it was identified that certain aspects of the ECC (20)02 regulation for RMR900 are not properly reflected.

Specifically, the OOB limits specified in ECC (20)02 table 5 are subject to the follwing note: “on a case-by

Decision: 		The document was not treated.

#### 5.4.2	UE RF requirements

R4-2200702	On RMR900 sensitivity

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

#### 5.4.3	BS RF requirements

R4-2201324	RMR 900 MHz - BS RF

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues related to BS RF requirements when introducing the new RMR 900MHz band

Decision: 		The document was not treated.

R4-2201325	Draft CR to TS 38.141-2: RMR 900MHz and 1900MHz bands introduction

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This a draft CR to TS 38.141-2 introducing support for the new RMR 900MHz band

Decision: 		The document was not treated.

R4-2201326	Draft CR to TS 36.104: RMR 900MHz and 1900MHz bands  introduction

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This a draft CR to TS 36.104 introducing support for the new RMR 900MHz band

Decision: 		The document was not treated.

R4-2201327	Draft CR to TS 36.141: RMR 900MHz and 1900MHz bands  introduction

Type: draftCR		For: Endorsement
					36.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This a draft CR to TS 36.141 introducing support for the new RMR 900MHz band

Decision: 		The document was not treated.

R4-2201807	Remaining aspects for BS RF requirements for RMR900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide discussion on the remaining issues for the RMR900-specific BS RF requirements.

Decision: 		The document was not treated.

R4-2201808	Draft CR to TS 38.104: RX requirements

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Draft CR to TS 38.104: implementation of RMR900 and RMR1900 bands

Decision: 		The document was not treated.

R4-2201809	TP to TR 38.853: BS RF requirements

Type: pCR		For: Approval
					38.853 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to TR 38.853: BS RF requirements

Decision: 		The document was not treated.

R4-2201810	Draft CR to TS 37.105: RMR implementation

Type: draftCR		For: Endorsement
					37.105 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Draft CR to TS 37.105: implementation of RMR900 and RMR1900 bands

Decision: 		The document was not treated.

R4-2201996	draft CR to 37.104 on introduction of n100 co-existence requirements

Type: draftCR		For: Endorsement
					37.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202003	draft CR to 38.104 on introduction of n100 (system parameters)

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202026	Draft CR to TS 37.145-1: RMR implementation

Type: draftCR		For: Endorsement
					37.145-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Implementation of RMR900 and RMR1900 co-location blocking to protect AAS BS receiver.

Decision: 		The document was not treated.

R4-2202027	Draft CR to TS 37.145-2: RMR implementation

Type: draftCR		For: Endorsement
					37.145-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Implementation of RMR900 and RMR1900 co-location blocking to protect AAS BS receiver.

Decision: 		The document was not treated.

#### 5.4.4	Others

R4-2201688	TR_38.853_changes_clause 7.1

Type: pCR		For: Approval
					38.853 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Decision: 		The document was not treated.

R4-2201689	TR_38.853_changes_clause 9

Type: pCR		For: Approval
					38.853 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Decision: 		The document was not treated.

R4-2201811	Consideration of coordinated/un-coordinated deployments for the RMR900 requirements in RAN4 specifications

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we discuss aspects related to the coordinated/un-coordinated deployments considerations for RMR900 BS and related implications on RAN4 specifications. Related TP to TR 38.853 is attached for approval.

Decision: 		The document was not treated.

R4-2201999	draft CR to 37.141 on introduction of n100 co-existence requirements

Type: draftCR		For: Endorsement
					37.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

### 5.5	Introduction of 1900 MHz spectrum to 5G NR applicable for Rail Mobile Radio

#### 5.5.1	General

R4-2201328	RMR 1900 MHz - TP to TR 38.852 on Regulatory background

Type: pCR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This is TP on the regulatory background for RMR band 1900MHz

Decision: 		The document was not treated.

R4-2201682	Revised TR 38.852 version 0.2.0

Type: draft TR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Abstract:

In the Copyright Notification section, update the copyright year to 2022.

Decision: 		The document was not treated.

R4-2201812	Cross-check on the implementation of ECC Decision (20)02 for RMR1900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we analyse progress on translation of the ECC decision (20)02 [1] into RAN4 BS RF requirements for RMR1900.

Decision: 		The document was not treated.

#### 5.5.2	UE RF requirements

#### 5.5.3	BS RF requirements

R4-2201329	RMR 1900 MHz - BS RF

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues related to BS RF requirements when introducing the new RMR 1900MHz band

Decision: 		The document was not treated.

R4-2201813	Remaining aspects for BS RF requirements for RMR1900

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we provide discussion on the remaining issues for the RMR1900-specific BS RF requirements.

Decision: 		The document was not treated.

R4-2201814	TP to TR 38.852: BS RF requirements

Type: pCR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to TR 38.852: BS RF requirements

Decision: 		The document was not treated.

R4-2201997	draft CR to 37.104 on introduction of n101 co-existence requirements

Type: draftCR		For: Endorsement
					37.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202004	draft CR to 38.104 on introduction of n101 (system parameters)

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 5.5.4	Others

R4-2201683	TR_38.852_changes_clause 7.1

Type: pCR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Decision: 		The document was not treated.

R4-2201685	TR_38.852_changes_clause 7.2

Type: pCR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Decision: 		The document was not treated.

R4-2201686	TR_38.852_changes_clause 9

Type: pCR		For: Approval
					38.852 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Union Inter. Chemins de Fer

Decision: 		The document was not treated.

R4-2201815	Consideration of coordinated/un-coordinated deployments for the RMR1900 requirements in RAN4 specifications

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution, we discuss aspects related to the coordinated/un-coordinated deployments considerations for RMR1900 BS and related implications on RAN4 specifications. Related TP to TR 38.852 is attached for approval.

Decision: 		The document was not treated.

R4-2202000	draft CR to 37.141 on introduction of n101 co-existence requirements

Type: draftCR		For: Endorsement
					37.141 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

### 5.6	Issues arising from basket WIs but not subject to block approval

#### 5.6.1	UE RF requirements

R4-2200698	Working procedures for updating release independence specification

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201440	Discussion on the working procedure for introducing release independent features

Type: discussion		For: Discussion
					Source: CHTTL

Decision: 		The document was not treated.

R4-2201804	Discussion on the Rel-17 specifications: 25-series and 34-series

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide inputs to the discussion on the list of RAN4 specifications to be promoted to Rel-17, in particular looking at 25-, and 34-series specifications.

Decision: 		The document was not treated.

R4-2202022	NR-U Contiguous UL-CA Measurements

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202028	Corrections to Intra-band CA MSD for CA_n5B and CA_n7B

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202034	Triple beat B3 MSD evaluation for DC_3A_n41C

Type: discussion		For: Approval
					38.101-3 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202038	MSD for DC_20A-38A_n8A

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202039	Intra-band CA REFSENS Ambiguity

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 5.6.2	NR-U intra-band contiguous UL CA

R4-2200851	On the MPR and A-MPR for intra-band UL CA with shared spectrum access

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose MPR and A-MPR requirements for interlaced transmissions and discuss the requirements for other resource-allocation types

Decision: 		The document was not treated.

### 5.7	NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)

#### 5.7.1	UE RF requirements for FR1

R4-2201104	TP to TR 38.717-01-01 Addition of CA_n40B

Type: pCR		For: Approval
					38.717-01-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, NBN

Decision: 		The document was not treated.

#### 5.7.2	UE RF requirements for FR2

### 5.8	NR inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2)

R4-2201796	TR 38.717-02-01 v0.8.0

Type: draft TR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

#### 5.8.1	NR inter band CA requirements without any FR2 band(s)

R4-2200060	TP to TR 38.717.02-01 for CA_n48-n96

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200061	TP to TR 38.717.02-01 for CA_n46-n48

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200193	Draft CR for 38.101-1: support of DC_n1A-n28A and DC_n1A-n41A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n1A-n28A and n1A-n41A are added based on relevant 2UL CA requirements..

Decision: 		The document was not treated.

R4-2200201	Draft CR for TS 38.101-1: Support of BCS2 in CA_n41-n79

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200706	n29 MSD in CA_n29-n71

Type: discussion		For: Discussion
					Source: Nokia, Dish

Decision: 		The document was not treated.

R4-2201022	Draft CR for 38.101-1 to introduce CA_n1A-n77(3A) and CA_n18A-n77(3A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201062	Draft CR for 38.101-1 to introduce new configurations of CA_n25-n77

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201088	draftCR to add DC_n1A-n7A to 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201090	draftCR to add DC_n7A-n46 to 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201094	TP to TR 38.717-02-01 Addition of BCS1 for CA_n25-n77

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Telus

Decision: 		The document was not treated.

R4-2201100	TP to TR 38.717-02-01 Addition of CA_n40-n77

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, NBN

Decision: 		The document was not treated.

R4-2201101	draftCR to add DC_n40B-n78 to 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, NBN

Decision: 		The document was not treated.

R4-2201105	draftCR 38.101-1 Addition of CA_n12A-n66(2A)(3A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201106	draftCR 38.101-1 Addition of CA_n2(2A)-n12

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201107	draftCR 38.101-1 Addition of CA_n29A-n66(3A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201546	draft CR 38.101-1 to add new configurations for CA_n25-n77 and CA_n41-n48

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Eridsson, T-Mobile US

Abstract:

draft CR 38.101-1 to add new configurations for CA_n25-n77 and CA_n41-n48

Decision: 		The document was not treated.

R4-2201554	TP for TR 38.717-02-01 to include CA_n41-n70

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-02-01 to include CA_n41-n70

Decision: 		The document was not treated.

R4-2201555	TP for TR 38.717-02-01 to include CA_n70-n78

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-02-01 to include CA_n70-n78

Decision: 		The document was not treated.

R4-2201556	draft CR 38.101-1 to add new configurations for CA_n3-n79

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

draft CR 38.101-1 to add new configurations for CA_n3-n79

Decision: 		The document was not treated.

R4-2201559	TP for TR 38.717-02-01 to include dual UL for CA_n3-n20

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-02-01 to include dual UL for CA_n3-n20

Decision: 		The document was not treated.

R4-2201561	TP for TR 38.717-02-01 to include CA_n1-n67

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-02-01 to include CA_n1-n67

Decision: 		The document was not treated.

R4-2201562	TP for TR 38.717-02-01 to include CA_n3-n67

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-02-01 to include CA_n3-n67

Decision: 		The document was not treated.

R4-2201564	TP for TR 38.717-02-01 to include dual UL for CA_n1-n20

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-02-01 to include dual UL for CA_n1-n20

Decision: 		The document was not treated.

R4-2201565	TP for TR 38.717-02-01 to include CA_n20-n67

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-02-01 to include CA_n20-n67

Decision: 		The document was not treated.

R4-2201681	Draft CR on CA_n1A-n8A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: China Unicom

Abstract:

Add BCS1 for CA_n1A-n8A.

Decision: 		The document was not treated.

R4-2202035	Measurements for CA_n29-n71 MSD

Type: discussion		For: Approval
					38.101 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202036	Measurements for CA_n5-n28 MSD

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2202037	CA_n29A-n71A MSD

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 5.8.2	NR inter band CA requirements with at least one FR2 band

R4-2200192	Draft CR 38.101-3: support of DC_n1A-n257D

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n1A-n257D is added.

Decision: 		The document was not treated.

R4-2200621	Draft CR for TS 38.101-3 on corrections to CA configurations of CA_n25-n260

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide draft CR for TS 38.101-3 on corrections to CA configurations of CA_n25-n260

Decision: 		The document was not treated.

R4-2201023	Draft CR for 38.101-3 to introduce new configuratuons of DC_n77-n257

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201102	draftCR to add CA_n40-n257 to 38.101-1

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, NBN

Decision: 		The document was not treated.

R4-2201348	Draft CR for TS 38.101-3 on corrections to CA configurations of CA_n77(3A)-n257A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

### 5.9	NR Inter-band Carrier Aggregation for 3 bands DL with 1 band UL

#### 5.9.1	UE RF requirements

R4-2200176	TP to TR TR38.717-03-01 for CA_n46-n48-n96

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200189	Draft CR for 38.101-3: support of n77(3A) in 1UL CA_n3A/n28A-n77-n257A/D/G/H/I

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

n77(3A) support is added to CA_n3/n28-n77-n257X in 1UL.

Decision: 		The document was not treated.

R4-2200200	Draft CR for TS 38.101-1: Addition of missing CA_n3-n28-n41 Tib values

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200606	Draft CR for TS 38.101-1: Adding same note for higher order combo of CA_n20-n28

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200607	Draft CR for TS 38.101-3: UL configuration correction of 48_n77's higher order combinations

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200711	TP to TR 38.717-03-01: CA_n41-n66-n260

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, TMO USA

Decision: 		The document was not treated.

R4-2200712	TP to TR 38.717-03-01: CA_n66-n77-n260

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, TMO USA

Decision: 		The document was not treated.

R4-2201024	TP for TR 38.717-03-01 CA_n1-n3-n18

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201025	TP for TR 38.717-03-01 CA_n1-n18-n28

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201026	TP for TR 38.717-03-01 CA_n1-n18-n41

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201027	TP for TR 38.717-03-01 CA_n1-n18-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201028	TP for TR 38.717-03-01 CA_n1-n28-n41

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201029	TP for TR 38.717-03-01 CA_n1-n28-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201030	TP for TR 38.717-03-01 CA_n1-n41-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201031	TP for TR 38.717-03-01 CA_n3-n18-n28

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201032	TP for TR 38.717-03-01 CA_n3-n18-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201033	TP for TR 38.717-03-01 CA_n18-n28-n41

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201034	TP for TR 38.717-03-01 CA_n18-n28-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201035	TP for TR 38.717-03-01 CA_n18-n41-n77

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201052	Draft CR for 38.101-1 to introduce new configurations to CA_n5-n48-n77 and CA_n48-n66-n77 with 1UL

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201063	Draft CR for 38.101-1 to introduce new configurations to CA_n5-n25-n77 and CA_n5-n66-n77 with 1UL

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201108	TP to TR 38.717-03-01 Addition of CA_n12-n30-n66

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201109	TP to TR 38.717-03-01 Addition of CA_n29-n30-n66

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201110	TP to TR 38.717-03-01 Addition of CA_n2-n12-n30

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201111	TP to TR 38.717-03-01 Addition of CA_n2-n12-n66

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201112	TP to TR 38.717-03-01 Addition of CA_n2-n29-n30

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201113	TP to TR 38.717-03-01 Addition of CA_n2-n29-n66

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201552	TP for TR 38.717-03-01 to include CA_n41-n70-n78

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-03-01 to include CA_n41-n70-n78

Decision: 		The document was not treated.

R4-2201566	TP for TR 38.717-03-01 to include CA_n1-n20-n67

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-01 to include CA_n1-n20-n67

Decision: 		The document was not treated.

R4-2201568	TP for TR 38.717-03-01 to include CA_n3-n20-n67

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-01 to include CA_n3-n20-n67

Decision: 		The document was not treated.

R4-2201724	TP for TR 38.717-03-01 to include CA_n2-n71-n78

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 38.717-03-01 to include CA_n2-n71-n78

Decision: 		The document was not treated.

R4-2201725	TP for TR 38.717-03-01 to include CA_n2-n66-n78

Type: pCR		For: Approval
					38.717-03-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 38.717-03-01 to include CA_n2-n66-n78

Decision: 		The document was not treated.

### 5.10	NR Inter-band Carrier Aggregation for 4 bands DL with 1 band UL

#### 5.10.1	UE RF requirements

R4-2200211	TP for TR 38.717-04-01: CA_n1-n3-n28-n77

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200212	TP for TR 38.717-04-01: CA_n1-n3-n28-n79

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200213	TP for TR 38.717-04-01: CA_n1-n3-n28-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200214	TP for TR 38.717-04-01: CA_n1-n3-n77-n79

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200215	TP for TR 38.717-04-01: CA_n1-n3-n79-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200216	TP for TR 38.717-04-01: CA_n1-n28-n77-n79

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200217	TP for TR 38.717-04-01: CA_n1-n28-n77-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200218	TP for TR 38.717-04-01: CA_n1-n28-n79-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200219	TP for TR 38.717-04-01: CA_n3-n28-n41-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200220	TP for TR 38.717-04-01: CA_n3-n41-n77-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200221	TP for TR 38.717-04-01: CA_n28-n41-n77-n257

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200356	TP for CA_n28-n77-n79-n257 for TR 38.717-04-01

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200357	TP for CA_n28-n78-n79-n257 for TR 38.717-04-01

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200724	draft CR to add NR Inter-band CA for 4 bands in TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2201065	TP for TR 38.717-04-01 CA_n5-n25-n66-n77

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201120	TP to TR 38.717-04-01 Addition of CA_n2-n12-n30-n66

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201121	TP to TR 38.717-04-01 Addition of CA_n2-n29-n30-n66

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201550	TP for TR 38.717-04-01 to include CA_n41-n66-n70-n78

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-04-01 to include CA_n41-n66-n70-n78

Decision: 		The document was not treated.

R4-2201726	TP for TR 38.717-04-01 to include CA_n25-n41-n71-n78

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 38.717-04-01 to include CA_n25-n41-n71-n78

Decision: 		The document was not treated.

R4-2201727	TP for TR 38.717-04-01 to include CA_n2-n66-n71-n78

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 38.717-04-01 to include CA_n2-n66-n71-n78

Decision: 		The document was not treated.

R4-2201891	TP for TR 38.717-04-01 to include CA_n2A-n5A-n30A-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n2A-n5A-n30A-n77A

Decision: 		The document was not treated.

R4-2201892	TP for TR 38.717-04-01 to include CA_n2A-n5A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n2A-n5A-n66A-n77A

Decision: 		The document was not treated.

R4-2201893	TP for TR 38.717-04-01 to include CA_n2A-n14A-n30A-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n2A-n14A-n30A-n77A

Decision: 		The document was not treated.

R4-2201894	TP for TR 38.717-04-01 to include CA_n2A-n14A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n2A-n14A-n66A-n77A

Decision: 		The document was not treated.

R4-2201895	TP for TR 38.717-04-01 to include CA_n5A-n30A-n66-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n5A-n30A-n66-n77A

Decision: 		The document was not treated.

R4-2201896	TP for TR 38.717-04-01 to include CA_n14A-n30A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-01 to include CA_n14A-n30A-n66A-n77A

Decision: 		The document was not treated.

### 5.11	NR Inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL

R4-2201797	TR 38.717-03-02 v0.8.0

Type: draft TR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

#### 5.11.1	UE RF requirements

R4-2200059	TP to TR 38.717.02-01 for CA_n46-n48--n96

Type: pCR		For: Approval
					38.717-02-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200190	Draft CR for 38.101-1: support of n77(2A) in 2UL CA_n1A-n77-n79A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

n77(2A) support is added to CA_n1-n77-n79.

Decision: 		The document was not treated.

R4-2200191	Draft CR for 38.101-1: support of 2UL in CA_n3A-n28A-n77(3A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

2UL support is added to CA_n3A-n28A-n77(3A).

Decision: 		The document was not treated.

R4-2200194	Draft CR for 38.101-1: support of 3 Bands DC_n1A-n3A-n77A and DC_n1A-n3A-n79A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n1A-n3A-n77A/n79A are added.

Decision: 		The document was not treated.

R4-2200195	Draft CR for 38.101-1: support of 3 Bands DC_n1A-n77A-n79A, DC_n3A-n77A-n79A and DC_n3A-n77(2A)-n79A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n1A-n77A-n79A and n3A-n77A/n77(2A)-n79A are added.

Decision: 		The document was not treated.

R4-2200196	Draft CR for 38.101-1: support of 3 Bands DC_n3A-n28A-n41A, DC_n3A-n41A-n77A and DC_n28A-n41A-n77A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n3A-n28A-n41A, DC_n3A-n41A-n77A and DC_n28A-n41A-n77A are added.

Decision: 		The document was not treated.

R4-2200202	TP for TR 38.717-03-02: CA_n1-n3-n79

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200203	TP for TR 38.717-03-02: CA_n1-n3-n257

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200204	TP for TR 38.717-03-02: CA_n1-n28-n41

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200205	TP for TR 38.717-03-02: CA_n1-n28-n77

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200206	TP for TR 38.717-03-02: CA_n1-n28-n79

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200207	TP for TR 38.717-03-02: CA_n1-n28-n257

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200208	TP for TR 38.717-03-02: CA_n1-n41-n77

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200209	TP for TR 38.717-03-02: CA_n1-n41-n257

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200210	TP for TR 38.717-03-02: CA_n41-n77-n257

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200351	BCS corrections for CA_n7-n66-n77 and CA_n66-n71-n77

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

CR implementation error is corrected.

Decision: 		The document was not treated.

R4-2200355	TP for CA_n28-n78-n79 for TR 38.717-03-02

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC. MediaTek Inc.

Decision: 		The document was not treated.

R4-2201036	TP for TR 38.717-03-02 CA_n1A-n3A-n18A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201037	TP for TR 38.717-03-02 CA_n1A-n18A-n28A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201038	TP for TR 38.717-03-02 CA_n1A-n18A-n41A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201039	TP for TR 38.717-03-02 CA_n1A-n18A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201040	TP for TR 38.717-03-02 CA_n1A-n28A-n41A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201041	TP for TR 38.717-03-02 CA_n1A-n28A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201042	TP for TR 38.717-03-02 CA_n1A-n41A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201043	TP for TR 38.717-03-02 CA_n3A-n18A-n28A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201044	TP for TR 38.717-03-02 CA_n3A-n18A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201045	TP for TR 38.717-03-02 CA_n18A-n28A-n41A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201046	TP for TR 38.717-03-02 CA_n18A-n28A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201047	TP for TR 38.717-03-02 CA_n18A-n41A-n77A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, KDDI

Abstract:

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

Decision: 		The document was not treated.

R4-2201048	Draft CR for 38.101-1 to introduce CA_n3A-n28A-n77(3A) and CA_n1A-n3A-n77(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201049	Draft CR for 38.101-3 to introduce new combination for NR inter-band CA DC 3 bands DL with 2 bands UL

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201053	TP for TR 38.717-03-02 CA_n2-n5-n48

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201054	TP for TR 38.717-03-02 CA_n2-n48-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201055	TP for TR 38.717-03-02 CA_n2-n48-n77

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201056	TP for TR 38.717-03-02 CA_n5-n48-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201057	TP for TR 38.717-03-02 CA_n5-n48-n77

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201058	TP for TR 38.717-03-02 CA_n48-n66-n77

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2201064	Draft CR for 38.101-1 to introduce new configurations to CA_n5-n25-n77 and CA_n5-n66-n77 with 2UL

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201086	TP to TR 38.717-03-02 Addition of CADC_n7A-n46-n78A

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201087	draftCR to add n78(2A) to CA_n7A-n28A-n78 already in 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201103	draftCR to add CA_n40-n78-n257 to 38.101-1

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, NBN

Decision: 		The document was not treated.

R4-2201114	TP to TR 38.717-03-02 Addition of CA_n12-n30-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201115	TP to TR 38.717-03-02 Addition of CA_n29-n30-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201116	TP to TR 38.717-03-02 Addition of CA_n2-n12-n30

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201117	TP to TR 38.717-03-02 Addition of CA_n2-n12-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201118	TP to TR 38.717-03-02 Addition of CA_n2-n29-n30

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201119	TP to TR 38.717-03-02 Addition of CA_n2-n29-n66

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201349	Draft CR for TS 38.101-3 Add a note for BCS in 3DL NR CA table

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201516	DraftCR for 38.101-1: Correction on CA_n7-n66-n78

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, Bell Mobility, Telus

Decision: 		The document was not treated.

R4-2201517	DraftCR for 38.101-1: CA_n25(2A)-n38A-n66(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, Bell Mobility, Telus

Decision: 		The document was not treated.

R4-2201518	DraftCR for 38.101-1: additional combinations for CA_n7-n25-n66

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, Bell Mobility, Telus

Decision: 		The document was not treated.

R4-2201547	draft CR 38.101-1 to add new configurations for CA_n25-n41-n66, CA_n25-n41-n71, CA_n25-n66-n71, CA_n41-n66-n71, CA_n66-n71-n77

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Eridsson, T-Mobile US

Abstract:

draft CR 38.101-1 to add new configurations for CA_n25-n41-n66, CA_n25-n41-n71, CA_n25-n66-n71, CA_n41-n66-n71, CA_n66-n71-n77

Decision: 		The document was not treated.

R4-2201548	TP for TR 38.717-03-02 to include CA_n41-n66-n260

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Eridsson, T-Mobile US

Abstract:

TP for TR 38.717-03-02 to include CA_n41-n66-n260

Decision: 		The document was not treated.

R4-2201549	TP for TR 38.717-03-02 to include DC_n41-n66-n260

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Eridsson, T-Mobile US

Abstract:

TP for TR 38.717-03-02 to include DC_n41-n66-n260

Decision: 		The document was not treated.

R4-2201553	TP for TR 38.717-03-02 to include CA_n41-n70-n78

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-03-02 to include CA_n41-n70-n78

Decision: 		The document was not treated.

R4-2201560	TP for TR 38.717-03-02 to include CA_n1-n3-n20

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-02 to include CA_n1-n3-n20

Decision: 		The document was not treated.

R4-2201563	TP for TR 38.717-03-02 to include CA_n1-n3-n67

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-02 to include CA_n1-n3-n67

Decision: 		The document was not treated.

R4-2201567	TP for TR 38.717-03-02 to include CA_n1-n20-n67

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-02 to include CA_n1-n20-n67

Decision: 		The document was not treated.

R4-2201569	TP for TR 38.717-03-02 to include CA_n3-n20-n67

Type: pCR		For: Approval
					38.717-03-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 38.717-03-02 to include CA_n3-n20-n67

Decision: 		The document was not treated.

R4-2201578	draft CR 38.101-3 to add new 3DL CA combinations with FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Telstra

Abstract:

draft CR 38.101-3 to add new 3DL CA combinations with FR2

Decision: 		The document was not treated.

R4-2201579	draft CR 38.101-3 to add new 3DL DC combinations with FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Telstra

Abstract:

draft CR 38.101-3 to add new 3DL DC combinations with FR2

Decision: 		The document was not treated.

### 5.12	NR inter-band Carrier Aggregation and Dual connectivity for DL 4 bands and 2UL bands

#### 5.12.1	UE RF requirements

R4-2200197	Draft CR for 38.101-3: support of 4 Bands DC_n28A-n77A-n79A-257A/G/H/I and DC_n28A-n77(2A)-n79A-257A/G/H/I

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n28A-n77A/(2A)-n79A-257A/G/H/I are added.

Decision: 		The document was not treated.

R4-2200198	Draft CR for 38.101-3: additions of Low+Low UL configurations for 4 Bands DC_n3A-n28A-n77A-257A/G/H/I and DC_n3A-n28A-n77(2A)-257A/G/H/I

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

UL conf of two low bands combinations, i.e., n3-n28, n3-n77 and n28-n77 are added to DC_n3A-n28A-n77A/(2A)-257A/G/H/I.

Decision: 		The document was not treated.

R4-2200199	Draft CR for 38.101-3: support of 4 Bands DC_n3A-n28A-n79A-257A/G/H/I

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

DC_n3A-n28A-n79A-257A/G/H/I are added.

Decision: 		The document was not treated.

R4-2200358	TP for CA_n28-n78-n79-n257 for TR 38.717-04-02

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200359	Draft CR for TS 38.101-3: DC_n28-n77-n79-n257 and DC_n28-n78-n79-n257

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2201050	Draft CR for 38.101-3 to introduce new combinations for NR inter-band CA DC 4 bands DL with 2 bands UL

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, KDDI

Decision: 		The document was not treated.

R4-2201066	TP for TR 38.717-04-02 CA_n5-n25-n66-n77

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201122	TP to TR 38.717-04-02 Addition of CA_n2-n12-n30-n66

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201123	TP to TR 38.717-04-02 Addition of CA_n2-n29-n30-n66

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, AT&T

Decision: 		The document was not treated.

R4-2201551	TP for TR 38.717-04-02 to include CA_n41-n66-n70-n78

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

TP for TR 38.717-04-02 to include CA_n41-n66-n70-n78

Decision: 		The document was not treated.

R4-2201580	draft CR 38.101-3 to add new 4DL CA combinations with FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Telstra

Abstract:

draft CR 38.101-3 to add new 4DL CA combinations with FR2

Decision: 		The document was not treated.

R4-2201581	draft CR 38.101-3 to add new 4DL DC combinations with FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Telstra

Abstract:

draft CR 38.101-3 to add new 4DL DC combinations with FR2

Decision: 		The document was not treated.

R4-2201897	TP for TR 38.717-04-02 to include CA_n2A-n5A-n30A-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n2A-n5A-n30A-n77A

Decision: 		The document was not treated.

R4-2201898	TP for TR 38.717-04-02 to include CA_n2A-n5A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n2A-n5A-n66A-n77A

Decision: 		The document was not treated.

R4-2201899	TP for TR 38.717-04-02 to include CA_n2A-n14A-n30A-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n2A-n14A-n30A-n77A

Decision: 		The document was not treated.

R4-2201900	TP for TR 38.717-04-02 to include CA_n14A-n30A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n14A-n30A-n66A-n77A

Decision: 		The document was not treated.

R4-2201901	TP for TR 38.717-04-02 to include CA_n2A-n14A-n66A-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n2A-n14A-n66A-n77A

Decision: 		The document was not treated.

R4-2201902	TP for TR 38.717-04-02 to include CA_n5A-n30A-n66-n77A

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 38.717-04-02 to include CA_n5A-n30A-n66-n77A

Decision: 		The document was not treated.

### 5.13	NR inter-band CA for 5 bands DL with x bands UL (x=1, 2)

#### 5.13.1	UE RF requirements

R4-2200222	TP for TR 38.717-05-01: CA_n3-28-n77-n79-n257

Type: pCR		For: Approval
					38.717-05-01 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

### 5.14	DC of 1 LTE band and 1 NR band

R4-2201362	TR 37.717-11-11 v1.1.0 Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)

Type: draft TR		For: Approval
					37.717-11-11 v1.0.0	  CR-  rev  Cat:  (Rel-17)

					Source: CHTTL

Decision: 		The document was not treated.

#### 5.14.1	EN-DC requirements without FR2 band

R4-2200185	Draft CR for 38.101-3: corrections of MOP in DC_42C_n1A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: SoftBank Corp.

Abstract:

A missed MOP for the combo is added.

Decision: 		The document was not treated.

R4-2200237	TP update for TR 37.717-11-11: DC_11_n1

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200827	TP for TR 37.717-11-11: DC_n28A_8C

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200828	TP for TR 37.717-11-11: DC_n41_39

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200829	TP for TR 37.717_11_11 DC_n41_3

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200830	TP for TR 37.717_11_11 DC_n41_8

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200831	TP for TR 37.717_11_11 DC_n41_40

Type: pCR		For: Approval
					37.717-11-11 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2201051	TP for TR 37.717-11-11 DC_5_n3

Type: pCR		For: Approval
					37.717-11-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Reliance Jio

Decision: 		The document was not treated.

R4-2201091	draftCR to add DC_3C_n20A to 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201557	TP for TR 37.717-11-11 to include DC_5_n1

Type: pCR		For: Approval
					37.717-11-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Airtel

Abstract:

TP for TR 37.717-11-11 to include DC_5_n1

Decision: 		The document was not treated.

R4-2201558	TP for TR 37.717-11-11 to include DC_5_n3

Type: pCR		For: Approval
					37.717-11-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Airtel

Abstract:

TP for TR 37.717-11-11 to include DC_5_n3

Decision: 		The document was not treated.

R4-2201728	draft CR 38.101-3 to add new configurations for DC_2_n78 and DC_71_n78

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Rogers

Abstract:

draft CR 38.101-3 to add new configurations for DC_2_n78 and DC_71_n78

Decision: 		The document was not treated.

#### 5.14.2	EN-DC requirements with FR2 band

### 5.15	DC of 2 LTE band and 1 NR band

R4-2200367	TP for TR 37.717-21-11 DC_n3A_1A-8A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200368	TP for TR 37.717-21-11: DC_n77A_1A-8A and DC_n77(2A)_1A-8A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200369	TP for TR 37.717-21-11 DC_n77A_3A_1A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200370	TP for TR 37.717-21-11 DC_n77A_3A-8A and DC_n77(2A)_3A-8A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200371	TP for TR 37.717-21-11: DC_n257A_3A-1A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200372	TP for TR 37.717-21-11 DC_n257A_3A-8A

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

#### 5.15.1	EN-DC requirements without FR2 band

R4-2200186	Draft CR for 38.101-3: support of n77(3A) in 2LTE+1NR DC_1A/3A/8A/11A_n77

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

n77(3A) support is added to 2 LTE + 1NR DC of 1A/3A/8A/11A_n77.

Decision: 		The document was not treated.

R4-2200235	TP for TR 37.717-21-11: EN-DC_8-41_n1

Type: pCR		For: Approval
					37.717-21-11 v0.7.1	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp., LG Electronics

Decision: 		The document was not treated.

R4-2200375	TP for TR 37.717-21-11: DC_1A_(n)3AA

Type: discussion		For: Approval
					37.717-21-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2200608	CR for TS 38.101-3: Adding same note for higher order combo of DC_20_n28

Type: CR		For: Endorsement
					38.101-3 v17.4.0	  CR-0672  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

This CR is for endorsement

Decision: 		The document was not treated.

R4-2201089	draftCR to add DC_n1A-n7A-n78A to 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201095	TP to TR 37.717-21-11 Addition of DC_2A-38A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R4-2201096	TP to TR 37.717-21-11 Addition of DC_2A-28A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R4-2201097	draftCR to add DC_2-7_n28 and DC_5-66_n78 to 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R4-2201356	TP for TR 37.717-21-11_DC_1A-38A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201357	TP for TR 37.717-21-11_DC_7A-38A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201573	TP for TR 37.717-21-11 to include DC_2-7_n25

Type: pCR		For: Approval
					37.717-21-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 37.717-21-11 to include DC_2-7_n25

Decision: 		The document was not treated.

R4-2201729	draft CR 38.101-3 to add new configurations for DC_12-66_n78, DC_2-71_n78, DC_7-12_n78, DC_7-71_n78, DC_66-71_n78

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Rogers

Abstract:

draft CR 38.101-3 to add new configurations for DC_12-66_n78, DC_2-71_n78, DC_7-12_n78, DC_7-71_n78, DC_66-71_n78

Decision: 		The document was not treated.

R4-2202005	TP update for TR 37.717-21-11: EN-DC_1-11_n79

Type: pCR		For: Approval
					37.717-21-11 v0.7.1	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp., LG Electronics

Decision: 		The document was not treated.

R4-2202007	TP update for TR 37.717-21-11: EN-DC_8-11_n79

Type: pCR		For: Approval
					37.717-21-11 v0.7.1	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp., LG Electronics

Decision: 		The document was not treated.

#### 5.15.2	EN-DC requirements with FR2 band

### 5.16	DC of 3 LTE band and 1 NR band

R4-2200373	TP for TR 37.717-31-11  DC_n77A_1A-3A-8A and DC_n77(2A)_1A-3A-8A

Type: discussion		For: Approval
					37.717-31-11 v	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

#### 5.16.1	EN-DC requirements without FR2 band

R4-2200609	CR for TS 38.101-3: Adding same note for higher order combo of DC_20_n28

Type: CR		For: Endorsement
					38.101-3 v17.4.0	  CR-0673  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

This CR is for endorsement

Decision: 		The document was not treated.

R4-2200623	Draft CR for TS 38.101-3 to add UL DC_20A_n78A support for DC_1A-20A-38A_n78A and DC_3A-20A-38A_n78A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide draft CR for TS 38.101-3 to add UL DC_20A_n78A support for DC_1A-20A-38A_n78A and DC_3A-20A-38A_n78A

Decision: 		The document was not treated.

R4-2200624	TP for TR 37.717-31-11_DC_1A-3A-38A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide TP for TR 37.717-31-11_DC_1A-3A-38A_n78A

Decision: 		The document was not treated.

R4-2200713	TP to TR 37.717-31-11: DC_2-5-48_n77

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200714	TP to TR 37.717-31-11: DC_5-48-66_n77

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200715	TP to TR 37.717-31-11: DC_2-13-48_n77

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2201059	Draft CR for 38.101-3 to introduce DC_7A-7A-29A-66A_n78A and DC_7C-29A-66A_n78A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201060	TP for TR 37.717-31-11 DC_2-7-29_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201098	TP to TR 37.717-31-11 Addition of DC_2-7-28_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R4-2201099	TP to TR 37.717-31-11 Addition of DC_5-7-66_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia

Decision: 		The document was not treated.

R4-2201233	Draft CR for 38.101-3 To configuration DC_3C-7A-20A_n28A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201234	Draft CR for 38.101-3 To configuration DC_3C-20A-32A_n1A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201235	Draft CR for 38.101-3 To add UL configuration DC_3C_n78A for DC_3C-7A-20A_n78A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201236	Draft CR for 38.101-3 To add UL configuration DC_3C_n28A for DC_1A-3C-20A_n28A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201358	TP for TR 37.717-31-11_DC_1A-7A-38A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201359	TP for TR 37.717-31-11_DC_7A-20A-38A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201574	TP for TR 37.717-31-11 to include DC_2-7-66_n25

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 37.717-31-11 to include DC_2-7-66_n25

Decision: 		The document was not treated.

R4-2201575	TP for TR 37.717-31-11 to include DC_2-7-13_n25

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 37.717-31-11 to include DC_2-7-13_n25

Decision: 		The document was not treated.

R4-2201730	TP for TR 37.717-31-11 to include DC_2-5-7_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-31-11 to include DC_2-5-7_n78

Decision: 		The document was not treated.

R4-2201731	TP for TR 37.717-31-11 to include DC_2-5-66_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-31-11 to include DC_2-5-66_n78

Decision: 		The document was not treated.

R4-2201732	TP for TR 37.717-31-11 to include DC_5-7-66_n78

Type: pCR		For: Approval
					37.717-31-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-31-11 to include DC_5-7-66_n78

Decision: 		The document was not treated.

#### 5.16.2	EN-DC requirements with FR2 band

R4-2201903	draft CR 38101-3  to add DC_2A-2A-5A-66A_n260M and DC_2A-2A-5A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_2A-2A-5A-66A_n260M and DC_2A-2A-5A-66A_n260M

Decision: 		The document was not treated.

R4-2201904	draft CR 38101-3  to add DC_2A-2A-12A-30A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_2A-2A-12A-30A_n260M

Decision: 		The document was not treated.

R4-2201905	draft CR 38101-3  to add DC_2A-2A-12A-66A_n260M and DC_2A-12A-66A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_2A-2A-12A-66A_n260M and DC_2A-12A-66A-66A_n260M

Decision: 		The document was not treated.

R4-2201906	draft CR 38101-3  to add DC_5A-30A-66A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_5A-30A-66A-66A_n260M

Decision: 		The document was not treated.

R4-2201907	draft CR 38101-3  to add DC_12A-30A-66A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_12A-30A-66A-66A_n260M

Decision: 		The document was not treated.

R4-2201908	draft CR 38101-3  to add DC_12A-30A-66A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_12A-30A-66A-66A_n260M

Decision: 		The document was not treated.

R4-2201961	draft CR 38101-3  to add DC_2A-2A-5A-30A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3  to add DC_2A-2A-5A-30A_n260M

Decision: 		The document was not treated.

### 5.17	DC of 4 LTE band and 1 NR band

#### 5.17.1	EN-DC requirements without FR2 band

R4-2200610	CR for TS 38.101-3: Adding same note for higher order combo of DC_20_n28

Type: CR		For: Endorsement
					38.101-3 v17.4.0	  CR-0674  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

This CR is for endorsement

Decision: 		The document was not treated.

R4-2200622	Draft CR for TS 38.101-3 to add UL DC_1A_n78A support for DC_1A-3A-20A-38A_n78A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide draft CR for TS 38.101-3 to add UL DC_1A_n78A support for DC_1A-3A-20A-38A_n78A

Decision: 		The document was not treated.

R4-2201061	TP for TR 37.717-41-11 DC_2-7-29-66_n78

Type: pCR		For: Approval
					37.717-41-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung, Telus, Bell Mobility

Decision: 		The document was not treated.

R4-2201733	TP for TR 37.717-41-11 to include DC_2-5-7-66_n78

Type: pCR		For: Approval
					37.717-41-11 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-41-11 to include DC_2-5-7-66_n78

Decision: 		The document was not treated.

#### 5.17.2	EN-DC requirements with FR2 band

R4-2201909	draft CR 38101-3 to add DC_2A-5A-30A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3 to add DC_2A-5A-30A-66A_n260M

Decision: 		The document was not treated.

R4-2201910	draft CR 38101-3 to add DC_2A-12A-30A-66A_n260M

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 38101-3 to add DC_2A-12A-30A-66A_n260M

Decision: 		The document was not treated.

### 5.18	DC of 5 bands LTE inter-band CA (5DL/1L) and 1 NR band (1DL/1UL)

#### 5.18.1	UE RF requirements

### 5.19	DC of x bands (x=1,2, 3, 4) LTE inter-band CA and 2 bands NR inter-band CA

R4-2200789	TR 37.717-11-21 v0.8.0 TR update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17

Type: draft TR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Provide TR to update DC band combinations for LTE(xDL/1UL)+ NR(2DL/1UL) in Rel-17

Decision: 		The document was not treated.

R4-2200791	Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) DC in Rel-17

Type: WID revised		For: Endorsement
					Source: LG Electronics France

Abstract:

Revised WID to update the status on each DC band combination for LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) DC in Rel-17.

Decision: 		The document was not treated.

R4-2200792	Introduction CR on new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) band combinations in Rel-17

Type: CR		For: Agreement
					38.101-3 v17.4.0	  CR-0675  rev  Cat: B (Rel-17)

					Source: LG Electronics France

Abstract:

Big CR to introduce new NR DC band combinations in TS38.101-3 in Rel-17.

Decision: 		The document was not treated.

#### 5.19.1	EN-DC requirements including NR inter CA without FR2 band

R4-2200225	TP for TR 37.717-11-21: EN-DC_1-8_n28-n79

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200226	TP for TR 37.717-11-21: EN-DC_1-8_n77-n79

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200227	TP for TR 37.717-11-21: EN-DC_3-8_n77-n79

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200228	TP for TR 37.717-11-21: EN-DC_8_n1-n3

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200231	TP for TR 37.717-11-21: EN-DC_8-41_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200232	TP for TR 37.717-11-21: EN-DC_8-42_n1-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200233	TP for TR 37.717-11-21: EN-DC_41_n1-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200234	TP for TR 37.717-11-21: EN-DC_42_n1-n3

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200236	TP for TR 37.717-11-21: EN-DC_11_n3-n79

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200716	TP to TR 37.717-11-21: DC_13-66_n5-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200717	TP to TR 37.717-11-21: DC_2-66_n5-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200718	TP to TR 37.717-11-21: DC_2-13_n66-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200719	TP to TR 37.717-11-21: DC_2-66_n66-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200720	TP to TR 37.717-11-21: DC_2-5-66_n66-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200721	TP to TR 37.717-11-21: DC_2-2-13-66_n66-n77

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200722	TP to TR 38.717-05-01: CA_n2-n5-n48-n66-n77

Type: pCR		For: Approval
					38.717-05-01 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200723	TP to TR 38.717-04-02: CA_n2-n5-n66-n77

Type: pCR		For: Approval
					38.717-04-02 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200725	draftCR 38.101-3 introduction of 2LTE and 2NR bandcombinations

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2200794	MSD test results and test configurations for new NR DC band combinations

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Add MSD test results and test configurations for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17.

Decision: 		The document was not treated.

R4-2201092	draftCR to add DC_3_n1A-n78(2A) to 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201093	draftCR to add DC_3_n28A-n78(2A) to 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, BT

Decision: 		The document was not treated.

R4-2201237	TP for TR 37.717-11-21:  DC_3A-32A_n1A-n28A and  DC_3C-32A_n1A-n28A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201238	TP for TR 37.717-11-21:  DC_20A-32A_n1A-n28A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201239	TP for TR 37.717-11-21:  DC_3A-20A-32A_n1A-n28A and  DC_3C-20A-32A_n1A-n28A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201240	TP for TR 37.717-11-21:  DC_1A_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201241	TP for TR 37.717-11-21:  DC_3A_n28A-n75A and DC_3C_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201242	TP for TR 37.717-11-21:  DC_1A-3A_n28A-n75A and DC_1A-3C_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201243	TP for TR 37.717-11-21:  DC_3A-20A_n28A-n75A and DC_3C-20A_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201244	TP for TR 37.717-11-21:  DC_1A-20A_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201245	TP for TR 37.717-11-21:  DC_1A-3A-20A_n28A-n75A and DC_1A-3C-20A_n28A-n75A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201246	Draft CR for 38.101-3 to add UL configuration DC_3C_n1A and DC_3C_n78A for DC_3C-7C_n1A-n78A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201350	TP for TR 37.717-11-21:  DC_1A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This band combination has existed in current Spec 38.101-3 v17.4.0. Moderator recommends it is noted.

Decision: 		The document was not treated.

R4-2201351	TP for TR 37.717-11-21_DC_1A-38A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201352	TP for TR 37.717-11-21_DC_7A-38A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201353	TP for TR 37.717-11-21_DC_20A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This band combination has existed in current Spec 38.101-3 v17.4.0. Moderator recommends it is noted.

Decision: 		The document was not treated.

R4-2201354	TP for TR 37.717-11-21_DC_20A-38A_n3A-n78A.

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201355	TP for TR 37.717-11-21_DC_38A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201519	TP for TR 37.717-11-21: DC_2-7-66_n66-n77

Type: pCR		For: Approval
					37.717-11-21 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon, Bell Mobility, Telus

Decision: 		The document was not treated.

R4-2201570	TP for TR 37.717-11-21 to include DC_20_n1-n67

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 37.717-11-21 to include DC_20_n1-n67

Decision: 		The document was not treated.

R4-2201571	TP for TR 37.717-11-21 to include DC_20_n3-n67

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 37.717-11-21 to include DC_20_n3-n67

Decision: 		The document was not treated.

R4-2201572	TP for TR 37.717-11-21 to include DC_3_n20-n67

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, BT plc

Abstract:

TP for TR 37.717-11-21 to include DC_3_n20-n67

Decision: 		The document was not treated.

R4-2201576	TP for TR 37.717-11-21 to include DC_2_n25-n66

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 37.717-11-21 to include DC_2_n25-n66

Decision: 		The document was not treated.

R4-2201577	TP for TR 37.717-11-21 to include DC_2-66_n25-n66

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 37.717-11-21 to include DC_2-66_n25-n66

Decision: 		The document was not treated.

R4-2201734	draft CR 38.101-3 to add new configurations for DC_2_n66-n78 and DC_2_n71-n78

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Rogers

Abstract:

draft CR 38.101-3 to add new configurations for DC_2_n66-n78 and DC_2_n71-n78

Decision: 		The document was not treated.

R4-2201735	TP for TR 37.717-11-21 to include DC_5_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_5_n2-n78

Decision: 		The document was not treated.

R4-2201736	TP for TR 37.717-11-21 to include DC_66_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_66_n2-n78

Decision: 		The document was not treated.

R4-2201737	TP for TR 37.717-11-21 to include DC_12_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_12_n2-n78

Decision: 		The document was not treated.

R4-2201738	TP for TR 37.717-11-21 to include DC_2-5_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-5_n66-n78

Decision: 		The document was not treated.

R4-2201739	TP for TR 37.717-11-21 to include DC_2-5_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-5_n2-n78

Decision: 		The document was not treated.

R4-2201740	TP for TR 37.717-11-21 to include DC_2-66_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-66_n2-n78

Decision: 		The document was not treated.

R4-2201741	TP for TR 37.717-11-21 to include DC_5-66_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_5-66_n2-n78

Decision: 		The document was not treated.

R4-2201742	TP for TR 37.717-11-21 to include DC_2-7_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-7_n2-n78

Decision: 		The document was not treated.

R4-2201743	TP for TR 37.717-11-21 to include DC_2-7_n71-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-7_n71-n78

Decision: 		The document was not treated.

R4-2201744	TP for TR 37.717-11-21 to include DC_2-12_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-12_n2-n78

Decision: 		The document was not treated.

R4-2201745	TP for TR 37.717-11-21 to include DC_2-12_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-12_n66-n78

Decision: 		The document was not treated.

R4-2201746	TP for TR 37.717-11-21 to include DC_2-66_n71-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-66_n71-n78

Decision: 		The document was not treated.

R4-2201747	TP for TR 37.717-11-21 to include DC_2-71_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-71_n2-n78

Decision: 		The document was not treated.

R4-2201748	TP for TR 37.717-11-21 to include DC_2-71_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_2-71_n66-n78

Decision: 		The document was not treated.

R4-2201749	TP for TR 37.717-11-21 to include DC_5-7_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_5-7_n2-n78

Decision: 		The document was not treated.

R4-2201750	TP for TR 37.717-11-21 to include DC_5-7_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_5-7_n66-n78

Decision: 		The document was not treated.

R4-2201751	TP for TR 37.717-11-21 to include DC_7-12_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-12_n2-n78

Decision: 		The document was not treated.

R4-2201752	TP for TR 37.717-11-21 to include DC_7-12_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-12_n66-n78

Decision: 		The document was not treated.

R4-2201753	TP for TR 37.717-11-21 to include DC_7-66_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-66_n2-n78

Decision: 		The document was not treated.

R4-2201754	TP for TR 37.717-11-21 to include DC_7-66_n71-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-66_n71-n78

Decision: 		The document was not treated.

R4-2201755	TP for TR 37.717-11-21 to include DC_7-71_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-71_n2-n78

Decision: 		The document was not treated.

R4-2201756	TP for TR 37.717-11-21 to include DC_7-71_n66-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_7-71_n66-n78

Decision: 		The document was not treated.

R4-2201757	TP for TR 37.717-11-21 to include DC_12-66_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_12-66_n2-n78

Decision: 		The document was not treated.

R4-2201758	TP for TR 37.717-11-21 to include DC_66-71_n2-n78

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include DC_66-71_n2-n78

Decision: 		The document was not treated.

#### 5.19.2	EN-DC requirements including NR inter CA with FR2 band

R4-2200793	TP on summary of self-interference analysis for new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17

Type: pCR		For: Approval
					37.717-11-21 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Add the self-interference analysis results in TR37.717-11-21.

Decision: 		The document was not treated.

### 5.20	DC of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA

#### 5.20.1	UE RF requirements

### 5.21	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)

#### 5.21.1	UE RF requirements

R4-2200187	Draft CR for 38.101-3: the support of n77(2A) in DC_8A_n3A-n77-n79A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: SoftBank Corp.

Abstract:

n77(2A) support is added to DC_8A_n3A-n77-n79A.

Decision: 		The document was not treated.

R4-2200224	TP for TR 37.716-11-31: EN-DC_1-8_n3-n77-n79

Type: pCR		For: Approval
					37.717-11-31 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200229	TP for TR 37.717-11-31: EN-DC_8_n3-n28-n79

Type: pCR		For: Approval
					37.717-11-31 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2200230	TP for TR 37.717-11-31: EN-DC_8_n28-n77-n79

Type: pCR		For: Approval
					37.717-11-31 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2201438	TP for TR 37.717-11-31: support of DC_3_n1-n8-n78, DC_3-3_n1-n8-n78, DC_7_n1-n8-n78, DC_7-7_n1-n8-n78

Type: pCR		For: Approval
					37.717-11-31 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CHTTL

Decision: 		The document was not treated.

R4-2201439	TP for TR 37.717-11-31: support of DC_3-7_n1-n8-n78, DC_3-3-7_n1-n8-n78, DC_3-7-7_n1-n8-n78, DC_3-3-7-7_n1-n8-n78

Type: pCR		For: Approval
					37.717-11-31 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CHTTL

Decision: 		The document was not treated.

### 5.22	DC of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL)

#### 5.22.1	UE RF requirements

### 5.23	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 4 bands NR inter-band CA (4DL/1UL)

#### 5.23.1	UE RF requirements

R4-2200223	TP for TR 37.717-11-41: EN-DC_1_n3-n28-n77-n79

Type: pCR		For: Approval
					37.717-11-41 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

### 5.24	Band combinations for SA NR supplementary uplink (SUL) NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)

#### 5.24.1	UE RF requirements

R4-2200188	CR for TS 38.101-1: Applicability correction for SUL_n41-n97

Type: CR		For: Endorsement
					38.101-1 v17.4.0	  CR-0985  rev  Cat: F (Rel-17)

					Source: MediaTek Inc.

Abstract:

This CR is for endorsement

Decision: 		The document was withdrawn.

### 5.25	Band combinations for Uu and V2X con-current operation

#### 5.25.1	UE RF requirements

R4-2200144	TR37.875, Band combinations of V2X con-current operation

Type: draft TR		For: Approval
					37.875 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Abstract:

[Email Approval]

Decision: 		The document was not treated.

R4-2200145	TP on coexistence study of V2X_n8A-n47A, V2X_8A_n47A and V2X_n8A_47A

Type: pCR		For: Approval
					37.875 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200146	Draft CR for TS 38.101-1, Introduce new band combinations of V2X_n8A-n47A

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200147	Draft CR for TS 38.101-3, Introduce new band combination of V2X_n8A_47A and V2X_8A_n47A

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200508	Calculation of MSD for V2X_1A-n47A and V2X_n1A-47A and accompanying TP

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Outlines MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A

Decision: 		The document was not treated.

### 5.26	Adding channel bandwidth support to existing NR bands

#### 5.26.1	UE RF requirements

R4-2200922	On new channel bandwidths narrower than 40 MHz of n79

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200923	Draft LS on CORESET#0 impact of CBW narrower than 40MHz of n79

Type: LS out		For: Approval
					to RAN1, cc RAN2
					Source: Samsung

Decision: 		The document was not treated.

##### 5.26.1.1	Addition of bandwidth and Tx/Rx requirements

R4-2201335	Equation Based for FR1 Minimum Output Power

Type: other		For: Approval
					Source: ZTE Corporation, Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2201336	Draft CR to TS38.101-1: Some corrections for the tables due to introduction of 35MHz_45MHz CBW

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: ZTE Corporation, Skyworks Solutions Inc.

Decision: 		The document was not treated.

##### 5.26.1.2	NR-U 100MHz bandwidth

R4-2200177	Further discussion on co-existence proposal between NR-U 100 MHz channel raster and Wi-Fi in 5 GHz (n46)

Type: Work Plan		For: Approval
					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200437	On intra-carrier guard bands for the 100MHz NR-U channel

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2200505	NR-U Punctured Channel SEM for 100 MHz Bandwidth

Type: discussion		For: Approval
					Source: CableLabs

Decision: 		The document was not treated.

#### 5.26.2	BS RF requirements

### 5.27	Introduction of bandwidth combination set 4 (BCS4) for NR

#### 5.27.1	UE RF requirements for BCS4/BCS5

R4-2200462	Handling of maximum aggregated channel bandwidth for BCS4/5

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses handling of maximum aggregated channel bandwidth for BCS4/5.

Decision: 		The document was not treated.

R4-2200463	MSD table improvements

Type: other		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses handling of MSD table based on the approved WF of [R4-2119878].

Decision: 		The document was not treated.

R4-2200464	Draft CR: Clarification on no simultaneous signalling of BCS4 and 5 for 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

The draft CR clarifies that BCS4 and BCS5 are not allowed to be signalled simultaneously.

Decision: 		The document was not treated.

R4-2200465	Draft CR: Clarification on no simultaneous signalling of BCS4 and 5 for 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

The draft CR clarifies that BCS4 and BCS5 are not allowed to be signalled simultaneously.

Decision: 		The document was not treated.

R4-2200619	Draft CR to TS 38.307 on Release independence of band combination set 4 and 5

Type: draftCR		For: Endorsement
					38.307 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide draft CR to TS 38.307 on Release independence of band combination set 4 and 5

Decision: 		The document was not treated.

R4-2201251	Discussion on simplifying extended MSD table

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201252	Draft CR for 38.307 to introduce release independent method for BCS4/5

Type: draftCR		For: Endorsement
					38.307 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201296	TP for TR 38.862 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

Type: pCR		For: Approval
					38.862 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2202040	Discussion on NR-CA and EN-DC MSD Table Simplifications

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

#### 5.27.2	Discussion of LS on NR CA capability for BCS5 (R2-1209073)

R4-2201295	Reply LS on NR CA capability for BCS5

Type: LS out		For: Approval
					to RAN2
					Source: Xiaomi

Decision: 		The document was not treated.

### 5.28	Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths

#### 5.28.1	UE RF requirements

R4-2201253	Draft CR for 38.101-3 to introduce MSD requirements for missing bandwidths

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

### 5.29	High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, Band 13, Band n5, Band n13, and Band n71

#### 5.29.1	General

R4-2200703	TR 37.828 v0.1.0

Type: draft TR		For: Approval
					37.828 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

In the Copyright Notification section, update the copyright year to 2022.

Decision: 		The document was not treated.

#### 5.29.2	Feasibility study

##### 5.29.2.1	Coexistence study between B5 and adjacent bands

R4-2200410	TP to TR 37.828: Coexistence study for High-power UE Vs adjacent channel Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 5 and Band n5

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides some discussion on the coexistence studies between HPUE in Band 5 and adjacent channel public safety operation in the same geographical area, and a text proposal is provided below to record the discussion into the TR 37.828 [2]

Decision: 		The document was not treated.

##### 5.29.2.2	Coexistence study between B13/n13 and adjacent bands

##### 5.29.2.3	Filter with smaller duplex for B13, n13 and n71

##### 5.29.2.4	PA related to MPR and A-MPR for B13, n13, and n71

#### 5.29.3	UE RF requirements

##### 5.29.3.1	UE REFSENS

##### 5.29.3.2	UE Tx requirements (MOP, MPR, A-MPR, and ACLR)

R4-2200704	MPR study for PC1 FWA device

Type: pCR		For: Approval
					37.828 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

### 5.30	High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink

R4-2200768	Draft TR 38.841 v0.6.0: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

Type: draft TR		For: Approval
					38.841 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Abstract:

[Email Approval] draft TR for email approval

Decision: 		The document was not treated.

R4-2200769	Draft CR to 38.101-1 Introduce RF requirements for HPUE CA with 2 bands downlink and x bands uplink (x =1,2)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: China Telecom

Abstract:

draft big CR for email approval

Decision: 		The document was not treated.

#### 5.30.1	UE RF requirements

R4-2200304	DraftCR to 38.101-1 for HPUE CA with 2 bands downlink and x bands uplink (x =1,2)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Verizon Denmark

Decision: 		The document was withdrawn.

R4-2201225	Draft CR to 38.101-1 corrections on Tx power configuration on IMD requirement for high power UE inter-band CA

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Xiaomi

Abstract:

For PC2 IMD requirments, MSD value is derived by assuming 3dB higher Tx power that that for PC3, therefore current Tx power configuration shall be change to min(+23 dBm, PCMAX_L,f,c) for PC2 case.

Decision: 		The document was not treated.

R4-2201680	TP for TR 38.841 Addition of CA_n29-n77

Type: pCR		For: Approval
					38.841 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: AT&T

Decision: 		The document was not treated.

R4-2201717	DraftCR to 38.101-1 for HPUE CA with 2 bands downlink and x bands uplink (x =1,2)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Verizon, AT&T

Decision: 		The document was not treated.

R4-2202041	Power Class notation for BCSs

Type: discussion		For: Approval
					Source: T-Mobile USA

Decision: 		The document was not treated.

R4-2202042	Draft CR for 38.101-1:  Addition of PC2 and PC1.5 for DL CA_n41(2A) UL n41

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: T-Mobile USA

Decision: 		The document was not treated.

R4-2202043	Draft CR for 38.101-1:  Addition of PC2 and PC1.5 for combinations with n25 and n41

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: T-Mobile USA

Decision: 		The document was not treated.

R4-2202044	Draft CR for 38.101-1:  Addition of PC2 and PC1.5 for combinations with n41 and n66

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: T-Mobile USA

Decision: 		The document was not treated.

R4-2202045	Draft CR for 38.101-1:  Addition of PC2 and PC1.5 for combinations with n41 and n71

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: T-Mobile USA

Decision: 		The document was not treated.

### 5.31	High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

#### 5.31.1	UE RF requirements

R4-2200305	TP for TR 37.826 for DL DC_48_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Decision: 		The document was not treated.

### 5.32	Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL

R4-2202020	MSD update needed for PC2 and PC1.5 UL configurations

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In R4#101 e-meeting two CRs [1,2] added single UL PC2 and PC1.5 configurations to the intra-band and inter-band RC CA combinations and configurations. At this point it is unclear if a new or increased MSD has been considered for these higher power UL conf

Decision: 		The document was not treated.

#### 5.32.1	UE RF requirements

### 5.33	Power Class 2 for EN-DC with xLTE band + yNR DL with 1LTE+1(TDD) NR UL band (x= 2, 3, 4, y=1; x=1, 2, y=2)

#### 5.33.1	UE RF requirements

R4-2200303	DraftCR to add EN-DC PC2 band combinations with more than 2 bands

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Verizon Denmark

Decision: 		The document was not treated.

R4-2200306	TP for TR 37.827 for DC_2-5_n2-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200308	TP for TR 37.827 for DC_2-5_n66-n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Decision: 		The document was not treated.

R4-2200309	TP for TR 37.827 for DC_2-5-66_n2-n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Decision: 		The document was not treated.

R4-2200310	TP for TR 37.827 for DC_2-5-66_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200311	TP for TR 37.827 for DC_2-5-66_n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200313	TP for TR 37.827 for DC_2-13_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200314	TP for TR 37.827 for DC_2-13-66_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200315	TP for TR 37.827 for DC_2-13-66_n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200316	TP for TR 37.827 for DC_2-66_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200317	TP for TR 37.827 for DC_5-66_n2-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200318	TP for TR 37.827 for DC_5-66_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200319	TP for TR 37.827 for DC_5-66_n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2200320	TP for TR 37.827 for DC_13-66_n66-n77

Type: discussion		For: Approval
					Source: Verizon, Samsung

Decision: 		The document was not treated.

R4-2201684	DraftCR 38.101-3 Addition of PC2 EN-DC Combinations

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: AT&T

Decision: 		The document was not treated.

### 5.34	High power UE for NR TDD intra-band carrier aggregation in frequency range FR1

#### 5.34.1	UE RF requirements

### 5.35	Increasing UE power high limit for CA and DC

#### 5.35.1	General and work plan

#### 5.35.2	Feasibility and impact study

R4-2200454	Comparison between variants of “the sum method”

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution addresses the issues captured in the approved WF of R4-2120064.

Decision: 		The document was not treated.

R4-2200494	Implementation aspects of increasing MOP for PC2 inter-band ULCA

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we further examine the different PC2 inter-band cases with a focus on the cases where one band is based on a 2Tx PC2 or PC1.5 implementation.

Decision: 		The document was not treated.

R4-2200852	Increasing UE power high limit for CA and DC by existing signaling

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose to reuse existing signaling mechanism to increase the power limit for CA and DC

Decision: 		The document was not treated.

R4-2200965	Further discussion on the increasing UE power high limit for CA and DC

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201278	R17 UE power class high limit

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201334	Further discussion on increase UE maximum power for NR uplink inter band CA

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201836	On Increasing MOP for NR inter-band CA

Type: discussion		For: Agreement
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

#### 5.35.3	UE RF requirements

R4-2200440	UE maximum output power for inter-band UL CA

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200455	UE RF requirements for the sum method

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution shares concrete examples of the requirements for PC2+PC3 based on the respective option 2 and 3 [R4-2200454].

Decision: 		The document was not treated.

R4-2201229	Discussion on increasing UE maximum power high limit

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201265	RF requirements impact for Increased MOP for CA and DC

Type: other		For: Approval
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R4-2201856	Higher output power for CA and DC

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

### 5.36	High power UE (power class 2) for NR FDD band

#### 5.36.1	General and work plan

#### 5.36.2	UE RF requirements

R4-2200908	Hybrid duplex operation for PC2 FDD bands

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2201226	Discussion on HP UE for FDD bands

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

##### 5.36.2.1	UE maximum output power and power tolerance

##### 5.36.2.2	A-MPR requirements

R4-2200444	HPUE A-MPR Proposal for NS_05

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200445	HPUE A-MPR Proposal for NS_48

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200446	HPUE A-MPR Proposal for NS_49

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200447	HPUE A-MPR Proposal for NS_100

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2201834	On A-MPR requirements for PC2 FDD bands

Type: discussion		For: Agreement
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2202008	A-MPR for Bandwidth Parts

Type: discussion		For: Discussion
					Source: Lenovo, Motorola Mobility

Decision: 		The document was not treated.

##### 5.36.2.3	PC2 MSD requirements (investigation for HD-FDD)

R4-2200441	NR FDD HPUE MSD and system performance analyses

Type: other		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200964	Further discussion on half duplex operation under HPUE

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201070	MSD analysis for PC2 FDD UE

Type: discussion		For: Approval
					Source: LG Electronics Inc.

Abstract:

In this contribution, we provide expected sensitivity degradation for PC2 FDD UE in NR band n1 and n3.

Decision: 		The document was not treated.

R4-2201338	Discussion on HPUE FDD MSD

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201835	On MSD requirements for PC2 FDD bands

Type: discussion		For: Agreement
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

### 5.37	Additional NR bands for UL-MIMO

#### 5.37.1	UE RF requirements

R4-2201759	Draft CR for n24 and n99 UL-MIMO PC3

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ligado Networks

Decision: 		The document was not treated.

### 5.38	Downlink interruption for band combinations to conduct dynamic Tx Switching

R4-2200770	Draft CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: China Telecom

Abstract:

draft big CR for email approval

Decision: 		The document was not treated.

#### 5.38.1	Determination of inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed

### 5.39	Simultaneous Rx/Tx band combinations for CA, SUL, MR-DC and NR-DC

R4-2200566	Clarification on per-band-pair simultaneous RxTx capability

Type: other		For: Approval
					Source: NTT DOCOMO INC.

Decision: 		The document was not treated.

#### 5.39.1	MSD threshold principle

R4-2200354	Discussion on the simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination

Type: other		For: Approval
					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2201067	Discussion on simultaneous RxTx capability for FR1 FDD-TDD band combination

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201230	Discussion on principle for simultaneous Rx Tx band combinations for CA, SUL, MR-DC and NR-DC

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201340	Simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201954	Further consideration on the MSD principle for FR1 FDD-TDD band combination

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201955	TP for TR 38.839: Principles for simultaneous RxTx capability

Type: pCR		For: Approval
					38.839 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 5.39.2	FR2 band combinations with simultaneous Rx/Tx

R4-2200567	View on FR2 simultaneous Tx/Rx discussion

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal: Add NOTE to clarify the minimum requirements apply only when there is non-simultaneous Rx/Tx operation for CA_n257-n259 and CA_n258-n260, as CA_n260-n261.

Decision: 		The document was not treated.

R4-2201341	DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA

Type: draftCR		For: Endorsement
					38.101-2 v16.10.0	  CR-  rev  Cat: F (Rel-16)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201342	DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: A (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201343	Draft CR to TS38.101-2[R17] On Simultaneous RxTx capability for FR2 inter-band CA_n257-n259 and CA_n258-n260

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201956	Further consideration on the simultaneous Rx/Tx capability for FR2 TDD-TDD band combination

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

### 5.40	4Rx support for NR band n8

#### 5.40.1	UE RF requirements (delta_R_IB,4Rx)

R4-2201347	On band n8 support 4Rx

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201372	draft CR for 4 Rx antenna ports support of band n8

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CHTTL

Decision: 		The document was not treated.

#### 5.40.2	Release independency

R4-2201413	Discussion on release independent for 4Rx support for NR band

Type: discussion		For: Approval
					Source: CHTTL

Decision: 		The document was not treated.

## 6	Rel-17 non-spectrum related work items for NR

### 6.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs

#### 6.1.1	General

R4-2200966	3GPP TS 38.151 v0.7.0

Type: draft TS		For: Approval
					38.151 v0.7.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200970	Discussion on framework for FR1 MIMO OTA performance

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201920	Illustration of Device Orientations for Select Test Points

Type: pCR		For: Approval
					38.151 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was not treated.

#### 6.1.2	Performance requirements

##### 6.1.2.1	Performance Requirements for FR1

R4-2200572	On framework for PAD alignment of NR UE FR1 MIMO OTA

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2200576	FR1 MIMO OTA channel validation

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: Adopt PDP pass/fail limits as below table.

Proposal2: Define cross-polarization pass/fail limit as [±1dB].

Observation: Channel validation result are submitted for all listed items as one of Lab volunteers.

Decision: 		The document was not treated.

R4-2200969	Pass/Fail limit for FR1 MIMO OTA lab alignment activity

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201282	Commercial devices preparation and data handling

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201602	Framework for FR1 MIMO OTA lab alignment activity

Type: discussion		For: Discussion
					Source: CAICT

Decision: 		The document was not treated.

##### 6.1.2.2	Performance Requirements for FR2

R4-2200580	FR2 MIMO OTA Simulation

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

We submit our FR2 MIMO OTA simulation result based on fundamental scenario for alignment.

Decision: 		The document was not treated.

R4-2200777	Discussion on FR2 MIMO OTA requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200778	Summary results for FR2 MIMO OTA simulation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201441	Discussion FR2 MIMO OTA performance requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

##### 6.1.2.3	MU assessment for FR1 and FR2

R4-2200779	Discussion on preliminary MU assessment for FR2 MIMO OTA

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200968	Proposal for MU budget of FR1 MIMO OTA

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

#### 6.1.3	Testing methodologies

##### 6.1.3.1	Testing parameters for Performance

R4-2200780	TP on TS 38.151 for test parameters of FR2 performance

Type: pCR		For: Approval
					38.151 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 6.1.3.2	Optimization of test methodologies

R4-2200731	Max downlink power verification of MIMO OTA test system

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200967	TP to TS38.151 on FR2 maximum downlink power and test procedure

Type: pCR		For: Approval
					38.151 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

##### 6.1.3.3	Channel model validation

R4-2200409	TP to TS38.151 on FR1 Spatial Channel Model Validation

Type: other		For: Approval
					Source: Spirent Communications

Abstract:

Based on the agreements in WF [1], the text proposals for updating the FR1 channel model validation annex of [2] is provided in this contribution.

Decision: 		The document was not treated.

R4-2200573	Further results on FR1 channel model validation

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2200832	FR1 channel model validation results for CMCC & BUPT joint lab

Type: discussion		For: Decision
					Source: CMCC   BUPT

Decision: 		The document was not treated.

R4-2200906	FR1 MIMO OTA Lab Alignment, Channel Model Validation

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2201494	Validation results and limits for FR1 CDL-C UMa channel model-v1

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201591	FR1 MIMO OTA channel model validation results and views on PDP pass/fail limits

Type: discussion		For: Discussion
					Source: CAICT

Decision: 		The document was not treated.

R4-2201676	Reference Channel Emulation PDP for Validation Purposes for FR1 CDL-C UMa

Type: discussion		For: Approval
					Source: CAICT, CMCC, Keysight Technologies, Spirent Communications

Decision: 		The document was not treated.

R4-2201919	Pass/Fail Limits for FR1 Channel Model Validation

Type: discussion		For: Approval
					Source: Keysight Technologies UK Ltd

Decision: 		The document was not treated.

### 6.2	Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

R4-2202049	TR 38.834 v0.3.0

Type: draft TR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

#### 6.2.1	General and work plan

R4-2200732	Discussion on hand phantom handling in FR1 TRP TRS

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2200733	TP to TR 38.834 on band parameters

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2200971	3GPP TS 38.161 v0.1.0

Type: draft TS		For: Approval
					38.161 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200975	Discussion and Text Proposals on band parameters

Type: other		For: Approval
					38.834 v	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200977	TP to TR 38.834 on Phantom definition

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200981	Updated workplan of TRP TRS WI

Type: Work Plan		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201285	TP to TR 38.834 on multi antenna

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201606	Progress on MU and recommendations from RAN5#93-e

Type: discussion		For: Approval
					38.834 v	  CR-  rev  Cat:  (Rel-17)

					Source: ROHDE & SCHWARZ

Abstract:

This contribution provides a brief summary of the MU progress in RAN5, together with some recommendations to RAN4 work identified during the discussion.

Decision: 		The document was not treated.

R4-2201649	TP to TR 38.834 on skeleton for Annex B

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: ROHDE & SCHWARZ

Abstract:

This contribution provides the Text Proposal endorsed by RAN5 during RAN5#93-e on the structure of Annex B of TR 38.834 which will contain the MU estimation.

Decision: 		The document was not treated.

#### 6.2.2	Test methodology

R4-2200935	TP to TR 38.834 on performance metrics

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: CAICT

Abstract:

This paper further refines the text proposal on performance metrics.

Decision: 		The document was not treated.

##### 6.2.2.1	SA test methodology

R4-2200450	TP to TR 38.834 on P-MPR handling

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200972	TP to TR 38.834 on SA TRP TRS test procedure

Type: pCR		For: Approval
					38.834 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo, CAICT

Decision: 		The document was not treated.

##### 6.2.2.2	EN-DC test methodology

R4-2200449	Views on EN-DC methodology

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200574	on remaining details of EN-DC TRP test configuration

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2200734	Discussion on ENDC power splitting for TRS

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200785	Discussion on EN-DC test methodology

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200973	Discussion and Text Proposals on EN-DC TRP TRS test procedure

Type: other		For: Approval
					38.834 v	  CR-  rev  Cat:  (Rel-17)

					Source: vivo, CAICT

Decision: 		The document was not treated.

R4-2201495	On EN-DC power split for TRS

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

##### 6.2.2.3	UE with multiple antennas test methodology

R4-2200786	Discussion on TAS ON test methodology

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201284	On the influence factors of Tx antenna switch

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201286	Consideration on Tx Diversity

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

##### 6.2.2.4	Test time reduction

R4-2200980	Discussion on alternative test method for TRP TRS testing time reduction

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

#### 6.2.3	Performance requirements

R4-2201283	On hand phantom selection for Rel-17 requirement

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

##### 6.2.3.1	Framework for lab alignment and requirements

R4-2200448	Further views on manufacturing tolerances

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200575	On framework for NR UE TRP/TRS lab alignment and requirements

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2200976	Working procedure for TRP TRS requirement development

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

##### 6.2.3.2	SA requirements

R4-2200974	TP to TS 38.161 on requirement applicability

Type: pCR		For: Approval
					38.161 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

##### 6.2.3.3	EN-DC requirements

### 6.3	RF requirements enhancement for NR frequency range 1 (FR1)

#### 6.3.1	General

R4-2200019	draft CR: UL MIMO coherence for Tx switching

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: China Telecom

Decision: 		The document was not treated.

R4-2200499	Requirement and signaling aspect of features requiring two transmit paths

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we highlight all the relevant cases for varying feature implementations with two transmit paths (TxD, UL MIMO, intra-band ULCA) and their R17 status, while identifying missing requirements. We also provide further clarification and g

Decision: 		The document was not treated.

R4-2200611	Discussion on capability signalling for HPUE NR DC with UE supporting TxD

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200612	Draft LS on signalling clarification of NR CA/DC power class in R17

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200958	TP for TR 38.837 on Power Class clarification

Type: pCR		For: Approval
					38.837 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2201228	PC2 PA configuration and signalling

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201590	3GPP TR 38.837 v0.3.0

Type: draft TR		For: Approval
					38.837 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

#### 6.3.2	RF core requirements

##### 6.3.2.1	UL MIMO configuration for SUL band configurations

##### 6.3.2.2	HPUE for TDD intra-band contiguous UL CA

R4-2200334	Requirements for different architectures and their capabilities

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200497	Signaling for contiguous ULCA cases

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

Although the MPR values are correct, some signaling aspects related to enabling implementations with 2LO with two PC2 PAs require additional consideration. In this contribution, we provide further clarification and guidance on related signaling aspects.

Decision: 		The document was not treated.

R4-2201593	Draft CR TS 38.101-1 R17: Introduction of PC2 contiguous ULCA MPR for 2Tx

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

Correction of Signaling aspects and addition of 2Tx MPR.

Decision: 		The document was not treated.

##### 6.3.2.3	HPUE for TDD intra-band non-contiguous UL CA

R4-2200335	draft CR to remove dualPA from Rel-16 specs

Type: draftCR		For: Endorsement
					38.101-1 v16.10.0	  CR-  rev  Cat:  (Rel-16)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200336	2CC LO location reportting and dualPA capability in rel-16

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200498	Requirement and signaling aspect of non-contiguous ULCA

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

PC2 non-contiguous ULCA MPR for 1LO architectures were agreed in [1], there was also an open aspect on how to differentiate 1LO and 2LO architectures that have a different MPR requirement. In this contribution, we provide further clarification on related

Decision: 		The document was not treated.

R4-2201674	Draft CR TS 38.101-1 R17: Addition of PC2 non-contiguous ULCA MPR requirements

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

PC2 non-contiguous ULCA MPR is captured for 1LO and 2LO architectures based on WF agreements.

Decision: 		The document was not treated.

R4-2201943	Big CR for PC2 intra-band non-contiguous UL CA

Type: CR		For: Agreement
					38.101-1 v17.4.0	  CR-0987  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, Qualcomm

Decision: 		The document was not treated.

R4-2201944	Consideration on signalling to differentiate MPR for different architectures

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.3.2.4	Intra-band UL contiguous CA for UL MIMO (n41C and n78C)

R4-2200493	Signalling on PC2 intra-band NC UL CA for FR1

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses signalling aspects based on approved WF of [R4-2119955].

Decision: 		The document was not treated.

R4-2200495	MPR for PC2 intra-band contiguous ULCA&MIMO 26+23 case

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we provide additional measured data for 2Tx PC2 contiguous UL CA for 26+23dBm PA architecture, compare them to PC2 1Tx and 23+23 2Tx results and make MPR proposals.

Decision: 		The document was not treated.

R4-2200956	Discussion on MPR applicability and reference architectures for CA +UL MIMO and TxD

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201069	MPR and capability signaling for 2Tx PC2 intra-band contiguous UL CA with UL MIMO

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201270	R17 FR1 UL CA and UL MIMO MPR

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201800	DraftCR on TS 38.101-1 on ULCA + ULMIMO

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201946	On RF requirements for PC2 intra-band UL CA with UL MIMO

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201947	Big CR for TS 38.101-1: contiguous CA with UL MIMO for power class 2

Type: CR		For: Agreement
					38.101-1 v17.4.0	  CR-0988  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.3.2.5	Solution preventing transmission power dropping on cell with lower priority

R4-2200337	Solution for SCell dropping

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200957	Further discussion on Scell dropping

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

###### 6.3.2.5.1	FR1 related

R4-2200853	Further details on resolving the Scell dropping (power prioritization) problem by power limits

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Further background to the solution of the Scell power prioritization problems by means of serving cell power limits (both FR1 and FR2 explained)

Decision: 		The document was not treated.

R4-2200854	Introduction of power limits for serving cells of UL CA

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to introduce power limits for serving cells of UL CA to prevent power reduction of serving cells for power limited UEs when the power reduction is enabled (FR1)

Decision: 		The document was not treated.

R4-2201068	Discussion on transmission power dropping on cell with low priority

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201945	On SCell dropping

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

###### 6.3.2.5.2	FR2 related

R4-2200855	Introduction of power limits for serving cells of UL CA

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to introduce power limits for serving cells of UL CA to prevent power reduction of serving cells for power limited UEs when the power reduction is enabled (FR2)

Decision: 		The document was not treated.

#### 6.3.3	RRM core requirements

#### 6.3.4	RRM performance requirements

### 6.4	NR RF requirement enhancements for frequency range 2 (FR2)

#### 6.4.1	General

R4-2200699	TR38.851 v0.3.0

Type: draft TR		For: Approval
					38.851 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

In the Copyright Notification section, update the copyright year to 2022.

Decision: 		The document was not treated.

#### 6.4.2	UE RF requirements for inter-band CA

##### 6.4.2.1	Inter-band DL CA requirements

R4-2200361	Sensitivity requirements for inter-band CA with CBM

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Abstract:

To discuss PSD condition for sensitivity requirements.

Decision: 		The document was not treated.

R4-2200940	Discussion on introducation of Fs, inter_CBM

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201970	dCR to 38.101-2 on requirements for UEs that support inter-band CA with CBM

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Abstract:

Cat B feature CR in draft form

Decision: 		The document was not treated.

###### 6.4.2.1.1	CA configurations within the same frequency group based on CBM

R4-2200362	UE capability for CA within same frequency group with CBM

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Abstract:

To discuss UE capability for frequency separation with CBM.

Decision: 		The document was not treated.

R4-2200466	UE requirements for CBM for the same frequency group

Type: other		For: Decision
					Source: Sony, Ericsson

Decision: 		The document was not treated.

R4-2200554	Discussion on CBM based inter-band DL CA within same frequency group

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses RF requirements for CBM based inter-band DL CA.

Decision: 		The document was not treated.

R4-2200579	Reference signal and Fs_intern_CBM of FR2 inter-band DL CA within same frequency group based on CBM

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Observation: Reference signal discussion shall be applied for both “different frequency groups” and “within same frequency group” based on CBM. Hence, we explain and propose the details in our paper of agenda item “6.4.2.1.2 CA configurations between diff

Decision: 		The document was not treated.

R4-2200939	Discussion on CBM within same frequency group

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201275	R17 FR2 CBM inter-band DL CA

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201337	Discussion on CBM for FR2 Inter-band DL CA

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201968	On delta(RIB) for n258+n261 DL inter-CA

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Inter-band CA for band-pairs within the same frequency group is a special case, as it allows a single receiver shared across both bands to become attractive and practical. We provide analysis for an example band combination from the same frequency group

Decision: 		The document was not treated.

###### 6.4.2.1.2	CA configurations between different frequency groups based on CBM

R4-2200439	Views on FR2 inter-band DL CA CBM for different band groups

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200467	Requirements for CBM UEs between different frequency group

Type: other		For: Decision
					Source: Sony, Ericsson

Decision: 		The document was not treated.

R4-2200577	Reference signal of FR2 inter-band DL CA between different frequency groups based on CBM

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: Configuration and side condition of reference signal of the Band_with_BMRS is as single-band beam correspondence operation

Proposal2: “QCLed with the other CC in Band_with_BMRS” shall be applied for the reference signal of Band_without_BMRS.

Pr

Decision: 		The document was not treated.

R4-2200700	TP to TR 38.851: Agreements made for CA configurations between frequency groups using CBM

Type: pCR		For: Approval
					38.851 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200735	Discussion on requirements of FR2 inter-band DL CA

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200941	Discussion on CBM between different frequency group

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201299	Rx requirements for inter-band DL CA with CBM

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201969	On delta(RIB) for DL inter-CA with CBM in n260+n261

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

delta(RIB) proposal for an example band combination, along with relevant considerations

Decision: 		The document was not treated.

###### 6.4.2.1.3	Feasibility study for DL inter-band CA for IBM within the same frequency group

###### 6.4.2.1.4	Rx beam switch value

R4-2200701	Discussion on UE Rx beam switch delay

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200945	Discussion on Rx beam switch time

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201594	Discussion on UE Rx beam switch delay

Type: other		For: Approval
					Source: Nokia

Abstract:

Revision of R4-2200701

Decision: 		The document was not treated.

##### 6.4.2.2	Inter-band UL CA requirements

###### 6.4.2.2.1	Inter-band UL CA for two bands

R4-2200345	FR2 inter-band UL CA framework

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Relexation framework is discussed without assuming total power concept.

Decision: 		The document was not treated.

R4-2200468	UE UL CA requirements based on IBM

Type: other		For: Decision
					Source: Sony, Ericsson

Decision: 		The document was not treated.

R4-2200736	Discussion on requirements of FR2 inter-band UL CA

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200942	Discussion on inter-band UL CA

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201276	R17 FR2 Inter-band UL CA requirements

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201291	Discussion on inter-band UL CA requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201300	Tx requirements for inter-band UL CA between different frequency groups based on IBM

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

###### 6.4.2.2.2	CA configuration CA_n257A-n259A based on IBM

R4-2200346	TP to TR 38.851 to introduce FR2 UL CA_n257A-n259A

Type: pCR		For: Approval
					38.851 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Text proposal to TR 38.851 is to included the spec impact of FR2 UL CA to TS 38.101-2.

Decision: 		The document was not treated.

R4-2200555	RF requirements for CA_n257A_n259A based on IBM

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses RF requirements for IBM based inter-band UL CA_n257-n259.

Decision: 		The document was not treated.

R4-2200569	View on factor of FR2 inter-band UL CA relaxation

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal: Detailed factors and values for inter-band UL CA relaxation value calculation shall base on below table.

Decision: 		The document was not treated.

R4-2201292	Discussion on relaxation value X&Y for CA_n257A_n259A

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201967	On UL power for FR2 inter-band ULCA

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

We evaluate if it is justified to defined non-zero flat reductions in min. peak EIRP for FR2+FR2 inter-band ULCA and share our MPR proposal based on measurements

Decision: 		The document was not treated.

#### 6.4.3	UL gaps for self-calibration and monitoring

R4-2200353	Discussion of UL gap in FR2 RF enhancement for RAN2 LS

Type: discussion		For: Discussion
					Source: Mediatek India Technology Pvt.

Decision: 		The document was not treated.

R4-2201693	Discussion on LS on UL gap in FR2 RF enhancement

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.4.3.1	UE Tx power management

R4-2200253	UL gaps for Tx power management RF aspect

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200255	Draft CR for UL gap for Tx power management RF aspect

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200383	Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

UE FR2 UL Gap for UE Tx power enhancement

Decision: 		The document was not treated.

R4-2200589	Discussion on UL gap for Tx power management

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200856	On UE Tx power management for MPE compliance

Type: other		For: Approval
					Source: Ericsson, Sony

Abstract:

In this constribution we make proposals on the Tx power manangement for MPE and P-MPR in particular

Decision: 		The document was not treated.

R4-2200943	Discussion and reply LS on FR2 UL gap

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201274	R17 FR2 UL gap for power management

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201443	Discussion on Tx power management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

##### 6.4.3.2	Coherent UL-MIMO

R4-2200254	UL gaps for coherent UL MIMO

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2201442	Draft CR to 38.101-2 on requirements for coherent UL MIMO

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201444	Discussion on UL coherent MIMO

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

#### 6.4.4	DC location for intra-band UL CA with > 2 CCs for both FR2 and FR1

R4-2200333	Further details and optimizations on DC location

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200456	Handling of multiple DC locations for intra-band configuration

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution shares our views on how to handle multiple DC locations for intra-band configuration.

Decision: 		The document was not treated.

R4-2200944	Discussion on DC location

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201273	R17 FR2 DC reporting

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201959	Further study on DC location reporting

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 6.4.5	CA BW classes

##### 6.4.5.1	New FR2 CA BW classes

R4-2200857	FR2 bandwidth classes covering up to 1600 MHz aggregated bandwidth with mixed carrier bandwidths

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose to reconsider the BW classes of the WF agreed at RAN4#100 in view of deployment aspects and number of CCs supported.

Decision: 		The document was not treated.

R4-2200858	FR2 CA BW classes up to 1600 MHz aggregated BW with mixed channel bandwidths

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to introduce FR2 CA BW classes up to 1600 MHz aggregated BW with mixed channel bandwidths.

Decision: 		The document was not treated.

R4-2200909	Alternatives of FR2 new CA BW classes

Type: other		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2201297	Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classesV, MF,ME, MD and MA

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

##### 6.4.5.2	Fallback group

R4-2200302	FR2 bandwidth class and fallback group

Type: discussion		For: Approval
					Source: Verizon, Qualcomm, MediaTek

Decision: 		The document was not treated.

R4-2200620	Further considerations on FR2 fallback group

Type: discussion		For: Approval
					Source: ZTE Corporation

Abstract:

In this paper, we further provide our opinion on FR2 fallback group.

Decision: 		The document was not treated.

R4-2201298	Discussion on fallback group for FR2 new CA BW classes

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

#### 6.4.6	RRM core requirements

R4-2201536	Draft Big CR on RRM requirements for FR2 Inter-band CA

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Draft Big CR on RRM requirements for FR2 Inter-band CA

Decision: 		The document was not treated.

##### 6.4.6.1	Inter-band DL CA requirements for CBM

###### 6.4.6.1.1	MRTD requirements

R4-2200391	Further considering on remaining issues for RRM requirements for inter-band DL CA in NR FR2

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200425	MRTD requirements for CBM based Inter-band DL CA

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200559	Discussion on MRTD for FR2 inter-band CA based on CBM

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses X value in MRTD requirements for CBM based FR2 inter-band CA.

Decision: 		The document was not treated.

R4-2200927	Discussion on CBM MRTD requirement for FR2 inter-band DL CA

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201129	MRTD requirements for FR2 inter-band DL CA enhancements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201537	discussion on MRTD  for CBM inter-band FR2 DL CA

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

discussion on MRTD requirements for inter-band FR2 DL CA for CBM

Decision: 		The document was not treated.

R4-2201538	draftCR on MRTD for CBM inter-band FR2 DL CA

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

draftCR on MRTD requirements for inter-band FR2 DL CA for CBM

Decision: 		The document was not treated.

R4-2201588	Timing requirements for inter-band DL CA

Type: other		For: Approval
					Source: Ericsson

Abstract:

Analysis of CBM requirements and remedies for MRTD=3 µs.

Decision: 		The document was not treated.

R4-2201589	Timing requirements for inter-band DL CA

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR for MRTD requirements for CBM UE.

Decision: 		The document was not treated.

R4-2201607	Discussion on MRTD requirements for FR2 inter-band DL CA with CBM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

###### 6.4.6.1.2	Other RRM requirements

R4-2200426	RRM requirements for CBM based Inter-band DL CA

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200742	Other RRM requirements for FR2 inter-band DL CA requirements for CBM

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200928	Discussion on CBM RRM requirements for FR2 inter-band DL CA

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200929	Introduction of SCell activation delay requirement for FR2 inter-band CA with common beam management

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201130	Other RRM requirements for FR2 inter-band DL CA enhancements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201373	Discussion on RRM requirements of inter-band DL CA for CBM UE

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, we provide our views on RRM requirements for FR2 inter-band DL CA UEs operating with CBM.

Decision: 		The document was not treated.

R4-2201374	Draft CR on scheduling restriction for FR2 inter-band DL CA for CBM UE

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

We provide draft CR to introduce Scheduling restriction requirements for CBM UE

Decision: 		The document was not treated.

R4-2201539	discussion on other RRM requirements  for CBM inter-band FR2 DL CA

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

discussion on other RRM requirements for inter-band FR2 DL CA for CBM

Decision: 		The document was not treated.

R4-2201540	draftCR on measurement restriction for CBM inter-band FR2 DL CA

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

draftCR on measurement restriction for inter-band FR2 DL CA for CBM

Decision: 		The document was not treated.

R4-2201608	Discussion on other RRM requirements for FR2 inter-band DL CA with CBM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201609	DraftCR on interruption requirements for FR2 inter-band CA with CBM

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.4.6.2	Inter-band UL CA for IBM

R4-2201131	Inter-band UL CA requirements for IBM

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201375	Discussion on RRM requirements for inter-band UL CA for IBM UE

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, we provide our views on number of UL carrier to be supported for FR2 inter-band UL CA for IBM UE.

Decision: 		The document was not treated.

R4-2201376	Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

We introduce draft CR for FR2 inter-band UL CA

Decision: 		The document was not treated.

R4-2201541	draftCR on RRM requirements for IBM inter-band FR2 UL CA

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

draftCR on RRM requirements for IBM inter-band FR2 UL CA

Decision: 		The document was not treated.

##### 6.4.6.3	UL gaps for self-calibration and monitoring

R4-2200256	UL gaps for Tx power management RRM aspect and draft reply LS

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200257	Draft CR for UL gap for Tx power management RRM aspect

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200384	Network impact of UE FR2 UL Gap for UE Tx power enhancements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200427	UL gaps for self-calibration and monitoring

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200590	Discussion on RRM impact of UL gap for Tx power management

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200605	Discussion on RRM impacts of UL gaps for self-calibration and monitoring

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201377	Discussion on UL gaps for self calibration and monitoring

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, we provide our views on UL gaps for self-calibration and monitoring and its impacts on other RRM requirements.

Decision: 		The document was not treated.

### 6.5	NR repeater

#### 6.5.1	General

##### 6.5.1.1	System parameters

R4-2200086	TP for TS 38.106:Clause 4 general

Type: pCR		For: Approval
					38.106 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200087	TP for TS 38.106:Clause 5 operating bands

Type: pCR		For: Approval
					38.106 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200089	Discussion of co-location requirements for multi-band repeaters

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200818	Discussion on co-located requirements for FR1 NR repeater

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201289	Discussion on co-location requirement for multi-band repeaters

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

##### 6.5.1.2	Repeater Class/Type

##### 6.5.1.3	TDD repeater switching requirements

R4-2200090	Discussion of TDD switching requirements

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200091	TP for TS 38.106:ON OFF mask

Type: pCR		For: Approval
					38.106 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200821	Discussion on switching requirements

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201290	Discussion on repeater switching requirement and power transients

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201526	Repeaters TDD aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining aspects for TDD switching

Decision: 		The document was not treated.

R4-2201544	Discussion on repeater switching requirements

Type: other		For: Approval
					Source: NEC

Decision: 		The document was not treated.

R4-2201656	TDD repeater switching requirements

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.5.1.4	Others

R4-2200088	Proposal to remove Annex A from the core spec

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2201529	Draft TP to TS 38.106: Frequency stability and out of band gain requirements

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

Draft spec text according to work split

Decision: 		The document was not treated.

R4-2201657	Reference points and regional requirements for NR repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201932	Repeater OTA declarations and specification points

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss how to specify the repeater OTA interface

Decision: 		The document was not treated.

#### 6.5.2	Conductive RF core requirements

R4-2201930	TP to TS 38.106 6.1 and 6.2

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to TS for allocated conducted section 6.1 general and 6.2 repeater output power

Decision: 		The document was not treated.

##### 6.5.2.1	Transmitted power related requirements

R4-2200178	Views on the requirements to avoid interference of NR repeater for UL

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200822	Discussion on repeater power related conducted requirements

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201459	Further discussions on power related requirements for conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201532	Repeater conducted power requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Open issues for power requirements

Decision: 		The document was not treated.

R4-2201658	Conducted power related requirements consideration for NR-Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.5.2.2	Emission requirements

R4-2200092	Discussion of remaining issues for FR1 emission requirements

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200823	Discussion on repeater emission related conducted requirements

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201460	Further discussions on emission related requirements for conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201528	Repeater conducted emissions requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Open issues for emissions requirements

Decision: 		The document was not treated.

R4-2201654	TP to TS 38.106 clause 6.5 Unwanted emissions conducted

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201660	Repeater conducted unwanted emissions

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201933	Repeater FR1 OOB gain and ACRR

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss list of assumptions in the WF for calculating OOB gain and ACRR requirements for FR1

Decision: 		The document was not treated.

R4-2201935	FR1 inband passband OBUE requirements and NF requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss inband OBUE requirement and their similarity with  NF requirements.

Decision: 		The document was not treated.

##### 6.5.2.3	Others

R4-2200093	Discussion of out of band gain and ACRR requirements for FR1

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200819	Discussion on other RF conducted requirements for NR repeater

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200825	TP to TS 38.106 conducted EVM and input IMD

Type: pCR		For: Approval
					38.106 v0.0.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2201461	Further discussions on other requirements of conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201527	Repeater conducted requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Open issues for ACRR, OOB gain requirement

Decision: 		The document was not treated.

R4-2201661	Low-power EVM and OOB gain considerations for FR1 NR Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201934	Discuss repeater co-location requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss co-location with BS and/or repeaters as discuss din WF from last meeting

Decision: 		The document was not treated.

#### 6.5.3	Radiated RF core requirements

R4-2201931	TP to TS 38.106 9.1and 9.2

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei

Abstract:

TP to TS for allocated radiated section 9.1 general and 9.2 repeater output power

Decision: 		The document was not treated.

##### 6.5.3.1	Transmitted power related requirements

R4-2201462	Further discussions on power related requirements for conducted repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201659	Radiated power related requirements consideration for NR-Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.5.3.2	Emission requirements

R4-2200824	Discussion on repeater emission related radiated requirements

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201463	Further discussions on emission related requirements for radiated repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201531	Repeaters radiated emissions requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Open issues for power requirement

Decision: 		The document was not treated.

R4-2201655	TP to TS 38.106 clause 7.5 Unwanted emissions radiated

Type: pCR		For: Approval
					38.106 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201662	Repeater radiated unwanted emissions

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201936	FR2 inside passband OBUE requirement and NF

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the inband OBUE and noise figure requirements for the FR2 repeater

Decision: 		The document was not treated.

R4-2201937	Repeater FR2 OOB gain and ACRR

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss list of assumptions in the WF for calculating OOB gain and ACRR requirements for FR2

Decision: 		The document was not treated.

##### 6.5.3.3	Others

R4-2200094	Discussion of out of band gain and ACRR requirements for FR2

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200820	Discussion on other RF radiated requirements for NR repeater

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200826	TP to TS 38.106 radiated EVM and input IMD

Type: pCR		For: Approval
					38.106 v0.0.0	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2201464	Further discussions on other requirements of radiated repeater

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201530	Repeaters radiated requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Propoals for other radiated requirements

Decision: 		The document was not treated.

R4-2201663	Signal quality and OOB gain considerations for FR2 NR Repeaters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201938	FR2 input IM requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the FR2 input IM signals and levels.

Decision: 		The document was not treated.

#### 6.5.4	EMC core requirements

R4-2200696	Discussion on test conditions and performance assessment for NR  repeater EMC tests

Type: discussion		For: Discussion
					Source: ZTE

Abstract:

In this document, we give some initial discussion on some test issue for different type NR repeaters EMC based on some agreements of RF requirements.

Decision: 		The document was not treated.

R4-2200697	TP to TS38.114: References

Type: pCR		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE

Abstract:

In this contribution, we provide a TP to TS38.114 for the Reference issue based on TS38.114V0.1.0.

Decision: 		The document was not treated.

R4-2200730	TS38.114V0.2.0 to capture RAN4#101-bis agreements

Type: draft TS		For: Approval
					38.114 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE

Abstract:

[Email Approval] To reflect the approved TP into TS 38.114 v0.2.0.

Decision: 		The document was not treated.

### 6.6	Introduction of DL 1024QAM for NR FR1

#### 6.6.1	General

#### 6.6.2	UE RF requirements maintenance

#### 6.6.3	BS TX RF requirements maintenance

#### 6.6.4	BS RF conformance testing

R4-2200134	Discussion on BS RF conformance testing for 1024QAM for NR FR1

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200135	Draft CR for 37.145-1 on BS RF conformance testing for 1024QAM  for NR FR1

Type: draftCR		For: Endorsement
					37.145-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2201458	Draft CR to TS 38.141-1: Introduction of 1024 QAM in FR1

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201489	CR to TS 38.141-2: Introduction of 1024 QAM in FR1

Type: CR		For: Endorsement
					38.141-2 v17.4.0	  CR-0380  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This is a draftCR that will not be agreed but endorsed. Based upon agreed worksplit: R4-2106121.  With inputs from WF on 1024 QAM conformance testsing (R4-2120659)

Decision: 		The document was not treated.

R4-2201514	Test procedure for support of 1024QAM

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201664	On 1024QAM for NR BS conformance tests

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201665	Draft CR to TS 37.145-2 with 1024QAM introduction

Type: draftCR		For: Endorsement
					37.145-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 6.6.5	Demodulation and CSI requirements

##### 6.6.5.1	General

R4-2201427	Summary of PDSCH simulation results for DL 1024QAM in FR1

Type: other		For: Information
					Source: Ericsson

Abstract:

This spread sheet summarizes the simulation results of PDSCH demodulation with DL 1024QAM.

Decision: 		The document was not treated.

R4-2201719	Simulation Results and views on 1024QAM FR1 UE Demod Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 6.6.5.2	PDSCH requirements

R4-2200265	Discussion on PDSCH demod requirements with 1KQAM

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200376	Discussion on the PDSCH requirements for 1024QAM

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201011	Discussion and simulation results on 1024QAM PDSCH

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201428	UE demodulation requirements for DL 1024QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issue of UE demodulation requirements for DL 1024QAM WI.

Decision: 		The document was not treated.

R4-2201783	Discussion on PDSCH requirements for DL 1024QAM in FR1

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.6.5.3	SDR requirements

R4-2200266	Discussion on SDR requirements with 1KQAM

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2201012	Discussion and simulation results on 1024QAM SDR

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201429	SDR requirements for DL 1024QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issues on SDR for DL 1024QAM.

Decision: 		The document was not treated.

R4-2201784	Discussion on SDR requirements for DL 1024QAM in FR1

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.6.5.4	CQI requirements

R4-2201013	Discussion and simulation results on 1024QAM CSI

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201430	CQI reporting requirements for DL 1024QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issues of CQI reporting requirements for DL 1024QAM.

Decision: 		The document was not treated.

### 6.7	UE RF requirements for Transparent Tx Diversity (TxD) for NR

#### 6.7.1	General

R4-2201941	Big CR for TS 38.101-1 Tx diversity requirements

Type: CR		For: Agreement
					38.101-1 v17.4.0	  CR-0986  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, Qualcomm, vivo

Abstract:

reserved CR for phase 2 requirements

Decision: 		The document was not treated.

#### 6.7.2	UE RF requirements for phase 1 (38.101-1)

R4-2201772	Draft CR TS 38.101-1 R17: moving 2Tx MPR to clause 6.2D and amending PC2 2TX MPR

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

Solving merge issue of R4-2119971 Draft CR on MPR of Tx Diversity (TxD) PC2 for two PC3 PA architecture and R4-2119977 Draft CR TS 38.101-1: Move PC1.5 MPR to Clause 6.2G. and clarify signaling aspects

Decision: 		The document was not treated.

##### 6.7.2.1	UL MIMO requirement for TxD except ULFPTx

R4-2200340	Draft CR for fixing MPRs in suffix D

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201267	R17 FR1 TxD requirements and signaling

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201269	Draft R17 CR on UL MIMO falllback to TxD

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

#### 6.7.3	UE RF requirements for phase 2 (38.101-1)

R4-2201308	TxD signalling and inter-band carrier aggregation

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

##### 6.7.3.1	SRS antenna switching related

R4-2200341	SRS virtualization for antenna switching

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200484	Relation of TxD and SRS antenna switching

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses relation between TxD and SRS antenna switching based on [R4-2120065].

Decision: 		The document was not treated.

R4-2200859	SRS antenna switching with antenna virtualization

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we discuss SRS swiching with virtualization (TxD) and FP UL-MIMO and propose corrections to the configured power for SRS with switching

Decision: 		The document was not treated.

R4-2200860	Pcmax for SRS usage set as antenna switching for TxD and UL-MIMO features

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to correct the Pcmax for SRS used for antenna switching for antenna virtualization and full-power UL-MIMO

Decision: 		The document was not treated.

R4-2200959	Further discussion on SRS antenna switching for TxD

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200960	Draft CR  on SRS antenna switching for TxD

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2201227	Discussion on Tx diversity SRS antenna switching

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201271	R17 SRS IL for TxD

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201272	Draft R17 CR on SRS IL for TxD

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201799	Further discussion on SRS antenna switching requirements for TxD

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201940	On SRS relaxation

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.7.3.2	ULFPTx related

R4-2200483	Relation of MOP between TxD and ULFPTx

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses relation of MOP between TxD and ULFPTx specifically for the case that a UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission based

Decision: 		The document was not treated.

R4-2200861	Single-antenna falback for TxD and UL-MIMO (including ULFPTx)

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose single-antenna port fall-back requirements and discuss the importance of consistent virtualization for SRS and PUSCH

Decision: 		The document was not treated.

R4-2200862	TxD and UL-MIMO requirements for single-port antenna transmission

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to correct the single-port requirements to accommodate TxD and (full-power) UL-MIMO modes

Decision: 		The document was not treated.

R4-2200961	Discussion on ULFPTx with TxD

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201268	R17 FR1 TxD and ULFPTx

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201762	Further Discussion on Transparent TxD – ULFPTx related

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201798	Further discussion on ULFPTx with TxD

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201942	On ULFPTx and applicable MPR requirements for different PA configurations

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2202051	ULFPTX Mode 2 and TxD

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

### 6.8	Enhancement for NR high speed train scenario in FR1

#### 6.8.1	General

#### 6.8.2	RRM core requirements

R4-2200062	Discussion on Rel-17 HST in FR1

Type: discussion		For: Discussion
					Source: MediaTek (Shenzhen) Inc.

Decision: 		The document was not treated.

R4-2200323	On NR FR1 HST RRM Requirements

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

##### 6.8.2.1	Intra-frequency measurements

R4-2200626	Draft CR on NSCC_SSB for CSSFoutside_gap,i

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

##### 6.8.2.2	Inter-frequency measurements

R4-2200095	Discussion on remaining issues for inter-frequency measurement for FR1 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200635	Discussion on NR HST RRM enhancement for inter-frequency measurement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200670	On R17 FR1 HST inter-frequency measurement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200876	Inter-frequency measurements enhancement for NR HST in FR1

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Inter-frequency measurements enhancement for NR HST in FR1

Decision: 		The document was not treated.

R4-2201172	Discussion on inter-frequency measurements for FR1 HST

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201173	Draft CR on inter-frequency measurements for FR1 HST

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.8.2.3	L1-SINR measurements

R4-2200096	Discussion on L1-SINR measurements for FR1 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200596	Discussion on L1-SINR measurements in R17 FR1 HST

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200671	On R17 FR1 HST L1-SINR

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200875	L1-SINR measurements requirements enhancement for NR HST in FR1

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

L1-SINR measurements requirements enhancement for NR HST in FR1

Decision: 		The document was not treated.

R4-2201132	L1-SINR requirements for Rel17 FR1 HST

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201582	Further discussion on L1-SINR measurements for Rel-17 FR1 HST CA

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

We present simulation results of L1-SINR and our views.

Decision: 		The document was not treated.

##### 6.8.2.4	Others

R4-2200097	Discussion on other remaining issues for FR1 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200246	On remaining issues for R17 FR1 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200247	CR on L1-RSRP measurement in FR1 HST

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200634	Discussion on general requirements for FR1 HST RRM

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200874	RRM requirements for NR HST in FR1

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Other requirments for NR HST in FR1

Decision: 		The document was not treated.

R4-2201133	Other RRM requirements for Rel17 FR1 HST

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201174	Discussion on remaining issues in FR1 HST

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

#### 6.8.3	UE demodulation requirements (38.101-4)

##### 6.8.3.1	General

R4-2201422	Summary for FR1 HST demodulation results

Type: other		For: Information
					Source: Ericsson

Abstract:

This spread sheet summarizes the simulation results of FR1 HST demodulation requirements.

Decision: 		The document was not treated.

##### 6.8.3.2	PDSCH requirements for CA scenarios

R4-2200267	Discussion on PDSCH CA Requirements in HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200268	Draft CR on HST DPS CA requirements for 4Rx

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was withdrawn.

R4-2200364	Views on HST CA tests for FR1

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200521	Discussion on HST FR1 CA PDSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200625	Draft CR on PDSCH requirements for HST-SFN CA requirements for 4Rx

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200633	Discussion on FR1 HST UE demodulation for CA scenario

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200835	Discussion on PDSCH requirements for CA scenarios

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200998	Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200999	Draft CR on HST FR1 DPS CA requirements for 2Rx (38.101-4)

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201423	PDSCH demodulation requirements for CA with HST-SFN scenario

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issues of the PDSCH demodulation requirements for CA with HST-SFN scenario.

Decision: 		The document was not treated.

R4-2201424	draft CR: FRC for CA PDSCH demodulation requirements for HST

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This draft CR provides FRC used for PDSCH demodulation requirements for HST with CA.

Decision: 		The document was not treated.

R4-2201876	draftCR to TS 38.101-4: HST-SFN CA requirements for 2Rx

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2202006	Views on FR1 HST PDSCH CA Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

### 6.9	NR support for high speed train scenario in FR2

#### 6.9.1	General

R4-2201696	TR for FR2 HST

Type: draft TR		For: Information
					38.854 v0.1.1	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Samsung

Abstract:

dradt TR 38.854; the version (v0.1.0) need to be checked and possibly increased to (v0.2.0).

Decision: 		The document was not treated.

#### 6.9.2	High speed train deployment scenario in FR2

R4-2201000	TP to TR 38.854 on Deployment Scenario Analysis for FR2 HST

Type: pCR		For: Approval
					38.854 v0.1.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201524	TP to TR 38.854: Coverage analysis

Type: pCR		For: Approval
					38.854 v0.1.1	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

Capture coverage analysis in the TR

Decision: 		The document was not treated.

#### 6.9.3	UE RF core requirements

R4-2200327	On FR2 HST RF Requirements

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

##### 6.9.3.1	UE Tx requirements

R4-2201764	Draft CR to introduce UE RF requirement for FR2 Power Class 6

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

###### 6.9.3.1.1	UE RF framework and power class

R4-2200347	UE RF requirement framework for FR2 HST

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

remaining issue on UE RF assumption is discussed.

Decision: 		The document was not treated.

###### 6.9.3.1.2	Spherical coverage requirements

R4-2200348	UE spherical coverage requirement for FR2 HST

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

spherical coverage shall be based on declaration of boresight beam

Decision: 		The document was not treated.

R4-2200836	Discussion on Spherical coverage requirements for HST_FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201525	FR2 HST UE requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining UE issues

Decision: 		The document was not treated.

R4-2201763	Further Discussion on Spherical Coverage Requirement for FR2 HST UE

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

###### 6.9.3.1.3	Beam correspondence

R4-2201765	Side Conditions for FR2 HST UE Beam Correspondence Requirement

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

##### 6.9.3.2	UE Rx requirements

#### 6.9.4	RRM core requirements

R4-2200328	On NR FR2 HST RRM Requirements

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

##### 6.9.4.1	General

R4-2200098	Discussion on general issues for FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200259	Discussion on general aspects for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200581	General RRM requirements for HST FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200627	Discussion on general RRM requirements for FR2 HST

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200877	CR On RRC_CONNECTED state mobility for HST FR2 RRM

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR On RRC_CONNECTED state mobility for HST FR2 RRM

Decision: 		The document was not treated.

R4-2200878	General requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General requirements for HST FR2

Decision: 		The document was not treated.

R4-2200883	LS on FR2 HST RRM

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

LS on network signalling for Rel-17 NR HST RRM

Decision: 		The document was not treated.

R4-2201156	Discussion on general requirements for FR2 HST

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201175	General discussion on network signaling and UE capability in FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201666	LS on network signalling for Rel-17 FR2 NR HST RRM

Type: LS out		For: Approval
					to RAN2
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201667	Discussion on network signalling for FR2 HST RRM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Addressing remaining open issues on network signalling.

Decision: 		The document was not treated.

R4-2201766	Discussion on network signaling and UE capability for FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201767	LS on Network Signalling for Rel-17 NR FR2 HST RRM

Type: LS out		For: Approval
					to RAN2
					Source: Samsung

Decision: 		The document was not treated.

R4-2201847	TP to TR 38.854 on Mobility Performance in HST FR2 Deployment Scenarios

Type: discussion		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.9.4.2	Number of RX beams

R4-2200099	Discussion on requirements on scenario-B with two-side RRH

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200260	Discussion on number of Rx beam for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200582	Discussion on RX beam number for HST FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200881	Number of RX beams for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discuss how to deal with requirments with different number of RX beams for HST FR2

Decision: 		The document was not treated.

R4-2201848	TP to TR 38.854 on the Number of Rx beams

Type: discussion		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.9.4.3	RRC Idle/Inactive and connected state mobility requirements

R4-2200100	Further discussion on RRC Idle/Inactive and connected state mobility requirements for FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200261	Discussion on mobility requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200583	Discussion on RRC Idle Inactive and Connected state mobility requirements for HST FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200628	Discussion on mobility requirements for FR2 HST

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200880	RRC Idle/Inactive and connected state mobility requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

RRC Idle/Inactive and connected state mobility requirements for HST FR2

Decision: 		The document was not treated.

R4-2201176	Discussion on RRC Idle/Inactive and connected state mobility requirements for HST in FR2

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201651	On RRM measurement requirements for FR2 HST

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Intra-frequency measurement requirements in connected and cell reselection requirements in idle mode.

Decision: 		The document was not treated.

R4-2201652	CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Enhancements on intra-frequency measurements for FR2 HST.

Decision: 		The document was not treated.

##### 6.9.4.4	Timing requirements

R4-2200101	Discussion on timing requirement for FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200262	Discussion on timing requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200584	Discussion on Timing Requirement for HST FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200882	On Timing requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Present our views on one shot timing adjustment

Decision: 		The document was not treated.

R4-2201768	Further Discussion on Timing Requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201769	Draft CR to introduce one shot large UL timing adjustment for FR2 HST UE

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2201846	On UL Timing Adjustment in HST FR2 Deployments

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.9.4.5	Signalling characteristics requirements

R4-2200102	Discussion on RLM/BFD requirement for FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200104	Draft CR on RLM/BFD requirement for FR2 HST

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200263	Discussion on signaling characteristic requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200585	Discussion on Signaling characteristics for HST FR2

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200884	Signalling characteristics requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Signalling characteristics requirements for HST FR2

Decision: 		The document was not treated.

R4-2201177	Discussion on signaling characteristics requirements for high speed train scenario in FR2

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201178	Discussion on RRM requirements for high speed train scenario in FR2

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201770	Remaining Issues on signaling characteristics requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201771	Draft CR to introduce active TCI state switching delay requirement for FR2 HST UE

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

##### 6.9.4.6	Measurement procedure requirements

R4-2200103	Further discussion on measurement procedure requirements for FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200264	Discussion on measurement procedure requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200879	Measurement procedure requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Measurement procedure requirements for HST FR2

Decision: 		The document was not treated.

#### 6.9.5	Demodulation requirements

##### 6.9.5.1	General

R4-2200743	Simulation results summary for Rel-17 FR2 HST

Type: other		For: Information
					Source: Samsung

Decision: 		The document was not treated.

R4-2200744	CR work split for Rel-17 FR2 HST

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201001	Discussion on general UE demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201002	Draft CR on minimum requirements for PDSCH HST-DPS (38.101-4)

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201003	Draft CR on HST FR2 BS manufacturer's declarations and applicability rule (38.141-2)

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

##### 6.9.5.2	UE demodulation requirements

R4-2200629	Discussion on UE demodulation for FR2 HST

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2201718	Addressing open issues on FR2 HST UE Demodulation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201877	Views on FR2 HST PDSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

###### 6.9.5.2.1	PDSCH requirements under Uni-directional scenario

R4-2200746	Discussion and simulation results of PDSCH requirement with Uni-directional scenario for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201004	Discussion on UE demodulation requirements for FR2 HST under Uni-directional scenario

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201425	PDSCH demodulation requirements for HST FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the UE demodulation requirements for HST FR2.

Decision: 		The document was not treated.

###### 6.9.5.2.2	PDSCH requirements under Bi-directional scenario

R4-2200745	Discussion and simulation results of PDSCH requirement with Bi-directional scenario for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200837	PDSCH requirements under Bi-directional scenario

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201005	Discussion on UE demodulation requirements for FR2 HST under Bi-directional scenario

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201426	PDSCH demodulation requirements for HST FR2 bi-directional scenario

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the UE demodulation requirements for HST FR2 specific to the bi-directional scenario.

Decision: 		The document was not treated.

##### 6.9.5.3	BS demodulation requirements

###### 6.9.5.3.1	PUSCH requirements

R4-2200128	Discussion on PUSCH demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200129	Simulation results for PUSCH demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200747	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201006	Discussion on PUSCH demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201522	HST PUSCH requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Remaining issues for PUSCH requirement

Decision: 		The document was not treated.

R4-2201844	On HST FR2 PUSCH Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201878	Views on FR2 HST PUSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

###### 6.9.5.3.2	PUSCH with UL timing adjustment requirements

R4-2200130	Simulation results for UL timing adjustment demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200748	Simulation results of UL timing adjustment requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201007	Discussion on PUSCH with UL timing adjustment requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201523	PUSCH with UL timing adjustment requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposed TA requirement

Decision: 		The document was not treated.

R4-2201845	On HST FR2 PUSCH UL Timing Adjustment Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

###### 6.9.5.3.3	PRACH requirements

R4-2200131	Simulation results for PRACH demodulation requirements for FR2 HST

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200749	Simulation results of PRACH requirement for Rel-17 FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201008	Discussion on PRACH demodulation requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201009	Draft CR on PRACH minimum requirements for high speed train (38.104)

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201010	Draft CR on PRACH test requirement for high speed train (38.141-2)

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201521	FR2 HST PRACH requirement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Proposed PRACH requirement

Decision: 		The document was not treated.

### 6.10	Further RRM enhancement for NR and MR-DC

#### 6.10.1	General

#### 6.10.2	RRM core requirements

##### 6.10.2.1	SRS antenna port switching

R4-2200068	Further discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200069	Interruption requirement to LTE serving cell, and impacts to other LTE RRM requirement

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200179	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200288	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200322	On SRS antenna switching

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

R4-2200531	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200561	Discussion on interruption due to SRS antenna port switching

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2200597	Discussion on RRM requirements for SRS antenna port switching

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200640	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200684	Further discussion on RRM requirements for SRS antenna switching

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200739	Discussion on SRS antenna port switching

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200895	Interruption requirements at SRS antenna port switching

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201134	RRM requirements for SRS ant port switch

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201201	Discussion on RRM requirements for SRS antenna port switching

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201378	RRM requirements for SRS antenna port switching

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide our views on the open issues of SRS antenna port switching

Decision: 		The document was not treated.

R4-2201379	Draft CR on Interruption requirement to NR serving cell, and impacts to other NR RRM requirement (if applicable)

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

We provide draft CR to introduce interruption requirements to NR serving cell

Decision: 		The document was not treated.

##### 6.10.2.2	HO with PSCell

R4-2200070	Further discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200289	Discussion on RRM requirement for handover with PSCell

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200290	Draft CR on HO with PSCell for NR SA to EN-DC_R17

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200416	HO with PSCell

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

There are open issues from the last meeting. Therefore, we continue to share our views of the RRM requirements for HO with PSCell in this paper.

Decision: 		The document was not treated.

R4-2200532	Discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200598	Discussion on RRM requirements for HO with PSCell

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200630	Discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200673	Further discussion on RRM requirements for handover with PSCell

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200740	Discussion on RRM requirements for HO with PSCell

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201135	RRM requirements for HO with PSCell

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201202	Discussion on RRM requirements for HO with PSCell

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201203	Draft CR on requirements for HO with PSCell from EN-DC to EN-DC

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201380	RRM requirements for handover with PSCell

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide our views on the open issues for HO with PSCell

Decision: 		The document was not treated.

R4-2201381	Drfat CR on HO with PSCell requirements for NE DC to NE-DC

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

We provide draft CR to introduce PSCell requirements for NE DC to NE-DC

Decision: 		The document was not treated.

R4-2201542	discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

discussion on HO with PSCell

Decision: 		The document was not treated.

R4-2201543	dratCR on HO with PSCell

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

DraftCR for HO with PSCell

Decision: 		The document was not treated.

R4-2201851	Discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

##### 6.10.2.3	PUCCH SCell activation/deactivation

R4-2200071	Further discussion on PUCCH SCell activation_deactivation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200072	PUCCH Scell activation delay requirements with multiple Scell

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200180	Discussion on PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200181	Draft CR for PUCCH SCell deactivation delay requirements in TS 38.133

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200291	Discussion on PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200352	Discussions on PUCCH SCell Activation/Deactivation delay requirements

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200390	Remaining issues for PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200418	PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200533	Discussion on PUCCH SCell activation

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200639	Discussion on PUCCH SCell activation/deactivation

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200674	Further discussion on SCell activation and deactication requirements for PUCCH SCell

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200741	Discussion on PUCCH SCell activation and deactivation

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200893	Discussion on the activation delay for deactivated PUCCH SCell

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200894	38.133 draft CR on PUCCH SCell activation delay requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201136	RRM requirements for PUCCH SCell Activation/Deactivation

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201204	Discussion on RRM requirements for PUCCH SCell activation

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201205	Draft CR on interruption of PUCCH SCell activation

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201382	RRM requirements for SCell activation/deactivation with PUCCH

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide our views on the RRM requirements for SCell (de)activation with PUCCH

Decision: 		The document was not treated.

R4-2201383	Draft CR on Interruption requirements to LTE serving cell

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

We provide draft CR to introduce Interruption requirements to LTE serving cell

Decision: 		The document was not treated.

### 6.11	NR and MR-DC measurement gap enhancements

#### 6.11.1	General

#### 6.11.2	RRM core requirements

##### 6.11.2.1	Pre-configured MG pattern(s)

R4-2200110	Further discussion on pre-configured MG pattern

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200111	Reply LS on R17 NR MG enhancements – Pre-configured MG

Type: LS out		For: Approval
					to RAN2
					Source: CATT

Decision: 		The document was not treated.

R4-2200112	Draft CR on measurement delay requirements with Pre-MG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200240	On Pre-MG pattern

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200241	CR on Pre-MG activation/deactivation delay

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200387	Remaining issues on pre configured MG patterns

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200486	Discussion on UE features for measurement gap enhancement

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200487	Discussion on pre-configured gap

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200488	Draft CR on 38.133 for L1 measurement impact of preconfigured gap

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200537	Discussion on pre-configured measurement gap

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200586	Views on pre-configured MG patterns

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200595	Draft Reply LS to Reply LS onViews on Concurrent MG

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200638	Discussion on pre-configured MG pattern(s)

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200675	Further discussion on pre-configured MG pattern for NR

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200676	DraftCR on inter-RAT measurement delay requirements with pre-configured gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200693	DraftCR on inter-frequency measurement requirements with pre-MG in NR

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200761	On pre-configured measurement gaps

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201137	On pre-configured MG pattern(s) for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201138	Draft CR to 38133 on gap interruption for Pre-MG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201621	Discussion on pre-configured MG

Type: LS out		For: Approval
					to RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201622	CR on pre-MG applicability

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201977	Discussion on Pre-configured MG patterns

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on remaining issues for pre-configured NR MG patterns

Decision: 		The document was not treated.

R4-2202009	Further analysis of pre-configured measurement gap pattern

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This document further analyzes RRM requirements for pre-configured MG in NR and MR-DC

Decision: 		The document was not treated.

R4-2202010	Measurement requirements for Pre-MG in TS 38.133

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

The CR defines RRM requirements for pre-configured MG in NR and MR-DC

Decision: 		The document was not treated.

##### 6.11.2.2	Multiple concurrent and independent MG patterns

R4-2200113	Further discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200114	Reply LS on R17 NR MG enhancements – Concurrent MG

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: CATT

Decision: 		The document was not treated.

R4-2200115	Draft CR on measurement delay requirements for concurrent MG patterns

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200242	On multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200243	CR on CSSF for concurrent gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200388	Remaining issues on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200404	Draft CR on inter-RAT measurement requirements with concurrent gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200489	Discussion on concurrent gaps

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200490	Draft CR on 38.133 for L1 measurement impact of concurrent gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200538	Discussion on multiple and independent concurrent measurement gaps in NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200560	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses multiple concurrent and independent MG patterns.

Decision: 		The document was not treated.

R4-2200587	Views on multiple concurrent and independent MGs

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200631	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200677	Further discussion on multiple concurrent and independent MG patterns for NR

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200678	DraftCR on inter-frequency measurement delay requirements with concurrent gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200694	DraftCR on positioning measurement requirements due to concurrent gap in NR

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200762	On multiple concurrent measurement gaps

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201139	On multiple concurrent and independent MG patterns for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201140	Draft CR to 38133 on CSI-RS based L3 measurement requirements with concurrent gap

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201213	Discussion on multiple concurrent MG patterns

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses concurrent gaps

Decision: 		The document was not treated.

R4-2201214	draftCR on concurrent gaps (9.1.2B)

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This draft CR introduce the measurement requirement skeleton for concurrent gaps

Decision: 		The document was not treated.

R4-2201623	Discussion on multiple concurrent MGs

Type: LS out		For: Approval
					to RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201624	CR on collision handling and MG related requirements for concurrent MGs

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201694	Discussion on concurrent measurement gap enhancements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201695	Discussion on LS on R17 NR Concurrent MG enhancements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 6.11.2.3	Network Controlled Small Gap

R4-2200116	Further discussion on Network Controlled Small Gap (NCSG)

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200117	Draft CR on measurement delay requirements with NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200244	On network controlled small gap

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200245	CR on NCSG applicability

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200329	On NCSG requirement

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

R4-2200389	Remaining issues for NCSG

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200405	Draft CR for interruption for de-activated SCell measurement due to NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200491	Discussion on NCSG

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200492	Draft CR on 38.133 for L1 measurement impact of NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200539	Discussion on NCSG in NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200588	Views on NCSG

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200632	Further discussion on Network Controlled Small Gap

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200695	DraftCR on interruption of NCSG in NR

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201157	Draft CR to UE behaviour to group the frequency layers with NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201158	Discussion on NCSG for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201232	Draft CR for UE behavior after the interruptions of NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201625	Discussion on NCSG

Type: LS out		For: Approval
					to RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201626	CR on use cases and CSSF for NCSG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201976	CR: NCSG scheduling restriction

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm communications-France

Decision: 		The document was not treated.

R4-2201978	Discussion on Network Controlled Small Gaps for NR

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on remaining issues for NR NCSG

Decision: 		The document was not treated.

R4-2202011	Further analysis of network controlled small gap

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This document further analyzes RRM requirements for NCSG in NR and MR-DC

Decision: 		The document was not treated.

R4-2202012	Measurement requirements for NCSG in TS 38.133

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

The CR defines RRM requirements for NCSG in NR and MR-DC

Decision: 		The document was not treated.

### 6.12	Further enhancement on NR demodulation performance

#### 6.12.1	General

R4-2200017	Draft TR 38.833 v1.1.0: Further enhancement on NR demodulation performance

Type: draft TR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Abstract:

[Email Approval] For email approval after the meeting, i.e., to implement the TPs approved during the meeting.

Decision: 		The document was not treated.

R4-2200018	TP to TR 38.833: Symbols and abbreviations

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Decision: 		The document was not treated.

R4-2200905	Updated work plan for Further enhancement on NR demodulation performance WI

Type: Work Plan		For: Approval
					Source: China Telecom

Decision: 		The document was not treated.

R4-2201603	Discussion on UE feature list and capability signalling for NR Demodulation Enhancements WI

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

#### 6.12.2	UE demodulation and CSI requirements

##### 6.12.2.1	MMSE-IRC receiver for inter-cell interference

###### 6.12.2.1.1	PDSCH requirements

R4-2200269	Discussion on PDSCH requirements in intercell interference scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200270	Draft CR on PDSCH demod requirements in ICI-FDD

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was withdrawn.

R4-2200366	Views on MMSE-IRC receiver for inter-cell interference test

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200377	Discussion on the PDSCH requirements for scenarios with inter-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200500	On Intercell PDSCH MMSE-IRC demodulation requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to Intercell PDSCH MMSE-IRC demodulation requirements. We make proposals concerning Common test parameters and Interference model for scenario 1.

Decision: 		The document was not treated.

R4-2200504	TP for 38.101-4 on NR Interference model for enhanced performance requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this TP we propose the structure for NR PUSCH performance requirements with CP-OFDM and FR1. Tables with test requirements are skeletons, an Interference model description for the NR UE enhanced performance requirements will be updated when agreements

Decision: 		The document was not treated.

R4-2200512	Discussion on PDSCH demodulation MMSE-IRC requirements for scenario with inter-cell interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200513	Summary of PDSCH simulation results for inter-cell interference suppression

Type: other		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200795	Draft CR for TS38.101-4 PDSCH TDD demodulation requirements for inter-cell interference MMSE-IRC

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200798	Discussion on R17 demodulation enhancement for inter-cell interference suppressing

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200902	On PDSCH requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200985	Discussion on Inter cell IRC demodulation requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200986	Simulation results for Inter cell IRC demodulation requirements

Type: other		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200988	Draft CR: Introduction of general and applicability section of inter-cell MMSE-IRC receiver in TS 38.101-4

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201215	Remaining issues on PDSCH requirement for inter-cell interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses PDSCH requirements for inter-cell IRC

Decision: 		The document was not treated.

R4-2201216	Simulation results on PDSCH performance for inter-cell interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for PDSCH demodulation for inter-cell IRC

Decision: 		The document was not treated.

R4-2201965	Views on Inter-cell Interference PDSCH Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

###### 6.12.2.1.2	CQI requirements

R4-2200271	Discussion on CSI reporting requirements in intercell interference scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200378	Discussion on the CQI requirements for scenarios with inter-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200501	On CQI requirements for intercell interference MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to CQI requirements for intercell interference MMSE-IRC. We make proposals concerning Antenna configuration and Requirement definition.

Decision: 		The document was not treated.

R4-2200514	Discussion on CSI MMSE-IRC requirements for scenario with inter-cell interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200799	Discussion on CQI requirements for inter-cell interference MMSE-IRC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200901	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200987	Discussion and simulations results for Inter cell IRC CSI

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201217	Remaining issues on CSI reporting requirements for inter-cell interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses CSI reporting requirements for inter-cell IRC

Decision: 		The document was not treated.

R4-2201218	Simulation results on CSI reporting for inter-cell interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for CSI reporting for inter-cell IRC

Decision: 		The document was not treated.

R4-2201221	draftCR on CSI reporting test case(TDD)

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This draftCR introduce the new CSI reporting test case.

Decision: 		The document was not treated.

R4-2201975	Views on Inter-cell Interference CQI Reporting Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 6.12.2.2	MMSE-IRC receiver for intra-cell inter-user interference

R4-2200272	Discussion on PDSCH requirements in MU-MIMO scenarios

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200379	Discussion on the PDSCH requirements for scenarios with intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200380	TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200515	Discussion on MMSE-IRC requirements for scenario with intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200516	TP to TR 38.833: Link level simulation results for Inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200800	Discussion on R17 demodulation enhancement for intra-cell inter-user interference suppressing

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200807	TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200903	Views on UE MMSE-IRC receiver for intra-cell inter-user interference suppression

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200904	TP to TR 38.833: Updated conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: China Telecom

Decision: 		The document was not treated.

R4-2200989	TP to TR 38.833: Introduction of simulation assumptions of intra cell inter user MMSE-IRC receiver

Type: pCR		For: Approval
					38.833 v1.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200990	Discussions on intra-cell inter user MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200991	Simulation results for intra cell inter user MMSE IRC receiver

Type: other		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200992	Draft CR: Introduction of MU-MIMO Beamforming model in TS 38.101-4

Type: draftCR		For: Endorsement
					38.101-4 v17.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2200993	Summary of simulation results for intra cell inter user MMSE receiver requirements

Type: other		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201219	Remaining issues on MMSE-IRC receiver for intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses PDSCH requirements for MU-MIMO

Decision: 		The document was not treated.

R4-2201220	Simulation results on PDSCH performance for intra-cell inter-user interference

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution submits our simulation results for PDSCH demodulation for intra-cell inter-user IRC

Decision: 		The document was not treated.

R4-2201966	Views on Intra-cell Inter-user Interference Scenarios

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 6.12.2.3	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR

R4-2201964	Views on CRS Interference Mitigation in NR

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

###### 6.12.2.3.1	General

R4-2200273	Discussion on CRS-IM with 30KHz SCS

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200502	On Receiver Assumptions for CRS-IM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to the receiver assumptions for CRS-IM.

Decision: 		The document was not treated.

R4-2200517	General discussion on CRS-IM requirements for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200801	Discussion on the receiver for CRS-IM

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200898	Discussion on the receiver assumption for CRS-IM requirement definition

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200982	Discussion on CRS-IM receiver for 30kHz SCS

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201414	CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201415	Simulation results on CRS-IM receiver

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201418	Discussion on remaining issues of receiver assumption for CRS-IM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discuss the remaining open issues of receiver assumption for CRS-IM

Decision: 		The document was not treated.

###### 6.12.2.3.2	Necessity of Network assistant signaling

R4-2200274	Discussion on Network Assistance for CRS-IM in NR

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200381	Views on the network assistance signalling for CRS-IM receiver

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200503	On Necessity of Network assistant signaling for CRS-IM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on the open issues for the subject of “Necessity of Network Assistant Signalling”. In addition, we have provided a proposal for a level of NWA, which we see as a compromise based on the previous RAN4 discuss

Decision: 		The document was not treated.

R4-2200518	Network assistant signaling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200802	Discussion on the network assistant signaling necessity for CRS-IM

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200899	Discussion on the necessity of network assistance signalling for CRS-IM

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200984	Discussion on necessity of NWA on CRS-IM receiver for 15kHz SCS

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201416	Discussion on the network assistance signalling for CRS-IM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the necessity of introducing the network asisstance signaling with additional simulations

Decision: 		The document was not treated.

R4-2201417	Simulation results for CRS interference handling

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution summarizes the additional simulations for information

Decision: 		The document was not treated.

###### 6.12.2.3.3	Test set-up

R4-2200275	Discussion on Test setup for CRS-IM requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200519	Test setup for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200803	Discussion on the test setup for CRS-IM

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200900	Discussion on the test setup for CRS-IM requirement definition

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200983	Discussion on test setup on CRS-IM receiver for 15kHz SCS

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201419	Discussion on the remaining issues of test setup for CRS-IM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues of test setup for CRS-IM

Decision: 		The document was not treated.

R4-2201601	On test setup for CRS-IM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on various open issues with relation to the Test Setup for CRS-IM. We make proposals concerning the area of Interference Model and Common parameters for target and interfering cells.

Decision: 		The document was not treated.

#### 6.12.3	BS demodulation requirements

##### 6.12.3.1	PUSCH demodulation requirements for FR1 256QAM

R4-2200132	Draft CR for TS 38.141-1 on manufacture declaration for FR1 PUSCH 256QAM performance requirements

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200133	Simulation results for FR1 PUSCH 256QAM performance requirement

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200148	Draft CR for TS 38.141-2, Introduction of manufacture declaration, MU and TT for FR1 PUSCH 256QAM performance requirements

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200474	draftCR for 38.141-1 MU and TT for PUSCH with 256QAM

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Measurement uncertainty and test torlerance for PUSCH with 256QAM

Decision: 		The document was not treated.

R4-2200520	Draft CR for TS 38.104: FR1 256QAM PUSCH requirements

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200752	draft CR on FR1 PUSCH 256QAM FRC for TS 38.104

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2200753	draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-1

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2200754	draft CR on FR1 PUSCH 256QAM FRC for TS 38.141-2

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2200755	Simulation results for FR1 PUSCH with 256QAM

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200796	Draft CR for TS38.141-2 Requirements for PUSCH with transform precoding disabled for BS type 1-O

Type: draftCR		For: Endorsement
					38.141-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200838	Simulation results for FR1 UL 256QAM demodulation requirement

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201018	Updated simulation results for FR1 PUSCH 256QAM performance requirements

Type: other		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201019	Summary of simulation results for FR1 PUSCH 256QAM performance requirements

Type: other		For: Information
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201020	draftCR: Introduction of conformance testing for FR1 PUSCH 256QAM with transform precoding disabled in TS 38.141-1

Type: draftCR		For: Endorsement
					38.141-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201801	Simulation results for FR1 UL 256QAM demodulation requirement

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201802	draftCR on FR1 PUSCH 256QAM FRC for TS 38.104

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

### 6.13	Solutions for NR to support non-terrestrial networks (NTN)

#### 6.13.1	General

R4-2201170	Draft text proposal to update TR 38.863

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201991	Considerations for TS 38.101-5 development

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

##### 6.13.1.1	System parameters

R4-2200162	On remaining open issue for NTN system parameters

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200163	TP to TR 38.863 on channel raster and sync raster

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200478	TP to TR 38.863 on operating bands and channel bandwidth

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200479	TP to TR 38.863 on transmitter characteristics for satellite access node

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2201074	On NTN System parameters

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201257	TP for 38.863 on system parameters to clarify “NTN satellite bands”

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201989	NTN system parameters remaining issues for n255

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

##### 6.13.1.2	NTN Satellite Access Node Class/Type

R4-2200165	Satellite Access Node Class/Type

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2201465	Further discussion on NTN gNB class

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

##### 6.13.1.3	Regulatory information

R4-2201075	TP to TR 38.863 on regulatory aspects for HAPS

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201314	NTN - Regulatory information - TP to TR 38.863

Type: pCR		For: Approval
					38.863 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This TP is suggesting text for the Regulatory section in TR 38.863

Decision: 		The document was not treated.

##### 6.13.1.4	Others

R4-2200363	Considerations on HAPS specific technical requirements

Type: discussion		For: Discussion
					Source: SoftBank Corp., KDDI Corporation, Intelsat

Decision: 		The document was not treated.

R4-2201076	TP to TR 38.863 on general aspects

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201263	Draft skeleton for TS 38.101-5

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Decision: 		The document was withdrawn.

R4-2201288	Draft skeleton for TS 38.101-5

Type: draft TS		For: Approval
					38.101-5 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: Samsung

Decision: 		The document was not treated.

R4-2201315	NTN - General aspects related to  BS and UE requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This TP is discussing general aspects when capturing requirements in BS and UE RF requirements

Decision: 		The document was not treated.

R4-2201830	Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1

Type: draft TS		For: Approval
					38.108 v0.0.1	  CR-  rev  Cat:  (Rel-17)

					Source: THALES

Abstract:

The attached skeleton is based on previous approved Way Forwards in RAN4#100-e (R4-2115641) and RAN4#101-e (R4-2120674, R4-2120675), as well as other RAN4#101-bis-e pre-meeting discussions. For further information please also check the Way Forwards on NTN

Decision: 		The document was not treated.

R4-2201838	Draft proposal to update TR 38.863 NTN related RF and co-existence aspects

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: THALES

Abstract:

This pCR is to provide B2 (Annex) Calibration Results of TN components with THALES calibration information provided prior to RAN4#101-e meeting before Sept. 30 2021, together with new variance and mean values.

Decision: 		The document was not treated.

#### 6.13.2	Coexistence aspects

##### 6.13.2.1	NTN coexistence scenarios and simulations

R4-2200164	NTN coexistence simulations

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200781	Coexistence simulation results for TN-NTN

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201072	NR-NTN co-ex study results

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201124	Collected NR-NTN co-ex results

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201222	Simulation result for coexistence study on NR to support non-terrestrial networks

Type: discussion		For: Information
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201255	NR NTN co-existence simulation Results

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201262	NTN coexistence results and observations

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

R4-2201316	NTN - Coexistence simulation results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our simulation results based on the agreed assumptions

Decision: 		The document was not treated.

R4-2201466	Simulation results for NTN coexistence study

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201842	NTN Simulation Assumptions

Type: discussion		For: Discussion
					Source: THALES

Abstract:

In this contribution we propose to discuss and modify some TN-NTN coexistence simulation assumptions/simulation methodologies from previous RAN4 meetings (see for example RAN4#101-e with approved R4-2120670, R4-2120671).

Decision: 		The document was not treated.

R4-2201843	NTN coexistence calibration data - THALES 28_09_2021

Type: discussion		For: Information
					Source: THALES

Abstract:

The Excel file with NTN calibration data provided by THALES prior to RAN4#101-e meeting (Nov. 2021). The document is for information only.

Decision: 		The document was not treated.

R4-2201852	NR-NTN coexistence results - THALES updates 10_01_2022

Type: discussion		For: Information
					Source: THALES

Abstract:

The Excel file contains TN-NTN coexistence results provided by THALES prior to RAN4#101-bis-e meeting (10th of January 2022). The contribution is for information purpose only.

Decision: 		The document was not treated.

##### 6.13.2.2	HAPS coexistence scenarios and simulations

R4-2200782	Coexistence simulation restuls for HAPS

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201077	HAPS simulation assumptions for coexistence study

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201078	TP to TR 38.863 on HAPS coexistence study

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201254	Discussion on HAPS requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.13.2.3	ACLR/ACS proposals

R4-2200166	ACLR/ACS proposal

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2201079	ACLR and ACS proposal for HAPS

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201126	NR-NTN co-ex results analysis and ACLR ACS proposal

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201127	Draft Text Proposal for TR 38.863 Chapter 6.3 and 6.4

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201256	Discussion on ACLR and ACS for NR NTN

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201317	NTN - ACLR/ACS proposals

Type: other		For: Approval
					Source: Ericsson

Abstract:

Based on our simulation results, this contribution is proposing ACLR and ACS limits for NTN satellite access node and NTN UE.

Decision: 		The document was not treated.

R4-2201467	Discussion on ACLR and ACS requirements for NTN

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201839	TN-NTN Coexistence Results Updates

Type: discussion		For: Agreement
					Source: THALES, Magister Solutions Ltd

Abstract:

New results based on new methodology from R4-2120671 (simulation assumptions RAN4#101-e), R4-2120670 (WF NTN_Solutions_Part2 RAN4#101-e).

Decision: 		The document was not treated.

#### 6.13.3	Satellite Access Node RF requirements

R4-2200167	Remaining issue for satellite access node RF requirement - Tx part

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200168	Remaining issue for satellite access node RF requirement - Rx part

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

##### 6.13.3.1	TX requirements for radiated characteristics

R4-2200169	Discussion on NTN BS RF requriement for type 1-O - Tx part

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2201258	Discussion on UE Tx requirements for satellite access

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201260	TP for 38.863 on UE transmitter characteristics for satellite access

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201318	NTN - Satellite Node Access - Tx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node access - Tx requirements

Decision: 		The document was not treated.

R4-2201468	Discussion on radiated Tx requirements of satellite access node

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201816	Discussion on Satellite Access Node radiated RF requirements: Tx

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further discussion on the radiated Tx requirements for the Satellite Access Node.

Decision: 		The document was not treated.

##### 6.13.3.2	RX requirements for radiated characteristics

R4-2200170	Discussion on NTN BS RF reqruiement for type 1-O - Rx part

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2201259	Discussion on UE Rx requirements for satellite access

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201261	TP for 38.863 on UE Receiver characteristics for satellite access

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201319	NTN - Satellite Node Access - Rx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node access - Rx requirements

Decision: 		The document was not treated.

R4-2201469	Discussion on radiated  Rx requirements of satellite access node

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201817	Discussion on Satellite Access Node radiated RF requirements: Rx

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further discussion on the radiated Rx requirements for the Satellite Access Node.

Decision: 		The document was not treated.

##### 6.13.3.3	Tx requirements for conducted characteristics

R4-2201320	NTN - Satellite Node Access - OTA Tx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node access - OTA Tx requirements

Decision: 		The document was not treated.

R4-2201470	Discussion on conducted Tx requirements of satellite access node

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201818	Discussion on Satellite Access Node conducted RF requirements: Tx

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further discussion on the conducted Tx requirements for the Satellite Access Node.

Decision: 		The document was not treated.

R4-2201819	Discussion on 64QAM support for the Satellite Access Node operation in FR1

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Abstract:

In this contribution we provide simulation results and related discussion to justify the performance gains of DL 64QAM support for the NTN Satellite Access Node operation in FR1.

Decision: 		The document was not treated.

##### 6.13.3.4	Rx requirements for conducted characteristics

R4-2201321	NTN - Satellite Node Access - OTA Rx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses Satellite node access - OTA Rx requirements

Decision: 		The document was not treated.

R4-2201471	Discussion on conducted RF requirements from satellite network perspective

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201820	Discussion on Satellite Access Node conducted RF requirements: Rx

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further discussion on the conducted Rx requirements for the Satellite Access Node.

Decision: 		The document was not treated.

#### 6.13.4	UE RF requirements

##### 6.13.4.1	TX requirements

R4-2200073	Further discussion on UE Tx RF requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200331	Discussion on UE TX requirements for NTN

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-17)

					Source: Mediatek India Technology Pvt.

Decision: 		The document was not treated.

R4-2200783	Considerations on NTN UE Tx requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201310	Discussion on UE Tx requirements for satellite access

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201312	TP for 38.863 on UE transmitter characteristics for satellite access

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201322	NTN - UE - Tx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses UE - Tx requirements

Decision: 		The document was not treated.

R4-2201472	Discussion on NTN UE Tx RF requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

##### 6.13.4.2	RX requirements

R4-2200074	Further discussion on UE Rx RF requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200332	Discussion on UE RX requirements for NTN

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-17)

					Source: Mediatek India Technology Pvt.

Decision: 		The document was not treated.

R4-2200784	Considerations on NTN UE Rx requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201223	TP for 38.863 on maximum input power for NTN UE

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201224	Discussion on Maximum input level for NTN UE

Type: other		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201311	Discussion on UE Rx requirements for satellite access

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201313	TP for 38.863 on UE Receiver characteristics for satellite access

Type: pCR		For: Approval
					38.863 v0.1.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201323	NTN - UE - Rx requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses UE - Rx requirements

Decision: 		The document was not treated.

R4-2201473	Discussion on NTN UE Rx RF requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

#### 6.13.5	RRM core requirements

##### 6.13.5.1	General

R4-2200075	Further discussion on  RRM requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200419	General and RRM requirements impacts

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200564	Discussion on NTN general requirements

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2200737	Discussion on General RRM Requirements for NTN UE

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200890	General requirements for NTN

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General requirements for NTN

Decision: 		The document was not treated.

R4-2200930	Discussion on general RRM requirements in NTN

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201141	Discussion on general RRM requirements for NTN

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201520	On the SMTC windows

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

We present our views on measurement gaps.

Decision: 		The document was not treated.

R4-2201587	Reply LS to RAN1: LS on open loop closed loop dual correction of timing

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

Draft Reply LS to RAN1 regarding open loop closed loop dual correction of timing.

Decision: 		The document was not treated.

R4-2201627	Discussion on general issues for NTN RRM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201628	CR on general issues for NTN

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.13.5.2	GNSS-related requirements

R4-2200076	Further discussion on GNSS-related requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200804	Discussion on NTN GNSS related issues

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2201142	Discussion on GNSS-related requirements for NTN

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201631	Discussion on GNSS related issue for NTN

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.13.5.3	Mobility requirements

R4-2200077	Further discussion on mobility requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200420	Mobility requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200523	Discussion on the mobility aspects for NR NTN UE

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200679	Further discussion on mobility requirements for NR NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200864	Discussion on mobility requirement for NR NTN

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was not treated.

R4-2200892	Mobility requirements for NTN

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Mobility requirements for NTN

Decision: 		The document was not treated.

R4-2201159	Discussion on mobility requirements for NTN

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201629	Discussion on mobility requirements for NTN RRM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201630	CR on mobility requirements for NTN

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.13.5.4	Timing requirements

R4-2200078	Further discussion on timing requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200297	Discussion on timing requirements for NR NTN

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200421	Timing requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200525	Discussion on the remaining issues for NTN timing requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200565	Discussion on NTN timing requirements

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2200680	Further discussion on timing requirements for NR NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200681	DraftCR on timing requirements for NR NTN

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200738	Discussion on timing requirements for NTN UE

Type: other		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200805	Discussion on NTN timing requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200931	Discussion on timing requirements in NTN

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201160	Discussion on UL timing requirements for NTN

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201445	On timing advance control

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we describe the problem of the combination of open and closed loop TA control and propose a way forward

Decision: 		The document was not treated.

R4-2201493	LS on Timing Advance control for Rel-17 NTN RRM

Type: LS out		For: Approval
					to RAN1
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201585	UE Timing requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Analysis of remaining NTN timing requirements.

Decision: 		The document was not treated.

R4-2201586	Reply LS to RAN1: LS on NTN UL time and frequency synchronization requirements (Timing)

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

Draft Reply LS to RAN1 regarding UE timing requirements.

Decision: 		The document was not treated.

R4-2201610	Discussion on UE timing related requirements for NR NTN

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.13.5.5	Measurement procedure requirements

R4-2200079	Further discussion on measurement procedure requirements for NTN

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200298	Discussion on measurement procedure requirements for NTN

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200422	Measurement procedure requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200524	Discussion on multiple SMTC and measurement gaps for NTN UE

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200682	Further discussion on measurement requirements for NR NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200865	Discussion on NTN measurement requirements

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was not treated.

R4-2200891	Measurement requirements for NTN

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Measurement requirements for NTN

Decision: 		The document was not treated.

R4-2201161	Discussion on measurement procedure requirements for NTN

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201632	Discussion on measurement requirements for NTN

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

#### 6.13.6	Demodulation requirements

##### 6.13.6.1	General

R4-2200475	Discussion on general issue for NTN NR

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General issue discusion for NTN NR

Decision: 		The document was not treated.

##### 6.13.6.2	Satellite Access Node demodulation requirements

R4-2200171	Discussion on Satellite Access Node demodulation requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200476	Discussion on satellite access node demodulation requirement for NTN NR

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Satellite access node demodulation issue discussion for NTN NR

Decision: 		The document was not treated.

R4-2201015	Discussion on UE NTN demod

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201785	Discussion on Satellite Access Node demodulation requirements for NR NTN

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.13.6.3	UE demodulation requirements

R4-2201016	Discussion on satellite NTN demod

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201420	Discussion on UE demodulation for NTN

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the general aspects for UE demodulation for NTN

Decision: 		The document was not treated.

R4-2201786	Discussion on UE demodulation requirements for NR NTN

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

### 6.14	UE Power Saving Enhancements for NR

#### 6.14.1	General

R4-2200599	LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: vivo

Decision: 		The document was not treated.

#### 6.14.2	RRM core requirements

R4-2200325	On Power Saving RRM Requirement

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

R4-2200530	Discussion on NR UE power saving for RLM and BM

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.14.2.1	UE measurements relaxation for RLM and/or BFD

R4-2200105	Further discussion on RLM/BFD relaxation for UE power saving enhancement

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200106	Draft CR on relaxed measurement criteria for BFD

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200258	UE measurements relaxation for RLM and/or BFD

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200600	Discussion on RLM and BFD relaxation for NR UE power saving

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200601	draft CR on CSI-RS RLM requirements relaxation for R17 UE power saving

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200685	Further discussion on UE measurements relaxation for RLM and/or BFD

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200686	Draft CR Minimum requirement for CSI-RS based beam failure detection for UE configured with relaxed measurement criterion

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200797	Draft CR for TS 38.133 Minimum requirement for SSB based BFD for UE configured with relaxed measurement criterion

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not treated.

R4-2200806	Discussion on RLM/BFD relaxation for NR power saving enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200896	Discussion about RLM/BFD measurement relaxation

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200897	38.133 draft CR on RLM relaxation criteria

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201143	Discussion on RRM requirements for R17 RLM/BFD relaxation

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201406	RLM and RLF relaxation for UE power saving

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201611	Discussion on UE measurement relaxation for RLM/BFD

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201612	DraftCR on SSB based relaxed RLM requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201867	Discussions on UE power saving for RLM and BFD

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss the remaining issues of Rel-17 UE power saving.

Decision: 		The document was not treated.

R4-2201868	Draft CR: Applicability rule for relaxed BFD requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Draft CR: Applicability rules for relaxed BFD requirements

Decision: 		The document was not treated.

R4-2201962	Discussion on Rel-17 RLM/BFD measurement relaxation

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201963	CR on TS38.133 for applicability of RLM measurement relaxation

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

### 6.15	NR Sidelink enhancement

#### 6.15.1	General

R4-2200833	TR38.785 v0.5.0 TR Update for SL enhancement in Rel-17

Type: draft TR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

[Email Approval] Provide revised TR to include these approved TPs in this meeting.

Decision: 		The document was not treated.

R4-2200946	TP for TR 38.785: Addition of definitions and symbols to Chapter 3

Type: pCR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

#### 6.15.2	UE RF requirements for NR SL enhancement

R4-2200842	TP on RF requirements for NR PS UE in n14 for NRSL_enh WI in Rel-17

Type: pCR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Propose TP to add the NR PS UE RF requirements in TR38.785 in Rel-17.

Decision: 		The document was not treated.

R4-2200848	Draft CR on RF requirements for SL enhancement for public safety service in n14

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Introduce NR PS UE RF requirements in TS38.101-1 in Rel-17.

Decision: 		The document was not treated.

##### 6.15.2.1	Configured Tx power requirements

##### 6.15.2.2	REFSENS requirements

R4-2200509	n14 REFSENS for PS in licensed band

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Presents REFSENSE numbers for n14 PS

Decision: 		The document was not treated.

##### 6.15.2.3	Other RF requirements

R4-2200138	Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-17)

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200139	Draft CR for TS 38.101-1, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16)

Type: draftCR		For: Endorsement
					38.101-1 v16.10.0	  CR-  rev  Cat: F (Rel-16)

					Source: CATT

Decision: 		The document was not treated.

R4-2200140	Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-17)

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200141	Draft CR for TS 38.101-3, Correction on MOP requirements for inter-band V2X con-current operation (Rel-16)

Type: draftCR		For: Endorsement
					38.101-3 v16.10.0	  CR-  rev  Cat: F (Rel-16)

					Source: CATT

Decision: 		The document was not treated.

R4-2201952	TP for 38.785: TxD requirements for NR V2X

Type: pCR		For: Approval
					38.785 v0.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201953	Big CR: introduction of TxD requirements for NR V2X

Type: CR		For: Agreement
					38.101-1 v17.4.0	  CR-0989  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 6.15.3	Intra-band con-current operation between NR SUL and NR Uu

R4-2200841	Draft CR on RF requirements for intra-band con-current V2X operation in Rel-17

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: LG Electronics France

Abstract:

Introduce RF core requirements for the intra-band con-current V2X UE in TS38.101-1 in Rel-17

Decision: 		The document was not treated.

##### 6.15.3.1	RF requirements for intra-band V2X con-current (including MPR)

R4-2200143	TP on configured transmitted power for intra-band V2X con-current operation

Type: pCR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200556	MPR for NR V2X intra-band con-current operation with Uu

Type: discussion		For: Approval
					Source: LG Electronics

Abstract:

It provides MPR for NR V2X intra-band con-current operation with Uu.

Decision: 		The document was not treated.

R4-2200834	RF requirements for intra-band con-current V2X operation in licensed band

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

This is a discussion and decision paper to finalize the ON/OFF time mask and configured Tx power for intra-band con-current V2X operation in the licensed band.

Decision: 		The document was not treated.

R4-2200840	TP on RF requirements for intra-band con-current V2X operation in licensed band

Type: pCR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Propose the ON/OFF time mask, MPR requirements, and configured Tx power for intra-band con-current V2X operation in the licensed band.

Decision: 		The document was not treated.

R4-2200947	Remaining issues for intra-band con-current operation

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201496	draft CR for TS 38.101-1 correctiron on intra-band concurrent operation

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201500	further discussion on configured power for intra-band concurrent operation

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201949	Draft CR for TS 38.101-1: configured transmitted power for intra-band con-current operation

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201950	MPR for intra-band con-current operation

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.15.3.2	Synchronous operation between SL and Uu (including switching time mask, SL transmission timing)

R4-2200142	Discussion on time mask for Uu and SL switching

Type: discussion		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200510	RF switching time for V2X intra-band con-current operation with different carriers in TDD bands and time masks for same carrier switching

Type: discussion		For: Approval
					38.101-2 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

RF switching requirements for intra-band con-current operation with different carrier is presented

Decision: 		The document was not treated.

R4-2201021	TP on sync issue for intra-band V2X operation

Type: pCR		For: Approval
					38.785 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2201497	draft CR for TS 38.101-1 on switching time mask between SL and Uu

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201502	further discussion on switching time mask between SL and Uu

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201948	On SL switching time mask

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 6.15.4	High power UE(PC2) for SL

##### 6.15.4.1	TX requirements (Power class)

R4-2201498	draft CR for TS 38.101-3 on Pcmax definition on inter-band V2X UE

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: F (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201501	further discussion on Pcmax definition on inter-band V2X UE

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

##### 6.15.4.2	Coexistence study

R4-2201499	further discussion on co-existence issue for HPUE

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201708	Co-channel existing

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our views on co-channel interference aspect

Decision: 		The document was not treated.

R4-2201951	On co-channel existence issue in RAN4

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.15.4.3	Others

#### 6.15.5	RRM core requirements

R4-2200326	On NR SL RRM Requirement

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

##### 6.15.5.1	Intra-band con-current V2X operation

R4-2200107	Draft CR on UE transmit timing requirements for sidelink enhancement

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200687	Further discussion on RRM requirements for intra-band con-current V2X operation

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201144	Discussion on RRM impact of intra-band concurrent V2X operation

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201365	Further discussion on Intra-band con-current V2X operation RRM requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201404	RRM requirements for FDM based intra-band con-current SL operation

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201613	Discussion on RRM requirements related to intra-band con-current V2X operation

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.15.5.2	SL-DRX

R4-2200108	Further discussion on RRM requirements related to SL-DRX

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200557	RRM requirements for SL-DRX

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses RRM core requirements related to SL-DRX.

Decision: 		The document was not treated.

R4-2200688	Further discussion on RRM requirements related to SL-DRX

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201162	Discussion on SL-DRX

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201366	Further discussion on SL-DRX RRM requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201367	Draft CR on Selection Reselction of V2X Synchronization Reference Source for sidelink enhancement

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2201403	Discussions on DRX in NR SL enhancement

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201614	Discussion on RRM requirements related to SL DRX

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201871	Discussions on SL DRX for Rel-17 SL operation

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Sidelink DRX was discussed at previous meeting and a number of open issues were identified in [1]. In this contribution, we discuss and provide our view on those.

Decision: 		The document was not treated.

##### 6.15.5.3	Others

R4-2200109	Discussion on L1-RSRP measurement for sidelink enhancement

Type: discussion		For: Discussion
					Source: CATT

Abstract:

Session Chair: Contribution is not available and will not be treated.

Decision: 		The document was not treated.

R4-2200558	draft CR on interruption requirement for SL

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: LG Electronics

Abstract:

It is draftCR to introduce interrutption requirement for NR SL enh.

Decision: 		The document was not treated.

R4-2200689	Draft CR on requirements for InitiationCease of SLSS Transmissions impact by SL-DRX

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201615	DraftCR on scheduling availability requirements for NR eV2X

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2202021	CR: SL autonomous resource allocation requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm communications-France

Decision: 		The document was not treated.

### 6.16	Extending current NR operation to 71GHz

#### 6.16.1	General

R4-2200469	Views on sensing beam selection on the UE side

Type: other		For: Discussion
					Source: Sony

Decision: 		The document was not treated.

R4-2200847	Draft LS on sensing beam characteristics to RAN1

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present some technical background related to beam quality properties previously discussed in RAN4 and our view on the need for beam quality requirements and corresponding test aspects relevant for BS and UE operating within the fre

Decision: 		The document was not treated.

R4-2200948	Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2201410	Discussion and draft reply LS on sensing beam selection from RAN1

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201533	Discussion and draft reply LS on sensing beam selection from RAN1

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201923	CR work split and UE feature list for NR ext. to 71GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

#### 6.16.2	Operation bands and system parameters (channelization, raster, CBW, etc)

R4-2200080	Discussion of channelization for up to 71 GHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200081	Draft LS for the channelization for up to 71 GHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200282	Channel and Sync rasters for NR operation in 52.6GHz - 71GHz

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200321	draft CR to 38.101-2 60 GHz UE general clauses

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

n263 CA, max TX BW, channel BWs added

Decision: 		The document was not treated.

R4-2200863	Channel arrangement and channel bandwidths for n263

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Draft CR to introduce channel and synchronisation rasters and channel bandwidths for n263

Decision: 		The document was not treated.

R4-2200949	Further discussion on channel raster and sync raster for 52.6~71 GHz

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201490	Draft CR to TS 38.104: Section 5.4 Channel arrangement

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Introduction of Channel Arrangement for NR extension to 71 GHz

Decision: 		The document was not treated.

R4-2201491	52.6-71 GHz System Parameters

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

This contribution will further highlight the design details of Option 1C and Option 1D using the above agreements as benchmarking criteria.

Decision: 		The document was not treated.

R4-2201592	60GHz channel and synchronization raster

Type: discussion		For: Approval
					Source: LG Electronics Finland

Abstract:

Channel raster and SSB raster for 57-71GHz frequency range is discussed and proposals for both are made.

Decision: 		The document was not treated.

R4-2201598	System parameters for a NR band in the range 52.6GHz – 71GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution synchronization raster, carrier aggregation and spectrum utilization in FR2-2 are discussed.

Decision: 		The document was not treated.

R4-2201924	Views on channelization for 52.6 to 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201985	Channelization and synchronization raster for 60GHz

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

R4-2202023	Discussion on the channel raster and sync raster in FR2-2

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

This contribution provides our views and proposals about channel raster and sync raster for band n263, and the possible licensed band in 66-71 GHz spectrum.

Decision: 		The document was not treated.

#### 6.16.3	UE RF requirements

R4-2200438	UE antenna module with 60 GHz integration

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

##### 6.16.3.1	TX requirements

R4-2200067	Views on UE antenna elements for FR2-2

Type: discussion		For: Discussion
					Source: Murata Manufacturing Co Ltd.

Decision: 		The document was not treated.

R4-2200238	60GHz UE TX

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Discuss various UE TX requirements

Decision: 		The document was not treated.

R4-2200312	draft CR to 38.101-2 60 GHz UE TX

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

EIRP, spherical coverage, SEM, ALCR, TX PSD requirements added

Decision: 		The document was not treated.

R4-2200453	Remaining issues with transient requirements for FR2-2

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200470	UE Array, EIRP level and Spherical Coverage at 60 GHz

Type: other		For: Decision
					Source: Sony, Ericsson

Decision: 		The document was not treated.

R4-2200570	View on TX requirements of FR2-2

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal: Antenna quantity assumption and Pout per RF chain assumption shall be agreed together as a package.

Decision: 		The document was not treated.

R4-2200950	Further discussion on handheld UE EIRP and spherical coverage requirements for 52.6~71 GHz

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201073	On UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201209	Discussion on Tx RF requirements in FR2-2

Type: discussion		For: Discussion
					Source: LG Electronics Finland

Abstract:

Views on Tx RF requirements for FR2-2

Decision: 		The document was not treated.

R4-2201411	On 60GHz UE Tx RF requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201534	On 60GHz UE Tx RF requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201925	UE Tx requirements for 52.6 to 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.16.3.2	RX requirements

R4-2200239	60 GHz UE RX

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Discuss various UE RX requirements

Decision: 		The document was not treated.

R4-2200307	draft CR to 38.101-2 60GHz UE RX

Type: draftCR		For: Endorsement
					38.101-2 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

define PC1, PC3 REFSENS and EIS spherical coverage

Decision: 		The document was not treated.

R4-2200360	Handheld UE antenna assumption for FR2-2

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Abstract:

To discuss antenna assumption for FR2-2 UE.

Decision: 		The document was not treated.

R4-2200951	Further discussion on handheld UE EIS requirements for 52.6~71 GHz

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201171	Discussion on Rx RF requirements in FR2-2

Type: discussion		For: Discussion
					Source: LG Electronics Finland

Abstract:

Views on Rx RF requirements for FR2-2.

Decision: 		The document was not treated.

R4-2201412	On 60GHz UE EIS requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2201535	On 60GHz UE EIS requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201926	UE EIS requirements for FR2-2

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

#### 6.16.4	BS RF requirements

##### 6.16.4.1	TX requirements

R4-2200136	Discussion on BS TX RF requirements for 52 6-71GHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200411	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides further proposals on BS transmitter requirements for extending current NR operation to 71 GHz according to the approved WF and the findings in the corresponding study item as recorded in TR 38.808.

Decision: 		The document was not treated.

R4-2200414	Draft CR to TR 38.104: Clauses 9.1 to 9.5

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Required changes to clauses 9.1 to 9.5 for extending current NR operation to 71 GHz

Decision: 		The document was not treated.

R4-2200843	On BS RF transmitter requirements for the frequency range 52 to 71 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present an overview of BS transmitter requirements, additional information and some proposals necessary to progress the work related to defining RF core requirements for the NR extension up to 71 GHz. To further stimulate the discu

Decision: 		The document was not treated.

R4-2200845	Draft CR to TS 38.104: Addition of requirements for NR extension up to 71 GHz in subclause 9.6 to 9.8

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

As part of the work to extend NR frequency range up to 71 GHz this draft CR adds requirements to subclauses 9.6 to 9.8. Additions will be collected in a “big” CR for the complete update of TS 38.104.

Decision: 		The document was not treated.

R4-2201456	Further discussion on BS Tx requirements for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201821	Discussion on the remaining BS RF requirements for FR2-2: Tx requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further inputs to the discussion on the BS RF transmitter requirements for the NR operation in FR2-2.

Decision: 		The document was not treated.

##### 6.16.4.2	RX requirements

R4-2200137	Discussion on BS RX RF requirements for 52 6-71GHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200151	Draft CR for TS 38.104 on introduction of BS RF Rx requirements for 57-71GHz in section 10.1 – 10.5

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200412	Proposals on BS receiver requirements for extending current NR operation to 71 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides further proposals on BS receiver requirements for extending current NR operation to 71 GHz according to the approved WF and the findings in the corresponding study item as recorded in TR 38.808.

Decision: 		The document was not treated.

R4-2200844	On BS RF receiver requirements for the frequency range 52 to 71 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we continue the discussion from [2] on BS RF receiver requirements relevant for the NR extension to support up to 71 GHz. The contribution presents an overview of base station receiver requirement changes compared to current version o

Decision: 		The document was not treated.

R4-2201457	Further discussion on BS Rx requirements for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201822	Discussion on the remaining BS RF requirements for FR2-2: Rx requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide further inputs to the discussion on the BS RF receiver requirements for the NR operation in FR2-2.

Decision: 		The document was not treated.

R4-2201823	Draft CR to TS 38.104: implementation of FR2-2 requirements: FRC annex

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Draft CR to TS 38.104 fot the FRC annex.

Decision: 		The document was not treated.

#### 6.16.5	Co-existence simulations

R4-2200039	Discussions on coexistence requirements for 60GHz

Type: other		For: Discussion
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R4-2200082	Discussion on ACIR requirement for 71 GHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200413	Proposals on coexistence simulation for extending current NR operation to 71 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides some preliminary simulation results based on the proposed assumptions and parameters in the approved WF and provides some proposals on coexistence simulation for extending current NR operation to 71.

Decision: 		The document was not treated.

R4-2200578	Discussion on ACIR requirements for 52.6-71 GHz

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision: 		The document was not treated.

R4-2200846	Update of coexistence simulation results relevant for NR extension to 71 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present additional results and summarize the current situation and propose a compromise to settle ACLR and ACS requirement for BS and UE.

Decision: 		The document was not treated.

R4-2200952	Discussion on DL/UL ACIR and BS/UE ACLR/ACS for FR2-2

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201455	Coexistence simulation results for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

#### 6.16.6	FR1+FR2-2 DC/CA band combinations

R4-2201599	How to introduce FR2-2 bands into 38.101-2 and combinations into 38.101-3

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In RAN4#101-e the changes to general parts of specifications due to introduction of FR2-2 was discussed and some draft CRs were endorsed. One of the open items was whether to separate FR2-2 bands and band combinations into separate tables and/or create fu

Decision: 		The document was not treated.

R4-2201916	Discussion on FR2-2 DC/CA with FR1 anchor

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on FR2-2 DC/CA with FR1 anchor

Decision: 		The document was not treated.

R4-2201917	draft CR 38.101-3 on FR2-2 DC/CA with FR1 anchor

Type: draftCR		For: Endorsement
					38.101-3 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

draft CR 38.101-3 on FR2-2 DC/CA with FR1 anchor

Decision: 		The document was not treated.

#### 6.16.7	RRM core requirements

R4-2200660	Reply LS on the minimum time gap for wake-up and Scell dormancy indication for NR operation in 52.6 to 71GHz

Type: LS out		For: Approval
					to RAN1
					Source: vivo

Decision: 		The document was not treated.

##### 6.16.7.1	General

R4-2200125	Further discussion on General RRM requirements for extension to 71GHz

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200562	Discussion on general RRM measurement requirements for extension to 71GHz

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2200654	Further discussion on RRM impacts for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200655	Draft CR to 38.133 Introducing applicability of requirements for FR2

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200888	General RRM requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General RRM requirements for extending NR operation to 71GHz

Decision: 		The document was not treated.

R4-2200889	On general measurement requriement for extending NR operation to 71GHz

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

On general measurement requriement for extending NR operation to 71GHz

Decision: 		The document was not treated.

R4-2200914	Discussion on general RRM requirements for extension to 71 GHz

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on general scenarios for FR2-2 and deriveSSB-IndexFromCell for RRM requriements

Decision: 		The document was not treated.

R4-2200932	Discussion on RRM requirements in FR2-2

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200933	Introduction of scheduling restriction for FR2-2

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201194	Discussion on general RRM requirements for FR2-2

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.16.7.2	Timing requirements

R4-2200126	Further discussion on RRM timing requirements for higher SCS

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200283	UE transmit timing for NR operation in 52.6GHz - 71GHz

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200563	Discussion on MTTD/MRTD for extension to 71GHz

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2200661	Further discussion on timing  for 52.6-71GHz

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200915	Discussion on RRM timing requirements for extension to 71 GHz

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on Te values, including proposals for boundaries and values to be applied when operating on FR2-2

Decision: 		The document was not treated.

R4-2200916	Draft CR adding timing requirements for FR2-2

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

The following additions are included by this CR:

-Placeholder for Te values for new SCSs

-Modified SSB availability for new SCSs in FR2-2

-Timing advance accuracy requirements as per agreements in the last meetings

Decision: 		The document was not treated.

R4-2200934	Discussion on timing requirements in FR2-2

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201195	Discussion on timing requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201407	On Te requirements for NR systems extended to 71 GHz

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201583	UE Timing requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Analysis of timing requirements.

Decision: 		The document was not treated.

R4-2201787	Discussion on timing requirements for NR 52.6 – 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2202033	Timing requirements in FR2-2

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss the UL timing error related issues

Decision: 		The document was not treated.

R4-2202048	Draft CR for timing requirements for FR2-2 – MRTD, MTTD

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

The draftCR updates clause 7.5 and 7.6 based on agreements related to MRTD and MTTD

Decision: 		The document was not treated.

##### 6.16.7.3	Interruption requirements

R4-2200656	Furthr discussion on interruption for 52.6-71GHz

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200657	Draft CR to 38.133 Introducing interruption requirements of FR2-2 to cover 52.6-71GHz

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200885	Interruption requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Interruption requirements for extending NR operation to 71GHz

Decision: 		The document was not treated.

R4-2200917	Discussion on interruption requirements for FR2-2

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Proposals to updated agreements reached for CA in previous meetings and apply them also for the DC cases that were missing.

Decision: 		The document was not treated.

R4-2201196	Discussion on interruption requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201788	Discussion on interruption requirements for NR 52.6 – 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.16.7.4	Active BWP switching delay requirements

R4-2200127	Discussion on cross-carrier active BWP switching for extension to 71GHz

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200284	Active BWP switch delay for NR operation in 52.6GHz - 71GHz

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200658	Further discussion on active BWP switching delay for 52.6-71GHz

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200872	On cross-carrier BWP switch delay for extension to 71 GHz

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200887	Active BWP switching delay requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Active BWP switching delay requirements for extending NR operation to 71GHz

Decision: 		The document was not treated.

R4-2201197	Discussion on BWP switching requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201198	Draft CR on BWP switching requirements for extending NR operation to 71GHz

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201409	Active BWP switching delay requirements for NR systems extended to 71 GHz

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201789	Discussion on BWP switching delay for NR 52.6 – 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 6.16.7.5	Measurement gap interruption requirements

R4-2200659	Further discussion on measurement gap interruption for 52.6-71GHz

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200886	Measurement gap interruption requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Measurement gap interruption requirements for extending NR operation to 71GHz

Decision: 		The document was not treated.

R4-2201199	Discussion on measurement gap interruption requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.16.7.6	LBT impacts on RRM requirements

R4-2200285	LBT impacts on RRM requirements for NR operation in 52.6GHz - 71GHz

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200873	LBT impacts on NR RRM requirements for extension to 71GHz

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201200	Discussion on LBT impacts on RRM requirements for extending NR operation to 71GHz

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201371	Discussion on LBT requirements for FR2-2

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201408	On LBT impacts on RRM for NR systems extended to 71 GHz

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201790	Discussion on LBT impacts on RRM requirements for NR 52.6 – 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201791	DraftCR on LBT impacts on RRM requirements for NR 52.6 – 71 GHz

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

#### 6.16.8	Others

R4-2200083	Discussion on the LBT requirement and the reply LS for sensing beam selection

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2200084	Draft CR for TS 37.107: introduction of LBT requirements for FR2-2

Type: draftCR		For: Endorsement
					37.107 v16.3.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200085	Draft CR for TS 37.106: introduction of LBT requirements for FR2-2

Type: draftCR		For: Endorsement
					37.106 v16.1.0	  CR-  rev  Cat: B (Rel-17)

					Source: CATT

Decision: 		The document was not treated.

R4-2200953	Discussion and draft reply LS on sensing beam selection

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201600	Sensing beam for LBT in FR2-2

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we discuss LBT and sensing beam from RAN4 perspective and propose a how to handle it in RAN4.

Decision: 		The document was not treated.

### 6.17	Enhancements to Integrated Access and Backhaul (IAB) for NR

#### 6.17.1	General

R4-2201702	LS response on range of power control parameters for eIAB

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present LS response on power control parameters

Decision: 		The document was not treated.

#### 6.17.2	RF requirements

##### 6.17.2.1	Impact for Simultaneous operation of IAB child and parent links

R4-2200936	Specification impact due to IAB simultaneous operation

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201595	RF requirements due to Tx power imbalance between IAB-MT and IAB-DU

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution power imbalance between IAB-MT and IAB-DU is discussed in it is proposed how to capture the restrictions to core specifications.

Decision: 		The document was not treated.

R4-2201700	RF core specification impact for Simultaneous operation of DU and MT

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on RF core spec for simultaneous operation of DU and MT

Decision: 		The document was not treated.

##### 6.17.2.2	Impact for Timing enhancement

R4-2200839	Discussion on timing issues for simultaneous operation of IAB

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200937	Discussion on timing enhancement

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201596	Discussion on timing error for eIAB

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we further discuss issue of IAB timing error between IAB-DU and IAB-MT and propose how to specify the requirements.

Decision: 		The document was not treated.

R4-2201701	IAB MT /DU case 6 timing

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on RF impact on simultaneous operation of DU and MT.

Decision: 		The document was not treated.

R4-2201939	eIAB Timing error between MT TX and DU TX

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discuss the open issue on the IAB MT Tx to DU TX timing error.

Decision: 		The document was not treated.

##### 6.17.2.3	Others

R4-2200938	Discussion on range of power control parameters for Rel-17 eIAB

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2201597	Draft reply LS to RAN1 on Power control parameters

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

RAN1 has sent a LS to RAN4 asking RAN4 to provide recommendation on min and max values for the desired DL Tx power adjustment, the actual DL Tx power adjustment and the desired UL Tx PSD range. In this contribution we discuss these and provide a draft rep

Decision: 		The document was not treated.

#### 6.17.3	RRM core requirements

R4-2201206	Discussion on RRM requirements for eIAB

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201207	Draft CR on timing requirements for Rel-17 IAB

Type: draftCR		For: Endorsement
					38.174 v16.5.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201405	On RRM for eIAB

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201849	On IAB Enhanced RRM Requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201850	TP to TS 38.174 on RRM Timing Requirements

Type: discussion		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2202019	Further analysis of RRM requirements for enhanced IAB

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

The paper further analyzes the impact of RRM on IAB enhancement

Decision: 		The document was not treated.

R4-2202052	CLI measurement requirement for R17 NR eIAB RRM

Type: discussion		For: Discussion
					Source: Qualcomm CDMA Technologies

Abstract:

Session Chair: Contribution is not available and will not be treated.

Decision: 		The document was not treated.

#### 6.17.4	Others

### 6.18	NR coverage enhancements

#### 6.18.1	General and CR structure

R4-2200020	Updated RAN4 RF work plan for NR coverage enhancements WI

Type: Work Plan		For: Approval
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200339	draft CR for EVM based requriements

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201706	simulation updated results for phase tolerance for PUSCH  repetition

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our updated simulation results

Decision: 		The document was not treated.

#### 6.18.2	UE RF requirements

R4-2201986	Some remaining open issues on coverage enhancements

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

##### 6.18.2.1	Requirements for non-scheduled gap

R4-2200021	RF requirements for the non-zero gap in between PUSCH/PUCCH transmissions

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200343	OFF power requirement for the gap in TX on case

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201705	RF impact on non-scheduled gap

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the RF requirement aspect of phase continuity.

Decision: 		The document was not treated.

##### 6.18.2.2	Tolerance for power consistency/phase continuity

R4-2200022	On phase continuity and power consistency tolerance

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200338	Discussion on UE requirement for JCE

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200471	Views on phase continuity and power consistency for PUSCH and PUCCH repetition

Type: other		For: Discussion
					Source: Sony

Decision: 		The document was not treated.

R4-2200926	Discussion on testability of coverage enhancement requirements

Type: discussion		For: Approval
					Source: ROHDE & SCHWARZ

Decision: 		The document was not treated.

R4-2201704	On JCE phase continuity and power consistency tolerance for PUCCH and PUSCH

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the RF requirement aspect on JCE phase continuity and power consistency tolerance for PUCCH and PUSCH repetition continuity

Decision: 		The document was not treated.

R4-2201840	Feasibility of proposed test methods for phase continuity and power consistency tolerance measurements

Type: discussion		For: Approval
					Source: Anritsu Limited

Decision: 		The document was not treated.

R4-2201958	On phase continuity for multiple transmissions

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.18.2.3	Maximum duration for joint channel estimation

R4-2200023	On maximum duration for joint channel estimation

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2201707	LS reply On maximum duration of phase continuity and power consistency for PUCCH and PUSCH repetition

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the FFS aspects of phase continuity and also our view on the LS questions.

Decision: 		The document was not treated.

##### 6.18.2.4	Others

R4-2200024	On UE autonomous adjustment and DL reception in-between transmission

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200344	Maximum duration handling for JCE

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201703	On measurement of the TX coherent transmission

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the RF requirement measurement aspect of phase /power discontinuity tolerance.

Decision: 		The document was not treated.

#### 6.18.3	BS demodulation requirements

R4-2200025	BS demodulation requirements for NR coverage enhancements

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2200408	BS demodulation requirements for NR coverage enhancements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we have provided an overview of NR coverage enhancements features and scenarios with a potential impact on the receiver demodulation performance.

Decision: 		The document was not treated.

R4-2200477	Discussion on BS demodulation requirements for NR coverage enhancement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

BS demodulatlion discussion for NR coverage enhancement

Decision: 		The document was not treated.

R4-2200756	View on demodulation requirement for Rel-17 coverage enhancement

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201017	Discussion on BS coverage enhancement demod

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201604	Discussion on scope of BS demodulation requirements for NR coverage enhancements WI

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

### 6.19	Further enhancements on MIMO for NR

#### 6.19.1	General

R4-2200276	Discussion on Impact to RF and RRM requirements with simultaneous reception

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200534	Discussion on FR2 simultaneous reception in NR FeMIMO

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200924	FR2 UE with the capability of simultaneous reception with different QCL Type-D RSs

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201266	Requirements for Simultaneous Reception in FR2

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201386	Discussion on Simultaneous reception with different QCL-type D

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribtuion, we discuss RRM requirements for simultaneous reception of channel/RS with different QCL type D

Decision: 		The document was not treated.

#### 6.19.2	UE RF requirements

R4-2200925	On RF requirements for further enhancements on MIMO

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

##### 6.19.2.1	Additional requirement for multi-panel reception

R4-2200568	View on additional FR2 FeMIMO multi-panel reception requirement

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal: No need to specify additional reception requirement for multi-panel reception UE.

Decision: 		The document was not treated.

R4-2200591	Discussion on Additional requirement for multi-panel reception

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200962	Further discussion on impact of multi-panel reception requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201957	RF requirements for further enhancements on MIMO

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 6.19.2.2	Impact of MPE enhancements

R4-2200301	On per beam based P-MPR reporting in FeMIMO

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200592	Discussion on Impact of MPE enhancements

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200963	Further discussion on impact of MPE requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

##### 6.19.2.3	SRS related impact

R4-2200342	Two sets of SRS sets and GP needed in between

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 6.19.3	RRM core requirements

##### 6.19.3.1	Unified TCI for DL and UL

R4-2200182	Discussion on unified TCI for DL and UL in R17 feMIMO

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200277	Discussion on RRM requirements for  Unified TCI

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200593	Discussion on Unified TCI for DL and UL

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200602	Discussion on RRM requirements for unified TCI in R17 feMIMO

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200642	Discussion on unified TCI

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200649	Discussion on requirements of unified TCI for DL and UL

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This paper discusses TCI switching delay requirements under Rel-17 unified TCI framework

Decision: 		The document was not treated.

R4-2200787	Discussion on Unified TCI state in NR FeMIMO

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201363	Unified TCI in FeMIMO

Type: discussion		For: Approval
					Source: Samsung

Abstract:

Overall specfication structure for unified TCI and proposals for open issues of unified TCI

Decision: 		The document was not treated.

R4-2201384	RRM requirements of unified TCI state for FeMIMO

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, we discuss RRM requirements for unified TCI state design requirements.

Decision: 		The document was not treated.

R4-2201616	Discussion on RRM requirements related to unified TCI framework for NR FeMIMO

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

##### 6.19.3.2	Inter-cell beam management

R4-2200183	Discussion on inter cell beam management in R17 feMIMO

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200278	Discussion on RRM requirements for  inter-cell beam management

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200594	Discussion on RRM requirements for inter-cell beam management

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2200603	Discussion on RRM requirements for inter-cell L1 beam measurements in R17 feMIMO

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200643	Discussion on inter-cell beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200650	Discussion on requirements of inter-cell beam management

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This paper discusses Rel-17 feMIMO beam management and L1-RSRP measurements

Decision: 		The document was not treated.

R4-2200788	Discussion on inter-cell beam management in NR FeMIMO

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201385	RRM requirements of inter-cell BM in FeMIMO

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, we discuss RRM requirements of inter-cell BM in FeMIMO

Decision: 		The document was not treated.

R4-2201617	Discussion on inter-cell beam management requirements for NR FeMIMO

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201960	Inter-cell beam management in FeMIMO

Type: discussion		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

##### 6.19.3.3	Others

R4-2200184	Discussion on general and RRM requirements impacts in R17 feMIMO

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2200279	Discussion on other RRM requirements for FeMIMO

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200535	Discussion on remaining issues in FeMIMO

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200536	DraftCR on TCI chain update for Rel-17 NR FeMIMO

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200604	Discussion on other RRM impacts in R17 feMIMO

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200651	Discussion on other items of Rel-17 feMIMO RRM requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This paper discusses Rel-17 feMIMO BFD/CBD/BFRQ and UE capability and requirements on simultaneous reception

Decision: 		The document was not treated.

R4-2201387	Discussion on FeMIMO other open issues

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribtuion, we discuss QCL definition update, and link recovery procedures for inter-cell beam management operation

Decision: 		The document was not treated.

R4-2201618	Discussion on other RRM requirements for NR FeMIMO

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

#### 6.19.4	UE Demodulation and CSI requirements

R4-2200280	On scope of demod requirements for FeMIMO

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200365	Views on Rel-17 HST-SFN scheme 1

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2200522	Discussion on demodulation performance requirements definition for Rel-17 NR feMIMO

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200644	Discussion on demodulation requirements for enhancement to support HST-SFN

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200767	Overview and work plan on performance requirements of Rel-17 FeMIMO WI

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2201014	Discussion on UE FeMIMO demod

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201421	Discussion on UE demodulation and CSI requirement for FeMIMO

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution  goes through all the new features of FeMIMO studied in RAN1 that are related to RAN4 UE demodulation, and proposes to define several requirements.

Decision: 		The document was not treated.

R4-2201841	Views on FeMIMO Tests

Type: discussion		For: Discussion
					Source: Qualcomm CDMA Technologies

Decision: 		The document was not treated.

R4-2201922	On UE Demodulation and CSI requirements for feMIMO

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have presented Nokia’s point of view concerning the CSI reporting performance requirement impact of the Rel-17 “further enhanced type II port selection codebook”, after giving some background on the new features and inner workings

Decision: 		The document was not treated.

### 6.20	Support of reduced capability NR devices

#### 6.20.1	General

R4-2200407	On NR RedCap general BS demodulation performance requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we have provided an overview of RedCap-related features and scenarios. We see that there is no potential impact on the BS demodulation performance.

Decision: 		The document was not treated.

#### 6.20.2	UE RF requirements

##### 6.20.2.1	FR1

R4-2201250	Draft CR for 38.101-1 to introduce RF requirements for RedCap UE

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, HiSilicon, CMCC, OPPO, CBN

Decision: 		The document was not treated.

R4-2201988	RedCap general UE RF requirements aspects

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

###### 6.20.2.1.1	Tx requirements (power class)

R4-2200496	RedCap UL Architecture and power class

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

A number of power class agreements were achieved in last meeting and, in this contribution, we further discuss the potential for PC2 implementations and preferred approach.

Decision: 		The document was not treated.

R4-2201279	R17 FR1 Redcap UE

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201301	Discussion on Tx requirements for FR1 Redcap UE

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201345	On FR1 Redcap UE 2Tx

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201709	CR on RedCap UE FR1-TX

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR on general and Tx  part for RedCap UE is introduced

Decision: 		The document was not treated.

###### 6.20.2.1.2	Rx requirements (REFSENS, etc)

R4-2200442	RedCap UE HD-FDD REFSENS requirements

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200472	Considerations on REFSENS for RedCap FR1

Type: other		For: Decision
					Source: Sony

Decision: 		The document was not treated.

R4-2201248	Discussion on FR1 REFSENS requirements for RedCap UE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201344	Further discussion on FR1 RedCap related requirements

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201710	CR on RedCap UE FR1-RX

Type: draftCR		For: Endorsement
					38.101-1 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR on general and  RX part for RedCap UE is introduced

Decision: 		The document was not treated.

##### 6.20.2.2	FR2

R4-2201249	Discussion on FR2 RF requirements for RedCap UE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

###### 6.20.2.2.1	Tx requirements (power class, UE type)

R4-2200473	Considerations on RF architecture for RedCap FR2

Type: other		For: Decision
					Source: Sony

Decision: 		The document was not treated.

R4-2200571	View on FR2 RedCap

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: For FR2 wearable, use “watch” as the typical assumption for requirement discussion.

Proposal2: Throughput, battery life, and UE implementation feasibility shall be considered together before specifying FR2 requirements for wearable.

Decision: 		The document was not treated.

R4-2200978	Discussion on FR2 RedCap Tx requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201302	Discussion on Tx requirements for FR2 Redcap UE

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201346	Discussion on FR2 RedCap UE

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201713	RF impact On FR2 RedCap

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the FR2 RedCap RF impact.

Decision: 		The document was not treated.

R4-2201972	On Redcap FR2-1 UE Tx RF assumptions

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Discussion on Tx topology, benefit of a lower TRP limit for a UE that is expected at the outset to be a low power device

Decision: 		The document was not treated.

###### 6.20.2.2.2	Rx requirements

R4-2200979	Discussion on FR2 RedCap Rx requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201303	Discussion on Rx requirements for FR2 Redcap UE

Type: discussion		For: Approval
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201712	On single polarization receiving on FR2 RedCap UE

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this paper, we present our view on single polarization receiving

Decision: 		The document was not treated.

R4-2201971	On Redcap FR2-1 UE Rx RF assumptions

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Discussion on whether a dual pol. receiver is appropriate for a RedCap UE

Decision: 		The document was not treated.

##### 6.20.2.3	Others

R4-2201711	RedCap FR1 Operating band n79

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on Redcap UE band n79

Decision: 		The document was not treated.

#### 6.20.3	RRM core requirements

R4-2201857	Updated WI work plan for RedCap for RRM

Type: other		For: Approval
					Source: Ericsson

Abstract:

Updated work plan and work split.

Decision: 		The document was not treated.

##### 6.20.3.1	Impacts from UE complexity reduction

###### 6.20.3.1.1	General

R4-2200392	On general aspects on complexity reduction of Redcap

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200692	Further discussion on general requirements on Redcap UE

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200808	On general requirements for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201183	Discussion on general RRM requirements impacts for RedCap UE

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201394	reply LS on capability related assumptions for RedCap

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: ZTE Corporation

Abstract:

We provide a draft LS reply to RAN2 LS R2-2109218 on UE capability assumptions.

Decision: 		The document was not treated.

R4-2201397	Discussions on general RRM aspects for RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201773	General discussion UE capability and scheduling availability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201858	Discussions on general requirements for RedCap

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

In this contribution we discuss general requirements that apply in all RRC states for RedCap.

Decision: 		The document was not treated.

R4-2201979	Discussion on PBCH simulation assumptions for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on PBCH simulation assumptions for NR_redcap

Decision: 		The document was not treated.

###### 6.20.3.1.2	Mobility requirements

R4-2200292	Discussion on mobility requirement for RedCap

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200393	On mobility requirements for Redcap

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200809	On mobility requirements for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200871	Discussion on mobility requirements for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201184	Discussion on UE mobility requirements due to UE complexity reduction

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201388	Requirements under RRC Connected mode for RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201774	Discussion on mobility requirements

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201859	Discussions on RedCap mobility requirements

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

In this contribution we discuss the mobility requirements for RedCap, HO, RRC re-establishment, RA and RRC connection release with redirection.

Decision: 		The document was not treated.

###### 6.20.3.1.3	Timing requirements

R4-2200810	On timings requirements for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201185	Discussion on UE timing requirements due to UE complexity reduction

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201389	Timing requirements for RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201775	UE complexity reduction impact on timing requirements

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201860	On UE transmit timing requirements in Redcap

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss the UE transmit timing requirements based on last WF.

Decision: 		The document was not treated.

###### 6.20.3.1.4	Signalling characteristics

R4-2200293	Discussion on signalling characteristics for RedCap

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200394	On signalling characteristics for Redcap

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200399	Simulation results for RLM and BFD

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200400	Simulation results for PDCCH RLM and BFD

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200811	On Signalling characteristics requirements for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200867	Simulation Results: RLM for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200868	Discussion on hypothetical PDCCH parameters for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201145	RRM impact from UE complexity reduction for Redcap UE - signaling characteristics

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201186	Discussion on signaling characteristics due to UE complexity reduction

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201187	Simulation results for RLM and BFD due to complexity reduction for RedCap UE

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201390	On Signalling characteristics of RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201776	Discussion on RLM and BFD in RedCap

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201861	Discussions on RedCap signaling characteristics

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss signaling characteristics for RedCap, e.g. RLM, link recovery etc.

Decision: 		The document was not treated.

R4-2202030	Impact of UE complexity reduction impact on signalling characteristics

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss  the simulation results for RLM

Decision: 		The document was not treated.

###### 6.20.3.1.5	Measurement procedure

R4-2200294	Discussion on cell identification and measurement for RedCap

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200395	On measurement procedure for Redcap

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200401	Simulation results for cell detection, PBCH detection, SSB measurement and L1 RSRP measurement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200812	On Measurement procedure for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200869	Discussion on measurement procedures for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200870	Simulation Results: cell detection performance for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201146	RRM impact from UE complexity reduction for Redcap UE - measurement requirements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201188	Discussion on measurement requirements due to UE complexity reduction

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201189	Simulation results for cell detection, L3 measurement and L1 RSRP measurement

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201391	Measurement procedure of RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201777	Simulation performance results for RedCap

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201862	Discussions on RedCap measurement procedure

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss CONNECTED mode measurement procedure for RedCap.

Decision: 		The document was not treated.

R4-2202031	Impact of UE complexity reduction on measurement procedures

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss  the simulation results for cell detection and PBCH decoding

Decision: 		The document was not treated.

##### 6.20.3.2	Extended DRX enhancements

R4-2200295	Discussion on RRM requirement with eDRX for RedCap

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200396	On Redcap eDRX enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200690	Further discussion on RRM requirements for extended DRX enhancements for RedCap

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200813	On Extended DRX cycle for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200866	Discussion on eDRX enhancements for RedCap

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201147	Extended DRX enhancements for Redcap UE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201190	Discussion on Extended DRX enhancements for RedCap UE

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201392	On extended DRX enhancements for RedCap

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201778	Extended DRX in IDLE mode and INACTIVE mode

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201863	Discussions on eDRX requirements for RedCap

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss eDRX requirements for RedCap

Decision: 		The document was not treated.

##### 6.20.3.3	RRM measurement relaxations

R4-2200296	Discussion on RRM relaxation requirement for RedCap

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200397	On Redcap RRM relaxation

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200691	Further discussion on RRM measurement relaxations for RedCap UE

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2200814	On RRM measurement relaxation for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201148	RRM measurement relaxations for RedCap UE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201191	Discussion on RRM measurement relaxations for RedCap UE

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201393	Discussions on RRM measurement relaxations for RedCap UEs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201779	RRM measurements relaxation for stationary criterion

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201864	Discussions on RRM measurement relaxations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss RRM measurement relaxation for RedCap.

Decision: 		The document was not treated.

R4-2201980	On RRM measurement relaxation for neighbouring cells

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on RRM relaxation for NR_redcap

Decision: 		The document was not treated.

R4-2202032	RRM relaxations enhancements for RedCap UE

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper we discuss RRM relaxations enhancements for RedCap UE

Decision: 		The document was not treated.

##### 6.20.3.4	Others

R4-2200398	On NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200815	Reply LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2201192	Discussion on NCD-SSB and CSI-RS

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201395	Reply LS on use of NCD-SSB for RedCap UE

Type: LS out		For: Approval
					to RAN1, cc RAN2
					Source: ZTE Corporation

Abstract:

We provide a draft LS reply to RAN1 LS R1-2110600 (R4-2117599). RAN4 provided some answers for part of the questions asked, and we propose to provide answers for all questions with another LS.

Decision: 		The document was not treated.

R4-2201396	reply LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Type: LS out		For: Approval
					to RAN1, cc RAN2
					Source: ZTE Corporation

Abstract:

We provide a draft LS reply to RAN1 LS R1-2112802 (R4-2200044) on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE.

Decision: 		The document was not treated.

R4-2201760	On capability and NCD-SSB design for RedCap RRM

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2201780	Discussion on the use of NCD-SSB

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2201865	RRM Discussions on RedCap UE capabilities

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

In this contribution we discuss the RRM aspects of this LS on UE capability and provide our view, and a draft response LS is provided in the Annex.

Decision: 		The document was not treated.

R4-2201981	Discussion on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion based on incoming RAN1 LS

Decision: 		The document was not treated.

#### 6.20.4	UE demodulation and CSI requirements

R4-2200281	On impact to demod requirements for Reduced capability devices in NR

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200382	Views on UE demodulation requirements for RedCap

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2200406	On NR RedCap general UE demodulation and CSI performance requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we have provided an overview of RedCap-related features with their potential impact on the UE demodulation and CSI performance requirements.

Decision: 		The document was not treated.

R4-2200816	On UE demodulation and CSI requirements for RedCap UE

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200994	Discussion on  UE demodulation and CSI requirements for RedCap

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201435	Work plan for UE demodulation requirements for RedCap

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution proposes the work plan of the demodulation performance part for RedCap WI.

Decision: 		The document was not treated.

R4-2201436	UE demodulation and CSI reporting requirements for RedCap

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our view on RedCap UE demodulation requirements.

Decision: 		The document was not treated.

R4-2201605	Discussion on scope of UE demodulation and CSI requirements for reduced capability NR devices

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201720	Introduction of RedCap Demodulation Requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

### 6.21	Positioning enhancements for NR

#### 6.21.1	General

R4-2200385	Work plan for Multi-SIM devices for LTE/NR

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was withdrawn.

#### 6.21.2	RRM core requirements

##### 6.21.2.1	UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

R4-2200118	Discussion on UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200119	Reply LS on the reporting of the Tx TEG association information

Type: LS out		For: Approval
					to RAN1, cc RAN2/RAN3
					Source: CATT

Decision: 		The document was not treated.

R4-2200540	Discussion on timing mitigating for NR positioning enhancement

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200668	Further discussion on timing delay error mitigation

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200757	On UE Rx/Tx timing error mitigation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201163	Discussion on UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201293	Impact of RX/TX TEG on UE requirements

Type: discussion		For: Discussion
					Source: Ericsson Inc.

Decision: 		The document was withdrawn.

R4-2201398	UE RxTx and gNB RxTx timing delay mitigation

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201636	Discussion on timing error mitigation for positioning

Type: LS out		For: Approval
					to RAN1, RAN2, cc RAN3
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201668	Impact of RX/TX TEG on UE requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution addresses open issues on TEG

Decision: 		The document was not treated.

R4-2201982	Discussion on timing error mitigation for NR positioning

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on timing error mitigation for NR positioning

Decision: 		The document was not treated.

R4-2202015	Work split on RRM core requirements for postioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

The paper provides structure of requirements and work split for CRs

Decision: 		The document was not treated.

R4-2202016	Big DraftCR on Positioning Enhancement

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This is Big Draft CR provided by rapporteur providing skeleton structure

Decision: 		The document was not treated.

##### 6.21.2.2	Latency reduction of positioning measurement

R4-2200120	Discussion on latency reduction of positioning measurement

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200541	Discussion on latency reduction for NR positioning enhancement

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200542	Simulation results summary for conditions with less PRS measurement samples

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200637	Discussion on latency reduction of positioning measurement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200645	Discussion on latency reduction of positioning measurement

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on latency reduction of Rel-17 positioning measurement

Decision: 		The document was not treated.

R4-2200646	Discussion on PRS measurement performance with reduced sample

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on simulation results of PRS measurements with reduced sample

Decision: 		The document was not treated.

R4-2200664	Simulation results for PRS measurement with reduced sample

Type: other		For: Information
					Source: vivo

Decision: 		The document was not treated.

R4-2200665	Further discussion on latency reduction of positioning measurement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200667	Reply LS on the condition of PRS measurement outside the MG

Type: LS out		For: Approval
					to RAN1
					Source: vivo

Decision: 		The document was not treated.

R4-2200758	Simulation results with reduced number of PRS samples

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200759	On latency reduction of NR positioning measurements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201164	Discussion on latency reduction of positioning measurements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201294	Link level simulation results for latency reduction for UE measurements

Type: discussion		For: Discussion
					Source: Ericsson Inc.

Decision: 		The document was withdrawn.

R4-2201306	On latency reduction for positioning measurement

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson Inc.

Decision: 		The document was withdrawn.

R4-2201370	Draft CR to 38.133 Introducing requirements for latency reduction of positioning measurement

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2201399	Discussions on latency reduction of positioning measurement

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201637	Simulation results for reduced sample number for PRS measurement

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201638	On latency reduction for positioning measurement

Type: LS out		For: Approval
					to RAN1
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201639	CR on latency reduction of positioning measurements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201669	Link level simulation results for latency reduction for UE measurements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses simulation results for latency reduction

Decision: 		The document was not treated.

R4-2201670	On latency reduction for positioning measurement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discussess impact of sample reduction on positioning accuracy

Decision: 		The document was not treated.

##### 6.21.2.3	Measurement in RRC_INACTIVE state

R4-2200121	Discussion on measurement in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200543	Discussion on measurements in RRC_INACTIVE for NR positioning enhancement

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200636	Discussion on positioning measurement in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2200760	On NR positioning measurements in RRC_INACTIVE

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201165	Discussion on PRS measurements in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201364	Further discussion on measurement in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201400	Positioning measurements in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201640	Discussion on PRS measurement in RRC_INACTIVE

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201641	CR on positioning measurements in RRC Inactive state

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201983	Discussion on measurement in RRC_INACTIVE state

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on positioning measurements in RRC_Inactive

Decision: 		The document was not treated.

R4-2202017	Analysis PRS measurement requirements in RRC inactive state

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

The paper further analyzes the positioning requirements in RRC inactive state

Decision: 		The document was not treated.

##### 6.21.2.4	Impact on existing UE positioning and RRM requirements

R4-2200122	Discussion on impact on existing UE positioning and RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2200647	Discussion on Impact on NR positioning RRM requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on Impact on existing UE positioning and RRM requirements including SRS antenna port switching

Decision: 		The document was not treated.

R4-2200663	Further discussion on impact to existing UE positioning and RRM requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201166	Impacts of SRS antenna port switching on positioning

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201307	Impact of enhanced positioning on existing RRM

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson Inc.

Decision: 		The document was withdrawn.

R4-2201401	Impact on existing UE positioning and RRM requirements

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201642	Discussion on RAN4 specific enhancements for positioning

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201671	Impact of enhanced positioning on existing RRM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discsses impact of enhanced positioning procedures on RRM procedures.

Decision: 		The document was not treated.

R4-2202018	Impact of RRM on positioning requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

The paper further analyzes the impact of RRM on positioning requirements

Decision: 		The document was not treated.

##### 6.21.2.5	Enhancements of A-GNSS positioning

##### 6.21.2.6	Others

R4-2200123	Reply LS on the condition of PRS measurement outside the MG

Type: LS out		For: Approval
					to RAN1
					Source: CATT

Decision: 		The document was not treated.

R4-2200124	Reply LS on lower Rx beam sweeping factor for latency improvement

Type: LS out		For: Approval
					to RAN1
					Source: CATT

Decision: 		The document was not treated.

R4-2200648	Discussion on other items of NR ePos requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on DL PRS path RSRP, UL SRS path RSRP and RSTD reporting enhancement

Decision: 		The document was not treated.

R4-2200662	Further discussion on First path PRS-RSRP requirments

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200666	Link level simulation assumption for first path PRS-RSRP measurement

Type: discussion		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201167	Other issues of positioning enhancements for NR

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201309	LS response on SRS on UL SRS-RSRPP definition

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson Inc.

Decision: 		The document was withdrawn.

R4-2201402	on PRS measurement outside the measurement gap

Type: discussion		For: Discussion
					Source: ZTE Corporation

Abstract:

This paper discusses the matter of PRS measurement outside the measurement gap based on RAN1 LS.

Decision: 		The document was not treated.

R4-2201643	Discussion on path RSRP

Type: LS out		For: Approval
					to RAN1
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201672	LS response on SRS on UL SRS-RSRPP definition

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

This contribution is a response to LS sent out by RAN1 on reference point for UL SRS-RSRPP measurement.

Decision: 		The document was not treated.

### 6.22	Multi-Radio Dual-Connectivity enhancements

#### 6.22.1	General

#### 6.22.2	RRM core requirements

##### 6.22.2.1	Efficient activation/de-activation mechanism for SCells

R4-2200063	Discussion on efficient activation/de-activation mechanism for SCell

Type: discussion		For: Discussion
					Source: MediaTek (Shenzhen) Inc.

Decision: 		The document was not treated.

R4-2200248	On efficient activation/de-activation mechanism for SCells

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200403	Draft CR for Interruption due to A-TRS based fast SCell activation

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200423	Efficient activation and deactivation mechanism for SCells

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201149	Discussion on efficient activation/de-activation mechanism for Scells

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201179	Discussion on efficient activation/de-activation mechanism for Scells

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201180	Draft CR on fast activation delay

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201690	Discussion on efficient activation/de-activation mechanism for Scells

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201886	On efficient (de)activation of Scell

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Scell activation delay discussion

Decision: 		The document was not treated.

##### 6.22.2.2	Efficient activation/de-activation mechanism for one SCG

R4-2200064	 Discussion on efficient activation/de-activation mechanism for one SCG

Type: discussion		For: Discussion
					Source: MediaTek (Shenzhen) Inc.

Decision: 		The document was not treated.

R4-2200249	On efficient activation/de-activation mechanism for one SCG

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200250	CR on interruptions due to SCG activation/deactivation

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200424	Efficient activation and deactivation mechanism for one SCG

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200526	Discussion on efficient activation for one SCG

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200527	DraftCR on interruptions due to RRM measurements on deactivated SCG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200652	Further discussion on efficient activation deactivation mechanism for one SCG

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201150	Discussion on efficient activation/de-activation mechanism for one SCG

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201181	Discussion on efficient activation/de-activation mechanism for one SCG

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201691	Views on efficient activation/de-activation mechanism for one SCG

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201761	CR on TS36.133 for interruptions due to SCG activation/deactivation

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2201887	On efficient (de)activation of SCG

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

SCG activation delay, interruption, deactivated status measurement relaxation

Decision: 		The document was not treated.

R4-2201890	Interruptions requirments due to RLM/BFD on deactivated SCG

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

draft CR for Interruptions due to RLM/BFD on deactivated SCG

Decision: 		The document was not treated.

##### 6.22.2.3	Conditional PSCell change and addition

R4-2200065	Discussion on conditional PSCell addition

Type: discussion		For: Discussion
					Source: MediaTek (Shenzhen) Inc.

Decision: 		The document was not treated.

R4-2200066	Draft CR on 38.133 for Conditional PSCell addition delay

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: MediaTek (Shenzhen) Inc.

Decision: 		The document was not treated.

R4-2200251	On conditional PSCell change and addition

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200653	Further discussion on conditional PSCell change and addition

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201151	Discussion on conditional PSCell change and addition

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201182	Discussion on conditional PSCell change and addition

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201692	Conditional PSCell change and addition

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201888	On conditional Pscell change addition

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Conditional Pscell change delay

Decision: 		The document was not treated.

##### 6.22.2.4	Others

R4-2201152	Draft CR to 38133 on SCG Activation and deactivation delay

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201153	Draft CR to 36133 on SCG Activation and deactivation delay

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

R4-2201889	LS on Measurement requirement for deactivated SCG

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

Draft LS to RAN 2 to clarify whether will introduce parameter for deactivated SCG measurement

Decision: 		The document was not treated.

### 6.23	Enhanced IIoT and URLLC support

#### 6.23.1	General

#### 6.23.2	RRM core requirements

##### 6.23.2.1	Propagation delay compensation enhancements

R4-2201168	Discussion on propagation delay compensation enhancements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201368	Further discussion on propagation delay compensation enhancements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201584	Propagation Delay Compensation Enhancements for Time Synchronization

Type: other		For: Approval
					Source: Ericsson

Abstract:

Requirements for RTT PDC.

Decision: 		The document was not treated.

R4-2201633	On RRM requirements for PDC enhancements

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201634	CR on requirements for UE Rx-Tx measurement for PDC

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201721	Discussion of RRM Requirements for Propagation Delay Compensation Enhancements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was withdrawn.

R4-2201723	Discussion of RRM Requirements for Propagation Delay Compensation Enhancements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2201781	RTT-based PDC enhancements in URLLC

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

##### 6.23.2.2	Reference point for Te requirements

R4-2200528	DraftCR to clarify timing reference point for UE UL timing test cases

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201369	Further discussion on reference point for Te requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201635	On reference point for Te requirements

Type: LS out		For: Approval
					to RAN1
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201673	Discussion on reference point for Te requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201722	Further discussion on the reference point for UE transmit timing requirement

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2201782	Reference point for Te requirements

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2202013	LS response on UE transmit timing error

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

This document further analyze the remaining issue of the reference point definition for UE timing error requirements. It is continuation of LS response to RAN1 in R4-2105850.

Decision: 		The document was not treated.

R4-2202014	Correction to reference point defintion for UE timing in TS 38.133

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, Intel, Huawei, HiSilicon

Abstract:

Definition of reference point for UE timing error is clarified

Decision: 		The document was not treated.

##### 6.23.2.3	Others

### 6.24	NR Sidelink Relay

#### 6.24.1	General

R4-2201154	CR to 38133 on measurement requirements for Selection/reselection of relay UE

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: OPPO

Decision: 		The document was not treated.

#### 6.24.2	RRM core requirements

R4-2200324	On NR SL relay RRM Requirement

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

R4-2200330	CR: SL discovery signal intra-frequency measurement accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Qualcomm, Inc.

Decision: 		The document was not treated.

R4-2201155	RRM requirements for SL relay (re)selection

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201619	Discussion on RRM requirements for R17 NR sidelink relay

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201620	DraftCR on interruption requirements for NR SL relay

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201872	RRM requirements for Rel-17 SL relay operation

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

The release 17 work item on NR sidelink relay was discussed at previous meeting, work plan was agreed and a number of issues were identified for further discussions in [1]. In this contribution, we discuss and provide our view on those.

Decision: 		The document was not treated.

### 6.25	NR small data transmissions in INACTIVE state

#### 6.25.1	General and work plan

R4-2200918	On BS demodulation performance requirements for CG-SDT

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion n requirements for CG-SDT and proposals on how to handle remaining timing offset errors in RAN4.

Decision: 		The document was not treated.

#### 6.25.2	RRM core requirements

R4-2200300	On RRM requirement for CG-SDT

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2200417	RRM requirements for NR-SDT

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

We continue to discuss the RRM requirments for NR-SDT

Decision: 		The document was not treated.

R4-2200919	TA validation requirements for CG-SDT

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on TA validation requirements, and proposals on improvements in relation to the LTE PUR requirements.

Decision: 		The document was not treated.

R4-2200920	Draft CR TA validation for Small Data Transmissions

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Introduction of TA validation core requirements for CG-SDT.

Decision: 		The document was not treated.

R4-2201644	Discussion on remaining issues for SDT RRM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201645	CR on UL timing requirements for SDT

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201792	Draft big CR for SDT RRM requirements

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Abstract:

Worksplit according to the approved workplan for SDT RRM requirements (R4-2120339)

Decision: 		The document was not treated.

R4-2201793	Further discussion on RRM requirements for SDT

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201853	RRM requirements for SDT

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2201869	Discussions on RRM requirements for Small Data Transmissions

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide an overview of the RRM requirements for CG-SDT that RAN4 needs to introduce.

Decision: 		The document was not treated.

R4-2201870	Draft CR to TS 38.133: Requirements on UE synchoronization for SDT

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

According to workplan in R4-2120339, requirements are UE synchonization for small data transmissions are introduced in this CR.

Decision: 		The document was not treated.

### 6.26	Support for Multi-SIM devices for LTE/NR

#### 6.26.1	General and work plan

R4-2200669	Work plan for Multi-SIM devices for LTE/NR

Type: Work Plan		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2201210	Further LS response on gap handling for MUSIM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the LS on gap handling for MUSIM

Decision: 		The document was not treated.

#### 6.26.2	RRM core requirements

R4-2200386	On MUSIM requirements

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2200402	Draft CR for introducing gap patterns for MUSIM

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: vivo

Decision: 		The document was not treated.

R4-2200428	Discussion on MUSIM requirements

Type: discussion		For: Discussion
					Source: Charter Communications, Inc

Decision: 		The document was not treated.

R4-2200529	Discussion on MUSIM RRM impacts

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200672	Discussion on RAN2 LS (R2-2108861) on gap handling for MUSIM

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2200683	Discussion on the gap pattern for MUSIM

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2201169	Discussion on RRM core requirements for Multi-SIM devices

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2201211	New gap pattern for MUSIM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the new MGPs for MUSIM

Decision: 		The document was not treated.

R4-2201212	draftCR on New gap pattern for MUSIM

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This draftCR introduce the new MGPs for MUSIM

Decision: 		The document was not treated.

R4-2201646	Discussion on gap patterns for MU-SIM

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201647	CR on measurement gap patterns for MUSIM

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201698	Requirements for MUSIM switching gaps

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201699	Introduction of MUSIM switching gaps

Type: draftCR		For: Endorsement
					38.133 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

## 7	Rel-17 Study Items for NR

### 7.1	Study on enhanced test methods for FR2 in NR

#### 7.1.1	Maintenance on objectives 1~6

R4-2200452	TP to TR 38.884 on release independence applicability of test method enhancements

Type: pCR		For: Approval
					38.884 v1.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

#### 7.1.2	OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz

##### 7.1.2.1	General

###### 7.1.2.1.1	Test system assumption

###### 7.1.2.1.2	UE types

R4-2201921	On Vehicular UEs

Type: discussion		For: Approval
					Source: Keysight Technologies UK Ltd

Decision: 		The document was not treated.

###### 7.1.2.1.3	MU assessment

###### 7.1.2.1.4	Others

R4-2201873	TP to TR 38.884 on extension of NR test methods to FR2-2

Type: pCR		For: Approval
					38.884 v1.2.0	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201927	UE types and permitted methods for FR2-2

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 7.1.2.2	Test methodology for UE RF

R4-2201990	Issues with MIMO EVM Measurement Using the Pseudo-Inverse

Type: discussion		For: Approval
					Source: Lenovo, Motorola Mobility

Decision: 		The document was not treated.

##### 7.1.2.3	Test methodology for RRM

R4-2201874	NR FR2-2 OTA test methods for RRM

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 7.1.2.4	Test methodology for UE demodulation and CSI

R4-2200907	On permitted test methods for demodulation in FR2-2

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2201875	NR FR2-2 OTA test methods for UE demodulation

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

### 7.2	Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

#### 7.2.1	General and TR

R4-2201485	draft TR 38.844 v0.0.6

Type: draft TR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

draft TR 38.844 v0.0.6 with implemented TPs from RAN4 #101-e.

Decision: 		The document was not treated.

R4-2201794	Revision on TR 38.344 Section 6.2.3

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2202046	TP for TR 38.844: Proposal for n12 and n85

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: T-Mobile USA

Decision: 		The document was not treated.

#### 7.2.2	Evaluation of use of larger channel bandwidths than licensed bandwidth

R4-2200913	Discussion on the widerCBW approach

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2201509	On the applicability of wider channel bandwidth

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 7.2.2.1	Channel filter assumptions and RB blanking with impacts on UE (ACS, blocking)

R4-2200911	Further input on performance when using the next larger channel

Type: pCR		For: Decision
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

##### 7.2.2.2	Signaling and configuration (RAN1/RAN2 impacts) aspects

R4-2201486	TP to TR 38.844: Section 6.1.2 Signalling for Larger Channel BW Approach

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

Based upon the LS Reply from RAN2 the following is a text proposal to include inputs for completeness.

Decision: 		The document was not treated.

R4-2201795	Further discussion on the WiderCBW approach

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Decision: 		The document was not treated.

R4-2201880	Views on signalling for Wider CBW method

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

##### 7.2.2.3	Other aspects such as detailed solution, complexity, legacy UE, etc

R4-2201995	On wider CBW open issues

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 7.2.3	Evaluation of use of overlapping UE channel bandwidths

##### 7.2.3.1	Overlapping CBWs from network perspective

R4-2200912	Further corrections to the solution based on overlapping channels from the network perspective

Type: pCR		For: Decision
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Apple

Decision: 		The document was not treated.

R4-2201510	Signaling aspect for overlapping from network perspective

Type: pCR		For: Approval
					38.844 v0.0.5	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201511	On overlapping CBWs from Network perspective

Type: pCR		For: Approval
					38.844 v0.0.5	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 7.2.3.2	Combined UE CBWs (one cell)

###### 7.2.3.2.1	Signaling and configuration (RAN1/RAN2 impacts) aspects

R4-2201488	TP to TR 38.844: Section 6.1.2 Signalling for Overlapping Channel BW from UE perspective Approach

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

Based upon the LS Reply from RAN2 the following is a text proposal to include inputs for completeness.

Decision: 		The document was withdrawn.

R4-2201883	Views on Overlapping UE CBWs method

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201993	TP to TR 38.844: on combined UE CBW (one cell) – signalling aspects

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

###### 7.2.3.2.2	Other aspects such as detailed solution, complexity, legacy UE, etc

R4-2201994	On combined UE CBW (one cell) gNB channel filter and UE requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 7.2.3.3	Overlapping CA (two cells)

###### 7.2.3.3.1	Signaling and configuration (RAN1/RAN2 impacts) aspects

R4-2201487	TP to TR 38.844: Section 6.1.2 Signalling for Overlapping CA Approach

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

Based upon the LS Reply from RAN2 the following is a text proposal to include inputs for completeness.

Decision: 		The document was not treated.

R4-2201884	Views on Overlapping CA method

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

###### 7.2.3.3.2	Other aspects such as detailed solution, complexity, legacy UE, etc

R4-2201333	TP to TR 38.844: Section 6.5.1 Overlapping CA (two cells)

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson

Abstract:

This TP is sugegsting text update to the overlapping CA / RAN1-RAN2 impact section for completeness

Decision: 		The document was not treated.

R4-2201512	RF requirements for overlapping CA

Type: pCR		For: Approval
					38.844 v0.0.5	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 7.2.3.4	Overall method comparisons

R4-2200817	Discussion on overall method comparisons for irregular bandwidth

Type: discussion		For: Decision
					Source: CMCC

Decision: 		The document was not treated.

R4-2200921	Considerations for wider CBW and overlapping UE CBW solutions

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2201264	Comparison of Different Schemes

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201492	Comparison Between Methods for Irregular BWs

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution further discusses the evaluation matrix introduced several meetings as a means to evaluate each method.

Decision: 		The document was not treated.

R4-2201513	Overall method comparisons

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2201885	TP to TR 38.844: Wider CBW method

Type: pCR		For: Approval
					38.844 v0.0.6	  CR-  rev  Cat:  (Rel-17)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201992	Irregular bandwidth – comparison between options

Type: discussion		For: Discussion
					Source: MediaTek (Chengdu) Inc.

Decision: 		The document was not treated.

### 7.3	Study on band combination handling in RAN4

#### 7.3.1	General and TR

R4-2200613	TR 38.862 V050 Band combination handling

Type: draft TR		For: Approval
					38.862 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

This paper is to provide TR 38.862 V050 for band combination handling to include the approved TP in this meeting.

Decision: 		The document was not treated.

R4-2200614	Motivation on further study on band combination handling in Rel-18

Type: discussion		For: Decision
					Source: ZTE Corporation

Abstract:

In this paper, we’d like to provide our opinions on the next phase of study for band combination handling in Rel-18.

Decision: 		The document was not treated.

R4-2200615	Draft New SID on further study on band combination handling in RAN4

Type: SID new		For: Discussion
					Source: ZTE Corporation

Abstract:

In this paper, we’d like to provide a draft SID for further study on band combination handling in Rel-18.

Decision: 		The document was not treated.

#### 7.3.2	Information of rules and guidelines of specifying band combinations (TP format, notation, band configurations, BCS)

R4-2200553	Adding contributions not for block approval

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we provide a status of the band combinations not for block approval and the related frameworks and formulate proposals to include in rules and guidelines of specifying band combinations TR for these cases.

Decision: 		The document was not treated.

R4-2200618	TP to TR38.862 on introdution to BCS4 and BCS5

Type: pCR		For: Approval
					38.862 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation, Xiaomi

Abstract:

In this proposal, a TP to capture the related agreements and some guidelines for BCS4/5 definition is proposed.

Decision: 		The document was not treated.

###### 7.3.2.1.1	Signaling and configuration (RAN1/RAN2 impacts) aspects

R4-2201881	Views on Signalling for Overlapping CBW from Network Perspective

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

###### 7.3.2.1.2	Other aspects such as detailed solution, complexity, legacy UE, etc

R4-2201882	Views on BS TX Channel BW Filters for Overlapping CBW

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

#### 7.3.3	Improving RAN4 specification structures and reducing redundant contents

##### 7.3.3.1	Optimization of delta TIB and delta RIB

R4-2200617	Band category for rule based approach for delta TIB and RIB

Type: discussion		For: Approval
					Source: ZTE Corporation

Abstract:

In this paper we discuss the band category for rule based approach for delta TIB and RIB.

Decision: 		The document was not treated.

R4-2200705	Statistics of dTib and dRib

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 7.3.3.2	Optimizations for other redundancy

R4-2200616	TP to TR38.862 on simplification for DC configuration table in Rel-18

Type: pCR		For: Approval
					38.862 v0.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: ZTE Corporation

Abstract:

In this paper, we would like to further discuss the possible optimizations for DC configuration table on the basis of [3] in the timeframe of Rel-18.

Decision: 		The document was not treated.

### 7.4	Optimizations of pi/2 BPSK uplink power in NR

#### 7.4.1	General and TR

R4-2200506	TR skeleton for SI on optimizations of pi_2 BPSK uplink power

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Workplan for ‘Optimizations of pi/2 BPSK uplink power in NR’ is presented

Decision: 		The document was not treated.

R4-2200507	TP for Pi/2 BPSK study item for TR38.868

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Presents items from company contributions that are to be included in TR38.868

Decision: 		The document was not treated.

#### 7.4.2	UE Tx power and related issues

R4-2200443	MPR region proposal for PI/2 BPSK power boosting

Type: discussion		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2200727	Transmitter performance for pi/2 BPSK with spectral shaping

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200954	Discussion on pi/2 BPSK UE Tx power

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2201837	On the remaining issues for Pi/2 BPSK Optimisations

Type: discussion		For: Agreement
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201879	Pi/2 BPSK combined Tx and Rx Link performance

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2202029	PC2 Pi/2 BPSK Power Boosting Measurements

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-17)

					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

#### 7.4.3	Evaluation of filter requirements applicable to identified new UE power capability

R4-2200511	Spectral flatness requirements

Type: discussion		For: Approval
					38.101-2 v	  CR-  rev  Cat:  (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Discussion on timing for SL transmission in Rel-17

Decision: 		The document was not treated.

R4-2200728	Shaping filter characteristics including transmitter and link performance

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 7.4.4	Link level simulations

R4-2200726	Receiver performance for pi/2 BPSK with spectral shaping

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2200955	Link level Simulation results for pi/2 BPSK

Type: other		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

#### 7.4.5	SAR analysis

#### 7.4.6	Identify RAN4 requirements

R4-2200729	Identify?potential changes for?RAN4 requirements

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

## 8	Rel-17 Work Items for LTE

### 8.1	LTE inter-band Carrier Aggregation for 2 bands DL with 1 band UL

#### 8.1.1	UE RF with harmonic, close proximity and isolation issues

#### 8.1.2	UE RF without specific issues

R4-2200349	TP to TR 36.717-02-01: CA_2-38

Type: pCR		For: Approval
					36.717-02-01 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

CA_2A-38A is introduced.

Decision: 		The document was not treated.

R4-2201911	TP for TR 36.717-02-01 to include CA_30A-48A

Type: pCR		For: Approval
					36.717-02-01 v0.6.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 36.717-02-01 to include CA_30A-48A

Decision: 		The document was not treated.

R4-2201912	TP for TR 36.717-02-02 to include CA_30A-48A

Type: pCR		For: Approval
					36.717-02-02 v0.3.0	  CR-  rev  Cat:  (Rel-17)

					Source: Ericsson, AT&T

Abstract:

TP for TR 36.717-02-02 to include CA_30A-48A

Decision: 		The document was not treated.

R4-2201914	draft CR 36101 to add CA_2A-2A-46E

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 36101 to add CA_2A-2A-46E

Decision: 		The document was not treated.

### 8.2	LTE inter-band Carrier Aggregation for 3 bands DL with 1 band UL

#### 8.2.1	UE RF with harmonic, close proximity and isolation issues

#### 8.2.2	UE RF without specific issues

R4-2200350	TP to TR 36.717-03-01: CA_2-7-38

Type: pCR		For: Approval
					36.717-03-01 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

CA_2A-7A-38A and CA_2A-7C-38A are introduced.

Decision: 		The document was not treated.

R4-2200707	DraftCR 36.101: Addition CA_5A-7A-7A-28A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2201915	draft CR 36101 to add CA_2A-2A-29A-66A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 36101 to add CA_2A-2A-29A-66A

Decision: 		The document was not treated.

### 8.3	LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL

#### 8.3.1	UE RF with 4 LTE bands CA

R4-2200708	DraftCR 36.101: Addition CA_1A-5A-7A-7A-28A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2200709	DraftCR 36.101: Addition CA_3A-5A-7A-7A-28A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2201913	draft CR 36101 to add CA_2A-2A-29A-30A-66A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Ericsson, AT&T

Abstract:

draft CR 36101 to add CA_2A-2A-29A-30A-66A

Decision: 		The document was not treated.

#### 8.3.2	UE RF with 5 LTE bands CA

R4-2200710	DraftCR 36.101: Addition CA_1A-3A-5A-7A-7A-28A

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

### 8.4	LTE inter-band Carrier Aggregation for 2 bands DL with 2 band UL

#### 8.4.1	UE RF with harmonic, close proximity and isolation issues

#### 8.4.2	UE RF without specific issues

### 8.5	LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 band UL

R4-2200773	TR 36.717-03-02 v0.6.0 TR update for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17

Type: draft TR		For: Approval
					36.717-03-02 v0.5.0	  CR-  rev  Cat:  (Rel-17)

					Source: LG Electronics France

Abstract:

Draft TR to update CA band combinations in Rel-17

Decision: 		The document was not treated.

R4-2200775	Revised WID on LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17

Type: WID revised		For: Endorsement
					Source: LG Electronics France

Abstract:

Revised WID to update the status on the LTE-A CA band combinations in Rel-17

Decision: 		The document was not treated.

R4-2200776	Introduction of LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL to TS36.101

Type: CR		For: Agreement
					36.101 v17.4.0	  CR-5847  rev  Cat: B (Rel-17)

					Source: LG Electronics France

Abstract:

Introduce new LTE-A CA band combinations in TS36.101 in rel-17

Decision: 		The document was not treated.

#### 8.5.1	UE RF with MSD

#### 8.5.2	UE RF without MSD

### 8.6	RRM for LTE CA basket WIs

#### 8.6.1	RRM Core (36.133)

#### 8.6.2	RRM Perf (36.133)

### 8.7	New WID on Additional LTE bands for UE category M1&M2 and/or NB1&NB2 in Rel-17

#### 8.7.1	RF requirements

#### 8.7.2	Others

R4-2201716	AMPR simulation results for Cat-M1 for B48

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

This paper presents results of A-MPR simulations for LTE Band 24 CAT-M1 UE with full-RB and sub-PRB allocation.

Decision: 		The document was not treated.

### 8.8	Upper 700MHz A Block new E-UTRA band in US

R4-2200766	TR skeleton for TR 36.779

Type: draft TR		For: Approval
					36.779 v0.0.3	  CR-  rev  Cat:  (Rel-17)

					Source: Puloli

Abstract:

In the Copyright Notification section, update the copyright year to 2022. Revised based on comments received from RAN4#101-e

Decision: 		The document was not treated.

#### 8.8.1	General

R4-2200765	Work plan for LTE_upper_700MHz_A WI

Type: Work Plan		For: Approval
					Source: Puloli

Decision: 		The document was not treated.

R4-2200771	TP to TR 36.779 on Frequency band arrangements and regulatory background

Type: pCR		For: Approval
					36.779 v0.0.3	  CR-  rev  Cat:  (Rel-17)

					Source: Puloli

Decision: 		The document was not treated.

R4-2200772	TP to TR 36.779 on Operating band, channel bandwidths, channel numbering

Type: pCR		For: Approval
					36.779 v0.0.3	  CR-  rev  Cat:  (Rel-17)

					Source: Puloli

Decision: 		The document was not treated.

R4-2202025	Draft CR to TS 36.104: exemplary implementation of LTE_upper_700MHz_A band

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei

Abstract:

Based on related discussion paper, we provide an exemplary implementation of the LTE_upper_700MHz_A band into the TS 36.104.

Decision: 		The document was not treated.

#### 8.8.2	Study for co-existence requirements

R4-2202024	Further discussion on the BS RF requirements for LTE_upper_700MHz_A

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide updated analysis of the co-location and co-ex requirements for BS RF.

Decision: 		The document was not treated.

#### 8.8.3	UE RF requirements

R4-2200004	Introduction of upper 700MHz A block into TS 36.101

Type: CR		For: Agreement
					36.101 v17.4.0	  CR-5846  rev  Cat: B (Rel-17)

					Source: Puloli

Decision: 		The document was not treated.

R4-2200774	TP to TR 36.779 on Analysis on UE requirements for upper 700MHz A block

Type: pCR		For: Approval
					36.779 v0.0.3	  CR-  rev  Cat:  (Rel-17)

					Source: Puloli

Decision: 		The document was not treated.

R4-2200790	draftCR to TS 36.101: Introduction of upper 700MHz A block

Type: draftCR		For: Endorsement
					36.101 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Puloli

Decision: 		The document was not treated.

#### 8.8.4	BS RF requirements

R4-2200457	draft CR on introduction of upper 700MHz A block for 36.104

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Baicells

Decision: 		The document was not treated.

R4-2200458	draft CR on introduction of upper 700MHz A block for 37.104

Type: draftCR		For: Endorsement
					37.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Baicells

Decision: 		The document was not treated.

R4-2200459	draft CR on introduction of upper 700MHz A block for 37.105

Type: draftCR		For: Endorsement
					37.105 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Baicells

Decision: 		The document was not treated.

R4-2200460	draft CR on introduction of upper 700MHz A block for 38.104

Type: draftCR		For: Endorsement
					38.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Baicells

Decision: 		The document was not treated.

R4-2200461	TP to TR 36.779 on BS aspect issues

Type: discussion		For: Approval
					Source: Baicells

Decision: 		The document was not treated.

R4-2201071	TP to TR 36.779 on Analysis on BS requirements for upper 700MHz A block

Type: discussion		For: Approval
					Source: Baicells

Decision: 		The document was not treated.

#### 8.8.5	Others

### 8.9	Additional enhancements for NB-IoT and LTE-MTC

#### 8.9.1	General and work plan

#### 8.9.2	Support of 16QAM in NB-IoT

##### 8.9.2.1	BS RF requirements

R4-2200415	Proposals on BS RF requirements for support of 16QAM in NB-IoT

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides further proposals on BS RF requirements for the support of 16-QAM in NB-IoT unicast in UL and DL according to the approved WF at TSG RAN4#98-bis-e and the agreements in RAN1.

Decision: 		The document was not treated.

R4-2201714	BS RF impact analysis on R17 NB_IoT

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the BS  RF impact on NB-IoT for this objective.

Decision: 		The document was not treated.

R4-2201831	Remaining issues for NB-IoT 16QAM BS RF requirements

Type: discussion		For: Agreement
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201832	Draft CR to TS36104 Addition of NB-IoT 16QAM

Type: draftCR		For: Endorsement
					36.104 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201833	Draft CR to TS36141 Addition of NB-IoT 16QAM

Type: draftCR		For: Endorsement
					36.141 v17.4.0	  CR-  rev  Cat:  (Rel-17)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

##### 8.9.2.2	UE RF requirements

#### 8.9.3	Support of power reduction for PRACH, PUCCH, and full-PRB PUSCH in MTC

##### 8.9.3.1	UE RF requirements

R4-2201287	On max power reduction for PRACH, PUCCH, and full-PRB PUSCH

Type: other		For: Decision
					Source: Sony

Decision: 		The document was not treated.

R4-2201715	RF impact analysis on R17 eMTC WID

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the RF impact for the Rel-17 eMTC.

Decision: 		The document was not treated.

#### 8.9.4	RRM core requirements

##### 8.9.4.1	Neighbour cell measurement in RRC Connected state for NB-IoT

R4-2200764	On NB-IoT neighbor cell measurements in RRC_CONNECTED

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2201208	Discussion on RRM requirements for Rel-17 NB-IoT

Type: discussion		For: Discussion
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201437	Draft CR on including channel quality table for 16 QAM for Rel-17 NB-IoT

Type: draftCR		For: Endorsement
					36.133 v17.4.0	  CR-  rev  Cat: B (Rel-17)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201866	Discussions on remaining issues of RRM requirements for NB-IoT

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contriution we discuss the open issues of Rel-17 NB-IoT.

Decision: 		The document was not treated.

#### 8.9.5	Others

R4-2201431	Introduction of channel quality report table for NB-IoT supporting 16QAM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution proposes how to capture the channel quality reporting mapping table in RAN4 specification.

Decision: 		The document was not treated.

#### 8.9.6	Demodulation requirements

##### 8.9.6.1	General

##### 8.9.6.2	Demodulation requirements for NB-IoT

###### 8.9.6.2.1	UE demodulation requirements

R4-2200763	On UE demodulation requirements for 16-QAM NB-IoT

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2200995	Discussion on UE demodulation requirements for additional enhancements of NB-IoT

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201432	View on Rel-17 NB-IoT UE demodulation requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our view on Rel-17 NB-IoT UE demodulation requirements.

Decision: 		The document was not treated.

###### 8.9.6.2.2	BS demodulation requirements

R4-2200751	View on demodulation requirement for Rel-17 NB-IoT

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200996	Discussion on BS demodulation requirements for additional enhancements of NB-IoT

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201433	View on Rel-17 NB-IoT BS demodulation requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our view on Rel-17 NB-IoT BS demodulation requirements.

Decision: 		The document was not treated.

R4-2201984	Discussion on BS demodulation requirements for Additional enhancements for NB-IoT

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on BS demodulation requirements for NB-IoT 16QAM

Decision: 		The document was not treated.

##### 8.9.6.3	Demodulation requirements for MTC

R4-2200750	View on demodulation requirement for Rel-17 eMTC

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2200997	Discussion on UE demodulation requirements for additional enhancements of LTE-MTC

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2201434	View on Rel-17 LTE-MTC demodulation requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides our view on Rel-17 eMTC UE/BS demodulation requirements.

Decision: 		The document was not treated.

## 9	Liaison and output to other groups

### 9.1	R17 related

#### 9.1.1	LS reply for beam correspondence with SDT in RRC_INACTIVE

R4-2200252	Discussion on RAN2 LS (R2-2108861) on gap handling for MUSIM

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was withdrawn.

R4-2201484	Discussion on reply LS on Beam correspondence with Small Data Transmission in Inactive State

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201973	On beam correspondence requirements and SDT

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

RAN1 has enquired about the beam correspondence functionality in context of the SDT feature. We provide our views on UE RF requirements for this type of feature

Decision: 		The document was not treated.

#### 9.1.2	RAN5 response LS on LTE REFSENS exception simplification (R5-215803)

R4-2201247	Discussion and draft LS on LTE REFSENS exception simplification

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 9.1.3	Others

R4-2201277	R17 FR2 power control for NR-DC and draft LS

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201339	Discussion on reply LS on configuration of p-MaxEUTRA and p-NR-FR1

Type: other		For: Approval
					Source: ZTE Corporation

Abstract:

Session Chair: This contribution (Rel-15 related) will not be treated in this meeting.

Decision: 		The document was not treated.

R4-2201477	Support of ATG for 5G Advanced

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201478	Motivation on study on NR NTN RF requirement for coexistence with TN standalone NB-IoT

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201479	New SI proposal: Study on NR NTN RF requirement for coexistence with TN standalone NB-IoT

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201480	Views on NR UE RF enhancement in Rel-18

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201481	Views on NR BS RF enhancement in Rel-18

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201482	Views on NR UE RRM enhancement in Rel-18

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201483	Motivation on the support of CRS-IM for Redcap UE

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201697	Incoming LS from RAN2 on L3 filter

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2201974	On reply to RAN5 on FR2 requirement applicability over ETC

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Session Chair: This contribution will not be treated in this meeting. RAN4 must first establish common understanding on any subject before responding externally.

Decision: 		The document was not treated.

## 10	Any other business

R4-2200172	Motivation for new WI on air-to-ground network for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2200173	New WID on air-to-ground network for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2200174	Motivation for new WID on Home Base Station for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2200175	New WID on Home Base Station for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2200287	Rel-18 proposal on FR2 coverage/performance enhancements

Type: discussion		For: Information
					Source: Apple

Decision: 		The document was not treated.

R4-2200299	Motivation paper for R18 RRM enhancement

Type: discussion		For: Information
					Source: Apple

Decision: 		The document was not treated.

R4-2200451	Views on handling ETC applicability to FR2 UE RF requirements

Type: discussion		For: Information
					Source: Apple

Decision: 		The document was not treated.

R4-2200545	Views on RAN4 Rel-18 scope

Type: discussion		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200546	Views on RAN4 Rel-18: FR2 multi-beam reception

Type: discussion		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200547	Views on RAN4 Rel-18: RRM enhancements

Type: discussion		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200548	Views on RAN4 Rel-18: Demodulation enhancements

Type: discussion		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200549	Draft New WID: Requirements for NR FR2 UEs with multi-Rx chain DL reception

Type: WID new		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200550	Draft New WID: NR and MR-DC RRM requirements enhancements

Type: WID new		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200551	Draft New WID: NR and MR-DC Measurement Gap further enhancements

Type: WID new		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2200552	Draft New WID: NR demodulation requirements enhancements

Type: WID new		For: Information
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2201193	[Draft] WID on FR2 RRM requirements evaluation

Type: WID new		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201280	R18 Motivation of 3Tx handheld UE

Type: discussion		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201281	R18 New WID on 3Tx handheld UE for NR

Type: WID new		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2201360	Motivation on basket WID on 4Rx and 8Rx bands

Type: other		For: Information
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2201361	New WID on 4Rx_8Rx support for NR bands

Type: other		For: Information
					Source: ZTE,China Telecom

Decision: 		The document was not treated.

R4-2201648	Motivation for FR2 RRM requirement enhancement

Type: discussion		For: Information
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2201650	Views on small A-MPR for Rel-18 RAN4 led WI

Type: discussion		For: Information
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution shares our views on smaller A-MPR, which is one of the topics in moderator summary in [RP-212682].

Decision: 		The document was not treated.

R4-2201653	New WID on Low MSD for CA and DC band combinations

Type: WID new		For: Information
					Source: Nokia, Nokia Shanghai Bell

Abstract:

A possible objective for a new WI for low MSD is shared.

Decision: 		The document was not treated.

R4-2201675	Motivation of new WID on High Altitude Platform Station (HAPS) for NR

Type: discussion		For: Information
					Source: China Unicom

Decision: 		The document was not treated.

R4-2201677	Rel-18 Draft New WID on High Altitude Platform Station (HAPS) for NR

Type: WID new		For: Information
					Source: China Unicom

Decision: 		The document was not treated.

R4-2201678	Draft Rel-18 new basket WID on high power UE (power class 2) for NR FDD band

Type: WID new		For: Information
					Source: China Unicom

Decision: 		The document was not treated.

R4-2201679	Draft Rel-18 new basket WID on High power UE (power class 1.5) for NR inter-band CA with 2 UL bands

Type: WID new		For: Information
					Source: China Unicom

Decision: 		The document was not treated.

R4-2202047	R18 New WID on NR MIMO OTA

Type: WID new		For: Approval
					Source: CAICT, OPPO

Decision: 		The document was not treated.

R4-2202050	R18 New WID on FR1 TRP TRS

Type: WID new		For: Approval
					Source: OPPO, CAICT

Decision: 		The document was not treated.

## 11	Close of the E-meeting

Report prepared by: MCC