TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2117031

Electronic Meeting, 1st – 12th November 2021

Source:	Qualcomm

Title:	NR coexistence simulation results for 60 GHz

Agenda item:	9.16.6

Document for:	Discussion

# Introduction

In RAN4#100-e meeting, coexistence simulation assumptions were discussed for 52.6-71 GHz were discussed [1]. After offline alignment of different metrics (e.g., pathloss, coupling loss, downlink and uplink SINR, and uplink power) with multiple companies, throughout this contribution we present simulation results for NR adjacent channel coexistence study at 60 GHz for indoor deployments. The simulation assumptions adopted in this study follow the agreed way forward from RAN4#100-e meeting [1].

# Discussion

## Indoor Deployment Scenarios

As agreed in [1] for the indoor deployments, we consider only indoor-C as mandatory, which is explained in detail in [2]. This scenario is based on the indoor 50m x 120m open space layout with a total of 12 base stations (BSs) per operator, 2 operators, with BS height at 3m (ceiling), and UE height of 1m. A pictorial representation of scenario C is shown in Figure 1.

Figure 1 Indoor scenario C for co-existence.

## Dense Micro Deployment Scenarios

As shown in our previous contribution [3], for dense deployment scenarios (i.e., coordinated, and uncoordinated), the system is noise limited in DL and UL. For UL, with the current assumptions is not possible to close the link budget. For downlink, a very low ACIR (around 6 dB) would be enough to keep degradation due to ACI within 5% loss. This is because the impact of noise is dominating, making adjacent channel interference less relevant in terms of relative throughput degradation. As a results, companies have agreed during RAN#100-e to focus on indoor deployments to derive the RF requirements for 60 GHz [1].

# Simulation Results

Throughout this section we provide the adopted list of simulation parameters, based on the agreed list in [1] and the simulation results based on those parameters. The list of the simulation assumptions for the considered scenarios is reported in Table 1. In addition, we follow the simulation methodology described in Section 5.3 in [4], where the RF parameters are determined based on the degradation cause by adjacent channel interference (ACI).

Table 1: Proposed list of coexistence simulation parameters

| System Parameters | Deployment | Indoor office C in TR 38.808 |
| --- | --- | --- |
|  | Carrier Frequency | 60 GHz, 70 GHz |
|  | Channel BW | 100Mhz and 400MHz |
|  | SCS | 120KHz for 100MHz and 960KHz for 400MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office model in TR 38.901. |
|  | LBT | No LBT considered |
|  | UE to BS 2D distance | 0 meters |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-7 in TR 38.802 for ceiling mount |
|  | EIRP limit | 40 dBm for indoor deployment |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in TR 38.802 |
|  | EIRP limit | 20 dBm |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability model defined in TR 38.803 |
|  | Maximum conducted power (per polarization) | 0 dBm |
|  | Minimum conducted power (per polarization) | -20 dBm |
|  | SNR target | 15 dB |
|  | CL-xile | 66 for 100 MHz and 60 for 400 MHz. |

## Indoor Results

In this section we present the simulation results for the indoor scenario C under the 100 and 400 MHz channel bandwidth configurations for 60 GHz and 70 GHz carrier frequencies. The measurements are conducted at the victim UEs (BSs), and the aggressor nodes are BSs (UEs) for DL (UL) operation in adjacent channel. The signal to noise (SNR) and signal to interference and noise ratio (SINR) cumulative distribution function (CDF) curves as well as the throughput loss for downlink are shown in Figures 3 and 4 for 60 GHz and 70 GHz, respectively. For the SINR/SNR plot, shown in Figure 3a, we observe that as the component carrier bandwidth narrows, the SINR and SNR are improved due to the impact of the thermal noise in the system (around 6 dB gap between the SNR curves for the 100 MHz and 400 MHz channel bandwidth). For the tail of the SINR curves, the bandwidth’s impact is negligible as the system is interference limited. Moving to the higher percentile of UEs, the impact of the channel bandwidth becomes higher. Figure 3b shows the DL throughput degradation as a function of the adjacent channel interference ratio (ACIR), where the mean and the 5%-tile TP are plotted. The results show that the TP loss is insensitive to the channel bandwidth.

| a) SINR/SNR curves considering only co-channel interference | b) DL throughput loss |
| --- | --- |

Figure 2 Downlink indoor results for 60 GHz

| - SINR/SNR curves considering only co-channel interference | b) DL throughput loss |
| --- | --- |

Figure 3 Downlink indoor results for 70 GHz

In Figures 4 and 5 the uplink simulation results for the different scenarios and bandwidth configurations are shown for 60 GHz and 70 GHz, respectively. From the SINR and SNR curves, an SNR target value of 15 dB is adopted as shown in Table 1. We observe the impact of the power control and minimum conducted power on the low and high percentile of the distributions. Observing the reported results in Figures 4 and 5, we can conclude that an EIRP of 20 dBm for the UE is sufficient to provide good performance and close the link budget in uplink transmissions.

| a) SINR/SNR curves considering only co-channel interference | b) UL throughput loss |
| --- | --- |

Figure 4 Uplink indoor results for 60 GHz

| a) SINR/SNR curves considering only co-channel interference | b) UL throughput loss |
| --- | --- |

Figure 5 Uplink indoor results for 70 GHz

Proposal 1: For 60 and 70 GHz, an ACIR of 15 and 13.8 dB would be enough to keep degradation due to ACI within 5% loss for DL and UL, respectively.

Proposal 2: For UL transmission in 52.6-71 GHz, an EIRP = 20 dBm is sufficient to close the link budget and provide low degradation in the degradation cause by adjacent channel interference.

In Table 2, the ACIR requirements for the considered simulation scenarios are summarized and compared to those values reported in Section 5.4 in TR 38.803. It is worth mentioning that In TR 38.803, carrier frequency of 30, 45, and 70 GHz were considered with a fixed channel bandwidth of 200MHz [5]. We compared our results with the ACIR values for DL and UL in the worst case across all scenarios considered in [5].

Table 3 ACIR requirement for 5% mean TP loss

| Case | ACIR requirement for 5% throughput loss |
| --- | --- |
| 60 GHz and 70 GHz scenario C with channel bandwidth = 100/400MHz | 15 dB for DL 13.8 dB for UL |
| 70 GHz scenario C reported results in Section 5.4, TR 38.803 [4] | 18.7 dB for DL 13.8 dB for UL |

Proposal 3: We can consider the ACIR limits considered in TR 38.803 for 70 GHz as a basis for 52.6-71. The ACIR limit is driven by indoor deployment scenario (while dense urban scenario is highly noise limited).

# Conclusion

Throughout this contribution, we provided indoor and outdoor coexistence simulation results for 52.6-71 GHz. Based on the results obtained so far, we have the following observation:

Proposal 1: For 60 and 70 GHz, an ACIR of 15 and 13.8 dB would be enough to keep degradation due to ACI within 5% loss for DL and UL, respectively.

Proposal 2: For UL transmission in 52.6-71 GHz, an EIRP = 20 dBm is sufficient to close the link budget and provide low degradation in the degradation cause by adjacent channel interference.

Proposal 3: We can consider the ACIR limits considered in TR 38.803 for 70 GHz as a basis for 52.6-71 GHz. The ACIR limit is driven by indoor deployment scenario (while dense urban scenario is highly noise limited).

# References

[1]	R4-2114993, “WF on co-existence simulation for NR_ext_to_71GHz”, Qualcomm.

[2]	TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz” 3GPP.

[3]	R4-2112146, “NR coexistence simulation results for 52.6-71 GHz”, Qualcomm.

[4] 	TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP.