3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117249

Electronic Meeting, 1 – 12 November 2021

Agenda Item:	8.16.5

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on coexistence simulation for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was approved at TSG RAN#91-e [1]. One of the objectives of this work item is to define BS and UE RF core requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. Whether new coexistence simulation should be performed or the results in TR 38.803 [2] can be reused to decide the requirements was further discussed at RAN4#100-e, and a way forward but was approved [3].

This contribution provides some preliminary simulation results based on the proposed assumptions and parameters in [3] and provides some proposals on coexistence simulation for extending current NR operation to 71.

# 2.	Discussion

It was agreed in [3] to focus on indoor deployments for coexistence simulation work and that:

- During the period between RAN4#100e and RAN4#101e: Offline calibrate between the companies.

- RAN4#101e: Calibrate and align the simulation results, try to agree preliminary ACIR.

Our simulation results for Scenario Indoor-C [5] which have been provided for the offline calibration are summarized in Table 1 below.

Table 1: Scenario Indoor-C simulation results summary

(a) 100MHz channel bandwidth at 60GHz carrier frequency

| Percentile | Path Loss | Coupling Loss | DL SINR | UE Tx power | UL SINR |
| --- | --- | --- | --- | --- | --- |
| 0 | -101.848 | -76.264 | 2.5419 | -20 | 0.8197 |
| 1 | -95.508 | -64.0799 | 9.7076 | -20 | 8.813 |
| 2 | -94.4096 | -62.7517 | 12.1632 | -20 | 10.5539 |
| 3 | -93.6223 | -61.7176 | 13.918 | -20 | 11.6843 |
| 4 | -92.9715 | -61.1364 | 15.3558 | -20 | 12.3214 |
| 5 | -92.396 | -60.7487 | 16.6082 | -20 | 12.7016 |
| 6 | -92.0437 | -60.3148 | 18.1975 | -20 | 13.0522 |
| 7 | -91.737 | -59.883 | 19.372 | -20 | 13.5188 |
| 8 | -91.2827 | -59.4434 | 20.1859 | -20 | 13.8607 |
| 9 | -90.9859 | -59.1994 | 21.1477 | -20 | 14.0262 |
| 10 | -90.7162 | -58.8849 | 21.8582 | -20 | 14.1597 |
| 11 | -90.4991 | -58.5532 | 22.4564 | -20 | 14.2703 |
| 12 | -90.2953 | -58.2604 | 23.2613 | -20 | 14.3587 |
| 13 | -89.9853 | -58.0335 | 23.8922 | -19.927 | 14.4483 |
| 14 | -89.7858 | -57.8045 | 24.4648 | -19.6152 | 14.4999 |
| 15 | -89.5661 | -57.6522 | 24.9757 | -19.3403 | 14.5641 |
| 16 | -89.4119 | -57.4702 | 25.5414 | -19.0606 | 14.6209 |
| 17 | -89.1389 | -57.2584 | 26.1064 | -18.7716 | 14.6713 |
| 18 | -88.9617 | -57.0622 | 26.6015 | -18.5181 | 14.7013 |
| 19 | -88.7922 | -56.8356 | 27.0067 | -18.3139 | 14.725 |
| 20 | -88.6721 | -56.6587 | 27.4354 | -18.1269 | 14.7591 |
| 21 | -88.5371 | -56.4355 | 27.7896 | -17.9062 | 14.7781 |
| 22 | -88.404 | -56.2719 | 28.1232 | -17.7108 | 14.8011 |
| 23 | -88.2689 | -56.1209 | 28.5572 | -17.4997 | 14.8287 |
| 24 | -88.1472 | -55.9361 | 28.8843 | -17.3851 | 14.8466 |
| 25 | -87.9242 | -55.8073 | 29.2098 | -17.2734 | 14.8621 |
| 26 | -87.7685 | -55.6574 | 29.6333 | -17.1132 | 14.8743 |
| 27 | -87.6621 | -55.5096 | 29.9952 | -16.9475 | 14.8859 |
| 28 | -87.5289 | -55.3395 | 30.442 | -16.8633 | 14.8948 |
| 29 | -87.3649 | -55.2106 | 30.7876 | -16.6646 | 14.9038 |
| 30 | -87.251 | -55.053 | 31.1181 | -16.4668 | 14.9136 |
| 31 | -87.161 | -54.9021 | 31.4875 | -16.2766 | 14.9218 |
| 32 | -87.021 | -54.7907 | 31.9097 | -16.0862 | 14.9278 |
| 33 | -86.8937 | -54.67 | 32.3611 | -15.9424 | 14.9344 |
| 34 | -86.7704 | -54.5341 | 32.7588 | -15.8077 | 14.94 |
| 35 | -86.6462 | -54.4054 | 33.1029 | -15.6928 | 14.9446 |
| 36 | -86.5036 | -54.2689 | 33.3584 | -15.5501 | 14.9507 |
| 37 | -86.3475 | -54.1313 | 33.6497 | -15.3739 | 14.9538 |
| 38 | -86.2268 | -54.0066 | 34.0519 | -15.2504 | 14.9581 |
| 39 | -86.1007 | -53.9043 | 34.3348 | -15.0981 | 14.9621 |
| 40 | -85.9789 | -53.7899 | 34.6597 | -14.9985 | 14.9664 |
| 41 | -85.8179 | -53.644 | 35.0868 | -14.8279 | 14.9699 |
| 42 | -85.7166 | -53.4942 | 35.3072 | -14.7252 | 14.9728 |
| 43 | -85.6017 | -53.3816 | 35.6866 | -14.5873 | 14.9751 |
| 44 | -85.5004 | -53.2629 | 35.9623 | -14.4416 | 14.9766 |
| 45 | -85.3569 | -53.1199 | 36.2576 | -14.2905 | 14.9782 |
| 46 | -85.2414 | -52.9649 | 36.521 | -14.112 | 14.9798 |
| 47 | -85.0943 | -52.8116 | 36.7395 | -14.0228 | 14.9814 |
| 48 | -84.9804 | -52.6541 | 37.0318 | -13.8982 | 14.9826 |
| 49 | -84.8727 | -52.5225 | 37.286 | -13.7514 | 14.9839 |
| 50 | -84.7625 | -52.3776 | 37.5534 | -13.625 | 14.9851 |
| 51 | -84.6365 | -52.2318 | 37.8004 | -13.4851 | 14.9864 |
| 52 | -84.5137 | -52.0767 | 38.0782 | -13.3137 | 14.9879 |
| 53 | -84.3913 | -51.89 | 38.3034 | -13.196 | 14.9889 |
| 54 | -84.2447 | -51.7402 | 38.5344 | -13.0674 | 14.9895 |
| 55 | -84.1155 | -51.6177 | 38.7442 | -12.9503 | 14.9905 |
| 56 | -83.9613 | -51.493 | 39.0124 | -12.7898 | 14.9915 |
| 57 | -83.8068 | -51.3714 | 39.2118 | -12.6751 | 14.9922 |
| 58 | -83.7035 | -51.2423 | 39.3915 | -12.5374 | 14.9927 |
| 59 | -83.5666 | -51.1107 | 39.6617 | -12.3974 | 14.9934 |
| 60 | -83.4055 | -50.987 | 39.9589 | -12.2839 | 14.9941 |
| 61 | -83.2599 | -50.8643 | 40.2216 | -12.1546 | 14.9945 |
| 62 | -83.1309 | -50.7332 | 40.4256 | -11.9833 | 14.995 |
| 63 | -82.9757 | -50.634 | 40.6664 | -11.8514 | 14.9954 |
| 64 | -82.8737 | -50.4809 | 40.8824 | -11.7551 | 14.9961 |
| 65 | -82.713 | -50.3315 | 41.1765 | -11.5995 | 14.9965 |
| 66 | -82.5823 | -50.1774 | 41.4769 | -11.4527 | 14.9967 |
| 67 | -82.4545 | -50.0578 | 41.673 | -11.3426 | 14.9972 |
| 68 | -82.3153 | -49.8962 | 41.9108 | -11.1806 | 14.9975 |
| 69 | -82.182 | -49.7664 | 42.1025 | -10.9609 | 14.9977 |
| 70 | -82.0652 | -49.5808 | 42.3941 | -10.8385 | 14.998 |
| 71 | -81.9549 | -49.4335 | 42.6314 | -10.6963 | 14.9982 |
| 72 | -81.797 | -49.2299 | 42.8408 | -10.4959 | 14.9984 |
| 73 | -81.688 | -49.0523 | 43.1322 | -10.3639 | 14.9986 |
| 74 | -81.5298 | -48.9111 | 43.4334 | -10.2222 | 14.9987 |
| 75 | -81.3737 | -48.6998 | 43.7075 | -10.1025 | 14.9989 |
| 76 | -81.2087 | -48.5347 | 43.9526 | -9.8942 | 14.9991 |
| 77 | -80.9868 | -48.3034 | 44.2468 | -9.7477 | 14.9992 |
| 78 | -80.8259 | -48.1104 | 44.5008 | -9.6206 | 14.9993 |
| 79 | -80.6251 | -47.942 | 44.7848 | -9.456 | 14.9994 |
| 80 | -80.4782 | -47.6842 | 45.0587 | -9.2767 | 14.9995 |
| 81 | -80.3129 | -47.4709 | 45.3791 | -9.1364 | 14.9996 |
| 82 | -80.1384 | -47.2886 | 45.7791 | -8.9962 | 14.9997 |
| 83 | -79.9268 | -47.123 | 46.1293 | -8.8263 | 14.9997 |
| 84 | -79.758 | -46.8642 | 46.4733 | -8.6813 | 14.9998 |
| 85 | -79.5114 | -46.6513 | 46.8264 | -8.4603 | 14.9999 |
| 86 | -79.2242 | -46.4471 | 47.2089 | -8.1571 | 14.9999 |
| 87 | -78.9711 | -46.1256 | 47.5345 | -7.9231 | 15 |
| 88 | -78.7042 | -45.8515 | 47.9597 | -7.7039 | 15 |
| 89 | -78.4256 | -45.5849 | 48.333 | -7.4867 | 15.3006 |
| 90 | -78.0869 | -45.2759 | 48.8741 | -7.1754 | 15.8076 |
| 91 | -77.7365 | -44.9092 | 49.3611 | -6.789 | 16.2175 |
| 92 | -77.3944 | -44.6017 | 49.9073 | -6.5202 | 16.4751 |
| 93 | -76.9746 | -44.132 | 50.5069 | -6.0671 | 16.8852 |
| 94 | -76.6574 | -43.6981 | 51.1537 | -5.7279 | 17.2294 |
| 95 | -76.1054 | -43.0589 | 52.0037 | -5.3127 | 17.5823 |
| 96 | -75.6002 | -42.3775 | 52.7406 | -4.2979 | 18.0654 |
| 97 | -75.0261 | -41.7153 | 53.6748 | -3.6358 | 18.6768 |
| 98 | -74.2943 | -40.9635 | 54.7677 | -2.5996 | 19.7229 |
| 99 | -73.1064 | -40.0699 | 56.3271 | -0.3193 | 21.6495 |
| 100 | -68.1115 | -32.3296 | 61.2607 | 0 | 26.5505 |

(b) 400MHz channel bandwidth at 60GHz carrier frequency

| Percentile | Path Loss | Coupling Loss | DL SINR | UE Tx power | UL SINR |
| --- | --- | --- | --- | --- | --- |
| 0 | -101.848 | -76.264 | 2.5364 | -20 | -2.4825 |
| 1 | -95.508 | -64.0799 | 9.6952 | -20 | 6.7215 |
| 2 | -94.4096 | -62.7517 | 12.1472 | -18.9792 | 9.3709 |
| 3 | -93.6223 | -61.7176 | 13.7518 | -17.8652 | 10.2721 |
| 4 | -92.9715 | -61.1364 | 15.2832 | -17.2316 | 11.335 |
| 5 | -92.396 | -60.7487 | 16.5686 | -16.8026 | 11.9323 |
| 6 | -92.0437 | -60.3148 | 18.0511 | -16.5271 | 12.1856 |
| 7 | -91.737 | -59.883 | 19.2446 | -16.0733 | 12.5402 |
| 8 | -91.2827 | -59.4434 | 20.0154 | -15.7088 | 12.7628 |
| 9 | -90.9859 | -59.1994 | 20.9549 | -15.3814 | 13.1135 |
| 10 | -90.7162 | -58.8849 | 21.7266 | -15.0728 | 13.4231 |
| 11 | -90.4991 | -58.5532 | 22.1912 | -14.6544 | 13.7292 |
| 12 | -90.2953 | -58.2604 | 23.0088 | -14.164 | 13.9331 |
| 13 | -89.9853 | -58.0335 | 23.6336 | -13.927 | 14.0595 |
| 14 | -89.7858 | -57.8045 | 24.1967 | -13.6152 | 14.1863 |
| 15 | -89.5661 | -57.6522 | 24.57 | -13.3403 | 14.2591 |
| 16 | -89.4119 | -57.4702 | 25.1216 | -13.0606 | 14.3379 |
| 17 | -89.1389 | -57.2584 | 25.5887 | -12.7716 | 14.4118 |
| 18 | -88.9617 | -57.0622 | 26.0437 | -12.5181 | 14.4646 |
| 19 | -88.7922 | -56.8356 | 26.4235 | -12.3139 | 14.5168 |
| 20 | -88.6721 | -56.6587 | 26.7652 | -12.1269 | 14.5608 |
| 21 | -88.5371 | -56.4355 | 27.1946 | -11.9062 | 14.6061 |
| 22 | -88.404 | -56.2719 | 27.5485 | -11.7108 | 14.6404 |
| 23 | -88.2689 | -56.1209 | 27.7946 | -11.4997 | 14.6797 |
| 24 | -88.1472 | -55.9361 | 28.1025 | -11.3851 | 14.7025 |
| 25 | -87.9242 | -55.8073 | 28.4308 | -11.2734 | 14.7338 |
| 26 | -87.7685 | -55.6574 | 28.6972 | -11.1132 | 14.7579 |
| 27 | -87.6621 | -55.5096 | 29.036 | -10.9475 | 14.7761 |
| 28 | -87.5289 | -55.3395 | 29.3998 | -10.8633 | 14.8003 |
| 29 | -87.3649 | -55.2106 | 29.7386 | -10.6646 | 14.8193 |
| 30 | -87.251 | -55.053 | 30.0287 | -10.4668 | 14.8368 |
| 31 | -87.161 | -54.9021 | 30.289 | -10.2766 | 14.8461 |
| 32 | -87.021 | -54.7907 | 30.6292 | -10.0862 | 14.86 |
| 33 | -86.8937 | -54.67 | 30.8805 | -9.9424 | 14.8715 |
| 34 | -86.7704 | -54.5341 | 31.1903 | -9.8077 | 14.8795 |
| 35 | -86.6462 | -54.4054 | 31.4736 | -9.6928 | 14.8884 |
| 36 | -86.5036 | -54.2689 | 31.8312 | -9.5501 | 14.8954 |
| 37 | -86.3475 | -54.1313 | 32.0568 | -9.3739 | 14.9025 |
| 38 | -86.2268 | -54.0066 | 32.3605 | -9.2504 | 14.9079 |
| 39 | -86.1007 | -53.9043 | 32.5387 | -9.0981 | 14.9142 |
| 40 | -85.9789 | -53.7899 | 32.7715 | -8.9985 | 14.9194 |
| 41 | -85.8179 | -53.644 | 33.0156 | -8.8279 | 14.9238 |
| 42 | -85.7166 | -53.4942 | 33.259 | -8.7252 | 14.9294 |
| 43 | -85.6017 | -53.3816 | 33.5299 | -8.5873 | 14.9337 |
| 44 | -85.5004 | -53.2629 | 33.7736 | -8.4416 | 14.9374 |
| 45 | -85.3569 | -53.1199 | 34.0395 | -8.2905 | 14.9411 |
| 46 | -85.2414 | -52.9649 | 34.2879 | -8.112 | 14.9451 |
| 47 | -85.0943 | -52.8116 | 34.444 | -8.0228 | 14.9484 |
| 48 | -84.9804 | -52.6541 | 34.6498 | -7.8982 | 14.9514 |
| 49 | -84.8727 | -52.5225 | 34.82 | -7.7514 | 14.9535 |
| 50 | -84.7625 | -52.3776 | 35.0053 | -7.625 | 14.9556 |
| 51 | -84.6365 | -52.2318 | 35.1872 | -7.4851 | 14.9574 |
| 52 | -84.5137 | -52.0767 | 35.3591 | -7.3137 | 14.9584 |
| 53 | -84.3913 | -51.89 | 35.5345 | -7.196 | 14.9597 |
| 54 | -84.2447 | -51.7402 | 35.6935 | -7.0674 | 14.9613 |
| 55 | -84.1155 | -51.6177 | 35.8606 | -6.9503 | 14.9627 |
| 56 | -83.9613 | -51.493 | 36.0615 | -6.7898 | 14.9636 |
| 57 | -83.8068 | -51.3714 | 36.2747 | -6.6751 | 14.9646 |
| 58 | -83.7035 | -51.2423 | 36.4231 | -6.5374 | 14.9658 |
| 59 | -83.5666 | -51.1107 | 36.6717 | -6.3974 | 14.9671 |
| 60 | -83.4055 | -50.987 | 36.8159 | -6.2839 | 14.968 |
| 61 | -83.2599 | -50.8643 | 36.9829 | -6.1546 | 14.9688 |
| 62 | -83.1309 | -50.7332 | 37.1561 | -5.9833 | 14.9697 |
| 63 | -82.9757 | -50.634 | 37.4316 | -5.8514 | 14.9704 |
| 64 | -82.8737 | -50.4809 | 37.6556 | -5.7551 | 14.9711 |
| 65 | -82.713 | -50.3315 | 37.8271 | -5.5995 | 14.9719 |
| 66 | -82.5823 | -50.1774 | 38.038 | -5.4527 | 14.9726 |
| 67 | -82.4545 | -50.0578 | 38.194 | -5.3426 | 14.9731 |
| 68 | -82.3153 | -49.8962 | 38.4436 | -5.1806 | 14.9736 |
| 69 | -82.182 | -49.7664 | 38.6165 | -4.9609 | 14.974 |
| 70 | -82.0652 | -49.5808 | 38.8539 | -4.8385 | 14.9745 |
| 71 | -81.9549 | -49.4335 | 39.0041 | -4.6963 | 14.9749 |
| 72 | -81.797 | -49.2299 | 39.204 | -4.4959 | 14.9755 |
| 73 | -81.688 | -49.0523 | 39.469 | -4.3639 | 14.9758 |
| 74 | -81.5298 | -48.9111 | 39.7513 | -4.2222 | 14.9761 |
| 75 | -81.3737 | -48.6998 | 39.9391 | -4.1025 | 14.9766 |
| 76 | -81.2087 | -48.5347 | 40.156 | -3.8942 | 14.9768 |
| 77 | -80.9868 | -48.3034 | 40.4467 | -3.7477 | 14.9771 |
| 78 | -80.8259 | -48.1104 | 40.6441 | -3.6206 | 14.9772 |
| 79 | -80.6251 | -47.942 | 40.8213 | -3.456 | 14.9775 |
| 80 | -80.4782 | -47.6842 | 41.1673 | -3.2767 | 14.9777 |
| 81 | -80.3129 | -47.4709 | 41.4366 | -3.1364 | 14.9779 |
| 82 | -80.1384 | -47.2886 | 41.7903 | -2.9962 | 14.978 |
| 83 | -79.9268 | -47.123 | 41.9688 | -2.8263 | 14.9782 |
| 84 | -79.758 | -46.8642 | 42.3047 | -2.6813 | 14.9784 |
| 85 | -79.5114 | -46.6513 | 42.587 | -2.4603 | 14.9785 |
| 86 | -79.2242 | -46.4471 | 42.8895 | -2.1571 | 14.9786 |
| 87 | -78.9711 | -46.1256 | 43.2831 | -1.9231 | 14.9787 |
| 88 | -78.7042 | -45.8515 | 43.6121 | -1.7039 | 14.9788 |
| 89 | -78.4256 | -45.5849 | 43.9562 | -1.4867 | 14.9789 |
| 90 | -78.0869 | -45.2759 | 44.4467 | -1.1754 | 14.979 |
| 91 | -77.7365 | -44.9092 | 44.7667 | -0.789 | 14.9791 |
| 92 | -77.3944 | -44.6017 | 45.2807 | -0.5202 | 14.9791 |
| 93 | -76.9746 | -44.132 | 45.9309 | -0.0671 | 14.9792 |
| 94 | -76.6574 | -43.6981 | 46.358 | 0 | 14.9792 |
| 95 | -76.1054 | -43.0589 | 47.002 | 0 | 14.9793 |
| 96 | -75.6002 | -42.3775 | 47.9034 | 0 | 14.9793 |
| 97 | -75.0261 | -41.7153 | 48.8577 | 0 | 14.9793 |
| 98 | -74.2943 | -40.9635 | 49.6382 | 0 | 14.9794 |
| 99 | -73.1064 | -40.0699 | 50.7318 | 0 | 15.6313 |
| 100 | -68.1115 | -32.3296 | 55.4421 | 0 | 20.5301 |

It can be seen from the tables that the simulation results are similar with the different channel bandwidths, thus there is no need to simulate both 100MHz and 400MHz channel bandwidths, hence only simulation results with 400MHz are provided below.

It was agreed in [4] that:

The new simulations results would need to provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting, showing the impact of the new results on the required ACIR comparing to the current ones in TR 38.803.

The current ACIR in TR 38.803 [2] are provided below for reference:

Table 5.5-4: Average ACIR values for UL in the worst case across all scenarios

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| ACIR value [dB] | 15.2 | 14.7 | 13.8 |

Table 5.5-5: UE ACLR

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| UE ACLR value [dB] | 17 | 16 | 15 |

Table 5.5-6: BS ACS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| BS ACS value [dB] | 23.5 | 22.5 | 21.5 |

Table 5.5-10: UE ACS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| UE ACS value [dB] | 22.5 | 21.5 | 20.5 |

Table 5.5-11: BS ACLR

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| BS ACLR value [dB] | 27.5 | 25.5 | 23.5 |

Our simulation results for Scenario Indoor-C [5] with 400MHz channel bandwidths at 60GHz or 70GHz carrier frequency are provided below. The simulation results obtained using the simulation assumptions and parameters agreed in [3].

2.1	Scenario Indoor-C (60GHz Downlink)

The Scenario Indoor-C simulation results of the downlink SINR of the victim UE with 400MHz channel bandwidths at 60GHz carrier frequency are provided in Figure 1 below. Here the downlink ACIR is obtained using 23.5dB BS ACLR and 20.5dB UE ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figures that enough downlink SINR (over 50dB) can be provided using (40-23=)17dBm BS conducted output power together with AAS BS beamforming, considering an additional 3dB gain is added to the total beamforming gain to account for the two polarization directions.

Figure 1: Scenario Indoor-C DL SINR of victim UE

The Scenario Indoor-C simulation results of the downlink throughput loss of the victim UE with different ACIR values and different ACIR offsets (compared to the ACIR using 23.5dB BS ACLR and 20.5dB UE ACS) with 400MHz channel bandwidth at 60GHz carrier frequency are provided, respectively, in Tables 1 and 2 below. It can be seen from the tables that the average and 5%-tile downlink throughput losses of the victim UE can only be limited to 5% in the simulated scenario with downlink ACIR value of 20dB or ACIR offset of 1dB (i.e., 1dB more stringent ACIR compared to the ACIR using 23.5dB BS ACLR and 20.5dB UE ACS).

| ACIR value [dB] | 20 | 19 | 18 | 17 | 16 | 15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 2.36% | 2.76% | 3.19% | 3.68% | 4.23% | 4.84% |
| 5%-tile throughput loss (400MHz) | 4.48% | 7.37% | 8.64% | 11.1% | 13.18% | 15.29% |

Table 1: Scenario Indoor-C DL throughput loss of victim UE with different ACIR values

| ACIR offset X [dB] | 1 | 0 | -1 | -2 | -3 | -4 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 2.34% | 2.87% | 3.49% | 4.25% | 5.17% | 6.3% |
| 5%-tile throughput loss (400MHz) | 4.33% | 7.4% | 9.61% | 13.21% | 16.58% | 19.59% |

Table 2: Scenario Indoor-C DL throughput loss of victim UE with different ACIR offsets

2.2	Scenario Indoor-C (70GHz Downlink)

The Scenario Indoor-C simulation results of the downlink SINR of the victim UE with 400MHz channel bandwidths at 70GHz carrier frequency are provided in Figure 2 below. Here the downlink ACIR is obtained using 23.5dB BS ACLR and 20.5dB UE ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figures that enough downlink SINR (over 50dB) can be provided using (40-23=)17dBm BS conducted output power together with AAS BS beamforming, considering an additional 3dB gain is added to the total beamforming gain to account for the two polarization directions.

Figure 2: Scenario Indoor-C DL SINR of victim UE

The Scenario Indoor-C simulation results of the downlink throughput loss of the victim UE with different ACIR values and different ACIR offsets (compared to the ACIR using 23.5dB BS ACLR and 20.5dB UE ACS) with 400MHz channel bandwidth at 70GHz carrier frequency are provided, respectively, in Tables 3 and 4 below. It can be seen from the tables that the average and 5%-tile downlink throughput losses of the victim UE can only be limited to 5% in the simulated scenario with downlink ACIR value of 20dB or ACIR offset of 1dB (i.e., 1dB more stringent ACIR compared to the ACIR using 23.5dB BS ACLR and 20.5dB UE ACS).

| ACIR value [dB] | 20 | 19 | 18 | 17 | 16 | 15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 2.34% | 2.73% | 3.16% | 3.64% | 4.18% | 4.78% |
| 5%-tile throughput loss (400MHz) | 4.24% | 6.96% | 8.33% | 10.73% | 12.64% | 14.43% |

Table 3: Scenario Indoor-C DL throughput loss of victim UE with different ACIR values

| ACIR offset X [dB] | 1 | 0 | -1 | -2 | -3 | -4 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 2.32% | 2.84% | 3.45% | 4.2% | 5.11% | 6.23% |
| 5%-tile throughput loss (400MHz) | 4.06% | 7.02% | 9.13% | 12.64% | 15.57% | 19.18% |

Table 4: Scenario Indoor-C DL throughput loss of victim UE with different ACIR offsets

2.3	Scenario Indoor-C (60GHz Uplink)

The Scenario Indoor-C simulation results of the victim and interfering UE conducted transmit power with 400MHz channel bandwidth at 60GHz carrier frequency are provided in Figure 3 below. Here the uplink ACIR is obtained using 15dB UE ACLR and 21.5dB BS ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figure that most of the UE are not transmitting at maximum conducted power because of the smaller propagation loss in this scenario.

Figure 3: Scenario Indoor-C UE conducted transmit power

The Scenario Indoor-C simulation results of the uplink SINR of the victim UE with 400MHz channel bandwidth at 60GHz carrier frequency are provided in Figure 4 below. Here the uplink ACIR is obtained using 15dB UE ACLR and 21.5dB BS ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figure that the target UL SINR of 15dB is achieved by most of the victim UE.

Figure 4: Scenario Indoor-C UL SINR of victim UE

The Scenario Indoor-C simulation results of the uplink throughput loss of the victim UE with different ACIR values and different ACLR offsets (compared to using 15dB UE ACLR and 21.5dB BS ACS) with 400MHz channel bandwidth at 60GHz carrier frequency are provided, respectively, in Tables 5 and 6 below. It can be seen from the table that the average and 5%-tile uplink throughput losses of the victim UE can only be limited to 5% in the simulated scenario with uplink ACIR value of 17dB or ACLR offset of 3dB (i.e., 3dB more stringent ACIR compared to the ACIR using 15dB UE ACLR and 21.5dB BS ACS).

| ACIR offset X [dB] | 18 | 17 | 16 | 15 | 14 | 13 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 0.86% | 1.04% | 1.24% | 1.49% | 1.77% | 2.09% |
| 5%-tile throughput loss (400MHz) | 3.22% | 4.28% | 5.04% | 6.28% | 8.76% | 10.75% |

Table 5: Scenario Indoor-C UL throughput loss of victim UE with different ACIR values

| ACLR offset X [dB] | 4 | 3 | 2 | 1 | 0 | -1 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 1.03% | 1.16% | 1.32% | 1.51% | 1.73% | 1.99% |
| 5%-tile throughput loss (400MHz) | 4.19% | 4.64% | 5.71% | 6.34% | 8.36% | 9.93% |

Table 6: Scenario Indoor-C UL throughput loss of victim UE with different ACLR offsets

2.3	Scenario Indoor-C (70GHz Uplink)

The Scenario Indoor-C simulation results of the victim and interfering UE conducted transmit power with 400MHz channel bandwidth at 70GHz carrier frequency are provided in Figure 5 below. Here the uplink ACIR is obtained using 15dB UE ACLR and 21.5dB BS ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figure that most of the UE are not transmitting at maximum conducted power because of the smaller propagation loss in this scenario.

Figure 5: Scenario Indoor-C UE conducted transmit power

The Scenario Indoor-C simulation results of the uplink SINR of the victim UE with 400MHz channel bandwidth at 70GHz carrier frequency are provided in Figure 6 below. Here the uplink ACIR is obtained using 15dB UE ACLR and 21.5dB BS ACS (proposed values in [2] at 70GHz carrier frequency). It can be seen from the figure that the target UL SINR of 15dB is achieved by most of the victim UE.

Figure 6: Scenario Indoor-C UL SINR of victim UE

The Scenario Indoor-C simulation results of the uplink throughput loss of the victim UE with different ACIR values and different ACLR offsets (compared to using 15dB UE ACLR and 21.5dB BS ACS) with 400MHz channel bandwidth at 70GHz carrier frequency are provided, respectively, in Tables 7 and 8 below. It can be seen from the table that the average and 5%-tile uplink throughput losses of the victim UE can only be limited to 5% in the simulated scenario with uplink ACIR value of 14dB or using 15dB UE ACLR and 21.5dB BS ACS.

| ACIR offset X [dB] | 18 | 17 | 16 | 15 | 14 | 13 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 0.84% | 1.02% | 1.22% | 1.46% | 1.73% | 2.05% |
| 5%-tile throughput loss (400MHz) | 2.31% | 2.4% | 2.84% | 3.4% | 4.77% | 6.54% |

Table 7: Scenario Indoor-C UL throughput loss of victim UE with different ACIR values

| ACIR offset X [dB] | 4 | 3 | 2 | 1 | 0 | -1 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (400MHz) | 1.01% | 1.14% | 1.29% | 1.48% | 1.7% | 1.95% |
| 5%-tile throughput loss (400MHz) | 2.4% | 2.48% | 2.86% | 3.67% | 4.76% | 5.99% |

Table 8: Scenario Indoor-C UL throughput loss of victim UE with different ACLR offsets

# 3.	Conclusion

This contribution has provided simulation results based on the proposed assumptions and parameters in [3]. The simulation results have shown that:

Observation 1) At 60GHz or 70GHz carrier frequency, the average and 5%-tile downlink throughput losses of the victim UE can only be limited to 5% in the simulated Scenario Indoor-C with downlink ACIR value of 20dB or ACIR offset of 1dB (i.e., 1dB more stringent ACIR compared to the ACIR using 23.5dB BS ACLR and 20.5dB UE ACS).

Observation 2) At 60GHz carrier frequency, the average and 5%-tile uplink throughput losses of the victim UE can only be limited to 5% in the simulated Scenario Indoor-C with uplink ACIR value of 17dB or ACLR offset of 3dB (i.e., 3dB more stringent ACIR compared to the ACIR using 15dB UE ACLR and 21.5dB BS ACS).

Observation 3) At 70GHz carrier frequency, the average and 5%-tile uplink throughput losses of the victim UE can be limited to 5% in the simulated Scenario Indoor-C with uplink ACIR value of 14dB or using 15dB UE ACLR and 21.5dB BS ACS.

Therefore, our preliminary simulation results show there is no technical justification to relax the required ACIR values compared to the current ones in TR 38.803 at 70GHz carrier frequency.

Observation 4) There is no technical justification to relax the required ACIR values compared to the current ones in TR 38.803 at 70GHz carrier frequency.

Hence it is proposed that:

Proposal 1) The proposed ACIR values in TR 38.803 at 70GHz carrier frequency can be reused as the required ACIR values for extending current NR operation to 71 GHz.

# References

[1]	RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC.

[2]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[3]	R4-2114993, “WF on co-existence simulation for NR_ext_to_71GHz”, Qualcomm Incorporated.

[4]	R4-2107915, “WF on 60 GHz coexistence, ACLR, and ACS”, Qualcomm Incorporated.

[5]	3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”.