3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117425

# Electronic Meeting, November 01-12, 2021

Agenda item:	5.1.3.1

Source:	Apple Inc.

Title:	On clarification of MRTD requirements applicability to multi-TRxP

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#100-e RRM requirements maintenance for eMIMO was discussed and way forward [1] was agreed. It was agreed to add clarification on MRTD applicability to multi-TRxP scenario into RAN4 specification. IN this contribution we provide our inputs on how the applicability should be clarified in TS 38.133.

# 2. Discussion

The agreements in [1] related to clarification on MRTD applicability to multi-TRxP:

With the intention to clarify that in NR-CA if any of the carriers are configured with multi-TRxP, option 2b above is more suitable. Also, option 2b clarifies that multi-TRxP is on the same carrier and not inter-cell which is Rel-17 feature.

Proposal #1: Add clarification on MRTD applicability to multi-TRxP for NR-CA as:

A UE shall be capable of handling a relative receive timing difference between slot timing boundaries of any one carrier and the closest slot timing boundary of another carrier in NR carrier aggregation; and if a UE is configured to receive multiple PDSCH from different TRP on the same carrier,  the UE shall be capable of handling a relative timing difference between any one of the slot timing boundaries of any one carrier with multiple PDSCH and the closest slot timing boundary of another carrier in NR carrier aggregation.

Multi-TRxP transmission could be possible in EN-DC, NR-DC and NR-DC operation as well, and hence a similar clarification should also be added to EN-DC, NE-DC and NR-DC operation in addition to NR-CA.

Proposal #2: Add clarification on MRTD applicability shall also be added to EN-DC and NE-DC operation.

Our proposals are captured in our companion CR [2].

# 3. Conclusion

In this paper, we provide our inputs on how the applicability of MRTD to multi-TRxP should be clarified in TS 38.133. Our proposals are captured below:

Proposal #1: Add clarification on MRTD applicability to multi-TRxP for NR-CA as:

A UE shall be capable of handling a relative receive timing difference between slot timing boundaries of any one carrier and the closest slot timing boundary of another carrier in NR carrier aggregation; and if a UE is configured to receive multiple PDSCH from different TRP on the same carrier,  the UE shall be capable of handling a relative timing difference between any one of the slot timing boundaries of any one carrier with multiple PDSCH and the closest slot timing boundary of another carrier in NR carrier aggregation.

Proposal #2: Add clarification on MRTD applicability shall also be added to EN-DC and NE-DC operation.

# Reference

- R4-2115299, “WF on NR eMIMO RRM requirement maintenance”, Samsung.

- R4-2117426, “Draft CR to 38.133 on applicability of MRTD requirements to multi-TRxP - R16”, Apple