3GPP TSG-RAN WG4 Meeting#101      	R4-2117570

E-meeting, 1st – 12th Nov, 2021

Title:	TP to TR 38.717-04-01: CA_n2-n48-n66-n77

Source:	Nokia, Verizon

Agenda item:	7.9.2

Document for:	Approval

# 1	Introduction

This contribution introduces CA_n2-n48-n66-n77 into TR 38.717-04-01.

*****************************Start of TP**************************************************

## 5.X	CA_n2-n48-n66-n77

### 5.X.1	Channel bandwidths per operating bands for CA

Table 5.30.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48A-n66A-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48B-n66A-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_ n48B Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48(2A)-n66A-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_ n48(2A) Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48A-n66A-n77C | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 | See CA_ n77C Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2	∆TIB,c and ∆RIB,c values

For CA_n2-n48-n66-n77 the TIB,c and RIB,c  values are shown in table 5.30.2-1 and  table 5.30.2-2, respectively. Values are derived from DC_2-48-66_n77.

Table 5.30.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n48-n66-n77 | n2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 5.30.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n48-n66-n77 | n2 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 5.X.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

*****************************End of TP**************************************************