3GPP TSG-RAN WG4 Meeting #101-e              	R4-2117596

Electrical Meeting, 1st Nov-12th Nov, 2021

Agenda item:	8.9.5.3.3

Source:	Samsung

Title:	Discussion and simulation results of PRACH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on PRACH requirement for Rel-17 FR2 HST were further discussed. The related agreement was captured in the WF [1] as

- Frequency offset - 19444Hz with 350km/h at 30GHz carrier frequency - Test Preamble Configuration for Ncs - Ncs = 0 as baseline - Timing offset configuration - Option 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e, 0.8us - Value of Timing offset start: 0 - Value of Timing offset step: 0.1us - Option 2: Configure the maximum timing offset (i.e., the end of the tested range) in HST FR2 testing setup equal to 4.6us - Value of Timing offset start: 0 - Value of Timing offset step: 0.48us - Note Scenario; Cell radius (m); Maximum timing offset (us) (RTT); Coverage of format C2; Maximum timing offset for C2 (us) (RTT). Scenario A (Ds =700m, Dmin =10m); 700; 4.6us; 1.15km; 7.6. Scenario B (Ds=700m, Dmin=150m); 716; 4.78us; 1.15km; 7.6. - Option 1 can be regarded as baseline. Companies are encouraged to bring the simulation results to compare the performance between option 1 and option 2 in the next meeting

In this contribution, the view on remaining issue is provided, meanwhile, the initial simulation results are provided to finalize the test parameters for PRACH requirement.

# 2	Discussion

Timing offset configuration

Regarding the timing offset, there are two principles need to be considered. One is the largest time offset is no larger than the CP length of PRACH as

12NcsLRA *2048*κ*2-μ*Tc+Δτ+τdelay<NCPRA*κ*2-μ*Tc

Another is the largest time offset with propagation delay cannot be large than the length of detection window (based on v and Ncs) as

12NcsLRA *2048*κ*2-μ*Tc+Δτ+τdelay<NcsLRA *2048*κ*2-μ*Tc

Based on this two principles, timing offset should be fulfilled with the following condition as

Δτ<minNCPRA*Tc-12NcsLRA *2048*κ*2-μ*Tc,12∙NcsLRA *2048*κ*2-μ*Tc-τdelay

During Rel-15 NR PRACH demodulation discussion, RAN4 has specified the requirement with different preamble formats with A1, A2. A3, B4, C0 and C2. In order to simply the test setup, a unified timing offset scheme for all the preamble formats was defined, where format A1 with 120 kHz SCS has the smallest timing offset (due to the short CP length), as  0.88us.

For FR2 HST WI, there are two deployment scenarios under discussion. Based on parameters (Ds, Dmin) for these two scenarios, the related maximum timing offset can be calculated as following

Table 1: Maximum timing offset for difference scenarios

| Scenario | Cell radius (m) | Maximum timing offset (us) (RTT) | Coverage of format C2 | Maximum timing offset for C2 (us) |
| --- | --- | --- | --- | --- |
| Scenario A (Ds =700m, Dmin =10m) | 700 | 4.6us | 1.15km | 7.6 |
| Scenario B (Ds=700m, Dmin=150m) | 716 | 4.78us | 1.15km | 7.6 |

From PRACH detection algorithm perspective, there is no different receiver behavior foreseen with different value of timing offset. The purpose is to verify PRACH receiver with large frequency offset impact.

# 3	Simulation Results

In this contribution, the initial simulation result was provided. The simulation assumption can be referred as following table

Table 1: Simulation assumption for PRACH

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Timing offset | Option 1: 0:0.1: Option 2: 0:0.46:4.6 |

Figure 1: Miss detection performance for PRACH with format C2

Table 2: Initial simulation results for PRACH with format C2

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Frequency offset | Timing offset range | SNR [dB] Ideal |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 19444Hz | 0: 0.1:0.8us | -12.49 |
| 1 | 2 | AWGN | 19444Hz | 0:0.46:4.6us | -12.56 |

Observation 1: Minor performance different between maximum timing offset as 0.8 and 4.6us

Therefore, we prefer to reuse the rel-15 FR2 timing offset configuration for PRACH.

Proposal 1:  Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e, 0.8us.

# 4	Conclusion

In this contribution, the view on the remaining issue for PRACH requirement. Meanwhile, the initial simulation results are provided for verify the impact on different timing offset configuration.

Observation 1: Minor performance different between maximum timing offset as 0.8 and 4.6us

Proposal 1:  Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us.

# References

R4-2115726, WF on FR2 HST demodulation, Samsung