3GPP TSG-RAN WG4 Meeting #101-e                	 R4-2117605

Electronic Meeting, November 01-12, 2021

Agenda Item:	8.11.2.3

Source:	MediaTek Inc.

Title:	Discussion on NCSG

Document for:	Discussion

# Introduction

In last meeting, a WF for Network Controlled Small Gap (NCSG) was approved [1]. In this paper, we will further discuss the detail for NCSG.

- Scenario and use cases

- NCSG pattern

- VIL, RRT and ML

- UE capability and network configuration of NCSG

- Measurement related requirements

- Other RRM requirements

# Scenario and use cases

The open issues in [1] are captured below.

## Issue 1-1: NCSG in FR2 - Agreement: - NCSG in FR2 shall be considered in this WI. How to indicate support of NCSG in FR2 is FFS. ## Issue 1-2: Use case for different types of measurement with NCSG - Agreement: - Confirm the agreements in RAN4#99e that NCSG can be used for: - Measurement on de-activated SCell - SSB based intra-frequency measurement with gap - SSB based inter-frequency measurement with gap - Inter-RAT E-UTRAN measurement - It is FFS whether NCSG can be used for measurement on dormant SCell. - It is FFS whether NCSG can be used for CSI-RS based inter-frequency measurement with gap. - NCSG will not be used for 2G/3G measurements and PRS measurements. ## Issue 1-3: Use case for different UE capability and network configuration - Open issues: - Option 1: RAN4 to define UE measurement requirements with different UE capabilities and NW configurations as in the following table: NW config UE capability; Case a: No MG nor NCSG; Case b: NCSG; Case c: MG. Case 1: gap; No requirement; No requirement; Measurement within MG. Case 2: no-gap-with-interruption; No requirement; Measurement within NCSG with only NCSG interruption allowed; Measurement within MG with only legacy gap interruption allowed. Case 3: no-gap-no-interruption; Measurement without MG; Measurement outside NCSG; Measurement outside MG. - Option 2: other solutions are not precluded. ## Issue 1-4: Whether NW should configure the legacy MG rather than NCSG even UE can support both of them - Agreement: - Even if UE supports NCSG then it is up to the NW implementation to configure the UE with legacy MG or with NCSG.” ## Issue 1-5: Other applicability issues - Agreement: - RAN4 to clarify whether to allow NCSG to be supported with NRDC and NEDC as well. - Open issues: - FFS: Existing gap applicability in Rel-16 for NR-only measurements and mandatory gap patterns is re-used for NCSG capable UEs. - FFS: Related to NCSG applicability and UE capability support: if UE supports NCSG, it is mandated to support MG patterns from per-UE gap patterns #0, #1, #4-9 for NCSG usage. In case UE supports NCSG and per-FR gap patterns, indicating this via capability, it is mandated to support also per-FR gap patterns #12-19 in FR2 for NCSG usage. - FFS: NCSG can be configured simultaneously with legacy MG pattern. - FFS: NCSG can be pre-configured and will reuse the activation/deactivation mechanism developed for pre-configured measurement gaps. - FFS: NCSG can be configured and activated together with concurrent measurement gaps.

Regarding FR2 support, this is up to the discussions on how to handle the CBM or IBM capability. If independent beams can be used for both serving cells and the target frequency layer, UE can perform measurement with rough beams at the same when receiving data from its serving cells with fine beams. However, if common beam is used for both serving cells and the target frequency, the benefit of NCSG is somehow questionable. UE either has to stop serving cell communications for measurement, or drop measurements for continuing serving cell data Tx/Rx. Therefore, we suggest to limit the NCSG only to the case that IBM is possible. In other words, UE should not claim the support of NCSG on the target band which shares common beam with any of UE’s serving cells in FR2.

Proposal 1: UE should not claim the support of NCSG on the target band which shares common beam with any of UE’s serving cells in FR2.

On the remaining open issues for the use case for different types of measurement, we are fine to exclude CQI measurements for dormant SCell. The main reason is that the CSI-RS for CQI/PMI/RI reporting may not be configured with the same time domain structure like SMTC. Therefore, it is not always guaranteed that the CSI-RS are available during the ML duration of NCSG. Not expecting CQI measurement to be done via NCSG implies the interruption requirements for CQI measurement defined in Section 8.2 of TS38.133 can still apply.

Proposal 2: CQI measurements for dormant SCells are not expected to be done via NCSG.

On the other hand, the RRM measurement for dormant SCell may still be done via NCSG. We suggest RAN4 to further check the feasibility.

Proposal 3: RAN4 to decide whether RRM measurements for dormant SCells should be done via NCSG.

Issue 1-3 is a more complicated issue. UE may report different UE capabilities for different bands. Some bands should always be gap-assisted, some bands can be done in NCSG, and some bands can be measured without any interruption. As network can only choose one configuration (no gap, gap, NCSG) for this UE, RAN4 needs to discuss the expected corresponding UE behaviours in different cases. This issue is roughly summarized in Table 1.

Table 1. Different UE behaviours due to different network configurations

The differences to the option 1 in [1] are highlighted in red. In short, the question is about whether UE needs to follow network’s configuration (gap or NCSG) to perform the measurement, or UE should perform the measurement outside NCSG or legacy gap. In our view, the principle we agreed in Rel-15 is to always follow network’s gap configuration, even UE does not need the gap. Therefore, here we suggest to follow the same principle for NCSG.

Proposal 4: When NCSG is configured to UE, UE will perform measurement within NCSG for those bands that can be measured without any interruption and those bands that can be measured with NCSG.

Proposal 5: When legacy gap is configured to UE, UE will perform measurement within gap for all target inter-frequency/RAT layers.

A slightly even more complicated case would be how to jointly consider the Rel-16 feature 9-4 ‘SSB based inter-frequency measurement without measurement gap’. In our view, we can check whether the SSB of the inter-frequency layer is within UE’s active BWP. If so, it is covered by this Rel-16 feature. If not, it is covered by Rel-17 NCSG discussion.

Proposal 6: If the SSB of the inter-frequency layer is within UE’s active BWP, whether UE can perform gapless measurement is determined by Rel-16 feature 9-4 ‘SSB based inter-frequency measurement without measurement gap’. Otherwise, it is determined by NCSG reporting.

# NCSG pattern

The open issues in [1] are captured below.

## Issue 2-1: supported NCSG patterns in R17 - Agreement: - No need to introduce NCSG patterns corresponding to legacy MG patterns #24 and #25. - It is FFS whether to introduce NCSG patterns with longer repetition periodicity (>160ms). - Define NCSG patterns corresponding to legacy patterns #0~#23 - FFS how to indicate the support of NCSG patterns - A subset of mandatory NCSG patterns for UEs supporting NCSG will be defined. FFS on the set of mandatory NCSG patterns. ## Issue 2-2: whether to define separate NCSG patterns for sync and async - Open issues: - Option 1: yes - Option 2: no - Option 3: different patterns in FR1 but same patterns in FR2 - Option 4: FFS ## Issue 2-3: whether to consider VIL as a part of NCSG pattern - Option 1: - NOT consider VIL as a part of NCSG pattern, i.e. only keep measurement length and repetition periodicity in the pattern design, and capture VIL separately as interruption requirements (similar to Table 9.1.2-4 in TS38.133). - Option 2: - Keep VIL as a part of the NCSG pattern.

Regarding how UE indicates the supported NCSG pattern, our original thinking is to inherit all Rel-15/16 legacy UE capability, including which gaps are mandatory. Given the comments received during several meetings, we are now also fine to introduce a separate UE capability reporting for supported NCSG patterns. And among all the NCSG pattern, we think pattern 0 and 1 should be conditional mandatory, i.e., they are mandatory for all UEs which support NCSG.

Proposal 7: Introduce a separate UE capability reporting for supported NCSG patterns. NCSG patterns #0 and #1 should be mandatory for all UEs which support NCSG.

Regarding Issue 2-2 and 2-3, we will discuss them together with other issues in Section 4.

# VI, RRT and ML

The open issues in [1] are captured below.

## Issue 3-1: whether to replace VIL (visible interruption length) with RRT (RF retuning time) - Open issues: - Option 1: Yes. Introduce absolute RRT to replace VIL. - Option 2: VIL and RRT can be defined separately. - Option 3: only capture VIL in RAN4 spec. RRT can be used to calculate ML in discussion. But no need to capture RRT in RAN4 spec. ## Issue 3-2: how to capture VIL in RAN4 spec - Note: issue 3-4 was concluded in the 1st round, which means VIL will be captured in RAN4 spec in terms of number of the interrupted slots. Therefore, no need to further discuss issue 3-2. ## Issue 3-3: relation between ML, MGL and RRT - Open issues: - Option 1: ML = MGL – 2*RRT - Option 1a: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL - Option 1b: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL, if VIL is defined as the number of interrupted slots - Option 1c: ML = MGL - RRT1 - RRT2 - Option 2: ML + VIL1 + VIL2 = MGL - Option 2a: ML = MGL – VIL1 – VIL2, if VIL is defined as the absolute time - Option 3: - Step 1: Define MLNCSG from legacy gap patterns by MLNCSG = MGLlegacy – 2*RRTlegacy, e.g., - Gap patterns 0-11: MLNCSG = MGLlegacy – 1 (ms) - Gap patterns 12-23: MLNCSG = MGLlegacy – 0.5 (ms) - Step 2: Define RRTNCSG before and after MLNCSG in FR1 and FR2 - Handled by Issue 3-5, e.g., same or longer than RRTlegacy. - Step 3: Define MGLNCSG as MLNCSG + 2* RRTNCSG. - Step 4: Define VIL - Handled by Issue 3-4 ## Issue 3-4: length of VIL - Agreements in the 1st round: - Translate [1ms] (FR1) and [0.75ms] (FR2) into the number of interrupted slots for defining the interruption requirements for the synchronous case and one more slot is added for asynchronous case. ## Issue 3-5: length of RRT - Agreements in the 1st round: - The following RRT time can be used as assumption to derive ML - RRT = 0.5 ms for FR1 and 0.25 ms for FR2 - Whether to capture above RRT time in RAN4 spec is FFS. ## Issue 3-6: impact from RTD - Open issues: - FFS: RAN4 to further discuss how to address RTD between time reference cell and victim cell. ## Issue 3-7: UL slot after VIL1 - Open issues: - FFS: RAN4 to further discuss how to address UL slot immediately after VIL1 in the interruption requirements.

The previous discussions on VIL, RRT and ML are not very efficient because different companies may have different definitions in mind. Therefore, we suggest to follow the step in Option 3 of Issue 3-3 to proceed the discussion.

The 1st issue is to ensure that NCSG has the same ML duration as legacy gap. This is an important restriction which helps us to create a 1-to-1 mapping between NCSG and legacy gap patterns.

Proposal 8: NCSG gap pattern #n should share the same ML duration of legacy gap pattern #n.

Given the ML duration confirmed, the next question is how long UE needs to get ready for starting/stopping measurement. (Let’s don’t call it RRT neither VIL for now to avoid confusion.) According to the discussions so far, UE may need to 2 preparations: 1) re-tune its RF and 2) re-allocate its baseband resources. RAN4 needs to agree on the length of this preparation time. With both ML and preparation time defined, the MGL for the NCSG is also well-defined. We have no strong view to call this preparation time by VIL or other different names.

Proposal 9: RAN4 to agree on the length of the preparation time for UE to start/stop measurement. The preparation time contains the RF re-tuning and BB resource re-allocation time.

In our understanding, during the preparation time, the only UE action causing interruption to other CCs is the RF re-tuning. Following Rel-15 agreements, RF re-tuning time (RRT) is 0.5ms for FR1 and 0.25ms for FR2. The exact location of this RF re-tuning time can be up to UE implementation if the preparation time is longer than RRT. But RAN4 can still discuss whether to fix its time location in order to simplify the calculation of the NCSG interruption requirement.

With the duration (and maybe the location) of RRT confirmed, RAN4 can start to discuss the interruption on other CCs caused by the RF re-tuning. The principle is no different to the interruption requirements for BWP switch. Some examples are provided in the Appendix for reference.

Proposal 10: The interrupted slots by NCSG gap are calculated based on the location and duration of RF re-tuning time.

# UE capability and network configuration of NCS

The open issues in [1] are captured below.

## Issue 4-1: whether additional NCSG capability for per-UE and per-FR differentiation is needed on top of existing per-UE and per-FR capability - Open issues: - Option 1: no. - Option 2: yes. Introduce per BC indication of per FR NCSG in Rel-17. The discussion can be postponed till progress is made towards per BC indication for per FR UE capability. ## Issue 4-2: how to indicate the support of NCSG pattern - Open issues: - Option 1: introduce new signalling (separately from NeedForGap) to indicate support of NCSG. - Option 2: introduce new element in NeedForGap to indicate support of NCSG. - Option 3: introduce new signalling (separately from NeedForGap) to indicate support of following cases - Case 1: gap - Case 2: no-gap-with-interruption (or NCSG) - Case 3: no-gap-no-interruption - Option 4: up to RAN2. ## Issue 4-3: whether RAN4 needs to decide whether UE is allow to report ‘no gap’, ’NCSG’ and ‘gap’ capabilities to different bands. The UE behaviour and the corresponding measurement requirements are highly depending on how signaling will be provided and UE capability will be reported. - Open issues: - Option 1: yes - Option 2: FFS ## Issue 4-4: configuration of NCSG. - Open issues: - Option 1: Support the explicit configuration for NCSG (detail to be left to RAN2). - Option 2: Introduce a single bit for existing MeasGapConfig to transform the legacy gap into NCSG (detail to be left to RAN2). - Option 3: postpone until NCSG pattern design as well as NCSG applicability and UE capability support are finalized ## Issue 4-5: Mapping/relation between NCSG and legacy MG patterns. - Open issues: - FFS: Introduce mapping table between legacy measurement gap patterns and corresponding NCSG patterns for the UE and gNB to determine the transform gap pattern. Details of mapping are FFS. - FFS: When UE supports NCSG, the supported gap pattern index shall be the same as its reported legacy MG pattern capability in Rel-15/16. - FFS: Use the same index for legacy MGP and its corresponding NCSG pattern.

We do not see a strong need to introduce additional per-UE or per-FR capability for NCSG. As the main bottleneck is the RF capability, if UE can already do FR1 and FR2 gap-based measurement in parallel, UE should have no problem even for the NCSG. But we are also fine to introduce the capability if companies have a strong concern.

Proposal 11: Do not introduce additional NCSG capability for per-UE and per-FR differentiation, unless there is a strong concern.

Regarding how UE reports the NCSG capability to network, we believe that this issue should be discussed from 3 different angles: 1) When should UE report this capability, 2) how UE capabilities for different bands are reported, and 3) what are in the content of the UE report.

- [When] Following NeedforGap, we think it is more efficient to only report after RRC re-configuration. This is to avoid the LTE reporting framework in which UE has to report whether to support ‘no gap’ between all pairs of the target band for measurement and all its supported band combinations for CA. The overhead is rather large and not all reported pairs will be used by network.

Proposal 12: RAN4 to suggest RAN2 to follow the same UE capability report frame work as NeedforGap, i.e., report only after RRC re-configuration.

- [How] It is also very important to discuss whether we allow UE to report different capability for different target bands for measurement, or we only ask UE to report a single capability for all target bands. In our view, as NeedforGap already provides such a flexibility to differentiate different target band, the reporting for NCSG can inherit it straightforwardly.

Proposal 13: Allow UE to report the NCSG capabilities separately for different target bands for measurement.

- [What] Regarding the content in the report, the key point is to allow UE to further indicate whether interruption is needed when ‘no-gap’ is reported for a certain band. We are fine to either add a new indication of NCSG in existing NeedforGap reporting or create a completely new IE like Option 3 in Issue 4-2.

Proposal 14: The UE capability reporting for NCSG should allow UE to further indicate whether interruption is needed or not when ‘no-gap’ is reported for a certain band.

Regarding the RRC configuration for NCSG, we think it is sufficient to introducing a single bit in the measurement gap configuration to differentiate legacy gap and NCSG. E.g., if the bit is 0, UE assumes legacy gap is configured, and if the bit is 1, UE NCSG is configured. In other words, all the other configurations (MGRP, MGL, offset, mgta) can be shared between legacy gap and NCSG.

Proposal 15: Introduce a single bit for existing MeasGapConfig to transform the legacy gap into NCSG (detail to be left to RAN2).

# Measurement related requirements

The open issues in [1] are captured below.

## Issue 5-1: CSSF design - Open issues: - Option 1: define a new CSSF dedicated for NCSG measurement - Option 2: When NCSG is configured, for a frequency layer that can be measured without MG, Kp = 1/(1- (SMTC period /VIRP)) applies when SMTC period < VIRP; when SMTC period >= VIRP, the frequency layer should be measured within NCSG and be accounted in the CSSF calculation. - Other options are not precluded. ## Issue 5-2: scheduling restriction - Open issues: - Option 1: during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply - Option 2 - For intra-frequency measurement, existing scheduling restriction requirements apply. - For inter-frequency measurement and the target carrier and the serving cell are in same band, existing scheduling restriction requirements apply except that all symbols in SMTC windows are restricted. - For inter-frequency measurement and the target carrier and the serving cell are in different bands, all symbols in SMTC windows are restricted when scheduling restrictions apply, and whether scheduling restrictions apply depends on UE capability. - NW should be informed whether UE needs scheduling restriction or not for a combination of an inter-frequency target carrier and a serving cell. - Option 3: The existing scheduling restriction requirements defined in TS 38.133 for FR1 shall apply during ML when serving and measured carriers are in FR1. No scheduling restriction is allowed for FR2 during ML when serving carrier and measured carriers are in FR2 and use IBM. - Option 4: Scheduling restriction for NCSG is FFS, and check with RAN2 on the feasibility of informing NW the CBM or IBM between inter-frequency measurements and serving cells in FR2.

Regarding CSSF, we think RAN4 needs to introduce a new CSSF for NCSG because the frequency layers to be measured via NCSG is not completely the same as Rel-15 CSSF within gap and CSSF outside gap. Furthermore, a new CSSF for NCSG also has a better forward compatibility when later RAN4 further extends NCSG to concurrent gaps. E.g., if network configures one legacy gap and one NCSG, then all the frequency layers to be measured need be categorized into 3 groups: outside gap, within NCSG and within gap.

Proposal 16: Introduce a separate CSSF for NCSG.

Then the next question is which frequency layers are assumed to be measured within NCSG. RAN4 has requirements for intra-frequency measurement without gap and inter-frequency measurements without gap. Theoretically, these frequency layers can also be measured within NCSG, but we suggest to keep them measured outside NCSG. The main reason is to follow Rel-15 principle. In Rel-15, we assume intra-frequency without gap will be measured outside gap, but actually it does not prohibit UE from measuring them within gap, as long as UE can meet the requirements. Similar principle can be applied here, UE can still measure within NCSG for those gapless frequency layers, but as a minimum requirement we do not need to mandate UE to do so.

Proposal 17: For those frequency layers that can be measured without MG nor NCSG, UE is assumed to only measure them outside of NCSG occasions.

# Other RRM requirements

The open issues in [1] are captured below.

## Issue 6-1: requirements for interruption due to NCSG based measurement on deactivated SCC - Open issues: - Option 1: existing requirements shall be revisited - Option 2: FFS

As long as NCSG is configured for de-activated SCell measurement, UE can follow the NCSG interruption to switch ON/OFF or re-tune its RF frequency, rather than follow the legacy interruption requirement. Therefore, we think it is reasonable to revise the current interruption requirement for de-activated SCell measurement. A note can be added, saying that the requirements are not applicable to the UEs configured with NCSG.

Proposal 18: The interruption for de-activated SCell measurement is not applicable to the UEs configured with NCSG.

# Conclusion

In the contribution, we discuss the issues for NCSG. We have the following observations and proposals:

Proposal 1: UE should not claim the support of NCSG on the target band which shares common beam with any of UE’s serving cells in FR2.

Proposal 2: CQI measurements for dormant SCells are not expected to be done via NCSG.

Proposal 3: RAN4 to decide whether RRM measurements for dormant SCells should be done via NCSG.

Proposal 4: When NCSG is configured to UE, UE will perform measurement within NCSG for those bands that can be measured without any interruption and those bands that can be measured with NCSG.

Proposal 5: When legacy gap is configured to UE, UE will perform measurement within gap for all target inter-frequency/RAT layers.

Proposal 6: If the SSB of the inter-frequency layer is within UE’s active BWP, whether UE can perform gapless measurement is determined by Rel-16 feature 9-4 ‘SSB based inter-frequency measurement without measurement gap’. Otherwise, it is determined by NCSG reporting.

Proposal 7: Introduce a separate UE capability reporting for supported NCSG patterns. NCSG patterns #0 and #1 should be mandatory for all UEs which support NCSG.

Proposal 8: NCSG gap pattern #n should share the same ML duration of legacy gap pattern #n.

Proposal 9: RAN4 to agree on the length of the preparation time for UE to start/stop measurement. The preparation time contains the RF re-tuning and BB resource re-allocation time.

Proposal 10: The interrupted slots by NCSG gap are calculated based on the location and duration of RF re-tuning time.

Proposal 11: Do not introduce additional NCSG capability for per-UE and per-FR differentiation, unless there is a strong concern.

Proposal 12: RAN4 to suggest RAN2 to follow the same UE capability report frame work as NeedforGap, i.e., report only after RRC re-configuration.

Proposal 13: Allow UE to report the NCSG capabilities separately for different target bands for measurement.

Proposal 14: The UE capability reporting for NCSG should allow UE to further indicate whether interruption is needed or not when ‘no-gap’ is reported for a certain band.

Proposal 15: Introduce a single bit for existing MeasGapConfig to transform the legacy gap into NCSG (detail to be left to RAN2).

Proposal 16: Introduce a separate CSSF for NCSG.

Proposal 17: For those frequency layers that can be measured without MG nor NCSG, UE is assumed to only measure them outside of NCSG occasions.

Proposal 18: The interruption for de-activated SCell measurement is not applicable to the UEs configured with NCSG.

# Reference

[1] R4-2115344, “WF on R17 NR MG enhancements –NCSG”, Apple

# Appendix

Figure 1. Gap interruption for Per-UE gap or FR1 gap under synchronous scenario without MGTA

Figure 2. Gap interruption for Per-UE gap or FR1 gap under synchronous scenario with MGTA

Figure 3. Gap interruption for Per-UE gap or FR1 gap under asynchronous scenario without MGTA

Figure 4. Gap interruption for Per-UE gap or FR1 gap under asynchronous scenario with MGTA