3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2117608

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-11: DC_n77A_3A

Agenda Item:	7.13

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR band n77 and LTE band3 for TR 37.717-11-11[1].

# Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.4.0

# Text Proposal

----- Start of TP -----

### 6.1.x	DC_n77A_3A

#### 6.1.x.1	Operating bands for DC

Table 6.1.37.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n77A_3A | 3 | n77 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.37.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n77A_3A | DC_n77A_3A | 3A | n77A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.37.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n77A_3A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n77A_3A are the same as DC_3A_n77A, which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

#### 6.1.x.5	MSD analysis for DC

The MSD analysis for DC_n77A_3A are the same as DC_3A_n77A which is available in 3GPP TR 37.716-11-11 V2.0.0:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2490 | 1515 | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4815 | 6690 | 780 | 270 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8310 | 10185 | 6720 | 7770 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8115 | 10890 | 930 | 2055 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11610 | 14385 | 8430 | 9555 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3030 | 4980 | 10020 | 11970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3840 | 2640 | 15090 | 11415 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1245 | 3270 | 9180 | 6330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10140 | 11340 | 14910 | 18585 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11730 | 13755 | 13320 | 16170 |

- 2nd order harmonics of band 3 could fall on the n77 downlink frequency range.

6.1.x.6	MSD

MSD values for DC_n77A_3A are the same as DC_3A_n77A, which have been already defined in the TS38.101-3 spec.

#### 6.1.x.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n77A_3A are the same as DC_3A_n77A, which have already been defined in the TS38.101-3 spec.

#### 6.1.x.8		Self-interference analysis

#### The analysis of DC_n77A_3A are the same as DC_3A_n77A which is available in 3GPP TR 37.716-11-11 V2.0.0.

#### 6.1.x.9	OOB blocking exception requirements

No OOB blocking exception for DC_n77A_3A.

----- End of TP -----