3GPP TSG-RAN WG4 Meeting #101-e	R4-2117647

1 –12 November 2021

Source:	Qualcomm Incorporated

Title:	n14 REFSENS for PS in licensed band

Agenda item:	8.15.3.2

Document for:	Approval

# 1.	Introduction

In RAN4#100-e the REFSENS for n14 for PS was discussed and no agreement was made [1]. This paper presents the development of n14 REFSENS for PS and proposes values for 5MHz and 10MHz operation.

# 2. 	Discussion

The REFSENS for PS was studied in detail for n14 in D2D [2]. This paper follows the general principles outlined for D2D in deriving the REFSENS for n14 PS.   In D2D the REFSENS was derived using an equation similar to that given below:

RefSensNRn14_PS = RefSensWAN + ∆SNRNRn14PS-WAN + ∆ILUL-DL + 10log10(LCRB/NRB)	(1)

The variables in the above equation are described below:

| RefSensNRn14_PS: | Reference sensitivity for NR n14 PS (Rx on FDD UL) with new reference measurement channels |
| --- | --- |
| RefSensWAN: | Reference sensitivity for WAN (Rx on FDD DL) currently specified in TS 38.101. |
| ∆SNRNRn14PS-WAN | Difference in decoding SNR requirements between the new reference measurement channels for NR PS and currently specified WAN DL reference measurement channels. |
| ∆ILUL-DL: | Difference in insertion loss of the UL filter compared to the DL filter of the band-duplexer (average over vendor data) and after shared pain |
| LCRB: | Transmission BW of the new NR n14 PS reference measurement channel |
| NRB: | Transmission BW configuration (since DL reference measurement channels are fully allocated) |

For D2D, based on company inputs, the ∆ILUL-DL for band14 was agreed to be ~0.1 dB [3]. It is proposed that the same value is used for n14 PS.

Proposal 1: The difference in insertion loss of the UL filter compared to the DL filter of the band-duplexer, ∆ILUL-DL =0.1 dB

For n14 PS the same SNRs will be adopted as for the corresponding NR V2X BW and SCS configurations. The NR V2X SNR and IM_relaxation requirements for a SCS=30kHz and BW of 10MHz is given in [4] and noted in table 1. The SNR value for a SCS=15kHz and a BW of 5MHz is assumed to be the same as for SCS=30kHz and BW=10MHz as the number of allocated RBs and hence the code length is the same between these 2 settings. It should be noted that when the number of RBs ≤24 the NR V2X SNR is relaxed by 0.5dB by increasing the IM by the same amount [4]. The following table outlines the parameters for each PS use case and the corresponding n14 PS RefSensNRn14_PS calculated using equation (1)

Table 1 – REFSENS calculation for n14 PS

Based on the above calculations the following is proposed

Proposal 2: Use the REFSENS (RefSensNRn14) values presented in table 1 for NR n14 PS

# Conclusion

This paper presents calculation of RefSensNRn14_PS based on the development used in D2D. Following this methodology, the REFSENS values given in table 1 are calculated and the following proposal is made:

Proposal 1: The difference in insertion loss of the UL filter compared to the DL filter of the band-duplexer, ∆ILUL-DL =0.1 dB

Proposal 2: Use the REFSENS (RefSensNRn14) values presented in table 1 for NR n14 PS

# References

[1] R4-2114979, WF on Pemax definition and NR PS REFSENS requirements for SL enhancement UE in n14, RAN4#100-e, August 16 -27 , 2021

[2] R4-150194, D2D Rx on FDD bands: Remaining details for REFSENS, RAN4#74, February 9-13, 2015

[3] TR36.877, LTE Device to Device (D2D) Proximity Services (ProSe); User Equipment (UE) radio transmission and reception, V12.0.0

[4] TR38.886, V2X Services based on NR; User Equipment (UE) radio transmission and reception, V16.2.0