3GPP TSG RAN WG4 Meeting #101-e	R4-2117650

Online, 1st – 12th Nov. 2021

Agenda item:	7.20.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-31: EN-DC_1_n3-n28-n79

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 3B NR of DC_1_n3-n28-n79 was approved in RAN#92 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_1_n3-n28-n79

6.X.1	Operating bands for DC

Table 6.X.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | CA_n3-n28-n79 |

6.X.2	Inter-band DC Configurations

Table 6.X.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |

6.X.3	Co-existence studies

Co-existence study for DC_1_n3-n28-n79 was covered by the studies for the fallback modes of DC_1_n3-n28, DC_1_n3-n79 and DC_1_n28-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.X.4	∆TIB and ∆RIB values

For DC_1_n3-n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.8 |
|  | n79 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28-n79 | 1 | 0 |
|  | n3 | 0 |
|  | n28 | 0.5 |
|  | n79 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, there is no need to specify additional MSD requirement for this UL DC configuration..

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-211056  Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL), ZTE