3GPP TSG RAN WG4 Meeting #101-e	R4-2117653

Online, 1st – 12th Nov. 2021

Agenda item:	7.18.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_1-8_n77-n79

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 2B NR of DC_1-8_n77-n79 was approved in RAN#92 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

7.X	DC_1-8_n77-n79

7.X.1	Operating bands for DC

Table 7.X.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.X.2	Channel bandwidths per operating band for DC

Table 7.X.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  |  |  | 8 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |

7.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n77, DC_1-8_n79, DC_1_n77-n79 and DC_8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.X.4	∆TIB and ∆RIB values

For DC_1-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

7.X.5	MSD

As mentioned in 7.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-211043  Revised WID on EN-DC of x bands (x=1,2,3,4) LTE and NR 2 bands, LG Electronics