3GPP TSG-RAN WG4 Meeting # 101-e      		R4-2117736

Electronic Meeting, November 01-12, 2021

Agenda Item:	8.12.2.1.1

Source: 	CMCC

Title: 	Simulation results for inter-cell interference suppressing

Document for:	Discussion

- Introduction

According to the assumption of test parameters and the interference model agreed in previous meetings [1], we provide our simulation results for inter-cell interference suppressing in this contribution.

- Simulation Results

Table 1. Simulation results of MMSE-IRC

| Case 1 | 2T2R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SNR=7.9 |
| --- | --- | --- |
| Case 2 | 2T2R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SNR=2.1 |
| Case 3 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SNR=2.6 |
| Case 4 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SNR=-3 |
| Case 5 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=4.9 |
| Case 6 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=-0.8 |
| Case 7 | 2T2R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=10.6 |
| Case 8 | 2T2R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=5 |

Table 2. Simulation results of MMSE-MRC

| Case 1 | 2T2R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SNR=9.9 |
| --- | --- | --- |
| Case 2 | 2T2R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SNR=3.3 |
| Case 3 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR 3.1 dB in case of 1 interference cell | SNR=5.5 |
| Case 4 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR 3.1 dB in case of 1 interference cell | SNR=-0.8 |
| Case 5 | 2T4R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=8.3 |
| Case 6 | 2T4R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=1 |
| Case 7 | 2T2R ULA low FDD 15kHz 10MHz MCS 13 (16QAM, CR=0.5) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=12.8 |
| Case 8 | 2T2R ULA low FDD 15kHz 10MHz MCS 4 (QPSK, CR=0.3) INR1/2 = 5.43/-1.50 dB in case of 2 interference cell | SNR=7.4 |

Observation 1: Performance gain from MMSE-IRC can be observed for both MCS 4 and MCS 13.

Observation 2: For MCS 4 configuration, the SNR at 70% throughput is lower than INR.

- Conclusion

In this contribution, we provide our simulation results for inter-cell interference suppressing. The observations are:

Observation 1: Performance gain from MMSE-IRC can be observed for both MCS 4 and MCS 13.

Observation 2: For MCS 4 configuration, the SNR at 70% throughput is lower than INR.

- Reference

[1] R4-2115730, Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel Corporation.

- Annex

Table 3. Simulation assumptions

| Network type | FDD sync |
| --- | --- |
| Antenna configuration | 2 Tx as baseline for serving cell and interfering cells UE with 2 and 4 RX ULA low correlation as baseline |
| SCS | FDD 15kHz Same SCS is used for the target and the interference cells |
| Channel bandwidth | FDD: 10MHz |
| TDD DL/UL configuration for 30kHz SCS | 7D1S2U(S=6D+4G+4U) |
| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID i for interferer i (where i ≥ 1, maximum number of i is FFS) |
| PDCCH and PDSCH allocation in time domain | Use symbols #0 and #1 of each slot for PDCCH PDSCH mapping type A, Start symbol 2, Duration 12 |
| PDSCH allocation in frequency domain | Full PRB |
| SSB configuration | All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources |
| TRS/CSI-RS configuration | TRS/CSI-RS colliding with TRS/CSI-RS interference |
| DMRS configuration | For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interferer |
| Propagation condition | TDLA30-10 |
| Transmission rank | rank 1 |
| MCS | MCS 4 (QPSK, CR=0.3) and MCS 13 (16QAM, CR=0.5) |
| Precoding model | Random precoder with Type I SP codebook |
| PRB bundle size | Set PRB bundle size as 2 for target PDSCH |
| HARQ process number | 4 for FDD |
| Performance measurement point | SNR at 70% TP |
| INR values for Homogeneous deployment assumptions initial simulations | Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| Modulation order of interfering PDSCH | Use 16QAM randomly modulated symbols for initial simulation assumption |
| Number of explicitly modeled interference cells | 1 and 2 interference cells |