3GPP TSG-RAN WG4 Meeting #101-e                                R4-2117918

Online Meeting, 1st -12th November, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_5-48_n77

Agenda item:			7.14.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_5-48_n77 according to the request in [1].

2. Reference

- RP-212174 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

- R4-2113051 TR 37.717-21-11 v0.6.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_5-48_n77

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-48A_n77A DC_5A-48C_n77A DC_5A-48D_n77A DC_5A-48A_n77C DC_5A-48C_n77C DC_5A-48D_n77C | DC_5A_n77A |

### 5.x.2	Co-existence studies

DC_48_n77 is fully limited for downlink only, and there is no simultaneous Tx/Rx operation between E-UTRA B48 and NR n77 carriers. Thus there is no additional co-existence study is needed for TDD Band 48.

### 5.x.3	∆TIB and ∆RIB values

For DC_5-48_n77, the ΔTIB,c and RIB,c values are specified in the tables below, reusing DC_5_n77 values.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n77 | 5 | 0.6 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-48_n77 | 5 | 0.2 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, REFSENS exceptions are not expected.

<End of Text Proposal>