3GPP RAN WG4 Meeting #101e		R4-2117951

Electronic meeting, 1st – 12th November 2021

Agenda item:	7.2.3

Source:	Apple

Title:	A-MPR for the 6GHz unlicensed band in Region 2 and Region 3 countries

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, this band is not supported in other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently (refer to the latest regulatory status updated in  [2]). Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band [3].

As discussed during the RAN4#101 meeting, several Region 2 countries, such as Peru and Chile, have the same LPI mode max EIRP and PSD as Brazil, but they do not have out-of-band emission requirements. Thus, a concern was raised whether we can re-use NS_53 for Peru and Chile or whether a new NS flag should be defined [4]. In this discussion paper we present A-MPR simulation results for Peru and Chile, based on which we suggest re-using NS_53 for the aforementioned countries.

# 2	A-MPR values

## 2.1	Simulation setup

All simulations are performed for the LPI mode with the following common operational parameters. In addition to that, country and regional specific parameters are also applied, such as maximum PSD restriction, OOBE restrictions, etc.

-	Power Class: PC5

-	Calibration: QPSK DFT-s-OFDM 100RB3 with 1dB backoff

-	ACLR: 27dB

-	SEM: NR-U

-	EVM: Not used for A-MPR simulations

-	Image rejection: -28dBc

-	Carrier leakage rejection: -28dBc

In all cases the PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum in-band power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the NR-U SEM mask. Simulations are conducted for the full carrier and interlace operation with the signal setup summarized in the Table 2-1 below.

Table 2-1: Signal setup.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 60 | 162RB0 | 30 |
|  | 6 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 8 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 9 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 10 | CP-OFDM | 60 | 162RB0 | 30 |
|  | 11 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 12 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 13 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 14 | DFT-s-OFDM | 60 | 1RB0 every 10RBs (30x) | 30 |
|  | 15 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 16 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 17 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 18 | CP-OFDM | 60 | 1RB0 every 10RBs (33x) | 30 |
|  | 19 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

## 2.2	Peru and Chilie

A-MPR simulation results for the LPI mode in accordance with the Peru and Chile regulatory requirements are presented in Figure 2.2-1 below, where X-axis denotes a particular simulation case index (refer to Table 2-1) and Y-axis indicates the required power back-off.

Figure 2.2-1: A-MPR simulation results (for Peru and Chile LPI).

Based on the presented simulation results, Table 2.2-1 below summarises required A-MPR values for Peru and Chile. The highlighted values are the ones that are smaller when compared to NS_53:

-	NS_53 defines 6.5dB A-MPR for 40MHz full allocation, while for Peru/Chile it can be relaxed to 6dB.

-	NS_53 defines 7dB A-MPR for 60MHz partial allocation with 256QAM, while for Peru/Chile it can be relaxed to 6.5dB

-	NS_53 defines 4dB A-MPR for 80MHz full allocation with 64QAM, while for Peru/Chile it can be relaxed to 3dB.

As can be seen, the difference is around 0.5dB for the 40-60MHz channel and 1dB in one particular case for the 80MHz channel. Since the difference is rather marginal and is only for a limited set of modulations and channels, it will be safe to re-use NS_53 for Peru and Chile without compromising final performance.

Table 2.2-1:  A-MPR values for LPI in Peru and Chile

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

Proposal 1:	Re-use NS_53 for Peru and Chile.

# 3	Conclusions

In this contribution we simulated the A-MPR values for the 6GHz unlicensed band required for the NR-U PC5 LPI operation in Peru and Chile. Based on the presented results, we suggest re-using existing NS_53 for these countries.

Proposal 1:	Re-use NS_53 for Peru and Chile.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- 3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.10.0

- RP-211602, "New WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- R4-2114884, "WF on introduction of the full unlicensed band", Apple, Skyworks Solutions Inc..