3GPP RAN WG4 Meeting #101-e		R4-2117978

Online, November 1st – 12th, 2021

Agenda item:	5.1.6.2.2

Source:	Apple

Title:	FR2 A-MPR requirements for intra-band non-contiguous UL CA

WI/SI:	NR_RF_FR2_req_enh-Core

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

FR2 intra-band non-contiguous UL CA feature has been introduced in Rel-16 specifications [1]. However, the A-MPR requirements for CA_NS_202 and CA_NS_203 has only been defined for intra-band contiguous UL CA but not for intra-band non-contiguous UL CA which in our view is an oversight when the feature was introduced. The concern over A-MPR not being defined would be that it could be misinterpreted as A-MPR not being required for intra-band non-contiguous UL CA which should not be the case. In this contribution, we provide our analysis on the A-MPR requirements for intra-band non-contiguous UL CA for both CA_NS_202 and CA_NS_203.

# Discussion

## 2.1	CA_NS_202

The additional spurious emission for CA_NS_202 follows the same requirements for NS_202 which is recaptured from TS 38.101-2 Table 6.5.3.2.3-1 as below,

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

Table 2.1-1 Additional spurious emission requirements for NS_202

For out-of-band emission, the A-MPR requirement for intra-band non-contiguous UL CA would be determined by the Tx 3rd order inter-modulation (IM3) power which needs to be below -10 dBm/100MHz when it is outside the FOOB and below +1 dBm/200MHz when it is inside the FOOB. The IM3 power can be estimated by the Tx output power and Tx output 3rd order intercept point (OIP3) based on the 2-tone equation,

PIM3 = 3*POUT – 2*OIP3

The Tx OIP3 is very much dominated by the power amplifier 3rd order non-linearity which has been estimated to be around 38 dBm for PC1 and 26 dBm for PC2/PC3/PC4.

Since the A-MPR is defined as the maximum power reduction allowed, it would be dominated by the -10 dBm/200MHz emission requirement. Table 2.1-2 summarizes the estimated A-MPR for CA_NS_202 for intra-band non-contiguous UL CA.

| Power Class | A-MPR (dB) |
| --- | --- |
| PC1 | 11 |
| PC2/PC3/PC4 | 6 |

Table 2.1-2 Estimated A-MPR for CA_NS_202 for intra-band non-contiguous UL CA

## 2.2	CA_NS_203

The additional spurious emission for CA_NS_203 follows the same requirement for NS_203 which is recaptured from TS 38.101-2 Table 6.5.3.2.4-1 as below,

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

Table 2.2-1 Additional spurious emission requirements for NS_203

Since the emission requirement is only specified for the range between 23.6 GHz and 24 GHz, the IM3 from non-contiguous UL allocations may not always overlap with this frequency range. However, when the IM3 overlaps with the specified frequency range, the A-MPR can be derived based on the same approach as for CA_NS_202 and that is summarized in Table 2.2-2.

| Power Class | A-MPR (dB) |
| --- | --- |
| PC1 | 6.5 |
| PC2/PC3/PC4 | 2.5 |

Table 2.2-2 Estimated A-MPR for CA_NS_203 for intra-band non-contiguous UL CA

For the case where the IM3 from the non-contiguous UL carriers does not overlap with the specified frequency range, the UL configuration may still contain contiguous or single carrier sub-block near the n258 lower band edge. In that case, the A-MPR would be determined by the contiguous CA or single carrier A-MPR whichever is higher.

## 2.3	Proposed CR for TS 38.101-2

Based on the above analysis, we have the following proposed CR for TS 38.101-2 to include the A-MPR requirements for intra-band non-contiguous CA where the numbers are temporarily specified in square brackets.

<<< Start of changed sections >>>

#### 6.2A.3.3	A-MPR for CA_NS_202

##### 6.2A.3.3.1	A-MPR for CA_NS_202 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 11.0 dB. For intra-band non-contiguous CA, A-MPR for CA_NS_202 shall be [11.0] dB.

##### 6.2A.3.3.2	A-MPR for CA_NS_202 for power class 2

For intra-band contiguous and non-contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

##### 6.2A.3.3.3	A-MPR for CA_NS_202 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 2.0 dB. For intra-band non-contiguous CA, A-MPR for CA_NS_202 shall be [6.0] dB.

##### 6.2A.3.3.4	A-MPR for CA_NS_202 for power class 4

For intra-band contiguous and non-contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

#### 6.2A.3.4	A-MPR for CA_NS_203

##### 6.2A.3.4.1	A-MPR for CA_NS_203 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 6.5 dB, if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB, otherwise.
The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the lowest CC among the configured UL CA.

For intra-band non-contiguous CA with single component carrier and contiguous component carrier sub-blocks, A-MPR for single carrier and contiguous CA whichever higher applies. In addition, if the 3rd order inter-modulation product from non-contiguous uplink sub-blocks overlaps with 23.6GHz to 24GHz frequency range, then A-MPR shall be [6.5] dB if single carrier and contiguous CA A-MPR is lower.

##### 6.2A.3.4.2	A-MPR for CA_NS_203 for power class 2

For intra-band contiguous and non-contiguous CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

##### 6.2A.3.4.3	A-MPR for CA_NS_203 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 2.5 dB, if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.25 GHz to to the lower edge of the lowest CC among the configured UL CA.

For intra-band non-contiguous CA with single component carrier and contiguous component carrier sub-blocks, A-MPR for single carrier and contiguous CA whichever higher applies. In addition, if the 3rd order inter-modulation product from non-contiguous uplink sub-blocks overlaps with 23.6GHz to 24GHz frequency range, then A-MPR shall be [2.5] dB if single carrier and contiguous CA A-MPR is lower.

##### 6.2A.3.4.4	A-MPR for CA_NS_203 for power class 4

For intra-band contiguous and non-contiguous CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

<<< End of changed sections >>>

# 3	Conclusion

In this contribution, we provide our analysis on the A-MPR requirements for intra-band non-contiguous UL CA for both CA_NS_202 and CA_NS_203 and propose a companion Rel-16 draft CR [2] and Rel-17 mirror draft CR [3] for TS 38.101-2 to include the A-MPR requirements for intra-band non-contiguous CA.

# 4	References

3GPP TS 38.101-2 V16.9.0 (2021-09)

R4-2117979 “Draft CR for TS 38.101-2: FR2 CA_NS_202 and CA_NS_203 A-MPR requirements for intra-band non-contiguous UL CA”, Apple, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2117980 “Draft CR for TS 38.101-2: FR2 CA_NS_202 and CA_NS_203 A-MPR requirements for intra-band non-contiguous UL CA”, Apple, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021