3GPP TSG RAN WG4 #101-e meeting 	R4-2118079

Electronic Meeting, 01st – 12th November, 2021

Agenda Item:	12.2.3

Source:	LG Electronics

Title:	Motivation on the simultaneous Rx/Tx Capability for intra-band non-contiguous CA/DC UE

Document for:	Information

- Introduction

In the last RAN #93-e meetings, RAN discussed the potential RAN4 enhancement WI/SI in Rel-18. In here, some companies request how to achieve the isolation levels in UE side and concerned operated in inter-operator deployed scenarios for simultaneous Rx/Tx capability in intra-band non-contiguous CA/DC UE. So, we provide the motivation on the simultaneous Rx/Tx capability and candidate deployment scenarios in FR1/FR2 and also propose how to achieve the reasonable isolation level in UE side.

Based on e-mail discussion, we summarized the companies’ opinions in previous RAN meeting as follow

- Clarification points on the simultaneous Rx/Tx capability in intra-band NC-CA/DC UE

  - Self-desense problems from Tx interference into own reception in UE (Skyworks, Apple, Qualcomm, Ericsson)

  - Deployment scenarios & Use case (Nokia, Skyworks, Intel)

In this contribution, we provide our view on the how to achieve the reasonable isolation level in Rx band and useful deployments scenarios and use cases for simultaneous Rx/Tx capability in intra-band non-contiguous CA/DC UE.

- How to mitigate the self-interference problem

Generally, RAN4 defined REFSENS requirements with UL configuration. In Rx part the dominant factor to define the REFSENS is the Rx noise level from the PA output.

In NR WI in Rel-15, RAN4 agreed the equation to specify the REFSENS requirements as follow

REFSENS=kTB + SNR +10log10(LCRB*SCS*12/RX_BW) +( NF+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1. For n77, 0.5dB additional relaxation allowed from n78.

-	IM: 2.5 dB is assumed.

-	Target SNR: -1.0 dB

-	Diversity gain: 3dB

Table 1. REFSENS requirements in n77 in 10MHz CBW (SCS 15kHz)

| NR band | n77 band |  |  |
| --- | --- | --- | --- |
|  | Primary RX |  | Diversity Rx |
| C/N requirement (dB) | -1.0 |  | -1.0 |
| Total NF(IM + RF NF=2.2+10.5) (dB) | 12.7 |  | 12.7 |
| Noise floor at Antenna connector(dBm/Hz) | -161.3 |  | -161.3 |
| Total RXBN at Antenna connector (dBm/Hz) | -161.3 |  | -161.3 |
| Sensitivity with RXBN (dBm/10MHz CBW) | -92.3 | (=-161.3+70-1) | -92.3 |
| REFSENS After MRC | -95.3 | *3GPP Ref sense |  |

In the equation, the NF, IM and Diversity gain are common levels regardless of the self-interference problem. Also target SNR is the derived by link budget to guarantee the UE performance.

The required thermal noise level in Rx part is -104 dBm for 10MHz CBW. To achieve the -104dBm/10MHz, we generally assumed -125dBm/Hz at PA &RFIC Rx noise level in FDD band respectively. In FDD band, the duplexer isolation was considered with 50dB, then total noise level -122 dBm/Hz (PA+RFIC) noise level will be assumed as the thermal noise level -122-50 =-172dBm/Hz. Therefore, the REFSENS requirements can be determined with the above equation in FDD band according to CBW.

However, there was no expected the duplexer isolation level such as 50dB isolation in TDD band.

So, if RAN4 only consider current Rx noise level as -122dBm/Hz in TDD band, then the expected the noise level at antenna connector is calculated as -122- 4 (RF FE IL) - 10dB (antenna isolation) = -136dBm/Hz with the current Rx (PA+RFIC) noise level.

Table 2. Expected Desense level in n77 in 10MHz CBW (SCS 15kHz)

| NR band | n77 band |  |  |
| --- | --- | --- | --- |
|  | Primary RX |  | Diversity Rx |
| C/N requirement (dB) | -1.0 |  | -1.0 |
| Total NF(IM + RF NF=2.2+10.5) (dB) | 12.7 |  | 12.7 |
| Noise floor at Antenna connector(dBm/Hz) | -161.3 |  | -161.3 |
| PA & RFIC RX Band Noise level at Antenna connector (dBm/Hz) | -136.0 | (-122dBm/Hz(PA +RFIC RxBN)-FE IL(4dB)-Ant ISO(10dB)) | -136.0 |
| Total RXBN at Antenna connector (dBm/Hz) | -136.0 |  | -136.0 |
| Sensitivity with RXBN (dBm/10MHz CBW) | -67.0 | (=-136+70-1) | -67.0 |
| REFSENS After MRC | -70.0 | *25.3dB De-sense level from -95.3dBm |  |

Therefore, we can expect 25.3dB MSD in n77 TDD band when RAN4 assumed the simultaneous Rx/Tx capability for intra-band NC-CA/DC UE with maximum Tx power.

Observation 1: RAN4 can expect 25.3dB MSD in n77 TDD band when RAN4 consider total noise level with -122dBm (PA + RFIC) without duplexer isolation.

The following figure is the PA noise level characteristics according to the frequency gap in B12.

We can see the common PA noise level is -129dBm/Hz in B12 with 30MHz frequency gap. And also can see the lower PA noise level when the frequency gap is larger than 30MHz in Band 12 in Figure 1.

Figure 1. B12 PA Rx noise level characteristics according to the frequency gap

In Figure 1, we can see the -139dBm/Hz PA noise level when the frequency gap is 153MHz from the Tx frequency band. Also -148dBm/Hz PA noise level is observed when the he frequency gap is 684MHz guaranteed.

We can summarized the noise level according to the frequency gap from the Tx band as in Table 3.

Table 3. PA Rx noise level characteristics in some FDD bands

| B12 PA characteristics |  |  |
| --- | --- | --- |
|  | Freq offset [MHz] | Noise power [dBm/Hz] |
| Ref point (716MHz) | 30M | -129 |
|  | 153M | -139 |
|  | 684M | -148 |
|  | 858M | -152 |
| B5 PA characteristics |  |  |
|  | Freq offset [MHz] | Noise power [dBm/Hz] |
| Ref point (849MHz) | 45M | -131 |
|  | 551M | -151 |
| B8 PA characteristics |  |  |
|  | Freq offset [MHz] | Noise power [dBm/Hz] |
| Ref point (915MHz) | 45M | -132 |
|  | 485M | -151 |
| B13 PA characteristics |  |  |
|  | Freq offset [MHz] | Noise power [dBm/Hz] |
| Ref point (787MHz) | 82M | -131 |
|  | 613M | -151 |
| B42 PA characteristics |  |  |
|  | Freq offset [MHz] | Noise power [dBm/Hz] |
| Ref point (3800MHz) | 700M | -133 |
|  | 1100M | -155 |

Based on the PA noise levels in Table 3, we can observed as

Observation 2: The noise level can be decreased by the frequency gap as shown in Table 3.

Observation 3: The -139dBm/Hz noise level in PA is expected when the frequency gap is acquired with 153MHz from the Tx band. Also when we consider the 500MHz frequency gap, the guaranteed noise level is -150dBm/Hz in some NR operating bands.

From the Table 3, we can derive the expected de-sense level with -139dBm noise level as shown in Table 4.

Table 4. Expected De-sense level with the 153MHz frequency gap in 10MHz CBW (SCS 15kHz)

| NR band | n77 band |  |  |
| --- | --- | --- | --- |
|  | Primary RX |  | Diversity Rx |
| C/N requirement (dB) | -1.0 |  | -1.0 |
| Total NF(IM + RF NF=2.2+10.5) (dB) | 12.7 |  | 12.7 |
| Noise floor at Antenna connector(dBm/Hz) | -161.3 |  | -161.3 |
| PA & RFIC RX Band Noise level at Antenna connector (dBm/Hz) | -150.0 | (-139+3) dBm/Hz(PA +RFIC RxBN)-FE IL(4dB)-Ant ISO(10dB)) | -150.0 |
| Total RXBN at Antenna connector (dBm/Hz) | -149.7 |  | -149.7 |
| Sensitivity with RXBN (dBm/10MHz CBW) | -80.7 | (=-149.7+70-1) | -80.7 |
| REFSENS After MRC | -83.7 | *11.6dB De-sense level from -95.3dBm |  |

Based on the Table 4, we can expected the reasonable MSD level when we assumed Tx/Rx frequency gap with 153MHz in Band 12.

In Table 5, we derive the expected de-sense level with 500MHz frequency gap in 10MHz CBW

Table 5. Expected De-sense level with the 500MHz frequency gap in 10MHz CBW (SCS 15kHz)

| NR band | n77 band |  |  |
| --- | --- | --- | --- |
|  | Primary RX |  | Diversity Rx |
| C/N requirement (dB) | -1.0 |  | -1.0 |
| Total NF(IM + RF NF=2.2+10.5) (dB) | 12.7 |  | 12.7 |
| Noise floor at Antenna connector(dBm/Hz) | -161.3 |  | -161.3 |
| PA & RFIC RX Band Noise level at Antenna connector (dBm/Hz) | -161.0 | (-150+3) dBm/Hz(PA +RFIC RxBN)-FE IL(4dB)-Ant ISO(10dB)) | -161.0 |
| Total RXBN at Antenna connector (dBm/Hz) | -158.1 |  | -158.1 |
| Sensitivity with RXBN (dBm/10MHz CBW) | -89.1 | (=-158.1+70-1) | -89.1 |
| REFSENS After MRC | -92.1 | *3.2dB De-sense level from -95.3dBm |  |

Observation 4: Based on the above Table 4 and 5, we can achieve 11.6dB MSD with 153MHz frequency gap and 3.2dB MSD with 500MHz frequency gap when simultaneous Rx/Tx capability is allowed in a NR operating band.

Therefore, we think that RAN4 need feasibility study whether to allow the simultaneous Rx/Tx capability in the intra-band NC-CA/DC UE and the detail MPR value and MSD level can specify to support simultaneous Rx/Tx capability.

Based on these observation, we proposed as

Proposal 1: RAN4 need to study for the self-interference problem when the simultaneous Rx/Tx capability is allowed for intra-band NC-CA/DC UE considering with the existing frequency gap between aggressor carrier and victim carrier.

Other solution can be further discussed in SI/WI phase to mitigate the desense level in victim carrier. Basically the uplink RB restriction and A-MPR approach or combination of these candidate solution should be discussed in SI/WI phase to allow the simultaneous Rx/Tx capability.

Specially, A-MPR value will be applied on top of the frequency gap to optimize the system performance.

Observation 5: Combination of MSD and A-MPR approach can be optimised the system performance for the simultaneous Rx/Tx capability intra-band NC-CA/DC UE.

Proposal 2: The frequency gap, UL RB restriction for REFSENS and A-MPR approach or combination of the candidate solutions will study to achieve the reasonable MSD values and cell coverage to keep the system performance.

- Candidate deployment scenarios & Use case

In previous discussion, some companies request to clarify the candidate deployment scenarios & use cases.

In this session, we provide detail deployment scenarios and use case.

- Deployment scenarios

  - Intra-band NC-CA/DC UE with wide frequency gap (in Figure 2)

    - Focus on the single operator usage with the existing n77 band + new candidate band in n77

      - Operator A + new band (intra-band NC-CA)

      - Operator B + new band (intra-band NC-CA)

      - Operator C + new band (only consider the intra-band NC-CA)

    - For the inter operator deployment scenarios, RAN4 only allow simultaneous Rx/Tx capability in the specific scenarios such as the Indoor-to-Macro scenarios and Indoor-to-Indoor scenarios. The Macro-to-Marco scenarios will be precluded due to cross link interference problem.

  - Intra-band con-current V2 UE (SL + NR Uu in different carriers) with wide frequency gap (in Figure 3)

    - CC1 (SL operation) + CC2 (NR Uu operation): Synchronous operation is baseline

    - If NR Uu allow the simultaneous Rx/Tx capability, then NR SL also can be consider the simultaneous Rx/Tx capability in intra-band

Figure 2. Deployment scenarios for intra-band NC-CA/DC UE operation

Figure 3. Use cases for intra-band con-current V2X operation within non-adjacent carrier

The operating scenario in Figure 2 is extended from the simultaneous Rx/Tx capability for inter-band CA/DC UE. This operation is target the CA/DC without any cancellation schemes in UE side.

In Figure 3, this operating scenarios are motivated from Rel-17 SL enh. WI where Uu+PC5 SL can be operated concurrently in an intra-band con-current operating band and thus simultaneous Tx/Rx can happen under this scenario. In SL enh. WI session, the interested companies are concluded that the simultaneous Rx/Tx capability scenario will be discussed in the general NR Uu session first. Then, SL can be considered the conclusion in NR Uu operation.

Based on the self-interference analysis in session 2, RAN4 can achieve the reasonable MSD level with frequency gap, and the uplink RB restriction and A-MPR approach are also considered to mitigate the self desense problems. Hence RAN4 needs the feasibility study whether allowing the simultaneous Rx/Tx capability or not in Rel-18.

From the detail deployment scenarios and use case, we propose as follow

Proposal 3: Based on the candidate deployment scenarios in Figure 2 and Figure 3, RAN4 needs the feasibility study whether allowing the simultaneous Rx/Tx capability UE in intra-band NC-CA/DC UE.

- Conclusions

In this paper, we provide our view on the simultaneous Rx/Tx capability for intra-band NC-CA/DC UE with wide frequency gap between CC#1 and CC#2. Based on the candidate deployment scenarios in session 3, RAN4 can study the feasibility on the simultaneous Rx/Tx capability.

Observation 1: RAN4 can expect 25.3dB MSD in n77 TDD band when RAN4 consider total noise level with -122dBm (PA + RFIC) without duplexer isolation.

Observation 2: The noise level can be decreased by the frequency gap as shown in Table 3.

Observation 3: The -139dBm/Hz noise level in PA is expected when the frequency gap is acquired with 153MHz from the Tx band. Also when we consider the 500MHz frequency gap, the guaranteed noise level is -150dBm/Hz in some NR operating bands.

Observation 4: Based on the above Table 4 and 5, we can achieve 11.6dB MSD with 153MHz frequency gap and 3.2dB MSD with 500MHz frequency gap when simultaneous Rx/Tx capability is allowed in a NR operating band.

Observation 5: Combination of MSD and A-MPR approach can be optimised the system performance for the simultaneous Rx/Tx capability intra-band NC-CA/DC UE.

Based on the above observations, we propose as following

Proposal 1: RAN4 need to study for the self-interference problem when the simultaneous Rx/Tx capability is allowed for intra-band NC-CA/DC UE considering with the existing frequency gap between aggressor carrier and victim carrier.

Proposal 2: The frequency gap, UL RB restriction for REFSENS and A-MPR approach or combination of the candidate solutions will study to achieve the reasonable MSD values and cell coverage to keep the system performance.

Proposal 3: Based on the candidate deployment scenarios in Figure 2 and Figure 3, RAN4 needs the feasibility study whether allowing the simultaneous Rx/Tx capability UE in intra-band NC-CA/DC UE.

Reference

[1] RAN Chair, RWS-210659, Summary of RAN Rel-18 Workshop, https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_AHs/2021_06_RAN_Rel18_WS/Docs

[2]	[RAN93e-R18Prep-17] Potential RAN4 enhancements, https://nwm-trial.etsi.org/#/documents/6540

[3]	TS38.101-1

[4]	TS38.101-3