3GPP TSG-RAN4 Meeting #101-e	R4-2118082

# Electronic Meeting, 1st Nov– 12th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2222 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SRVCC TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-Perf |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Event B1 is used in TC A.6.6.5.1, However, b2-threshold is also configured in this TC. It may cause confusion. - To correct typos. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 6.3.1.6: - typos are corrected; - RF channels for UTRA Cell is added in Table A.6.3.1.6-4. For 6.5.5.1: - Threshold for event B2 is removed. - RF channel for UTRA Cell is corrected. - Typos are corrected. - NR measurement quantity is removed since event B1 doesn't need NR intra-frequency measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.6, A.6.6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.6.3.1.6	 SA NR - UTRAN FDD handover

A.6.3.1.6.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT UTRAN FDD handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to UTRAN FDD handover requirements as specified in clause 6.1.2.2.1.

The test comprises of one NR carrier and one UTRA FDD carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.6-1. General test parameters are provided in Table A.6.3.1.6-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.6-3 and A.6.3.1.6-4 respectively.

Table A.6.3.1.6-1: Supported test configurations for SA inter-RAT UTRAN FDD handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.6-2: General test parameters for SA inter-RAT UTRAN FDD handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  |  | 2 | 1 UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| Inter-RAT (UTRAN FDD) measurement quantity |  |  | CPICH Ec/N0 |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.6-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2UTRA-FDD |  | dB | -18 | Absolute UTRAN CPICH Ec/Io threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.6-3: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3 | 1 |  |  |
| Duplex mode |  |  | 1 | FDD |  |  |
|  |  |  | 2, 3 | TDD |  |  |
| TDD Configuration |  |  | 2 | TDDConf.1.1 |  |  |
|  |  |  | 3 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1 | SR.1.1 FDD |  |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1 | CR.1.1 FDD |  |  |
|  |  |  | 2 | CR.1.1 TDD |  |  |
|  |  |  | 3 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
|  |  |  | 3 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
|  |  |  | 3 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3 | -100 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, | -100 |  |  |
|  |  |  | 3 | -97 |  |  |
| Ês/Noc |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
|  |  |  | 3 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -70.59 | -70.59 |
|  |  | dBm/38.16 MHz | 3 | -53.68 | -64.49 | -64.49 |
| Propagation condition |  |  | 1, 2, 3 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.6-4: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| UTRA RF Channel Number |  | 2 |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| DCH_Ec/Ior | dB | N/A | N/A | Note 1 |
| OCNS_Ec/Ior | dB | -0.941 | 0.941 | Note 2 |
|  | dB | -infinity | -1.8 | -1.8 |
|  | dBm/3,84 MHz | -70 | -70 | -70 |
| CPICH_Ec/Io | dB | -infinity | -14 | -14 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |  |

A.6.3.1.6.2	Test Requirements

The UE shall start to transmit the UL DPCCH to Cell 2 less than 190 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms, which is specified in clause 5.3.1.1.1.

Tinterrupt = 140 ms in the test; Tinterrupt is defined in clause 5.3.1.1.2. 	This gives a total of 190 ms.

<End of modified section 1>

<Start of modified section 2>

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1

A.6.6.5.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN FDD measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clause 9.4.6.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.5.1.1-1. General test parameters are provided in Table A.6.6.5.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4, respectively.

Table A.6.6.5.1.1-1: Supported test configurations in SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRA FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRA FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRA FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.5.1.1-2: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

Table A.6.6.5.1.1-3: PCell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.5.1.1-4: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

A.6.6.5.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section 2>