3GPP TSG-RAN WG4 Meeting #101-e	R4-2118166

# Electronic meeting, November 1st – 12th, 2021

Source:		Ericsson

Title:			RMR 900MHz band – TP to TR 38.852

Agenda item:		7.4.4

Document for:		Approval

# Introduction

The 1900MHz NR band for European RMR WI ([1]) has been revised in RAN#92e meeting to introduce a new 1900MHz band for Railway in Europe.

In last RAN4#100-e meeting, the TR skeleton was approved. This contribution is a text proposal to TR focusing on the Regulatory aspects.

# Text proposal

<Start of changes>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

<End of changes>

<Start of changes>

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Advanced Antenna Systems

CEPT	Conférence Européenne des administrations des Postes et Télécommunications

EC	European Commission

ECC	Electronic Communications Committee

EIRP	Effective Isotropic Radiated Power

RMR	Rail Mobile Radio

<End of changes>

<Start of changes>

# 4	Regulatory background

## 4.1	EC Decision 2021/1703

On September 28th 2021, the European Commission Decision 2021/1703 ([2]) establishes the harmonised conditions for the availability and efficient use of radio spectrum for the Railway Mobile Radio (RMR) in the band 1900 – 1910 MHz. By 1 January 2022, Member States shall designate and make available on a non-exclusive basis the unpaired frequency band 1900 – 1910 MHz for Railway Mobile Radio, in accordance with the technical conditions listed in subclause 4.3

## 4.2	ECC Decision(20)02

The ECC Decision(20)02 addresses the designation of the unpaired frequency band 1900-1910 MHz to be used for Railway Mobile Radio (RMR) on a CEPT wide basis.

By this Decision, the CEPT administrations shall designate the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) on a non-exclusive basis, according to the technical conditions listed in subclause 4.3.

This Decision entered into force on November 20th, 2020.

## 4.3	Technical conditions

### 4.3.1	Base stations using wideband technologies

Only non AAS Base Stations are considered.

Table 9: General in-block requirement mandatory for uncoordinated deployment

| RMR channel BW | Maximum EIRP |
| --- | --- |
| 10 MHz | 65 dBm/10 MHz (Note 1) |
| Note 1: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Table 10: Baseline requirement

| Frequency Range | EIRP limit |
| --- | --- |
| 1920 - 1980 MHz | -43 dBm/5 MHz |

Table 11: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805 - 1880 MHz | -20 dBm |
| The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. |  |

### 4.3.2	Terminals other than cab-radios using wideband technologies

| Maximum output power | 23 dBm |
| --- | --- |
| ACLR | 30dB minimum |
| Other | Uplink power control is mandatory and shall be activated. |

<End of changes>

<Start of changes>

# 5	Frequency band arrangement

The new RMR band is within the range of FR1 and is proposed as a FDD band (Table 5-1).

Table 5-1: New NR RMR band in FR1

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| [RMR_1900] | 1900 – 1910 MHz | 1900 – 1910 MHz | TDD |

<End of changes>