3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118210

# Electronic meeting,  1st - 12th Nov, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0954 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R17_3BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Completed inter-band CA combinations for 3DL with 2 bands UL are introduced into TS 38.101-1 from RAN4 #101-e meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following approved contributions of inter-band CA for 3 bands DL with 2 bands UL are added from RAN4 #101-e. - R4-2117158 DraftCR on introducing NR inter-band CA for 3DL Bands and 2UL band combination for 38.101-1, Verizon Denmark - R4-2117906 DraftCR for 38.101-1 to add additional combinations for CA_n5-n66-n78, Huawei, HiSilicon, Bell Mobility, Telus - R4-2117907 DraftCR for 38.101-1: Correction on CA_n5A-n25A-n77A, Huawei, HiSilicon, Bell Mobility, Telus - R4-2118505 draft CR 38.101-1 to correct 3DL/2UL band combinations, Ericsson - R4-2118508 draft CR 38.101-1 to add dual UL to CA_n25-n66-n77, Ericsson, Bell Mobility - R4-2118509 draft CR 38.101-1 to add dual UL to CA_n7-n25-n78, Ericsson, Bell Mobility - R4-2118510 draft CR 38.101-1 to add dual UL to CA_n7-n66-n77, Ericsson, Bell Mobility - R4-2118511 draft CR 38.101-1 to add CA_n25A-n41A-n77(2A), Ericsson, Bell Mobility - R4-2118517 draft CR 38.101-1 to add BCS and configuration for CA_n25-n41-n77, Ericsson, T-Mobile US - R4-2118521 draft CR 38.101-1 to add new BCSs and new configurations for CA_n41-n66-n77, Ericsson, T-Mobile US - R4-2118637 draftCR to add CA_n25A-n66A-n77(2A) to 38.101-1, Nokia, Bell - R4-2118646 draftCR to add missing UL configurations for 3 band CA to 38.101-1, Nokia, DISH Network - R4-2118647 draftCR 38.101-1 Addition of CA_n2-n5-n77, Nokia, AT&T - R4-2118648 draftCR 38.101-1 Addition of CA_n2-n12-n77, Nokia, AT&T - R4-2118649 draftCR 38.101-1 Addition of CA_n2-n14-n77, Nokia, AT&T - R4-2118650 draftCR 38.101-1 Addition of CA_n2-n30-n77, Nokia, AT&T - R4-2118651 draftCR 38.101-1 Addition of CA_n2-n66-n77, Nokia, AT&T - R4-2118652 draftCR 38.101-1 Addition of CA_n5-n12-n77, Nokia, AT&T - R4-2118653 draftCR 38.101-1 Addition of CA_n5-n14-n77, Nokia, AT&T - R4-2118654 draftCR 38.101-1 Addition of CA_n5-n30-n77, Nokia, AT&T - R4-2118655 draftCR 38.101-1 Addition of CA_n5-n66-n77, Nokia, AT&T - R4-2118656 draftCR 38.101-1 Addition of CA_n12-n30-n77, Nokia, AT&T - R4-2118657 draftCR 38.101-1 Addition of CA_n12-n66-n77, Nokia, AT&T - R4-2118658 draftCR 38.101-1 Addition of CA_n14-n30-n77, Nokia, AT&T - R4-2118659 draftCR 38.101-1 Addition of CA_n14-n66-n77, Nokia, AT&T - R4-2118660 draftCR 38.101-1 Addition of CA_n30-n66-n77, Nokia, AT&T - R4-2119760 TP for TR 38.717-03-01: CA_n24-n41-n77 combinations, Ligado Networks - R4-2119761 TP for TR 38.717-03-02: CA_n24-n41-n48 combinations, Ligado Networks - R4-2119762 TP update for TR 38.717-03-02: CA_n3-n77-n79, SoftBank Corp., LG Electronics - R4-2117904 TP for TR 38.717-03-02: CA_n30A-n66A-n77(2A), Huawei, HiSilicon, Bell Mobility, Telus - R4-2117924 TP for TR 38.717-03-02 CA_n1A-n3A-n77A, Samsung, KDDI - R4-2119765 TP for TR38.717-03-02_CA_n28A-n40A-n79A, ZTE Corporation - R4-2118543 TP to TR 38.717-03-02 for CA_n28-n46-n78 and DC_n28-n46-n78, Huawei,HiSilicon, BT - R4-2119752 TP to TR 38.717-03-02 Addition of CA_n1A-n40-n78A, Nokia - R4-2119753 TP to TR 38.717-03-02 Addition of CA_n28A-n40-n78A, Nokia - R4-2119757 TP to TR 38.717-03-02 Addition of CA_n7-n25-n77, Nokia, Bell - R4-2118669 TP to TR 38.717-03-01 Addition of CA_n29-n66-n77, Nokia, AT&T - R4-2118670 TP to TR 38.717-03-02 Addition of CA_n2-n29-n77, Nokia, AT&T - R4-2118671 TP to TR 38.717-03-02 Addition of CA_n5-n29-n77, Nokia, AT&T - R4-2118672 TP to TR 38.717-03-02 Addition of CA_n29-n30-n77, Nokia, AT&T - R4-2118673 TP to TR 38.717-03-02 Addition of CA_n29-n66-n77, Nokia, AT&T |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for above band combinations are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3.2, 7.3A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration |  | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n5A |  | CA_n1A-n3A CA_n1A-n5A CA_n3A-n5A | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n7A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n1A-n3A-n7B |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n1A-n3A CA_n1A-n7A CA_n3A-n7A CA_n7B | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n8A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n20A |  | - | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n28A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 2 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 201 |  |  | 301 |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n41A |  | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n3A-n77A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | CA_n1A-n3A CA_n1A-n77A CA_n3A-n77A |  | n1 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n3A-n77(2A) |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n78A |  | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n3A-n78(2A) |  | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n5A-n7A |  | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n1A-n5A-n7B |  | CA_n1A-n5A CA_n1A-n7A CA_n5A-n7A CA_n7B | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n5A-n78A |  | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| CA_n1A-n8A-n77A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n8A-n77(2A) |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7A-n28A |  | CA_n1A-n7A CA_n1A-n28A CA_n7A-n28A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7B-n28A |  | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A CA_n7B | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n7A-n78A |  | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 901 | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 901 | 100 |  |
| CA_n1A-n7B-n78A |  | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A CA_n7B | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| CA_n1A-n7A-n78(2A) |  | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n78A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n8A-n78(2A) |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n8A-n79A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n1A-n20A-n78A |  | - | n1 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n28A-n40A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
| CA_n1A-n28A-n40B |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n40 | See CA_n40B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n28A-n78A |  | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n28A-n78(2A) |  | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n40A-n78A |  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | CA_n1A-n40A CA_n1A-n78A CA_n40A-n78A |  | n1 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n40 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 80 |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 | 80 | 90 | 100 |  |  |
| CA_n1A-n40B-n78A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n40 | See CA_n40B Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n1A-n77A-n79A4 |  | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n1A-n78A-n79A5 |  | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n1A-n78(2A)-n79A |  | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n2A-n5A-n30A |  | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A-n48A |  | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n5A-n48B |  | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A-n48(2A) |  | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n5A-n30A |  | CA_n2A-n5A CA_n2A-n30A CA_n5A-n30A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A-n66A |  | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2(2A)-n5A-n66A |  | CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n5A-n66(2A) |  | CA_n2A-n5A CA_n2A-n66A CA_n5A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A-n77A |  | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n5A-n77C |  |  | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n5A-n77(2A) |  |  | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n5A-n77A |  | CA_n2A-n5A CA_n2A-n77A CA_n5A-n77A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n12A-n77A |  | CA_n2A-n12A CA_n2A-n77A CA_n12A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2(2A)-n12A-n77A |  | CA_n2A-n12A CA_n2A-n77A CA_n12A-n77A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n12A-n77(2A) |  |  | CA_n2A-n12A CA_n2A-n77A CA_n12A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n14A-n30A |  | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n14A-n30A |  | CA_n2A-n14A CA_n2A-n30A CA_n14A-n30A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n14A-n66A |  | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2(2A)-n14A-n66A |  | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n14A-n66(2A) |  | CA_n2A-n14A CA_n2A-n66A CA_n14A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n14A-n77A |  | CA_n2A-n14A CA_n2A-n77A CA_n14A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n14A-n77(2A) |  |  | CA_n2A-n14A CA_n2A-n77A CA_n14A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n14A-n77A |  | CA_n2A-n14A CA_n2A-n77A CA_n14A-n77A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n29A-n77A |  |  | CA_n2-n77 |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2(2A)-n29A-n77A |  | CA_n2-n77 | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n29A-n77(2A) |  |  | CA_n2-n77 |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n30A-n66A |  | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
| CA_n2(2A)-n30A-n66A |  | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
| CA_n2A-n30A-n66(2A) |  | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n30A-n77A |  | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n30A-n77(2A) |  |  | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n30A-n77A |  | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n48A-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n48B-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n48(2A)-n66A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n2A-n48A-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48B-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n48(2A)-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n66A-n77A |  | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2(2A)-n66A-n77A |  | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n2A-n66(2A)-n77A |  |  | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n66A-n77C |  |  | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n2 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2A-n66A-n77(2A) |  | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n5A-n7A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
| CA_n3A-n5A-n7B |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n3A-n5A CA_n3A-n7A CA_n5A-n7A CA_n7B | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n5A-n78A |  | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n7A-n28A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n3A-n7A CA_n7A-n28A CA_n7A-n28A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 2 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7B-n28A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n7 |  | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A CA_n7B |  | n3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n7A-n78A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n7B-n78A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n8A-n77A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n8A-n77(2A) |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n78A |  | CA_n3A-n8A CA_3A-n78A CA_n8A-n78A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n18A-n41A |  | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n20A-n78A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n28A-n41A |  | CA_n3A-n28A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n28A-n77A |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n28A-n77(2A) |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | CA_n77(2A) | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A |  | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 2 |
|  |  |  | n28 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n28A-n78(2A) |  | CA_n3A-n78A CA_n28A-n78A | n3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 202 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n79A |  | CA_n3A-n28A CA_n3A-n79A CA_n28A-n79A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  |  |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n3A-n77A-n79A4 |  |  | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A |  | n3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n3A-n77(2A)-n79A4 |  |  | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A |  | n3 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n3A-n40A-n41A |  | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n41A-n77A |  | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | CA_n3A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n77A | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n41A-n77(2A) |  | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | CA_n3A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n77A | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n41A-n78A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | CA_n3A-n78A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n78A | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n41A-n78(2A) |  | CA_n3A-n41A | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | CA_n3A-n78A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n78A | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n41A-n79A |  | - | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n3 | 5 | 10 | 15 | 20 | 25 |  | 30 |  |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n5A-n7A-n78A |  | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| CA_n5A-n7B-n78A |  | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | CA_n5A-n7A CA_n5A-n78A CA_n7A-n78A CA_n7B | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| CA_n5A-n12A-n77A |  | CA_n5A-n12A CA_n5A-n77A CA_n12A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n12A-n77(2A) |  |  | CA_n5A-n12A CA_n5A-n77A CA_n12A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n14A-n77A |  | CA_n5A-n14A CA_n5A-n77A CA_n14A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n14A-n77(2A) |  |  | CA_n5A-n14A CA_n5A-n77A CA_n14A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n66A |  | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66A |  | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n25A-n66(2A) |  | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A) |  | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n77A |  | CA_n5A-n25A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | CA_n5A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | CA_n25A-n77A | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n78A |  | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25(2A)-n78A |  | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n78(2A) |  | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n78(2A) |  | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n29A-n77A |  |  | CA_n5A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n29A-n77(2A) |  |  | CA_n5A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n30A-n66A |  | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
| CA_n5A-n30A-n66(2A) |  | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n30A-n77A |  | CA_n5A-n30A CA_n5A-n77A CA_n30A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n30A-n77(2A) |  |  | CA_n5A-n30A CA_n5A-n77A CA_n30A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48A-n66A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n48B-n66A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n48(2A)-n66A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n5A-n48A-n77A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n48B-n77A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n48(2A)-n77A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n66A-n77A |  | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n66(2A)-n77A |  |  | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n66A-n77C |  |  | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n77(2A) |  | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66A-n78A |  | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n66(2A)-n78A |  |  | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n5A-n66A-n78(2A) |  |  | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n66(2A)-n78(2A) |  |  | CA_n5A-n66A CA_n5A-n78A CA_n66A-n78A |  | n5 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n66A |  | CA_n7A-n25A CA_n7A-n66A CA_n25A-n66A | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n77A |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7A-n25(2A)-n77A |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7A-n25A-n77(2A) |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25(2A)-n77(2A) |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25A-n77A |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7(2A)-n25(2A)-n77A |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7(2A)-n25A-n77(2A) |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n25(2A)-n77(2A) |  |  | CA_n7A-n25A CA_n7A_n77A CA_n25A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n25A-n78A |  |  | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 704 |  | 80 |  | 904 |  | 100 |  |  |  |
| CA_n7A-n25A-n78(2A) |  |  | CA_n7A-n25A CA_n7A-n78A CA_n25A-n78A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n28A-n78A |  |  | - |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| CA_n7B-n28A-n78A |  | - | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A CA_n7B | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 704 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7A-n66A-n77A |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7A-n66(2A)-n77A |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7A-n66A-n77(2A) |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n66(2A)-n77(2A) |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n7 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66A-n77A |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7(2A)-n66(2A)-n77A |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n7(2A)-n66A-n77(2A) |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A)-n77(2A) |  |  | CA_n7A-n66A CA_n7A-n77A CA_n66A-n77A |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n7 |  | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7A-n66A-n78A |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n66A-n78(2A) |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66A-n78A |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n66(2A)-n78A |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n66(2A)-n78A |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n66A-n78(2A) |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A)-n78(2A) |  | CA_n7A-n66A CA_n7A-n78A CA_n66A-n78A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n28A-n78A |  | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n8A-n39A-n41A |  | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  |  |  |  |  |
| CA_n8A-n40A-n41A |  | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n8A-n41A-n79A |  | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n8A-n78A-n79A |  | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n8A-n78(2A)-n79A |  | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n12A-n30A-n77A |  |  | CA_n12A-n30A, CA_n12A- n77A, CA_n30A-n77A |  | n12 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n12A-n30A-n77(2A) |  |  | CA_n12A-n30A, CA_n12A- n77A, CA_n30A-n77A |  | n12 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n12A-n66A-n77A |  |  | CA_n12A-n66A, CA_n12A-n77A, CA_n66A-n77A |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n12A-n66(2A)-n77A |  |  | CA_n12A-n66A, CA_n12A-n77A, CA_n66A-n77A |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n12A-n66A-n77(2A) |  |  | CA_n12A-n66A, CA_n12A-n77A, CA_n66A-n77A |  | n12 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n13A-n25A-n66A |  | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n13A-n25A-n77A |  | - | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n66A-n77A |  | - | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n14A-n30A-n66A |  | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n14A-n30A-n66(2A) |  | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n14A-n30A-n77A |  |  | CA_n14A-n30A, CA_n14A-n77A, CA_n30A-n77A |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n14A-n30A-n77(2A) |  |  | CA_n14A-n30A, CA_n14A-n77A, CA_n30A-n77A |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n14A-n66A-n77A |  |  | CA_n14A-n66A, CA_n14A-n77A, CA_n66A-n77A |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n14A-n66(2A)-n77A |  |  | CA_n14A-n66A, CA_n14A-n77A, CA_n66A-n77A |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n14A-n66A-n77(2A) |  |  | CA_n14A-n66A, CA_n14A-n77A, CA_n66A-n77A |  | n14 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n20A-n28A-n78A |  | - | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n24A-n41A-n48A |  |  | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n48 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n24A-n41(2A)-n48A |  |  | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  | See CA_n41(2A) BCS1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n48 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n24A-n41A-n48(2A) |  |  | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n48 |  | See CA_n48(2A) BCS0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41(2A)-n48(2A) |  |  | CA_n24A-n41A CA_n24A_n48A CA_n41_n48A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  | See CA_n41(2A) BCS1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n48 |  | See CA_n48(2A) BCS0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41A-n77A |  | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n24A-n41(2A)-n77A |  |  | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n41 | See CA_n41(2A) BCS1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n24A-n41A-n77(2A) |  |  | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41(2A)-n77(2A) |  |  | CA_n24A-n41A CA_n24A_n77A CA_n41_n77A |  | n24 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  | See CA_n41(2A) BCS1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) BCS0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41(2A)-n77A |  | - | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n24A-n41A-n77(2A) |  | - | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n41(2A)-n77(2A) |  | - | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n29A-n66A |  | CA_n25A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n38A-n66A |  | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25(2A)-n38A-n66A |  | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n38A-n66(2A) |  | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n38A-n78A |  | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n38A-n78(2A) |  | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n38A-n78A |  | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25(2A)-n38A-n78(2A) |  | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n66A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n41C-n66A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 | See CA_n41C Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n66A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | CA_n25A-n71A |  | n71 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71B |  |  | - |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n71 |  | See CA_n71B bandwidth combination set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n71(2A) |  |  | - |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n71A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 | See CA_n41(2A) bandwidth combination set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n71A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n71 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n41A-n71A |  | - | n25 | See CA_n25(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41A-n77A |  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41(2A)-n77A |  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n25A-n41C-n77A |  | CA_n41C CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n41 |  | See CA_n41C Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n25A-n41A-n77(2A) |  |  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n41A-n77A |  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | See CA_n25(2A) bandwidth combination set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41A-n78A |  | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41A-n78(2A) |  | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48A-n66A |  | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n48 | 5 | 10 | 15 | 20 |  |  |  | 40 | 50 |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n48 | 5 | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n48(2A)-n66A |  | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n48C-n66A |  | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
| CA_n25A-n66A-n71A |  | - | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66(2A)-n71A |  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A-n77A |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n25A-n66(2A)-n77A |  |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n25A-n66A-n77(2A) |  |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66(2A)-n77(2A) |  |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  | n25 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66A-n77A |  |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A |  | n25 |  | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n25(2A)-n66(2A)-n77A |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25(2A)-n66A-n77(2A) |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66(2A)-n77(2A) |  | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66A-n78A |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25(2A)-n66A-n78A |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n66(2A)-n78A |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n66A-n78(2A) |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66(2A)-n78A |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25(2A)-n66A-n78(2A) |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n66(2A)-n78(2A) |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25(2A)-n66(2A)-n78(2A) |  | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n71A-n77A |  | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n71A-n78A |  | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n71A-n78(2A) |  | - CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66A-n70A |  | CA_n26A-n66A CA_n26A-n70A | n26 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n26A-n66(2A)-n70A |  | CA_n26A-n66A CA_n26A-n70A | n26 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n40A-n78A |  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  | n40 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A |  | n28 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  |  |  |  |  | n40 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n40A-n79A |  |  | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A |  | n28 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n40 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n28A-n41A-n77A |  | CA_n28A-n41A | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | CA_n28A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n77A | n77 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n41A-n77(2A) |  | CA_n28A-n41A | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | CA_n28A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | CA_n41A-n77A | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n41A-n78A |  | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  |  | 90 | 100 |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n28A-n41A-n78(2A) |  | CA_n78(2A) | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n41A-n79A |  | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5 | 10 | 15 | 20 |  |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n28A-n46A-n78A |  |  | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  | n28 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n46 |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  | 40 |  |  |  | 60 |  |  |  | 80 |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n28A-n46C-n78A |  |  | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  | n28 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n28A-n46D-n78A |  |  | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A |  | n28 |  | 5 |  | 10 |  | 15 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n28A-n77A-n79A4 |  | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n77 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n28A-n77(2A)-n79A4 |  | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n28A-n78A-n79A |  | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  | 60 |  |  |  | 80 |  |  |  | 100 |  |  |  |
| CA_n29A-n30A-n77A |  |  | CA_n30A-n77A |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n29A-n30A-n77(2A) |  |  | CA_n30A-n77A |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66A-n70A |  | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66B-n70A |  | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66(2A)-n70A |  | - | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n29A-n66A-n77A |  |  | CA_n66A-n77A |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n29A-n66(2A)-n77A |  |  | CA_n66A-n77A |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n29A-n66A-n77(2A) |  |  | CA_n66A-n77A |  | n29 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n30A-n66A-n77A |  | CA_n30A-n66A CA_n30A-n77A CA_n66A-n77A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n30A-n66(2A)-n77A |  |  | CA_n30A-n66A CA_n30A-n77A CA_n66A-n77A |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n30A-n66A-n77(2A) |  |  | CA_n30A-n66A CA_n30A-n77A CA_n66A-n77A |  | n30 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n38A-n66A-n78A |  | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n38A-n66A-n78(2A) |  | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n38A-n66(2A)-n78A |  | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n38A-n66(2A)-n78(2A) |  | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n39A-n40A-n41A |  | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n39A-n40A-n79A |  | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n39A-n41A-n79A |  | - | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n39 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n40A-n41A-n79A |  | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 |  | 80 |  |  | 0 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
|  |  |  | n40 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  |  | n41 |  | 10 | 15 | 20 |  |  |  | 40 | 50 | 60 |  |  |  |  |  |
|  |  |  | n79 |  |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n41A-n66A-n71A |  | - | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n71A |  | CA_n41A-n66A CA_n66A-n71A CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A-n71A |  | - | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n66A-n71A |  | - | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n77A |  | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
|  |  |  |  |  | n41 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | 1 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |  |  |
| CA_n41A-n66A-n77(2A) |  | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n77A |  | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66(2A)-n77(2A) |  | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n66A-n77A |  | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n66A-n77A |  | CA_41C CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n78A |  | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n78(2A) |  | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n78A |  | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66(2A)-n78(2A) |  | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n71A-n77A |  | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n71A-n77(2A) |  | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 |  | 10 | 15 | 20 |  | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n71A-n77A |  | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41C-n71A-n77A |  | CA_41C CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n71A-n78A |  | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n71A-n78(2A) |  | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66A-n70A |  | CA_n48A-n66A CA_n48-n70A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66(2A)-n70A |  | CA_n48A-n66A CA_n48-n70A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A-n70A |  | CA_n48A-n66A CA_n48-n70A | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n48B-n66A-n70A |  | CA_n48A-n66A CA_n48-n70A | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66A-n71A |  | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66(2A)-n71A |  | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A-n71A |  | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48B-n66A-n71A |  | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66A-n71(2A) |  | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66A-n77A |  | - | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n48B-n66A-n77A |  | - | n48 | See CA_n48B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n48 | See CA_n48B Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n48(2A)-n66A-n77A |  | - | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n48A-n70A-n71A |  | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n70A-n71A |  | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48B-n70A-n71A |  | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n70A-n71(2A) |  | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5 | 10 | 15 | 20 |  |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n70A-n71A |  | CA_n66A-n71A CA_n70A-n71A | n66 | 5 | 10 | 15 | 20 |  |  |  | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n70A-n71(2A) |  | CA_n66A-n71A CA_n70A-n71A | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A-n71A |  | CA_n66A-n71A CA_n70A-n71A | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A-n71A |  | CA_n66A-n71A CA_n70A-n71A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n70 | 5 | 10 | 15 | 201 | 251 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A-n77A |  | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n71A-n77A |  | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n71A-n77(2A) |  | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A-n77(2A) |  | CA_n66A-n71A, CA_n66A-n77A, CA_n71A-n77A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A-n78A |  | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66A-n71A-n78(2A) |  | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | 5 | 10 | 15 | 20 | 25 |  | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n71A-n78A |  | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n66(2A)-n71A-n78(2A) |  | CA_n66A-n78A CA_n66A-n71A CA_n71A-n78A | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 5: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

<<unchanged texts are omitted>>

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n41 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  |  | n41 |  | 2507.5 |  | 10 |  | 25 |  | 2507.5 |  | 5.0 |  | TDD |  | IMD5 |  |
| CA_n1-n3-n77 |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | FDD |  | N/A |  |
|  |  | n3 |  | 1750 |  | 5 |  | 25 |  | 1845 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 3700 |  | 10 |  | 50 |  | 3700 |  | 28.4 |  | TDD |  | IMD22 |  |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | N/A |  | FDD |  | N/A |  |
|  |  | n3 |  | 1712.5 |  | 5 |  | 25 |  | 1807.5 |  | 31.5 |  | FDD |  | IMD21,2 |  |
|  |  | n77 |  | 3757.5 |  | 10 |  | 50 |  | 3757.5 |  | N/A |  | TDD |  | N/A |  |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 31.0 |  | FDD |  | IMD21 |  |
|  |  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 3915 |  | 10 |  | 50 |  | 3915 |  | N/A |  | TDD |  | N/A |  |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |
| CA_n1-n5-n7 | n1 | 1968 | 5 | 25 | 2158 | N/A | FDD | N/A |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 1.0 | FDD | IMD5 |
| CA_n1-n5-n78 | n1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | TDD | IMD3 |
| CA_n1-n7-n28 | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2533 | 10 | 50 | 2653 | 30.0 | FDD | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | FDD | IMD5 |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 |
| CA_n1-n28-n78 | n1 | 1960 | 5 | 25 | 2150 | 15.7 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | 739 | 5 | 25 | 794 | 4.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  |  | n78 |  | 3416 |  | 10 |  | 50 |  | 3416 |  | 15.7 |  | TDD |  | IMD3 |  |
| CA_n1-n40-n78 |  | n1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | TDD |  | N/A |  |
|  |  | n78 |  | 3480 |  | 10 |  | 50 |  | 3480 |  | 9.8 |  | TDD |  | IMD41 |  |
|  |  | n1 |  | 1930 |  | 5 |  | 25 |  | 2120 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2340 |  | 5 |  | 25 |  | 2340 |  | 10.6 |  | TDD |  | IMD4 |  |
|  |  | n78 |  | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | TDD |  | N/A |  |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 9.1 |  | FDD |  | IMD4 |  |
|  |  | n40 |  | 2380 |  | 5 |  | 25 |  | 2380 |  | N/A |  | TDD |  | N/A |  |
|  |  | n78 |  | 3450 |  | 10 |  | 50 |  | 3450 |  | N/A |  | TDD |  | N/A |  |
| CA_n1-n77-n79 | n1 | 1950 | 5 | 25 | 2140 | 6.0 | FDD | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n5-n30 | n2 | 1870 | 5 | 25 | 1959 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 9.7 | FDD | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | FDD | N/A |
| CA_n2-n5-n66 | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 7.2 | FDD | IMD4 |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | 842.5 | 5 | 25 | 887.5 | 3.8 | FDD | IMD5 |
|  | n77 | 3305 | 5 | 25 | 3305 | N/A | TDD | N/A |
|  | n2 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD31 |
| CA_n2-n12-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD32 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | TDD | IMD31,2 |
| CA_n2-n14-n66 | n2 | 1874 | 5 | 25 | 1954 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1762 | 5 | 25 | 2162 | 7.6 | FDD | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | 7.2 | FDD | IMD4 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| CA_n2-n14-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3466 | 10 | 50 | 3466 | 16.0 | TDD | IMD31 |
| CA_n2-n30-n77 | n2 | 1906 | 5 | 25 | 1986 | 8.6 | FDD | IMD4 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | 2309 | 5 | 25 | 2354 | 10.6 | FDD | IMD41 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | 29.4 | TDD | IMD22 |
| CA_n2-n66-n77 | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 31.2 | FDD | IMD2 |
|  | n77 | 4010 | 10 | 50 | 4010 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.3 | FDD | IMD4 |
|  | n77 | 3480 | 10 | 50 | 3480 | N/A | TDD | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 2.8 | FDD | IMD5 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n5-n7 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n7 | 2505 | 10 | 50 | 2625 | 30.0 | FDD | IMD24 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 19.0 | FDD | IMD3 |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
| CA_n3-n5-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | TDD | IMD5 |
|  | n3 | 1767 | 5 | 25 | 1862 | 15.7 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n7-n28 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 741 | 5 | 25 | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | 2562 | 5 | 25 | 2682 | 17.0 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 16.5 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
| CA_n3-n7-n78 | n3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD3 |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | 4.5 | TDD | IMD5 |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n18-n41 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | [N/A]1 | TDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | 16.0 | TDD | IMD3 |
|  | n18 | 820 | 5 | 25 | 865 | 28.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | 830 | 5 | 25 | 875 | [19.0] | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | IMD3 |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.3 | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A | n3 |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A | n28 |
|  | n79 | 4585 | 40 | 216 | 4585 | 9.4 | IMD41¦ | n79 |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A | n3 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A | n79 |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 | n28 |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A | n28 |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A | n79 |
|  | n3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 | n3 |
| CA_n3-40-n41 | n3 | 1747.5 | 5 | 25 | 1842.5 | 1.0 | FDD | IMD5 |
|  | n40 | 2347.5 | 5 | 25 | 2347.5 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | FDD | IMD3 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | 16.8 | TDD | IMD31 |
| CA_n3-n41-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | TDD | IMD3 |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | TDD | IMD3 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | FDD | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | FDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | TDD | IMD31, 2 ¦2*fBn77-fBn79¦ |
| CA_n5-n7-n78 | n5 | 834 | 5 | 25 | 879 | 30.2 | FDD | IMD2 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | 3.3 | FDD | IMD5 |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | FDD | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | TDD | IMD2 |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | 9.7 | TDD | IMD4 |
| CA_n5-n12-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 4.4 | FDD | IMD5 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | 4.4 | TDD | IMD5 |
| CA_n5-n14-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 11.6 | FDD | IMD41 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3298 | 10 | 50 | 3298 | 10.3 | TDD | IMD41 |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n25-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  |  | n78 |  | 3560 |  | 10 |  | 50 |  | 3560 |  | 16.1 |  | TDD |  | IMD3 |  |
| CA_n5-n29-n77 |  | n5 |  | 845 |  | 5 |  | 25 |  | 890 |  | N/A |  | FDD |  | N/A |  |
|  |  | n29 |  | N/A |  | 5 |  | N/A |  | 720 |  | 4.4 |  | SDL |  | IMD56 |  |
|  |  | n77 |  | 4100 |  | 10 |  | 50 |  | 4100 |  | N/A |  | TDD |  | N/A |  |
| CA_n5-n30-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 4 | FDD | IMD5 |
| CA_n5-n30-n77 | n5 | 835 | 5 | 25 | 880 | 15.2 | FDD | IMD3 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n66-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.1 | TDD | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4192 | 10 | 50 | 4192 | 8.2 | TDD | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3590 | 10 | 50 | 3590 | 3.3 | TDD | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 14.4 | FDD | IMD3 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n5-n66-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | 16.1 | TDD | IMD3 |
| CA_n5-n66-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | 13.2 | FDD | IMD3 |
|  |  | n78 |  | 3780 |  | 10 |  | 50 |  | 3780 |  | N/A |  | TDD |  | N/A |  |
| CA_n7-n25-n77 |  | n7 |  | 2520 |  | 5 |  | 25 |  | 2640 |  | 5.3 |  | FDD |  | IMD5 |  |
|  |  | n25 |  | 1870 |  | 5 |  | 25 |  | 1950 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 4125 |  | 10 |  | 50 |  | 4125 |  | N/A |  | TDD |  | N/A |  |
|  |  | n7 |  | 2550 |  | 5 |  | 25 |  | 2670 |  | N/A |  | FDD |  | N/A |  |
|  |  | n25 |  | 1870 |  | 5 |  | 25 |  | 1950 |  | 8.6 |  | FDD |  | IMD4 |  |
|  |  | n77 |  | 3525 |  | 10 |  | 50 |  | 3525 |  | N/A |  | TDD |  | N/A |  |
|  |  | n7 |  | 2520 |  | 5 |  | 25 |  | 2640 |  | N/A |  | FDD |  | N/A |  |
|  |  | n25 |  | 1905 |  | 5 |  | 25 |  | 1985 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 3750 |  | 10 |  | 50 |  | 3750 |  | 4.5 |  | TDD |  | IMD5 |  |
| CA_n7-n25-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n25 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n78 | 3750 | 10 | 50 | 3750 | 4.5 | TDD | IMD5 |
| CA_n7-n28-n78 | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 28.8 | FDD | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | N/A | FDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | FDD | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n28 | 720 | 5 | 25 | 775 | N/A | FDD | N/A |
|  | n78 | 3714 | 10 | 50 | 3714 | 9.7 | TDD | IMD4 |
| CA_n7-n66-n77 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD3 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | 3.4 | FDD | IMD5 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 4040 | 10 | 50 | 4040 | 4.2 | TDD | IMD5 |
| CA_n7-n66-n78 | n7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD3 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
| CA_n12-n30-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3913 | 10 | 50 | 3913 | 16.0 | TDD | IMD3 |
| CA_n12-n66-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | 1746 | 5 | 25 | 2146 | 13.2 | FDD | IMD3 |
|  | n77 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | FDD | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | FDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | TDD | IMD31,2 |
| CA_n13-n25-n66 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7..2 | FDD | IMD4 |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n13 | 780 | 10 | 50 | 749 | N/A | FDD | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | FDD | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
| CA_n13-n25-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3444 | 10 | 50 | 3444 | 17.3 | TDD | IMD31,2 |
|  | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 16.0 | FDD | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | TDD | N/A |
| CA_n13-n66-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1736 | 5 | 25 | 2136 | 17.1 | FDD | IMD3 |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n13 | 781 | 5 | 25 | 750 | 15.2 | FDD | IMD3 |
|  | n66 | 1710 | 5 | 25 | 2110 | N/A | FDD | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | TDD | N/A |
|  | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3334 | 10 | 50 | 3334 | 16.3 | TDD | IMD31,2 |
| CA_n14-n30-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3896 | 10 | 50 | 3896 | 16.0 | TDD | IMD3 |
| CA_n14-n66-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | 13.2 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  |  | n77 |  | 3341 |  | 10 |  | 50 |  | 3341 |  | 16.0 |  | TDD |  | IMD31,2 |  |
| CA_n24-n41-n48 |  | n24 |  | 1649 |  | 5 |  | 25 |  | 1528.5 |  | N/A |  | FDD |  | N/A |  |
|  |  | n41 |  | 2610 |  | 5 |  | 25 |  | 2610 |  | N/A |  | TDD |  | N/A |  |
|  |  | n48 |  | 3571 |  | 10 |  | 50 |  | 3571 |  | 16.8 |  | TDD |  | IMD3 |  |
|  |  | n24 |  | 1630 |  | 5 |  | 25 |  | 1528.5 |  | N/A |  | FDD |  | N/A |  |
|  |  | n41 |  | 2500 |  | 5 |  | 25 |  | 2500 |  | 5.3 |  | TDD |  | IMD5 |  |
|  |  | n48 |  | 3695 |  | 10 |  | 50 |  | 3695 |  | N/A |  | TDD |  | N/A |  |
|  |  | n24 |  | 1631.5 |  | 5 |  | 25 |  | 1530 |  | 16.4 |  | FDD |  | IMD3 |  |
|  |  | n41 |  | 2592.5 |  | 5 |  | 25 |  | 2592.5 |  | N/A |  | TDD |  | N/A |  |
|  |  | n48 |  | 3655 |  | 10 |  | 50 |  | 3655 |  | N/A |  | TDD |  | N/A |  |
| CA_n24-n41-n77 |  | n24 |  | 1630 |  | 5 |  | 25 |  | 1528.5 |  | N/A |  | FDD |  | N/A |  |
|  |  | n41 |  | 2685 |  | 5 |  | 25 |  | 2685 |  | N/A |  | TDD |  | N/A |  |
|  |  | n77 |  | 3735 |  | 10 |  | 50 |  | 3735 |  | 16.8 |  | TDD |  | IMD31,5 |  |
|  |  | n24 |  | 1630 |  | 5 |  | 25 |  | 1528.5 |  | N/A |  | FDD |  | N/A |  |
|  |  | n41 |  | 2610 |  | 5 |  | 25 |  | 2610 |  | 5.3 |  | TDD |  | IMD55 |  |
|  |  | n77 |  | 3755 |  | 10 |  | 50 |  | 3755 |  | N/A |  | TDD |  | N/A |  |
|  |  | n24 |  | 1630 |  | 5 |  | 25 |  | 1528.5 |  | 16.4 |  | FDD |  | IMD32,5 |  |
|  |  | n41 |  | 2500 |  | 5 |  | 25 |  | 2500 |  | N/A |  | TDD |  | N/A |  |
|  |  | n77 |  | 3465 |  | 10 |  | 50 |  | 3465 |  | N/A |  | TDD |  | N/A |  |
| CA_n25-n38-n78 | n25 | 1852.5 | 5 | 25 | 1932.5 | 16.4 | FDD | IMD3 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | TDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1880 | 5 | 25 | 1960 | 8.6 | TDD | IMD4 |
|  | n38 | 2570 | 5 | 25 | 2570 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |
| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n77 | 3180 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | TDD | N/A |
| CA_n25-n41-n78 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n78 | 3180 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | TDD | N/A |
| CA_n25-n48-n66 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n48 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD21 |
|  | n48 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
| CA_n25-n66-n77 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n25-n71-n78 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD3 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  |  | n78 |  | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | TDD |  | N/A |  |
| CA_n28-n40-n78 |  | n28 |  | N/A |  | 5 |  | 25 |  | 800.5 |  | 11 |  | IMD3 |  | IMD3 |  |
|  |  | n40 |  | 2302.5 |  | 5 |  | 25 |  | 2302.5 |  | N/A |  | N/A |  | N/A |  |
|  |  | n78 |  | 3795 |  | 10 |  | 50 |  | 3795 |  | N/A |  | N/A |  | N/A |  |
|  |  | n28 |  | 708 |  | 5 |  | 25 |  | 2120 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2310 |  | 5 |  | 25 |  | 2310 |  | N/A |  | TDD |  | N/A |  |
|  |  | n78 |  | 3736 |  | 10 |  | 50 |  | 3736 |  | 16.0 |  | TDD |  | IMD32 |  |
|  |  | n28 |  | 708 |  | 5 |  | 25 |  | 763 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2134 |  | 5 |  | 25 |  | 2134 |  | 15.7 |  | TDD |  | IMD3 |  |
|  |  | n78 |  | 3550 |  | 10 |  | 50 |  | 3550 |  | N/A |  | TDD |  | N/A |  |
| CA_n28-n40-n79 |  | n28 |  | 730 |  | 5 |  | 25 |  | 785 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2350 |  | 5 |  | 50 |  | 2350 |  | N/A |  | TDD |  | N/A |  |
|  |  | n79 |  | 4540 |  | 40 |  | 216 |  | 4540 |  | 10.7 |  | TDD |  | IMD4 |  |
|  |  | n28 |  | 720 |  | 5 |  | 25 |  | 775 |  | N/A |  | FDD |  | N/A |  |
|  |  | n40 |  | 2340 |  | 5 |  | 50 |  | 2340 |  | 9.2 |  | TDD |  | IMD4 |  |
|  |  | n79 |  | 4500 |  | 40 |  | 216 |  | 4500 |  | N/A |  | TDD |  | N/A |  |
| CA_n28-n41-n77 |  | n41 |  | 2642 |  | 5 |  | 25 |  | 2642 |  | N/A |  | TDD |  | N/A |  |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 28.2 | TDD | IMD24 |
| CA_n28-n41-n78 | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD21 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD22 |
| CA_n28-n41-n79 | n28 | 725 | 5 | 25 | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4480 | 40 | 216 | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2645 | 10 | 50 | 2645 | 10.4 | TDD | IMD4 |
|  |  | n79 |  | 4850 |  | 40 |  | 216 |  | 4850 |  | N/A |  | TDD |  | N/A |  |
| CA_n28-n46-n78 |  | n28 |  | 710 |  | 5 |  | 25 |  | 765 |  | N/A |  | FDD |  | N/A |  |
|  |  | n46 |  | 5170 |  | 20 |  | 100 |  | 5170 |  | N/A |  | FDD |  | N/A |  |
|  |  | n78 |  | 3750 |  | 10 |  | 50 |  | 3750 |  | 17 |  | TDD |  | IMD31 |  |
|  |  | n28 |  | 725 |  | 5 |  | 25 |  | 780 |  | 16 |  | FDD |  | IMD3 |  |
|  |  | n46 |  | 5900 |  | 20 |  | 100 |  | 5900 |  | N/A |  | FDD |  | N/A |  |
|  |  | n78 |  | 3340 |  | 10 |  | 50 |  | 3340 |  | N/A |  | TDD |  | N/A |  |
|  |  | n28 |  | 740 |  | 5 |  | 25 |  | 795 |  | N/A |  | FDD |  | N/A |  |
|  |  | n46 |  | 5900 |  | 20 |  | 100 |  | 5900 |  | 22 |  | TDD |  | IMD31,2 |  |
|  |  | n78 |  | 3320 |  | 10 |  | 50 |  | 3320 |  | N/A |  | TDD |  | N/A |  |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | N/A | n77 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A | n79 |
|  |  | n28 |  | 745 |  | 5 |  | 25 |  | 800 |  | 16.2 |  | IMD21,2 |  | n28 |  |
| CA_n29-n30-n77 |  | n29 |  | N/A |  | 5 |  | N/A |  | 722 |  | 15.2 |  | SDL |  | IMD31 |  |
|  |  | n30 |  | 2310 |  | 5 |  | 25 |  | 2355 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 3898 |  | 10 |  | 50 |  | 3898 |  | N/A |  | TDD |  | N/A |  |
| CA_n29-n66-n77 |  | n29 |  | N/A |  | 5 |  | N/A |  | 722 |  | 15.2 |  | SDL |  | IMD36 |  |
|  |  | n66 |  | 1734 |  | 5 |  | 25 |  | 2134 |  | N/A |  | FDD |  | N/A |  |
|  |  | n77 |  | 4190 |  | 10 |  | 50 |  | 4190 |  | N/A |  | TDD |  | N/A |  |
| CA_n30-n66-n77 | n30 | 2310 | 5 | 25 | 2355 | 29.2 | FDD | IMD21 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 8.7 | FDD | IMD4 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4055 | 10 | 50 | 4055 | 28.4 | TDD | IMD21 |
| CA_n38-n66-n78 | n38 | 2550 | 5 | 25 | 2550 | N/A | TDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3460 | 10 | 50 | 3460 | 15.0 | TDD | IMD3 |
| CA_n39-n40-n79 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | TDD | IMD4 |
| CA_n40-n41-n79 | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | 30.5 | TDD | IMD2 |
| CA_n41-n66-n77 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n66-n78 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31 |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n41-n71-n78 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n48-n66-n70 | n48 | 3625 | 10 | 50 | 3625 | N/À | TDD | N/A |
|  | n66 | 1742.5 | 5 | 25 | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
| CA_n48-n66-n71 | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | 1761.5 | 5 | 25 | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | 3695 | 10 | 50 | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n48-n70-n71 | n48 | 3694 | 10 | 50 | 3694 | 9 | TDD | IMD41 |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | 3724 | 10 | 50 | 3724 | 9 | TDD | IMD41 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: This band is subjected to 2nd order IMD but is not expected for the operating frequency range of n77 within USA (3450 – 3550 MHz, 3700 – 3980 MHz). NOTE 6: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>