3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118618

# Electronic Meeting, Nov. 1-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On band definition for the lower 6GHz NR unlicensed operation

Agenda item:	7.1.2

Document for:	Approval

# Introduction

During RAN#89-e a WID was approved for a lower 6GHz NR unlicensed operation intended for deployment in Europe. The start of the WID was delayed pending administrations in Europe have had unlicensed operation in the range 5925 to 6425 MHz for consultation. The result of this consultation is that at the November 2020 meeting the ECC with ECC Decision (20)01 “on the harmonized use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)” approved unlicensed operation in the range 5945 MHz to 6425 MHz. Given the ECC decision the work was at RAN#90 approved to start in RAN4#98 [1].

NR-Unlicensed operation is standardized in Rel-16 [3] with the definition of band n96 covering the spectrum range 5925 MHz -7125 MHz which is currently applicable in the USA only subject to FCC Report and Order FCC 20-51. The ECC decision covers the range 5945 MHz to 6425 MHz which is therefor a sub-set of the already defined band n96.

The objectives for the core part from the WID [1] relevant for the UE are given as:

- Depending on the details of the European regulatory requirements, determine whether they are best handled by relevant updates (if any) of band n96 or whether a new band is needed.

  - If a new band is needed, determine the band plan for unlicensed operation in the range 5945-6425 MHz

- Define or update (if needed) system parameters such as channel bandwidths and channel arrangements

- Define or update (if needed) transmitter and receiver characteristics requirements for the UE

During RAN4#98 a WF was agreed in [4] including the following general agreements:

General - Band plan:

- Unlicensed operation in the range 5945-6425 MHz can be introduced by:

  - Option 1: Re-using already defined band n96

    - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

  - Option 2: Defining a new band n[xx]

    - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

- NR-ARFCN and GSCN already defined for n96 should be re-used in the frequency range applicable to 6GHz NR unlicensed operation in Europe. This is independent if n96 is reused or a new band is defined.

General - LPI and VLP deployment:

- LPI devices to be supported by 3GPP Specifications

- VLP devices can be supported by the 3GPP specification, VLP deployments and devices are subject to further checking of regulations

- 0 dBi antenna gain is assumed for the purpose of deriving 3GPP specifications on MOP, PSD, and ASE for LPI deployments and, if introduced, also VLP deployments.

- VLP deployment and device types are subject to further checking of regulations

During RAN4#98bis a WF was agreed in [5] including the following general agreements:

General - Band plan:

- Unlicensed operation in the range 5945-6425 MHz can be introduced by (Agreed in R4-2103229):

  - Option 1: Re-using already defined band n96

    - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

  - Option 2: Defining a new band n[xx]

    - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

- NR-ARFCN and GSCN already defined for n96 should be re-used in the frequency range applicable to 6GHz NR unlicensed operation in Europe. This is independent if n96 is reused or a new band is defined.

General - Note for n96:

- Currently the note in 38.101-1 reads:
NOTE 14: This band is applicable in the USA only subject to FCC Report and Order [FCC 20-51]

  - Option 1: Modify the note according to R4-2106273:

    - This band is applicable in the USA subject to FCC Report and Order FCC 20-51 and in Europe subject to CEPT ECC Decision (20)01 for the 5945 to 6425 MHz frequency range.

  - Option 2: Modify the note to generic text:

    - This band is only applicable subject to regional and/or country specific restrictions.

General - LPI and VLP deployment:

- Inclusion of VLP deployment to 3GPP specification:

  - Option 1: VLP deployment is subject to further checking of regulations

  - Option 2: VLP deployment can be included in specification given the available regulations

During RAN4#99 a WF was agreed in [6] including the following general agreements:

General - Band plan:

- Option 1: Re-using already defined band n96, for the frequency range 5945 MHz to 6425 MHz 
(HPE, Nokia, Intel, Skyworks, Facebook, Charter, CableLabs, Apple, Qualcomm, OPPO)

  - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

- Option 2: Defining a new band n[xx], for the frequency range 5945 MHz to 6425 MHz 
(Ericsson, ZTE, Huawei, BT, Orange, TIM, OPPO, DT)

  - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

- Note that selecting any of the options above shall not in any way interfere with regulatory activities and timelines for the 6 GHz range. (As per RAN agreement)

General - LPI and VLP deployment:

- VLP deployment can be included in specification given the available regulations.

  - Further discussion is needed in future meeting about the possible regulation issues.

During RAN4#100 the following agreements were made during GTW:

Agreements:

- The same hardware of UE as for n96 may be reused on the frequency range 5945MHz to 6425MHz no matter whether to define a new band or define new NS for the existing n96.

- For outdoor UEs connecting to the indoor LPI base stations, no solution is needed to meet the regulatory requirements in Rel-17.

- A-MPR for VLP as captured in TR 38.849 (R4-2113692)

During RAN#93 a WF was agreed in [7] and GTW agreements captured in chair minutes were:

Agreements:

- RAN4 is asked to define a new band n[xx] for lower 6GHz NR unlicensed operation for Europe in order to support

  - The definition of alternate UE receiver blocking requirements.

  - Definition of this new band is dependent on completion of the task agreed in RP 212589 [7] (Option 1, below)

- Option 1: Task RAN4 with the evaluation of an achievable UE receiver blocking level based on existing n96 hardware. The blocking level is targeted to be as close as possible to -30 dBm.

The objectives of the WID together with the agreements so far are discussed further in this contribution.

# Discussion

#### Band Plan

As listed in the WFs [4 - 6] two options have been discussed for the introduction of a NR band for unlicensed operation in Europe in the frequency range 5945 MHz to 6425 MHz. The choices were either leverage already defined n96 or introduce a new band. No progress was made on this choice during the RAN4 meetings why the question was raised to RAN.

As argued in previous meetings our opinion is that there is no need for a new band as we can utilize n96 with additional restrictions specific for Europe (EU) by the Harmonised Standard (HS) for access to radio spectrum in the 6 GHz range [2]. However, we acknowledge the strong preferences from some sides for a new band.

At RAN#93 it was decided to progress with defining a new band but with the precondition that UE hardware supporting already defined n96 could be reused. In practice this means UE receiver blocking requirements shall be defined close enough to the -44dBm used for n96 to allow reuse of band n96 hardware (filter, LNA, etc.). Then to ensure most possible protection from a potential licensed operation in the upper 6 GHz band it was also agreed to target a blocking level as close as possible to -30dBm.

Observation 1: 	Hardware designed for n96 shall be possible to reuse for a new band defined for unlicensed operation in the range 5945-6425 MHz.

Observation 2: 	A new band for unlicensed operation in the range 5945-6425 MHz cannot be introduced before blocking requirements have been agreed.

Proposal 1:	Decide blocking requirements in the range -44 to -30dBm for a new band defined for unlicensed operation in the range 5945-6425 MHz.

The difference from band n96, originally intended for US deployments, and the frequency range for EU is that band n96 starts at 5925 MHz while the EU range starts at 5945 MHz and that the US range goes to 7125 MHz while the EU range stops at 6425 MHz. Even though the starting frequency is different for the US and EU ranges the channel and synchronization raster are still aligned since the 20 MHz shift have already been accounted for in the design of band n96. The main difference is that for the US range will have additional available raster points corresponding to the upper range of band n96 (i.e., 6425 MHz – 7125 MHz).

Observation 3: 	The channel raster’s for n96 are already aligned to the channels defined by the European harmonized standard EN 303 687.

Proposal 2:	A new band for unlicensed operation in the range 5945-6425 MHz shall reuse channel and synchronization raster points of n96 corresponding to the reduced frequency range as captured in TR 38.849, Clause 5.2

# Conclusion

In this contribution, we discuss the 6 GHz band for unlicensed operation in Europe. We have made following observations and proposals:

Observation 1: 	Hardware designed for n96 shall be possible to reuse for a new band defined for unlicensed operation in the range 5945-6425 MHz

Observation 2: 	A new band for unlicensed operation in the range 5945-6425 MHz cannot be introduced before blocking requirements have been agreed.

Proposal 1:	Decide blocking requirements in the range -44 to -30dBm for a new band defined for unlicensed operation in the range 5945-6425 MHz

Observation 3: 	The channel raster’s for n96 are already aligned to the channels defined by the European harmonized standard EN 303 687.

Proposal 2:	A new band for unlicensed operation in the range 5945-6425 MHz shall reuse channel and synchronization raster points of n96 corresponding to the reduced frequency range as captured in TR 38.849, Clause 5.2

# References

- RP-212625, WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

- Draft EN 303 687 v0.0.13, ETSI BRAN

- RP-200312 Revised WID on NR-based Access to Unlicensed Spectrum; rapporteur: Qualcomm

- R4-2103229, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Qualcomm, Charter Communications Inc.

- R4-2105383, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

- R4-2108020, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

- RP-212589, Agreement regarding discussion [93e-26-6GHz-NR-U] Topic #1: Consideration of Aspects beyond ECC Decision (20)01, RAN vice-chair (AT&T)