3GPP TSG-RAN WG4 Meeting # 101-e                                                                                         R4-2118716

Electronic Meeting, 1–12 November, 2021

Agenda item:	8.13.1.1

Source:	Huawei

Title:	TP for 38.863 on system parameters on satellite bands

Release:	Rel-17

Document for:	Approval

1	Introduction

RAN4 has discussed the system parameters for the satellite communications. Some agreements have been recorded into the WF [1]. This contribution provides the agreed system parameters for satellite communications as a text proposal to the new TR 38.863.

2	Discussion

In RAN4#98bis, there are two options on NTN channel raster: option 1 15kHz and option 2 100kHz. However, RAN4 agreed to specify 100kHz channel raster for S-band and L-band in RAN4#99 meeting. Currently, the sync raster below 3GHz was defined based on the 100kHz channel raster considering LTE re-farming bands. If RAN4 choose 15kHz channel raster, the sync raster need to be revisited below 3GHz.

Observation 1: Specifying 100kHz channel raster is aligned with the definition of sync raster below 3GHz.

3	References

R4-2115640	Way Forward on NTN_solutions_Part1, Moderator (THALES)

Text Proposal

<< Start of text proposal  >>

6.1.3	Channel raster and sync raster

6.1.3.1	Analysis

There are two candidate options about channel raster for S-band and L-band: Option 1 15kHz and Option 2 100kHz. The relationship between channel raster and sync raster can be found as below equation.

Maximum SS raster: ΔFSC,Raster ≤ BWConfig + ΔFCH,Raster – BWPBCH

Since the minimum BWConfig = 4.5MHz, and BWPBCH =3.6MHz, when we input 15kHz or 100kHz for ΔFCH,Raster , we can get the maximum sync raster as below

Table 6.1.3.1-1: The comparison for different channel raster

| Channel Raster | 15kHz | 100kHz |
| --- | --- | --- |
| Sync Raster | 915kHz | 1000kHz |

In order to be compatible with current design of sync raster below 3GHz, option 2 100kHz should be chosen as the channel raster for NR NTN bands below 3GHz.

<< end of text proposal  >>