3GPP TSG-RAN WG4 Meeting # 101-e                                                            R4-2118721

Electronic Meeting, 1– 12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | DraftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-3 to correct the MSD due to Tx non-linearity interference in 1st and 2nd adjacent channel of UL band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN4#99 meeting, companies had comments on the definition of CIM5 based on the paper R4-2109155. And the definition of MSD due to counter intermodulation interference may not reflect the real situation accurately based on the E-mail discussion summary R4-2107912 in clause 6.3.1. The term “Tx non-linearity interference in 1st and 2nd adjacent channel of UL band” is proposed to reflect the reason why we specify the MSD. Since it’s clarified “CIM5 appears on the other side of CIM3”, the “minus mark” is changed to “plus mark” for the equation in the note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To impove the wording of MSD due to counter intermodulation interference. - The counter intermodulation interference is replaced by Tx non-linearities interference in 1st and 2nd adjacent channel of UL band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of new MSD may cause some confusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change1>>

7.3B.2.3.6	Reference sensitivity exceptions due to Tx non-linearity interference in 1st or 2nd adjacent channel of UL band for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by Tx non-linearity interference in 1st or 2nd adjacent channel from another UL band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (DL band) are specified in Table 7.3B.2.3.6-1 with uplink configuration of the aggressor band (UL band) specified in Table 7.3B.2.3.6-1.

Table 7.3B.2.3.6-1: Reference sensitivity exceptions (MSD) due to Tx non-linearity interference in 1st or 2nd adjacent channel of UL band for EN-DC in NR FR1

| UL band | DL band | SCS of UL band (kHz) | LCRB of UL band | Applicable UL BW(MHz) | MSD value of DL band (dB) | The adjacent channel of UL band |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 3 | 15 | 25 | ≥ 25 | 4.5 | 2nd adjacent channel |
| n1 | 3 | 15 | 25 | 50 | 17 | 1st adjacent channel |
| n5 | 28 | 15 | 6 | ≥ 15 | 7.9 | 2nd adjacent channel |
| n40 | 1 | 30 | 25 | ≥ 70 | 21.5 | 2nd adjacent channel |
| NOTE 1: For interference in 2nd adjacent channel, the MSD exceptions are applicable to the case that interference in 2nd adjacent channel of UL band falls into the DL channels. (The victim frequency of DL band can be expressed as fCIM5DL=FC_UL+-5(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). NOTE 2: For interference in 1st adjacent channel, the MSD exceptions are applicable to the case that interference in 1st adjacent channel of UL band falls into the DL channels. (The victim frequency of DL band can be expressed as fCIM5DL=FC_UL-3(fUL-FC_UL), where FC_UL is the centre frequency of UL channel and fUL is the allocated transmission frequency of UL band). |  |  |  |  |  |  |

<<End of Change1>>