3GPP TSG-RAN WG4 Meeting #101-e	R4-2118868

Electronic Meeting, November 1-12, 2021

Source:	China Telecom

Title:	Simulation results on PUSCH FR1 256QAM demodulation requirements

Agenda Item:	8.12.3.1

Document for:	Discussion

- Introduction

In the RAN4#100-e meeting, we have made conclusion on all the test parameter, simulation assumptions and applicability for the PUSCH FR1 256QAM demodulation test cases and the WF was approved in [1].

This paper we provide link-level simulation results for all the agreed test cases.

- Simulation Results

In the last meeting, we have provided ideal and impairment simulation results with PUSCH mapping type A, MCS 20 for 15kHz SCS 10MHz and 30kHz SCS, 40MHz and for 2/4/8 Rx in [2]. Based on the agreed test parameters in [1], we provide simulation results for other test cases as follows:

Table 1 Ideal and impairment SNR (dB) @ 70% max TP for PUSCH Mapping Type A

| 1T2R | PUSCH Mapping Type A |  | Ideal | Impairment |
| --- | --- | --- | --- | --- |
|  |  | 15kHz, 5MHz | 17.14 | 19.14 |
|  |  | 30kHz, 10MHz | 17.18 | 19.18 |
|  |  | 30kHz, 100MHz | 18.31 | 20.31 |
| 1T4R | PUSCH Mapping Type A | 15kHz, 5MHz | 12.97 | 14.97 |
|  |  | 30kHz, 10MHz | 13.22 | 15.22 |
|  |  | 30kHz, 100MHz | 14.09 | 16.09 |
| 1T8R | PUSCH Mapping Type A | 15kHz, 5MHz | 9.50 | 11.50 |
|  |  | 30kHz, 10MHz | 9.81 | 11.81 |
|  |  | 30kHz, 100MHz | 10.35 | 12.35 |

Table 2 Ideal and impairment SNR (dB) @ 70% max TP for PUSCH Mapping Type B

| 1T2R | PUSCH Mapping Type B |  | Ideal | Impairment |
| --- | --- | --- | --- | --- |
|  |  | 15kHz, 5MHz | 17.06 | 19.06 |
|  |  | 15kHz, 10MHz | 17.32 | 19.32 |
|  |  | 30kHz, 10MHz | 17.14 | 19.14 |
|  |  | 30kHz, 40MHz | 17.95 | 19.95 |
|  |  | 30kHz, 100MHz | 18.42 | 20.42 |
| 1T4R | PUSCH Mapping Type B | 15kHz, 5MHz | 12.99 | 14.99 |
|  |  | 15kHz, 10MHz | 13.34 | 15.34 |
|  |  | 30kHz, 10MHz | 13.23 | 15.23 |
|  |  | 30kHz, 40MHz | 13.51 | 15,51 |
|  |  | 30kHz, 100MHz | 15.47 | 17.47 |
| 1T8R | PUSCH Mapping Type B | 15kHz, 5MHz | 9.55 | 11.55 |
|  |  | 15kHz, 10MHz | 9.84 | 11.84 |
|  |  | 30kHz, 10MHz | 9.82 | 11.82 |
|  |  | 30kHz, 40MHz | 9.98 | 11.98 |
|  |  | 30kHz, 100MHz | 10.40 | 12.40 |

- Conclusion

This paper we provide link-level simulation results for all the agreed test cases.

- Reference

R4-2115748, Way forward on PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#100-e, Nov 2021.

R4-2112147, Initial simulation results and discussion on PUSCH FR1 256QAM demodulation requirements, China Telecom, RAN4#100-e, Nov 2021.