3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118929

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	On RRM for eIAB

Agenda item:	8.17.3

Document for:	Approval

# 1	Introduction

In RAN4 101-e, there is an incoming LS from RAN1 on inter-donor mitigation [1]. Some content are copy pasted below.

For Alt2, RAN1 understands that only the cell from one of the two logical DUs is active at one time using the same physical cell resources. RAN1 has not reached consensus on how the two logical DUs share the same physical cell resources. - Understanding 1: The two DUs can be switched ON and OFF in a dynamic manner. This means that a UE may stay in CONNECTED mode during the migration but it cannot identify both of cells at one time. - Understanding 2: The two DUs can be switched between ON and OFF only once. - When two cells use the same PCI, this may not necessarily require all the UEs to switch to another cell at one time if RAN2 can confirm that the current specification enables a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI. - When two cells use the different PCIs, this will require all the UEs to perform HO to another cell at one time, which pose a high load to RACH. For Alt2 - RAN1 would like to ask RAN3 which of the above understandings is in line with the assumption made in RAN3. - RAN1 would like to ask RAN4 whether RLM and RRM will be impacted for legacy UEs. - It is RAN1's understanding that the feasibility of Alt2 is dependent on whether HO can be performed without negatively impacting legacy UEs, regardless if the same or different PCIs are used for the two DUs.

- In the LS RAN1 provides some of their understandings and also asks RAN4 on whether RLM and RRM would be impacted. This paper discusses the issues listed above and provide our views.

2	Discussions

The question from RAN1 is about whether RLM and RRM (in general) will be impacted. They also point out that they key is whether handover will be impacted, which we already provided our answer in the previous LS.

During inter-donor mitigation, NCGI will change. There is process of CGI reading in RLM. Thus, if CGI changes during RLM then the UE behavior is unclear. In other words, RLM is impacted.

RLM will be impacted for legacy UEs.

For general aspects of RRM, especially HO, RAN4 has already provided understanding in the previous reply LS [2].

Observation 1: RAN4 has already provided answers in the previous reply LS [2] regarding general RRM and HO.

Since RAN1 asks a specific question on RLM, which is not covered by our previous LS, it is proposed that RAN4 sends a reply LS to RAN1 providing again our analysis along with answers regarding RLM process. RAN1 might be waiting for our answers in order to resolve further issues.

RAN4 sends a reply LS to RAN1 providing again our analysis along with answers regarding RLM process.

A draft LS is provided in the Annex of this paper.

3	Conclusion

- Proposal 1: RLM will be impacted for legacy UEs.

Observation 1: RAN4 has already provided answers in the previous reply LS [2] regarding general RRM and HO.

Proposal 2: RAN4 sends a reply LS to RAN1 providing again our analysis along with answers regarding RLM process

References

- R4-2117010

- R4-2115354

Annex

Title:	Reply LS on inter-donor migration

Reply To:	R1-2108529 (R4-2117010)

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN WG4

To:	RAN WG1

Cc:			RAN WG3, RAN WG2

Contact Person:

Name:	Richie Leo

E-mail Address:	richie.leo@zte.com.cn

Attachments:

1. Overall Description:

RAN4 would like to thank RAN1 for their LS on inter-donor migration [1]. RAN4 would like to provide the following observation and responses to the RAN1 questions:

- Question: For Alt2, RAN1 would like to ask RAN4 whether RLM and RRM will be impacted for legacy UEs.

[RAN4 Response]: Answers on general RRM aspects and handover can be found in a previous LS [2].

On RLM, RLM will be impacted for legacy UEs since NCGI reading is a part of RLM.

- Actions:

To RAN WG1 group.

ACTION: 	RAN4 kindly asks RAN1 to take the above feedback into account on their future work.

- Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #101-bis-e			17th – 25th January 2022 		Online

TSG-RAN4 #102-e		21th February – 03rd March 2022 		Online

- References:

- R4-2117010

- R4-2115354