3GPP TSG-RAN WG4 Meeting #101-e	R4-2119021

# Electronic Meeting, 1st - 12th Nov, 2021

Title: 	Discussion on NR FR2 HST deployment Scenario-A

Source: 	Huawei, HiSilicon

Agenda item:	8.9.2.1

Document for:	Discussion

Background

During RAN4#100-e meeting, WF [1] on remaining issues on FR2 HST deployment scenario and channel modelling was approved. In this contribution, we share our views about NR FR2 HST deployment Scenario-A as per agreements reached in RAN4#100-e.

- No dedicated performance RAN4 requirements will be specified for Bi-directional deployment for Scenario A by assuming the requirements will be specified under uni-directional deployment which pending on further confirmation in RRM session for the feasibility of uni-directional deployment. - Capture relevant information for the analysis of all possible deployment and schemes into TR, and some comparison analysis can be also included.

Discussion

Bi-directional

There are two schemes for Bi-directional deployment. Considering very small Dmin, the angle between BS-UE line and the railway can be negligible, so only Scheme-1 is for further analysis.

Figure 2.1-1 Scheme-1 for Bi-directional deployment

For Scheme-1, 1 beam per BS panel and 6 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to the railway at the distance of Ds. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.1-2 below.

Figure 2.1-2 Link budget for Scheme-1

For Scheme-1, the power of side-lobes for different beams change rapidly when UE is near to the RRH and the minimum time duration for the best beam with same beam index can be far less than 160 ms that is the L1-RSRP measurement period for HST FR2 scenario as analysis in [2]. It is a great challenge for the UE to ensure the performance not to degrade in such location. UE can use different strategy by implementation, such as select the best beam as per RSRP measurement result or directly switch the UE beam point to the main-lobe beam transmission from the next RRH. For the former one, the best beam may be unavailable with high probability once UE beam switching has been performed.

For Bi-directional Scenario-A deployment for FR2 HST, due to the rapidly changed power of side-lobes when UE is near to the RRH, it is a great challenge for the UE to ensure the performance not to degrade in such location.

Uni-directional

For Uni-directional deployment, 1 beam per BS panel and 2 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to 0 degrees. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.2-1 below.

Figure 2.2-1 Link budget for Uni-directional deployment

The link budget remaining and the minimum beam dwelling time for Uni-directional deployment is shown as Table 2.2-1 below.

Table 2.2-1 Link budget remaining and minimum beam dwelling time

| link budget remaining[dB] | minimum beam dwelling time[s] | Beam switching point[m] |
| --- | --- | --- |
| 19.2 | 7.20 | 50 |

For Uni-directional Scenario-B deployment for FR2 HST, it is feasible to provide sufficient link budget remaining while it requires only one switching which leads to larger minimum beam dwelling time.

Proposals

In this contribution, we discuss on NR FR2 HST deployment Scenario-A. Our observations and proposals are:

- For Bi-directional Scenario-A deployment for FR2 HST, due to the rapidly changed power of side-lobes when UE is near to the RRH, it is a great challenge for the UE to ensure the performance not to degrade in such location.

For Uni-directional Scenario-B deployment for FR2 HST, it is feasible to provide sufficient link budget remaining while it requires only one switching which leads to larger minimum beam dwelling time.

Reference

R4-2115725, WF on remaining issues on FR2 HST deployment scenario and channel modeling, RAN4#100-e, Samsung

R4-2108660, WF on FR2 HST Deployment Scenario Analysis, RAN4#99-e, Samsung

- Annex Simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Ds | 700m |
| Dmin | 10m |
| RRH Tx power | 47dBm |
| RRH height | 15m |
| RRH antenna array | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] |
| Path Loss | RMa LoS |
| UE antenna height | 5m |
| UE antenna array | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] |
| UE noise figure | 10dB |
| ILs | 13 dB |
| SNR | 18.6dB (i.e. FR2 Test 2-6, 64QAM CR=0.43 and Rank2 in TS 38.101-4) |
| Others | In WF [2] |