3GPP TSG-RAN WG4 Meeting#101-e                               R4-2119207

E-meeting,1st Nov– 12th Nov, 2021

Agenda item:	8.5.2.1

Source: 	ZTE Corporation

Title: 	Discussions on power related requirements for conducted repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of discussion on repeater conducted output power and emission requirements and there were lots of open issues left for further discussions. Therefore in this contribution, we want to share some further considerations on those remaining open issues.

Discussion

- 2.1. UL output power levels

- During the last meeting, it was agreed to have two different output power levels for FR1 repeater in UL which is quite similar as IAB-MT defined in Rel-16. One is with power limitation for LA IAB-MT and another one is without power limitation for WA IAB-MT. However for repeater class with power limitations, it’s not clear whether to define with power class per band basis or with fixed values across all bands. From our understandings, repeater functionality in UL should be similar as IAB-MT as network equipment, therefore we don’t see the necessity to define power limitation per band basis with mandatory power class definition and the rated transmitted power in UL of repeater should be based on vendor’s declaration instead of with mandatory power class definition per band.

- Proposal 1: for repeater class with power limitation, propose to define with fixed values;

- Introduce two classes, one with power limitation and another one without power limitation.

- For the class with power limitation: the exact power limitation can be further discussed

- Option 1: With fixed values

- Option 2: With maximum value over the supported classes as per band basis

- Other options not precluded

- Regarding the power limitations for repeater UL in TDD bands, we prefer to reuse the 24dBm as the same as IAB-MT. Indeed this has been agreed for repeater in FDD band with 24dBm cap and we don’t see much necessity for further increase it for TDD bands. The potential consequence of further increasing output power of repeater UL as PC2 26dBm or PC1.5 29dBm, then it will result in more “visible fixed HPUE” in the network as shown in Figure 1 which might result in more co-channel interference for other neighbour cells and adjacent channel interference to other operating carriers.

- Proposal 2: for TDD band, reuse 24dBm as power limitation;

- Figure 1. the illustration of repeater UL for coverage enhancement

- For FDD, align with IAB-MT requirements and use 24dBm as the power limitation

- For TDD,

- Option 1: reuse 24dBm the same as IAB-MT

- Option 2: UE power class based power limitation, e.g. 26dBm or 29dBm

RAN4 will further discuss the antenna gain assumption for repeater and associated co-existence impact

- 2.2. Power accuracy requirement for both DL and UL

Power accuracy requirement is mainly based on output power detection performance within BS type 1-C. which is mainly used to detect and maintain the output power to certain setting power or rated output power, therefore we don’t see the much difference between DL and UL. It’s also not necessary to consider power accuracy requirement of repeater UL similar as the UE power control tolerance since UE power control tolerance e.g. power Absolute power tolerance 9dB for FR1 also includes the downlink RSRP estimation errors for uplink power control, however for repeater UL, power control is not depending on the DL RSRP measurement.

Proposal 3: for power accuracy requirements for repeater UL, propose to be same as power accuracy as DL.

WF:

- Define output power accuracy instead of gain accuracy for FR1 repeater DL and UL

- Define the same output power accuracy as BS type 1-C for DL.

- FFS for UL power accuracy until UL output power definition is finished. The UL power accuracy requirement definition considers the following options,

  - Use the same power accuracy as DL

  - Other options are not precluded.

- 2.3. ALC/AGC

Based on our understandings, the concern raised for appropriate ALC/ACG implementation should be taken into account in case of just clipping of output signal instead of retunning Tx gain based on received RSSI within Rx chain Therefore at least one more requirement like EVM requirement should be tested to verify the appropriate implementation of ALC/AGC.

Proposal 4: to verify the ALC/AGC functionality of repeater, both output power and EVM requirement should be tested;

WF:

- Test ALC using the same approach with E-UTRA repeater, i.e. using an input signal at 10dB compared to the maximum power level.

- FFS which requirements need to be checked with when the power of all signals is increased by 10dB:

    - Option 1: Output power only

    - Option 2; (in addition to output power) some or all of ACLR, EVM, OBUE within 1st MHz from the passband, spurious emission

- No other requirement is defined related to ALC/AGC.

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

- Proposal 1: for repeater class with power limitation, propose to define with fixed values;

- Proposal 2: for TDD band, reuse 24dBm as power limitation;

- Proposal 3: for power accuracy requirements for repeater UL, propose to be same as power accuracy as DL.

- Proposal 4: to verify the ALC/AGC functionality of repeater, both output power and EVM requirement should be tested;

References

- R4-2115720,WF on conducted output power and emission requirements,CATT, approved.

- R4-2115721,WF on other conducted requirements,approved.

- R4-21xxxx, Discussions on repeater conducted emission requirements, ZTE corporation